# 3GPP TSG-RAN WG4 Meeting #112	R4-2411538

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Sony

Title:	views on requirements of the low-power wake-up receiver

Agenda item:	8.22.2

Document for:	Approval

- Background

The RF requirements of low-power wake-up signal (LP-WUS) and low-power wake-up receiver (LP-WUR) for NR were discussed in RAN4 #111 [1], and a WF was agreed [2]. According to the WF, the link level simulation assumption for ACS/ASCS evaluation has been agreed, but some prominent issues for REFSENS, which includes reference architecture, NF, and IM, still need to be further discussed.

In this contribution, we further discuss the REFSENS and other RF requirements of LP-WUR and provide our initial link-level simulation results.

- REFSENS for LP-WUR

## Reference architecture for LP-WUR

Considering that the threshold SNR for defining the REFSENS requirement will be based on the input from RAN1 [2], RAN4 can first focus on choosing a reference architecture for the LP-WUR receiver to derive the REFSENS requirements. Based on the technical report 38.869 [3], different LP-WUS receiver designs perform significantly differently in terms of noise figure (NF), power consumption, frequency selectivity, and coverage. Therefore, without a common assumption on the reference architecture, it would be difficult for RAN4 to further refine the REFSENS requirements even with an SNR input from RAN1.

Observation 1: A reference architecture for the LP-WUR receiver is needed to define the REFSENS requirement.

Based on the discussion during the study phase [3], Although the envelope detector (ED)-based receiver suffers higher NF than OFDM-based receivers, they also provide a significant power-saving gain. Considering the primary purpose of supporting LP-WUS is to save the power over the main receiver (MR), it may be more feasible to adopt an ED-based receiver to define the REFSENS requirement for LP-WUS receivers unless it would be mandatory for UE to decode any OFDM symbols in this LP-WUS.

Proposal 1: 	It is proposed that an ED-based receiver be adopted to define the REFSENS requirement to ensure sufficient power-saving gain unless it is mandatory for UE to decode any OFDM symbols in this LP-WUS.

Among ED-based receivers, it is challenging for the RF-ED receivers to support operation on multiple frequency bands or carriers since the RF-ED receiver does not have any frequency selectivity, and it will require integration of extremely high Q filters that match the BW of LP-WUS to suppression the interference on neighboring spectrum. Practically, since it has been agreed that the WUS signal will not be limited to a specific band(s) [2] and can be configured at different RB allocations within each frequency band, it is questionable if such an LP-WUR architecture would be feasible. Therefore, it is proposed that the RF-ED receiver be down-selected.

Proposal 2	It is proposed that the RF-ED receiver be down-selected due to its poor frequency selectivity for being used to derive the REFSENS requirements.

On the other hand, it is more practical to integrate high Q filters in intermediate frequency (IF) or baseband (BB). Therefore, heterodyne and/or homodyne ED receivers with IF and BB filtering will be more practical.

In addition to the power consumption and frequency selectivity, the receiver's expected coverage is another essential aspect when determining a feasible reference architecture. Although the coverage performance is tightly connected to the REFSENS level itself, initial estimations can still be made. As analyzed in [4], it is possible for an ED-based LP-WUS receiver with heterodyne or homodyne architecture to reach the coverage target under a 10-16 dB NF, which is the same as Msg. 3. Therefore, it can be concluded that the heterodyne and homodyne architecture can be used as a reference architecture for LP-WUR.

Observation 2	The heterodyne and homodyne ED-based receivers provide significant power-saving gain compared to OFDM-based receivers but are still capable of meeting the coverage target of LP-WUS with NF assumption in the range of 10-16 dB.

Proposal 3: 	As a starting point, the heterodyne and homodyne ED based receivers can be used as the reference architecture to derive the REFSENS requirement.

Another issue that has been brought up in the last couple of meetings is the consideration of antenna sharing [5], where a reference architecture is shown in Fig. 1.

Fig. 1. The reference architecture of heterodyne LP-WUR with antenna sharing with the MR.

In our view, antenna sharing would be necessary if the RRM measurement needs to be offloaded to the LP-WUS. Otherwise, the measured results would not be valid due to the possible different channels observed by different antennas. In such a case, an antenna switching needs to be implemented, where additional insertional loss may need to be considered.

Proposal 4:	It is proposed that RAN4 to include additional switch in the RF reference architecture to support antenna sharing between the MR and LP-WUR to support RRM measurement offload.

Practically, the antenna switching is already taken into account in the MR REFSENS, therefore such an antenna sharing scheme should not affect the performance MR significantly, and thus no impact on the MR performance should be considered.

Proposal 5: 	No impact on MR requirements should be considered.

## REFSENS performance metric

## REFSENS performance metric

In RAN4#110bis, the following aggreements for the performance metric of REFSENS have been made based on the agreed WF [2]:

Although the miss detection rate (MDR) has been agreed as the sole metric for LP-WUS Rx RF requirement, the MDR value is still for FFS. Besides the 1% MDR that has been used as a simulation assumption for determining SNR during the study phase, a higher percentage MDR, e.g., 5% or 10%, was also proposed in the discussion.

The reason for choosing a higher percentage instead of a lower value (e.g., 1%) is to facilitate testing, in our understanding. To obtain a 1% test results, it is intuitive to understand that at least 100 LP-WUS sequences are required to be measured in the conformance test. Moreover, based on the rule of thumb, the number of tests is preferred to be even ten times the least required number of sequences, e.g., 1000 LP-WUS sequences may be needed to obtain the performance of 1%MDR. Therefore, a high percentage, e.g., 10%, might be more feasible from the testability of view.

Observation 3	Adopting a relatively small percentage value on the MDR may lead to an excessive test time.

However, one concern regarding adopting a larger percentage in the core requirement, which needs to be discussed further, is how to ensure that the REFSENS requirement defined in RAN4 fulfills the coverage target of LP-WUS. It is our understanding that the LP-WUS should reach the same coverage of Msg. 3 with 1% MDR. Therefore, one possibility is still to define the core requirement according to 1% MDR and scale the corresponding REFSENS level to 10 % MDR (e.g., scaling according to different SNR levels of 1% and 10% MDR) in the conformance test.

Observation 4	The target coverage of LP-WUS is to meet Msg. 3 coverage with 1% MDR.

Proposal 6	Use 1% MRD as REFSENS metric for LP-WUR as a starting point to define the core requirement and further study if a higher percentage can be used while fulfilling the coverage target of LP-WUS.

Proposal 7	RAN4 may consider adopting a higher percentage MDR value, e.g., 5 %or 10 %, in the conformance test by scaling the REFSENS level accordingly.

## One or two sets of requirements

In addition, the number of requirements has also been discussed, and one proposal is to define sets of requirements to accommodate receivers for both OOK type and OFDM type LP-WUS. In general, both types of WUS signal and the corresponding receivers need to support sufficient coverage of LP-WUS (as PUSCH of Msg 3). In addition, as we already discussed in section 2.1, the OFDM-based LP-WUR is unlikely to perform worse than ED-based LPWUR due to improved NF. Having in mind that RAN4 only defines minimum requirements, it should be sufficient to define only one set of requirements covering all types of receivers.

Observation 5: 	All types of WUS signals and the corresponding receivers shall meet the same coverage target, and thus, one set of minimum requirements is required to ensure this coverage is sufficient.

Proposal 8	RAN4 should aim to define one set of minimum requirements covering all types of LP-WUS receivers.

- ACS and ASCS Requirement

## Requirement for ACS and ACSC

From the network co-existence aspect, the LP-WUS is expected to co-exist with other signals and suffer the same level of interference as a normal NR receiver. Thus, setting the same ACS level for the LP-WUS receiver as MR would be reasonable. Meanwhile, ACSC's requirement is to tolerate the in-band interference that the network can control, so it could potentially be more relaxed than ACS. Therefore, RAN4 should further investigate if ACSC requirements should be set like ACS requirements.

Observation 6	Since the LP-WUS needs to co-exist with other NR signals, it is reasonable to set the same ACS level for the LP-WUS receiver as the main receiver.

Proposal 9	Define the ACS requirement for LP-WUS as 33 dB and further investigate if the ACSC should be set to the same value as the ACS requirement.

Proposal 10	RAN4 shall derive the number of guard RB based on some practical filter assumption once the ACS/ASCS requirement is agreed.

## Initial link-level simulation results

In this section, we present initial simulation results for ASCS analysis with OOK-4 waveform. Our simulations are based on the simulation assumption for link level simulation (LLS) agreed during RAN4#111, shown below:

| Attributes | Assumptions |
| --- | --- |
| Case name (waveform) | OOK-4 waveform |
| Center frequency | 2.6GHz |
| Chip rate | M=4 |
| WUS duration | 126 bits ~ 1mSec |
| Coding | 1/2 rate Manchester coding |
| Time error | 0 |
| Residual Frequency error | 0 |
| SCS | 30kHz |
| UE Channel BW | 20MHz (51 RB)-case 1 |
| WUS RB | - Fixed 11RB ~ 3.96MHz |
| Position within channel | - Center of the channel |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth |
| Filter | - 5th Order Butterworth matching fixed 3.96MHz RF bandwidth |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same average PSD with WUS signal |
| Sampling rate | 7.68MHz |
| ADC bit width | Ideal ADC |
| Phase noise | Not modelled |
| Non-linearities | Not modelled |
| Power boosting | No power boosting |
| Channel Model | TDL-C 300 and AWGN |
| Performance metric | 1% MDR with false alarm rate below 1% |

As indicated in the table, the LP-WUS waveform is generated based on OOK-4 principles and for M = 4. The LP-WUS includes two parts, a preamble used for synchronization and cell ID identification and a data for UE group wake-up/identification. Both sequences are chosen from Kasami sequences of length 63. PN sequences (m-sequences) are further used as overlaid sequences and applies to the ON duration of each Manchester coded OOK symbol. We have used ideal ADC but it is expected that the ADC performance would not degraded our MDR performance significantly The order of lowpass Butterworth is set to 5 and we assume no frequency and timing error.

We study the results for AWGN and TDL-C 300 channel for two guard band settings of 0 RB and 1 RB on each side of LP-WUS bandwidth. The achieved false-alarm rate for the studied scenarios is below 0.1%.

Figure 1 - Miss detection rate (MDR) for AWGN (left) and TDLC (right) channel models.

Based on this initial simulation results, it can be observed that the number of guard RB does not affect the MDR performance significantly. This may be caused by the fact we use a correlation detector rather than any CRC based detector, where the processing gain of the receiver would overcome the error floor in terms of bit error rate due to the interference from adjunct NR signals.

Observation 7	The number of guard RB may not affect the receiver performance significantly depends on the processing in the receiver.

- OBB/IBB and other receiver requirements

Similar logic to ACS, it is expected that the blocking signal level in IBB and OBB should be the same as the MR. The exact location of the blocking signal can be further discussed. Assuming the LP-WUS receiver is capable of meeting the MR ACS requirement with configured guard RBs, it is proposed that the configured guard RB should also be configured in the OBB and IBB test., e.g., the offset of blocking signal for LP-WUS OBB/IBB test is shifted further away from the LP-wanted signal with the number of guard RB than in the MR case.

Proposal 11	Define the blocking signal level the same as the MR for LP-WUR, and further study if the offset of the blocking signal to the wanted signal should be shifted with guard RB.

- Side condition of LP-WUS

The side condition of the LP-WUS requirement has also been brought up in RAN4#111, and whether RAN4 should consider both idle and connected mode conditions in the LP-WUS requirement was discussed. For most of the devices, it is our understanding that the LP-WUS is targeted mainly for idle mode. Meanwhile, the connected mode of LP-WUS is designed to support XR devices due to its unique traffic model, and it is unclear how this would affect the RF requirements. Therefore, it is suggested that RAN4 focus on the idle mode and FFS if the connected mode needs to be addressed separately later on and if any impact on the RF requirement of LP-WUR is identified.

Observation 8  	Connected mode LP-WUS is mainly for XR devices.

Proposal 12  	RAN4 can focus on the idle mode first and FFS if the connected mode needs to be addressed separately later on if any impact on the RF requirement of LP-WUR would be identified.

- Testability of LP-WUS

From the testability aspect, different from the legacy REFSENS or ACS test for the MR, there will be no ACK/NACK for the LP-WUS receiver, and therefore, how the test equipment (TE) shall be able to count the missed detection rate is unclear. One possible solution is to implement a dedicated test mode to allow the UE to provide feedback on the received wake-up signal. As an alternative, 3GPP may also investigate whether succeeding transmissions (e.g., PRACH) on the main radio can be detected by the TE once the UE has been woken up. In either case, RAN4 may consider leaving the testability discussion to RAN5.

Observation 9	As there is no feedback on the LP-WUS receiver upon the wake-up signal, it may need a test mode so that the TE can measure the missed detection rate.

Proposal 13	As an alternative to the test mode, 3GPP may also investigate whether succeeding transmissions from the main radio can be detected by the TE to measure the missed detection rate once the UE has been woken up.

Proposal 14	RAN4 may consider leaving the testability discussion to RAN5.

# Conclusion

In this contribution, we have discussed LP-WUS and LP-WUR for NR. The following observations and proposals have been made:

Observation 1    A reference architecture for the LP-WUR receiver is needed to define the REFSENS requirement.

Observation 2	The heterodyne and homodyne ED-based receivers provide significant power-saving gain compared to OFDM-based receivers but are still capable of meeting the coverage target of LP-WUS with NF assumption in the range of 10-16 dB.

Observation 3	Adopting a relatively small percentage value on the MDR may lead to an excessive test time.

Observation 4	The target coverage of LP-WUS is to meet Msg. 3 coverage with 1% MDR.

Observation 5: 	All types of WUS signals and the corresponding receivers shall meet the same coverage target, and thus, one set of minimum requirements is required to ensure this coverage is sufficient.

Observation 6	Since the LP-WUS needs to co-exist with other NR signals, it is reasonable to set the same ACS level for the LP-WUS receiver as the main receiver.

Observation 7	The number of guard RB may not affect the receiver performance significantly depends on the processing in the receiver.

Observation 8  	Connected mode LP-WUS is mainly for XR devices.

Observation 9	As there is no feedback on the LP-WUS receiver upon the wake-up signal, it may need a test mode so that the TE can measure the missed detection rate.

Proposal 1  	It is proposed that an ED-based receiver be adopted to define the REFSENS requirement to ensure sufficient power-saving gain unless it is mandatory for UE to decode any OFDM symbols in this LP-WUS.

Proposal 2	It is proposed that the RF-ED receiver be down-selected due to its poor frequency selectivity for being used to derive the REFSENS requirements.

Proposal 3 	As a starting point, the heterodyne and homodyne ED based receivers can be used as the reference architecture to derive the REFSENS requirement.

Proposal 4	It is proposed that RAN4 to include additional switch in the RF reference architecture to support antenna sharing between the MR and LP-WUR to support RRM measurement offload.

Proposal 5 	No impact on MR requirements should be considered.

Proposal 6	Use 1% MRD as REFSENS metric for LP-WUR as a starting point to define the core requirement and further study if a higher percentage can be used while fulfilling the coverage target of LP-WUS.

Proposal 7	RAN4 may consider adopting a higher percentage MDR value, e.g., 5 %or 10 %, in the conformance test by scaling the REFSENS level accordingly.

Proposal 8	RAN4 should aim to define one set of minimum requirements covering all types of LP-WUS receivers.

Proposal 9	Define the ACS requirement for LP-WUS as 33 dB and further investigate if the ACSC should be set to the same value as the ACS requirement.

Proposal 10	RAN4 shall derive the number of guard RB based on some practical filter assumption once the ACS/ASCS requirement is agreed.

Proposal 11	Define the blocking signal level the same as the MR for LP-WUR, and further study if the offset of the blocking signal to the wanted signal should be shifted with guard RB.

Proposal 12  	RAN4 can focus on the idle mode first and FFS if the connected mode needs to be addressed separately later on if any impact on the RF requirement of LP-WUR would be identified.

Proposal 13	As an alternative to the test mode, 3GPP may also investigate whether succeeding transmissions from the main radio can be detected by the TE to measure the missed detection rate once the UE has been woken up.

Proposal 14	RAN4 may consider leaving the testability discussion to RAN5.

- References

R4-2408947		Topic summary for [111][136] NR_LPWUS_UERF	Moderator(vivo)

R4-2410569		WF on NR LP-WUS UE RF	Vivo

TR 38.830		"Study on NR coverage enhancements, (Release 17)"

R4-2407412		On receiver sensitivity of the low-power wake-up receiver	Sony

R4-2407894		Discussion on REFSENS for LP-WUR	Samsung