3GPP TSG-RAN WG4 Meeting #112		                R4-2411539

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Sony

Title:	UE RF requirement for NTN Redcap and eRedcap

Agenda item:	8.25.2.1

Document for:	Approval

# Background

In RAN4#111, the RF requirement for NTN Redcap in Rel-19 has been further discussed [1]. Multiple agreements have been made that create a basis of NTN Redcap RF requirements [2], with the major left-over issues including HD-FDD REFSENS for 2 Rx, frequency error, and maximum output power.

In this contribution, we provide our view on the remaining missing RF requirement for NTN Redcap and eRedcap in Rel-19.

# REFSENS for HD-FDD

In RAN4#111, it has been agreed:

- [0.5] dB tightening for n254 and n256, FFS for n255. n256 to be confirmed in RAN4#112.

The 0.5 dB tightening on n254 and n256 followed the same principle as in the TN case, which was originally proposed in [3], and the principle is summarized in the table below:

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

In our view, it is essential that the same approach should be taken as in the TN case so that the RF front end of TN Redcap and NTN Redcap can be largely reused to facilitate the cost and time-to-market of Redcap/eRedcap NTN system.

Observation 1: Adopting the same principle in defining the TN Redcap and NTN Redcap REFSENS can facilitate the time-to-market of the Redcap NTN system and reduce its cost.

From the coverage perspective, it is our understanding that the uplink coverage is the bottleneck for both legacy TN Redcap and legacy NTN networks. Therefore, further stringent REFSENS requirements may not significantly benefit network coverage.

Observation 2: Making the REFSENS for HD-FDD NTN Redcap more stringent may not significantly benefit network coverage as the uplink is usually the bottleneck.

Therefore, it is proposed that the same approach can also be used to derive the REFSENS for NTN Redcap HD-FDD, where RAN4 shall confirm 0.5 dB tightening for n254 and n256, and adopt 0 dB tightening for n255.

Proposal 1: RAN4 confirms the 0.5 dB tightening for n254 and n256, and 0 dB tightening for n255 for the REFSENS of HD-FDD Redcap NTN, e.g., -100 dBm for 5MHz BW for n256, n255 and n254.

For reduced baseband bandwidth eRedcap, the 5MHz REFSENS requirement applies to 5, 10, 15, and 20 MHz RF bandwidth as the PDSCH is limited to 5MHz regardless of the RF bandwidth for 15kHz SCS. For larger SCS, e.g., 30 kHz, where 5MHz channel RF bandwidth does not apply, the REFSENS of 10 MHz is applied with 3 dB relaxation. In our view, those principles can be directly reused for NTN eRedcap UE, and the detailed description is captured in 7.3I.3 of 38.101-1.

Proposal 2: Reuse the same way as in Rel-18 eRedcap REFSENS (as in 7.3I.3 of 38.101-1) to define the NTN eRedcap REFSENS.

# GNSS operation

In RAN4#111, it is identified that the GNSS operate in the frequency range 1559~1610MHz can’t operate simultaneously with frequency band n255 due to the NTN DL interference and n254 due to the UL interference [4], and it was proposed that relaxed frequency error requirement shall be applied to accommodate this issue.

Observation 3: It is challenging for GNSS and NR NTN to operate simultaneously in some frequency bands due to the extremely closed frequency allocation.

On the other hand, it is explicitly stated in the WI that the simultaneous GNSS and NR-NTN operation are supported in RedCap/eRedCap UE [5]. Such a restriction was not required explicitly for normal NR NTN UEs, and it was explicitly not required at all for IoT NTN UEs. Considering the complexity of Redcap UE is somewhere between normal NR UE and IoT UEs, it is a bit unclear what the implication of having this requirement for Redcap NTN devices.

Considering the Redcap NTN is a lower device than normal NR NTN, it is also a question of whether the Redcap NTN can fully support the Doppler and TA pre-compensation in the same way as normal NR UEs. Therefore, RAN4 shall investigate how to handle the scenario in which the GNSS can’t operate simultaneously with NR communication.

Proposal 3: RAN4 shall further study the impact if GNSS can’t operate simultaneously with NR NTN.

# Maximum output power

Another open issue from RAN4#111 is whether to consider an increased PC3 power level for NRN Redcap UE for HD-FDD operation. In our view, the output power for each power class is carefully defined and justified. The HD-FDD front end is intended to provide a more efficient frontend solution than FD-FDD with reduced insertion loss but is not designed to support higher output power. Therefore, there is no reason to change the maximum output power of the power class, and a new power class would be needed if any change of maximum output power would be needed.

Proposal 4: Keep the output power level of PC3 for Redcap NTN.

# Conclusion

In this paper, we presented our view on the RF requirement for NTN Redcap and eRedcap, where the following observations and proposals are made:

Observation 1: Adopting the same principle in defining the TN Redcap and NTN Redcap REFSENS can facilitate the time-to-market of the Redcap NTN system and reduce its cost.

Observation 2: Making the REFSENS for HD-FDD NTN Redcap more stringent may not significantly benefit network coverage as the uplink is usually the bottleneck.

Observation 3: It is challenging for GNSS and NR NTN to operate simultaneously in some frequency bands due to the extremely closed frequency allocation.

Proposal 1: RAN4 confirms the 0.5 dB tightening for n254 and n256, and 0 dB tightening for n255 for the REFSENS of HD-FDD Redcap NTN, e.g., -100 dBm for 5MHz BW for n256, n255 and n254.

Proposal 2: Reuse the same way as in Rel-18 eRedcap REFSENS (as in 7.3I.3 of 38.101-1) to define the NTN eRedcap REFSENS.

Proposal 3: RAN4 shall further study the impact if GNSS can’t operate simultaneously with NR NTN.

Proposal 4: Keep the output power level of PC3 for Redcap NTN.

# References

- R4-2408948  Topic summary for [111][137] NR_NTN_Ph3_UERF	Moderator(Qualcomm)

- R4-2410574   WF on Rel-19 phase 3 NR-NTN UE RF requirements	Qualcomm

- R4-2200442   RedCap UE HD-FDD REFSENS requirements   Apple

- R4-2409336   Discussion on the other potential impacts for Rel-19 NTN RedCap UE    Huawei, HiSilicon

- RP-234078  New WID: Non-Terrestrial Networks (NTN) for NR Phase 3