3GPP TSG-RAN WG4 Meeting #112				                              R4-2411540

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Sony

Title:	emission limit for HPUE in IoT NTN

Agenda item:	8.8.2.2

Document for:	Discussion

# Background

In RAN#103, a new WID: enhanced requirements and test methodology for NR and IoT NTN has been approved [1]. From core requirement, the target of this WID is to support HPUE for both NR and IoT NTN and also introduce the 3MHz channel BW into the NTN system.

In this contribution, we express our views regarding the emission limit for HPUE in IoT NTN system.

# Discussion

The objective of HPUE in the WI is defined as below:

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes

  - Evaluate the impact on the uplink performance of TN (Terrestrial network)

    - Example bands include bands n256 and 256, n255 and 255

    - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS

    - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-handheld UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

Unlike NR UEs, no HPUE is defined for eMTC and NB-IoT. Therefore, there is a lack of HPUE TN requirements as a reference for IoT NTN, and the corresponding MPR and AMPR values of the HPUE need to be re-evaluated based on reasonable implementations of IoT devices.

Observation 1: Due to the lack of TN requirement as a reference, the MPR and AMPR values of the HPUE need to be re-evaluated based on reasonable implementations of IoT devices.

Another critical aspect of IoT NTN, which has already been encountered even for PC3, is to meet several ETSI emission limits. A couple of problematic scenarios have been illustrated in Fig.1 and Fig. 2 for b255 and b254 for PC3, respectively. It can be expected that those scenarios will be even more challenging for HPUEs. For the out-of-band emission limit, a guard band on the edge of the band might be helpful. For example, around 200 kHz guard band is needed for b255 to meet the OOB emission limit from ETSI EN 301 681, as shown in Fig. 1. However, the in-band emission limit would be tricky to resolve, as shown for the b254 v.s. ETSI EN 301 441 in Fig. 2. Based on the replied LS from ETSI [2][3], some actions are ongoing in ETSI to create a new harmonized standard for IoT NTN operation. It is suggested 3GPP wait for the ETSI progress before we capture any ETSI limits in 3GPP HPUE for IoT NTN. For other regulatory requirements applicable to IoT NTN, 3GPP should also scrutinize them to determine if the existing solution (guard band or AMPR) would be sufficient for HPUE.

Fig. 1. The comparison of ETSI OOB emission limit in ETSI EN 301 681 vs. 3GPP SEM for NB-IoT in b255.

Fig. 2. The comparison of ETSI IB and OOB emission limit in ETSI EN 301 441 vs. 3GPP SEM for NB-IoT in b254.

Proposal 1: RAN4 shall carefully study the corresponding emission limit from regulators and specify the method to ensure the 3GPP HPUE IoT NTN does not violate them.

Proposal 2: RAN4 may wait for ETSI to specify the new harmonized standard for HPUE IoT NTN before capturing any emission limit from ETSI.

# Conclusion

In this contribution, we make the following observations and conclusions:

Observation 1: Due to the lack of TN requirement as a reference, the MPR and AMPR values of the HPUE need to be re-evaluated based on reasonable implementations of IoT devices.

Proposal 1: RAN4 shall carefully study the corresponding emission limit from regulators and specify the method to ensure the 3GPP HPUE IoT NTN does not violate them.

Proposal 2: RAN4 may wait for ETSI to specify the new harmonized standard for HPUE IoT NTN before capturing any emission limit from ETSI in HPUE.

# References

- RP-240857 New WID: Enhanced requirements and test methodology for NR and IoT NTN

- R4-2317932	LS reply on 3GPP NTN requirements and potential impact on specification work in ETSI TC SES	ETSI TC SES

- R4-2411017	Blocking requirements for extended L band 	ETSI TC ERM