3GPP TSG-RAN WG4 Meeting #112				                            R4-2411541

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Sony

Title:	Supporting 3MHz BW for Redcap/eRedcap NTN

Agenda item:	8.8.3.2

Document for:	Discussion

# Background

In RAN#103, a new WID: enhanced requirements and test methodology for NR and IoT NTN has been approved [1]. From core requirement-wise, the target of this WID is to support HPUE for both NR and IoT NTN, and also introduce the 3MHz channel BW into the NTN system.

In this contribution, we discuss the possibility of including the Redcap/eRedcap to be supported by 3MHz channel BW in the NTN system.

# Discussion

3MHz channel BW has been specified for NR TN operation since Rel-18 to support fragmented spectrums, especially for some re-farmed bands. In Rel-19, it is proposed to add this 3MHz channel BW for NR NTN operation as well to facilitate a more efficient usage of spectrum for NTN bands, with the objective as below:

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signaling to support the above objectives

It is unclear of Redcap NTN is in the scope of the WI as Redcap NTN was not specified when the WI was approved. However, after RAN4#111 and RAN4#111bis, the majority of RF requirements for Redcap NTN have been agreed with only HD-FDD REFSENS left and some ambiguity regarding GNSS operation. Therefore, it should be feasible to include the Redcap NTN into the scope of the work.

Observation 1: The major part of the core requirements for Redcap NTN UE has been settled.

From a technical perspective, we understand that the major changes on the UE implementation required for 3MHz channel BW is on the baseband, where it should be able to decode the punctured SSB. However, very little change is needed in terms of RF implementation.

Observation 2: From the RF perspective, it is feasible for Redcap NTN UE to support a smaller BW.

In addition, it would make sense for Redcap devices to operate in a smaller bandwidth so their deployment can be more flexible to improve spectral efficiency. Also, power consumption can be reduced with a narrow bandwidth as well. Therefore, it is proposed that the Redcap/eRedcap NTN in Rel-19 be considered to support the 3Mhz channel BW.

Observation 3: Supporting smaller BW can facilitate power consumption and deployment flexibility for Redcap NTN UE.

Proposal 1: It is proposed that the 3Mhz channel BW for Redcap/eRedcap NTN in Rel-19 be specified as well.

# Conclusion

In this contribution, we make the following observations and conclusions:

Observation 1: The major part of the core requirements for Redcap NTN UE has been settled.

Observation 2: From the RF perspective, it is feasible for Redcap NTN UE to support a smaller BW.

Observation 3: Supporting smaller BW can facilitate power consumption and deployment flexibility for Redcap NTN UE.

Proposal 1: It is proposed that the 3Mhz channel BW for Redcap/eRedcap NTN in Rel-19 be specified as well.

# References

- RP-240857 New WID: Enhanced requirements and test methodology for NR and IoT NTN