3GPP TSG-RAN WG4 Meeting #112	R4-2411544

# Maastricht, Netherlands, 19 – 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0040 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (IoT_NTN_FDD_LS_band) CR to 36.102 for IoT-NTN UE RF 1-tone A-MPR requirements (Rel-18) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_LS_band |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The 1-Tone allocation A-MPR values for band 254 is not analyzed for NS_04N/NS_05N. To provide the evaluated result is necessary for Cat-M1 and Cat-NB1/NB2 UE. Based on evalaution, no NS_04N/NS_05N A-MPR for Cat-M1. But NS_04N/NS_05N A-MPR for Cat-NB1/NB2 is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclause 6.2B.3, regarding Cat-NB1/NB2 UE, the missing 1-Tone allocation A-MPR values for band 254 for NS_04N/NS_05N is further updated based on simulation results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The required NS_04N/NS_05N A-MPR for Cat-NB1/NB2 1-Tone operation for IoT-NTN band 254 (L+S band) is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TR 36.764... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====================Start of the change 1==========================

6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2B.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2-1.

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5B.4.4.5 | 254 | Table 6.2B.3-4 |  |
| NS_05N | 6.5B.4.4.6 | 254 | Table 6.2B.3-5 |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |

Table 6.2B.3-4: 1-Tone A-MPR for "NS_04N"

| Modulation | π/4 QPSK |  |  |
| --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/3/4/5/6/7/8/9 |
| A-MPR | [≤ 0.4 dB] | [0 dB] | [0 dB] |

Table 6.2B.3-5: 1-Tone A-MPR for "NS_05N"

| Modulation | π/4 QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/9 | 3/4/5/6/7/8 |
| A-MPR | [≤ 1.4 dB] | [≤ 0.7 dB] | [0 dB] | [0 dB] |

====================End of the change 1==========================