3GPP TSG-RAN WG4 Meeting #112	R4-2411548

# Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	7.18.2

Source:	MediaTek Inc.

Title:	Discussion on an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) UE RF requirements

Document for:	Approval

- Introduction

In RAN#104 plenary meeting, the spectrum WID on introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) was approved in [1]. One of the main objectives is to specify UE RF core requirements for fulfilling the urgent deployment demand.

In the contribution, the consideration for an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) UE RF requirements are provided for discussion.

- Discussion

2.1		Spectrum WI scope for an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)

First, regarding UE RF requirements, the objective of the WI RF core part is shown below for reference.

- Specify a new E-UTRA NTN FDD band for NB-IoT/eMTC NTN operation, with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario; - Support UE Categories NB1, NB2, M1; - Support UE Power Class 3 (+23dBm) and Power Class 5 (+20dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. The specification work of this WI shall leverage the studies and requirements for NR/LTE NTN n256/B256, n255/B255, n254/B254, where applicable.

2.2		UE RF system parameters

2.2.1	Band and band numbering

In TS 36102 [2], IoT NTN band 252 is not used yet.

Proposal 1: Regarding band number, specify IoT-NTN band number [252] for an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) if feasible.

.

Therefore, as for an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), the following row may be added to the operating table in the TS 36.102 [2] after further discussion.

TS 36.102 Table 5.2-1 E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 2532 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525 MHz | FDD |
| [252] | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |

2.2.2	Channel bandwidth

The current specification specifies channel bandwidth for category M1 and category NB1/NB2 in a band agnostic way, therefore, the addition of an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) does not introduce any change on the channel bandwidth sub-clauses.

Proposal 2: Regarding CBW, the addition of an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) does not have any impact on the CBW sub-clauses for category M1 or category NB1/NB2.

2.2.3	Tx-Rx frequency separation

Proposal 3: As for an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), to set the default Tx-Rx frequency separation as fixed value first. Further study variable Tx-Rx frequency separation.

2.3		UE RF TX/RX requirements

For the eMTC/NB-IoT NTN UE RF requirements, we distinguish band agnostic requirements from TS 36.102 [2]. In the table below, we provide comments on reusing these requirements if possible.

Table 2.3-1

| Requirement name | Band-agnostic? | Reuse 36.102 requirement? | Comments |
| --- | --- | --- | --- |
| Clause 6: Transmitter Characteristics |  |  |  |
| 6.2A.1/6.2B.1: UE maximum output power | Yes | See comments | Discussion on Proposal 5. |
| 6.2A.1/6.2B.1: UE maximum output power tolerance | See comments | See comments | Discussion on Proposal 5. |
| 6.2A.2/6.2B.2: UE maximum output power reduction | Yes | Yes | Different MPR tables for category M1 and NB1/NB2 separately. |
| 6.2A.3/6.2B.3: UE additional maximum output power reduction | No | See comments | Need further discussion based on UE-UE coexistence analysis |
| 6.3: Output power dynamics | Yes | Yes |  |
| 6.4: Transmit signal quality | Yes | Yes |  |
| 6.5A.3.2/6.5B.3.2: Spectrum emission mask | Yes | Yes |  |
| 6.5A.3.4/6.5B.3.4: Adjacent Channel Leakage Ratio | Yes | Yes |  |
| 6.5A.4.3/6.5B.4.3: Spurious emissions for UE co-existence | No |  |  |
| 6.5A.4.4/6.5B.4.4: Additional spurious emissions | No |  |  |
| 6.6: Transmit intermodulation for category NB1 and NB2 | Yes | Yes |  |
| 7: Receiver characteristics |  |  |  |
| 7.2: Diversity characteristics | Yes | Yes |  |
| 7.3: Reference sensitivity power level | No | See comments | Because the new band’s 2180-2200 MHz DL frequency range is overlapping with band 256 DL, to consider putting band 256 REFSENS into square bracket as starting point for the new band. |
| 7.4: Maximum input level | Yes | Yes |  |
| 7.5: Adjacent Channel Selectivity | Yes | Yes |  |
| 7.6A.2/7.6B.2: In-band blocking | No | Yes if there is no specific concern | Different band group may have different In-band blocking tables for Cat-M1 and NB1/NB2 separately. Currently, bands 256,255,254, and 253 apply the same IBB requirements. |
| 7.6A.3/7.6B.3: Out-of-band blocking | No | See comments | Different Out-of-band requirements for IoT NTN bands; whether to leverage requirement from band 256 needs further discussion. |
| 7.6A.4/7.6B.4: Narrow band blocking | No | Yes if there is no specific concern |  |
| 7.7: Spurious response | Yes | Yes |  |
| 7.8: Intermodulation characteristics | Yes | Yes |  |
| 7.9: RX spurious emission | No | Yes if there is no specific concern |  |

Observation 1: Regarding an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) UE RF requirements, to distinguish band-agnostic UE RF requirements from TS 36.102 [2] is provided in Table 2.3-1.

Proposal 4: To agree the proposals in the Table 2.3-1 if there is no specific concern, or at least use them as the starting point for further discussions on an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) UE TX and RX RF requirements.

Observation 2: In TS 36.102 [2], regarding IoT NTN Cat-M1 UE reference sensitivity difference between FDD and HD-FDD duplex modes, HD-FDD REFSENS is 0.8dB better than FDD REFSENS. The lower HD-FDD RF front-end insertion loss compared to FDD insertion loss contributes the HD-FDD REFSENS improvement.

Table 7.3A-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 253 | -102.7 | FDD |
| 254 | -102.2 | FDD |
| 255 | -102.7 | FDD |
| 256 | -102.2 | FDD |

Table 7.3A-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 253 | -103.5 | HD-FDD |
| 254 | -103.1 | HD-FDD |
| 255 | -103.5 | HD-FDD |
| 256 | -103 | HD-FDD |

Observation 3: The HD-FDD RF front-end (FE) insertion loss is lower compared to FD-FDD FE insertion loss, which could improve ~0.8dB the HD-FDD UE RX REFSENS and TX maximum output power simultaneously.

Observation 4: For IoT-NTN Cat-M1 UE, it can be observed that TX maximum output power increases when switching FD-FDD mode to HD-FDD mode. The Tx output power improvement could be 0.4dB.

Proposal 5: Regarding IoT-NTN UE which supports both HD-FDD and FD-FDD duplex modes for the IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), consider whether PC3 maximum output power would be feasible as:

Option 1: PC3 of [23.4] dBm.

| EUTRA band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| [252] | [23.4] | +/-2 |

Option 2: PC3 of 23 dBm with +[2.4]/-2 dB power tolerance.

| EUTRA band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| [252] | 23 | +[2.4]/-2 |

Option 3: PC3 of [23.2] dBm with +[2.2]/-2 dB power tolerance.

| EUTRA band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| [252] | [23.2] | +[2.2]/-2 |

Proposal 6: Regarding IoT-NTN UE which only supports HD-FDD duplex mode for the IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), consider whether PC3 maximum output power would be feasible as:

Option 1: PC3 of [23.4~23.8] dBm.

| EUTRA band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| [252] | [23.4~23.8] | +/-2 |

Option 2: PC3 of 23 dBm with +[2.8]/-2 dB power tolerance.

| EUTRA band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| [252] | 23 | +[2.8]/-2 |

Option 3: PC3 of [23.4] dBm with +[2.4]/-2 dB power tolerance.

| EUTRA band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| [252] | [23.4] | +[2.4]/-2 |

3 Conclusion

In this paper, our views on an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) UE RF requirements are provided.

Proposal 1: Regarding band number, specify IoT-NTN band number [252] for an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) if feasible.

Proposal 2: Regarding CBW, the addition of an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) does not have any impact on the CBW sub-clauses for category M1 or category NB1/NB2.

Proposal 3: As for an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), to set the default Tx-Rx frequency separation as fixed value first. Further study variable Tx-Rx frequency separation.

Observation 1: Regarding an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) UE RF requirements, to distinguish band-agnostic UE RF requirements from TS 36.102 [2] is provided in Table 2.3-1.

Proposal 4: To agree the proposals in the Table 2.3-1 if there is no specific concern, or at least use them as the starting point for further discussions on an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) UE TX and RX RF requirements.

Observation 2: In TS 36.102 [2], regarding IoT NTN Cat-M1 UE reference sensitivity difference between FDD and HD-FDD duplex modes, HD-FDD REFSENS is 0.8dB better than FDD REFSENS. The lower HD-FDD RF front-end insertion loss compared to FDD insertion loss contributes the HD-FDD REFSENS improvement.

Table 7.3A-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 253 | -102.7 | FDD |
| 254 | -102.2 | FDD |
| 255 | -102.7 | FDD |
| 256 | -102.2 | FDD |

Table 7.3A-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 253 | -103.5 | HD-FDD |
| 254 | -103.1 | HD-FDD |
| 255 | -103.5 | HD-FDD |
| 256 | -103 | HD-FDD |

Observation 3: The HD-FDD RF front-end (FE) insertion loss is lower compared to FD-FDD FE insertion loss, which could improve ~0.8dB the HD-FDD UE RX REFSENS and TX maximum output power simultaneously.

Observation 4: For IoT-NTN Cat-M1 UE, it can be observed that TX maximum output power increases when switching FD-FDD mode to HD-FDD mode. The Tx output power improvement could be 0.4dB.

Proposal 5: Regarding IoT-NTN UE which supports both HD-FDD and FD-FDD duplex modes for the IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), consider whether PC3 maximum output power would be feasible as:

Option 1: PC3 of [23.4] dBm.

| EUTRA band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| [252] | [23.4] | +/-2 |

Option 2: PC3 of 23 dBm with +[2.4]/-2 dB power tolerance.

| EUTRA band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| [252] | 23 | +[2.4]/-2 |

Option 3: PC3 of [23.2] dBm with +[2.2]/-2 dB power tolerance.

| EUTRA band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| [252] | [23.2] | +[2.2]/-2 |

Proposal 6: Regarding IoT-NTN UE which only supports HD-FDD duplex mode for the IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), consider whether PC3 maximum output power would be feasible as:

Option 1: PC3 of [23.4~23.8] dBm.

| EUTRA band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| [252] | [23.4~23.8] | +/-2 |

Option 2: PC3 of 23 dBm with +[2.8]/-2 dB power tolerance.

| EUTRA band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| [252] | 23 | +[2.8]/-2 |

Option 3: PC3 of [23.4] dBm with +[2.4]/-2 dB power tolerance.

| EUTRA band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| [252] | [23.4] | +[2.4]/-2 |

Reference

- RP-241662, “New WID on Introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)”, DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- TS 36.102, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access”