3GPP TSG RAN WG4 #112															     R4-2411550

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:		7.19.1

Source:				Inmarsat, Viasat

Title:	MSS L-band and Extended L-band band plan and background

Document for:		Discussion

# Background

The WI to define new bands for NR NTN, including a new NR NTN band supporting the MSS Extended L-band and new NR NTN bands supporting the combinend MSS NTN L-band and Extended L-band as a singular combined band was approved.

This paper first clarifies the band plan, applicable the regulations to derive RF requirements and the typical deployment scenario between TN and NTN to derive UE Coexistence requirements. The papers further propose to relax UE to UE Coexistence requirement according to the TN – NTN typical deployment scenario.

# Band Plan

Based on the WIDs, the following band plan (range and number) in Table 1 for NR-NTN and in Table 2 for IoT-NTN can be introduced.

Extended L-band (n253 which has been specified for NB-IoT and is yet to be specified for NR) is reserved, and S-band is proposed to use n252. The nature of the extended L-band and L-band spectrum is that the uplink is non-contiguous, with a gap of 7.5MHz between bands, and the downlink is contiguous. It has been decided that the best way of addressing this issue is to define two bands, one which utilises the n253 uplink (n250) and one which utilises the n255 uplink (n251). Both defined bands use the full combined n253 and n255 downlink spectrum.

Table 1: NR-NTN satellite bands in FR1-NTN

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high |  | Duplex mode |
| --- | --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz |  | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz |  | FDD |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz |  | FDD |
| n253 | 1668MHz – 1675MHz | 1518MHz – 1559MHz |  | FDD |
| n252 | Reserved for S-band |  |  | FDD |
| n251 | 1626.5MHz – 1660.5MHz |  | 1518MHz – 1559MHz | FDD |
| n250 | 1668MHz – 1675MHz |  | 1518MHz – 1559MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |  |

Proposal 1: Band number n251 for n255 UL and n255+n253 (specified for NB-IoT – to be specified for NR) DL and band number n250 for n253 UL and 255+n253DL.

As the newly defined band is a combination of n253 and n255, a lot of the specification will be shared between the new n251 and n250 bands and the n253 and n255 specification.

In RAN4#111 it was expressed that there may be self interference issues associated with flexible Tx-Rx separation for n255, so this will need to be addressed. In addition, coexistence between the defined band and adjacent terrestrial deployments, particularly in the spectrum below n253, as per the WID [1] relevant studies carried out for the adoption of n253 for NB-IoT will be leveraged for this new band definition.

The changes required to the RF specification for both NR and NB-IoT are addressed below:

Table 3: Required changes compare to current spec TS 38.101-5 [3] 
(Note: applicable to respective sections of TS 36.102)

| Section | Requirement | Proposal for NR-NTN (Note: applicable also for IoT-NTN in respective sections of TS36.102) |
| --- | --- | --- |
| Tx requirements |  |  |
| 6.2.1 | UE maximum output power | As proposed in the WID [1] PC3 for NR-NTN |
| 6.2.2 | UE maximum output power reduction | No changes |
| 6.2.3 | UE additional maximum output power reduction | FFS (Depends on Regulations requirement and UE Coexistence discussion) |
| 6.2.4 | Configured transmitted power | No changes |
| 6.3.1 | Minimum output power | No changes |
| 6.3.2 | Transmit OFF power | No changes |
| 6.3.3 | Transmit ON/OFF time mask | No changes |
| 6.3.4 | Power control | No changes |
| 6.4 | Transmit signal quality | No changes |
| 6.5.1 | Occupied bandwidth | No changes |
| 6.5.2 | Out of band emission (ACLR) | No changes |
| 6.5.3.1 | General Spurious emissions | No changes |
| 6.5.3.2 | Spurious emissions for UE co-existence | FFS (Depends on Regulations requirement and UE Coexistence discussion) |
| 6.5.3.3 | Additional spurious emissions | FFS (Depends on Regulations requirement and UE Coexistence discussion) |
| 6.5.4 | Transmit intermodulation | No changes |
| Rx requirements |  |  |
| 7.3 | Reference sensitivity | FFS (tied to Tx-Rx separation discussion) |
| 7.4 | Maximum input level | No changes |
| 7.5 | Adjacent channel selectivity | No changes |
| 7.6.1 | General | No changes |
| 7.6.2 | In-band blocking | No changes |
| 7.6.3 | Out-of-band blocking | FFS (due to sub-n253 blocking requirement) |
| 7.6.4 | Narrow band blocking | Borrowed from n255 |
| 7.7 | Spurious response | No changes |
| 7.8 | Intermodulation characteristics | No changes |

Proposal 2: General requirements do not need to be changed. Areas of concern are spectrum /coexistence related specification e.g. Spurious emissions, and some Rx requirements, REFSENS due to self interference and out of band blocking.

# 3 Regulatory Information

Regulatory information, in particular spectrum emissions requirements, are used to assess the performance requirements of the device, especially when mapping A-MPR values to different RB sizes/locations and channel bandwidths as seen in TR 38.741. The regulatory documentation concerning itself with n255 and n253 are ETSI EN 301 681 and FCC Part 25 clause 202. Furthermore ETSI EN 301 681 contains information about the protection of radioastronomy bands, in particular from 1660MHz to 1670MHz

### 3.1 ETSI EN 301 681

The ETSI EN 301 681 harmonized standard provides regulatory requirements for band 255 and 253. Depending on which frequency band is explicitly used, different tables within the standard are mandated. In this harmonized standard, devices are separated into two categories, those which are only capable of transmitting in n255, and those which are capable of transmitting within all or any part of n255 or n253.

Table 1 ETSI EN 301 681 Requirements vs Band

| Band | Uplink [MHz] | Downlink [MHz] | In-band SEM Table | Out-of-band SEM Table |
| --- | --- | --- | --- | --- |
| n255 | 1626.5-1660.5 | 1525-1559 | Table 4a | Table 3 |
| n253 | 1668-1675 | 1518-1525 | Table 4a | Table 3a |

In addition, the ETSI EN 301 681 standard section 4.2.6 contains a clause about the protection of radioastronomy bands operating in the range of 1660MHz – 1670MHz:

“

To protect the radio astronomy service in the 1 660 MHz to 1 660,5 MHz band and the 1 668,0 MHz to 1 670,0 MHz band from emissions produced by MESs the transmissions in these frequency bands shall be capable of being disabled in the vicinity of RA stations recorded in the ITU Master International Frequency Register.

“

And section 4.2.4.4.1 stipulates that the mechanism by which this is done is via an ‘enable/disable command’ sent to the device. If a disable command is sent to the device it shall enter carrier-off state after no more than 1 second after the disable command was received.

Proposal 3: ETSI EN 301 681 shall be used to assess the regulatory compliance of this new band, and shall be used to inform decisions on spectrum emissions related requirements such as A-MPR. In addition, ETSI EN 301 681 shall be used to dictate the functionality of a device when concerned with radioastronomy protection within the 1660MHz-1670MHz band.

### 3.2 FCC Part 25 Clause 202

The FCC part 25 specification clause 202 covers the FCC requirements for n253 and n255. Condensed into a table the requirements are as follows, for an authorised bandwidth B, with a transmitter power W:

| Frequency from edge of authorised bandwidth | Attenuation [dB] | Measurement BW [kHz] |
| --- | --- | --- |
| 0.5B - B | 25 | 4 |
| B – 2.5B | 35 | 4 |
| >=2.5B | 43-10log10(W) | 4 |

Proposal 4: FCC Part 25 clause 202 shall be used to assess the regulatory compliance of this new band, and shall be used to inform decisions on spectrum emissions related requirements such as A-MPR.

## 3.3 ECC Report 263

The ECC report 263 [2] outlines a coexistence report between a TN mobile system with its downlink deployed at 1492-1518MHz and a mobile satellite service with its downlink deployed at 1518-1559MHz (the proposed downlink band).

The report concluded that several mitigation strategies can be used to reduce the effect of the TN service as an interferer on the MSS service as follows:

- The interference due to IMT OOB emissions can be reduced by improved filtering on the IMT base station;

- The interference due to blocking can be reduced by improving the MES resilience to IMT signals in the adjacent band;

- Either adding location based frequency allocation to MSS to avoid the use of the lower couple of MHz and/or, implementing interference avoidance which would in addition allow for a better frequency utilisation of the lower part of the 1518-1559 MHz frequency band for MSS. The feasibility and impact of these techniques have not been assessed.

The study only considers the TN system as being the interferer and the MSS system as the victim. N.B. this report is yet to be verified and endorsed by ETSI.

Observation 1: ECC Report 263 considers MSS systems as victims to TN systems deployed directly adjacent to n253 downlink lower edge and has offered mitigation strategies.

# Typical TN-NTN Deployment Scenario

Deriving the parameters of a deployment scenario is necessary to derive the necessary RF requirements which  would permit a real world deployment and enable coexistence between other systems/technologies. TR 38.863 offers a complete coexistence scenario assessment between TN and NTN. The scope of the study was defined as follows:

“

NOTE:	Satellite operation is used as complementarily to Terrestrial Network operation, and in separate location when satellite operates in an adjacent channel/band of terrestrial network. For instance, NTN satellite UE is assumed to connect and transmit to the satellite when it is outside a Terrestrial Network operating in an adjacent channel/band. The satellite beam might still overlap the terrestrial network.

“

In section 6.2.1 of TR 38.863, the assumptions used for coexistence studies are expanded upon. One assumption is that the TN UE will be spatially isolated from an NTN UE. In the figure below, the isolation distance is the distance between the TN deployment (denoted by the dotted blue circle) and the NTN deployment (denoted by the solid red circle):

Figure 1 TR 38.863 Isolation Distance

For case 1 in TR 38.863, which defines the TN deployment as an urban scenario, the isolation distance is 1500m. This is considered to accurately reflect the NTN-TN selection algorithm at the border between the two technologies, assuming that the TN system will always be prioritized over the NTN system for a UE with dual-technology capability.The mechanism to ensure this priority, as of Rel-17/18 is available in both idle and connected state.

Proposal 5: The scope of the study carried out in TR 38.863 shall also be true for this band definition when concerned with UE-UE coexistence:

“Satellite operation is used as complementarily to Terrestrial Network operation, and in separate location when satellite operates in an adjacent channel/band of terrestrial network. For instance, NTN satellite UE is assumed to connect and transmit to the satellite when it is outside a Terrestrial Network operating in an adjacent channel/band. The satellite beam might still overlap the terrestrial network”.

# TN-NTN Coexistence

The following band structure is related to n253 (extended L-band) and n255 (L-band):

Figure 2 n253/n255 Band Structure

When concerned with the n255 UL, there are no adjacent or co-located bands so further study of the coexistence is unnecessary and can be ported from previous studies carried out for the definition of n255.

When concerned with the n253 UL, the only adjacent band is n70 which is 20MHz away, which is not of concern.

When concerned with n255 DL, there are several TN bands ≈10MHz away, the coexistence with which has already been carried out for the definition of n255.

When concerned with n253 DL, it lies directly adjacent to n74, n94, n75 and n50. The coexistence analysis for n253 NB-IoT which has already been carried out can be utilised for the definition of this band, but additional analysis may be required for NR deployments for n253, though it is expected in tandem with the ECC report 263 [2] that the MSS will be a victim in to these TN deployments.

Thusly, only the following coexistence scenarios need to be addressed:

- Coexistence protection for TN DL n74

- Coexistence protection for TN DL n94, n92, n75 and n50

Proposal 6: Only n253 NR coexistence needs to be considered for the definition of these bands, and only for bands n74, n84, n92, n75 and n50.

# Conclusions

The proposals and conclusions drawn from the above are as follows:

Proposal 1: Specify two new NR NTN bands, with band number n251 for n255 UL and n255+n253 (specified for NB-IoT – to be specified for NR) DL and band number n250 for n253 UL and 255+n253DL.

Proposal 2: General requirements do not need to be changed. Areas of concern are spectrum /coexistence related specification e.g. Spurious emissions, and some Rx requirements, REFSENS due to self interference and out of band blocking.

Proposal 3: ETSI EN 301 681 shall be used to assess the regulatory compliance of this new band, and shall be used to inform decisions on spectrum emissions related requirements such as A-MPR.

Proposal 4: FCC Part 25 clause 202 shall be used to assess the regulatory compliance of this new band, and shall be used to inform decisions on spectrum emissions related requirements such as A-MPR.

Observation 1: ECC Report 263 considers MSS systems as victims to TN systems deployed directly adjacent to n253 downlink lower edge and has offered mitigation strategies.

Proposal 5: The scope of the study carried out in TR 38.863 shall also be true for this band definition when concerned with UE-UE coexistence:

“Satellite operation is used as complementarily to Terrestrial Network operation, and in separate location when satellite operates in an adjacent channel/band of terrestrial network. For instance, NTN satellite UE is assumed to connect and transmit to the satellite when it is outside a Terrestrial Network operating in an adjacent channel/band. The satellite beam might still overlap the terrestrial network”.

Proposal 6: Only n253 NR coexistence with n253 NTN UE as victim needs to be considered for the definition of these bands, and only for bands n74, n84, n92, n75 and n50.

# References

[1] RP-241532 New WID  NR_NTN_LbandExtension

[2] ECC Report 263