3GPP TSG RAN WG4 Meeting #112		 R4-2411555

Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	8.6.2

Source: 	MediaTek Inc.

Title: 	Discussion on methods for reducing the number of UE Rx chains

Document for:	Approval

1. Introduction

It had been discussed in previous RAN plenary meeting that considering the spectrum hold be North America operators are usually non-contiguous blocks as stated in the reference [2][3]. In the RANP#104, a new SID[1] to study on NR FR1 DL fragmented carriers was approved. We provide our views and discussion points in this contribution.

2. Discussion

The scope of the study item Rel-19 NR FR1 DL fragmented carriers were copied below:

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

2.1 Clarification on the scope of the study

The study would focus on single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario with considering bullets mentioned in the SID scope. With the DL band frequency span limitation, all FDD and SDL bands are applicable. But for TDD bands n41, n46, n48, n77, n78, n79, n90, n96, n102 and n104, due to their DL range frequency span exceed 100MHz, are not applicable for the study scope.

Proposal 1: All FDD and SDL bands are applicable for the study. TDD bands n41, n46, n48, n77, n78, n79, n90, n96, n102 and n104 are not applicable for the study.

2.2 Methods to reduce number of UE Rx chains

There are different UE implementation for DL NCCA configuration. Some typical methods are listed below:

- Separated Rx chain architecture

- Partially-shared Rx chain architecture

- Fully-shared Rx chain architecture

It is foreseen fully separated Rx chain architecture may have best performance with less degradation on REFSENS when there is blocking noise toward the wanted carriers. Such architecture can even handle asynchronous DL intra-band carriers as discussed in Rel-18 non-collocated WI. A fully-shared Rx chain may have difficulty to achieve comparable performance as those implemented with separated Rx chain. And performance of partially shared Rx chain is in between the above-mentioned two architectures.

Figure 1. Receiver architectures for single band DL NCCA configuration

The single band DL NCCA had been discussed and evaluated in TR36.823 and requirements had been specified in the TS36.101. In the previous evaluation, though is was said separated Rx architecture, the UE architecture was actually partially-shared Rx chain. Since there’s no restriction on UE implementation UE may use fully-separated/partially-separated Rx chains to implement DL NCCA combos with best performance. However when the number of bands for CA configuration grows up more than three, the required number of Rx chains may exceed the number of Rx chains one UE can support. In order to support higher order CA configurations for such UE, fully-shared Rx chain would be considered intuitively.

For UE supporting Rel-18 and onward, it is possible that new UE design includes more Rx chains so that UE may be able to support more bands CA configurations with partially-shared/fully-separated Rx chains architecture. There would be no requirements impact for the powerful new UE capable with more Rx chains.

Observation 1: The existing DL NCCA requirements are based on partially-shared Rx architecture

Proposal 2: Fully-shared Rx chain will be considered for the method to reduce number of Rx chains. It does not preclude using existing partially-shared Rx chain architecture or fully-separated Rx chain for new designed UE if capable.

3. Conclusion

Proposal 1: All FDD and SDL bands are applicable for the study. TDD bands n41, n46, n48, n77, n78, n79, n90, n96, n102 and n104 are not applicable for the study.

Observation 1: The existing DL NCCA requirements are based on partially-shared Rx architecture

Proposal 2: Fully-shared Rx chain will be considered for the method to reduce number of Rx chains. It does not preclude using existing partially-shared Rx chain architecture or fully-separated Rx chain for new designed UE if capable.

Proposal 2: Fully-shared Rx chain will be considered for the method to reduce number of Rx chains. It does not preclude using existing partially-shared Rx chain architecture or fully-separated Rx chain for new designed UE if capable.

Reference:

- RP-241669 New SID: Study on NR FR1 DL Fragmented Carriers, RANP#104

- RP-233374, “Fragmented carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular

- R4-2402309: “Study on effective utilization of fragmented FR1 carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular