3GPP TSG RAN WG4 Meeting #112		 R4-2411556

Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	8.6.3

Source: 	MediaTek Inc.

Title: 	Discussion on UE RF requirements and DL performance impacts

Document for:	Approval

1. Introduction

It had been discussed in previous RAN plenary meeting that considering the spectrum hold be North America operators are usually non-contiguous blocks as stated in the reference [2][3]. In the RANP#104, a new SID[1] to study on NR FR1 DL fragmented carriers was approved. We provide our views and discussion points on UE RF requirements and DL performance impacts in this contribution.

2. Discussion

The scope of the study item Rel-19 NR FR1 DL fragmented carriers were copied below:

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

2.1 Discussion on equal PSD assumption

In the scope of the study, it was assumed “up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs”. However if we look at the existing REFSENS of UE RF requirements over all supported channel bandwidth, many of them are not scalable which means the REFSENS itself are not equal PSD according to previous evaluation due to impact from self-band uplink transmission. This is highlighted with red colour in the table below:

Table 1. Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n1 | 15 |  | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 |  | -90.6 | -90.1 | -89.6 |
| n2 | 15 |  | -98 | -94.8 | -93 | -91.8 | -90.7 | -84.1 | -83.6 | -81.5 |  |  |
| n3 | 15 |  | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 | -86.2 | -82.3 | -81.3 | -79.7 |
| n5 | 15 |  | -98.0 | -94.8 | -93.0 | -86.8 | -84.8 |  |  |  |  |  |
| n71 | 15 |  | -98.0 | -94.8 | -93.0 | -91.8 | -90.7 | -89.9 | -89.2 | -88.6 |  | -81.5 |
| n8 | 15 |  | -97.0 | -93.8 | -91.4 | -85.8 | -83.6 | -81.3 | -78.4 |  |  |  |
| n12 | 15 |  | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
| n13 | 15 |  | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
| n14 | 15 |  | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
| n18 | 15 |  | -100.0 | -96.8 | -95.0 |  |  |  |  |  |  |  |
| n20 | 15 |  | -97.0 | -93.8 | -91.0 | -89.8 |  |  |  |  |  |  |
| n24 | 15 |  | -100.0 | -96.8 |  |  |  |  |  |  |  |  |
| n25 | 15 |  | -96.5 | -93.3 | -91.5 | -90.3 | -89.3 | -82.2 | -81.7 | -79.5 | -77.6 |  |
| n26 | 15 | -99.7 | -97.56 | -94.56 | -92.76 | -87.6 | -84.5 | -81.7 |  |  |  |  |
| n28 | 15 | -100.2 | -98.5 | -95.5 | -93.5 | -90.8 | -84.2 | -78.5 |  |  |  |  |
| n30 | 15 |  | -99.0 | -95.8 |  |  |  |  |  |  |  |  |
| n31 | 15 | -95.7 | -93.5 |  |  |  |  |  |  |  |  |  |
| n65 | 15 |  | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  | -89.2 |
| n66 | 15 |  | -99.5 | -96.3 | -94.5 | -93.3 | -92.2 | -91.4 | -90.7 | -90.1 | -89.6 |  |
| n70 | 15 |  | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 |  |  |  |  |  |
| n71 | 15 |  | -97.2 | -94.0 | -91.6 | -86.0 | -84.19 -74.810 | -82.59 -67.110 | -80.79 -64.010 |  |  |  |
| n72 | 15 | -95.7 | -93.5 |  |  |  |  |  |  |  |  |  |
| n74 | 15 |  | -99.53 | -96.33 | -94.53 | -89.33 |  |  |  |  |  |  |
| n85 | 15 | -99.2 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
| n100 | 15 | -102.2 | -100 |  |  |  |  |  |  |  |  |  |
| n105 | 15 |  | -97.28 | -94.0 | -91.6 | -86.9 | -85.1 | -83.8 | -82.5 |  |  |  |
| n106 | 15 | -99.2 |  |  |  |  |  |  |  |  |  |  |

Observation 1: For FDD bands, the REFSENS level over different DL CBWs are usually not equal PSD duel to interference from self-band uplink transmission

Several possible options may be considered to continue evaluation for the impact on DL performance if equal PSD condition is required:

- Option 1: Equal PSD shall only apply for CBW of the two non-contiguous CCs in one DL band are equal

- Option 2: Equal PSD is applicable for REFSENS can be scalable. No evaluation on other cases.

- Option 3: Equal PSD is applicable for REFSENS can be scalable. Case by case discussion on other CBWs in WI phase

- Option 4: Start from equal PSD cases. FFS on other un-equal PSD cases

- Option 5: Study start from existing test configurations

- *Other options are not precluded if any

Considering discussion on option 1 to 4 may not be easy to converge in the meeting progress and the purpose of the study is not to re-evaluate everything in single band DL NCCA configurations. And to control workload to proper range, we propose option 5, the evaluation only focus on existing test configurations. As for equal PSD, it may apply for BS Tx perspective for single band DL transmission non-contiguous carriers.

Observation 2: Equal PSD may apply for BS Tx perspective for single band DL transmission non-contiguous carriers

Proposal 1: We propose option 5 : The study starts from existing test configurations

Proposal 2: The evaluation assumptions on UL Tx shall be consistent with previous RAN4 assumptions. For ex: PC3 PA ACPR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dBc

2.2 Example band consideration for evaluation discussion

Though the example band for study purpose was not indicated in the SI scope, it would be beneficial if we can have one example band for evaluation for impacts on UE DL performance evaluation. For example, n25 is a typical FDD band that its Tx-Rx gap is small that self-band UL Tx has significant impact on DL carriers. The existing UE Rx requirements are to be considered to analyse including ΔRIBNC on PCC/SCC, ACS IBB and NBB.

Table 2. Existing requirements on intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |

Proposal 3: In order to evaluate impacts on UE DL performance for the study, company may select preferred example band to evaluate impacts on UE RF requirements and DL performance

2.3 Aspects of impacts on DL Rx requirements

In our view, n25 is a good example band that we would like to start the study with. With above proposal 1 & 2, let’s look at the three test configurations in existing requirements. The Rx architecture assumption of existing requirements is based on partially-shared Rx chains. It is foreseen both CCs may have good performance w.r.t ACS/IBB/NBB as well as rejection to self-band UL transmission interference. If the Rx chain is shared by the two non-contiguous CCs, (fully-shared Rx chain), the channel selection filter would need to be adjusted to wider bandwidth to accommodate the two DL carriers including Wgap between the two CCs. The Tx rejection of shared Rx chain would be much worse. Not only SCC would still impacted by self-band UL Tx, PCC may also be impacted due to worse filter rejection ratio toward UL Tx transmission. Another potential degradation may happen on in-gap ACS/IBB/NBB performance, these requirements would also need to be evaluated.

Figure 1. Rx requirements to be evaluated

Figure 2. Illustration on the two BCS for DL NCCA test

Observation 3: Not only SCC is impacted by self-band UL Tx, PCC may also be impacted by it due to channel selection filter would be adjusted to wider total bandwidth in fully-shared Rx chain architecture

Proposal 4: Impacts on DL performance shall also consider degradation on PCC due to wider channel selection filter configured under fully-shared Rx chain architecture

We may also consider evaluation on following terms for the example band evaluation:

- REFSENS relaxation for PCC (5M-5M / 5M-40M)

- REFSENS relaxation for SCC (5M-5M / 40M-5M)

  - For 40M-5M case, partial RB uplink IMD provided by : Rapp (3GPP) PA model

- Blocking characteristic

  - Evaluate on (ACS1/2, IBB1/2, NBB)

  - Symmetrical CC BW: 5M-5M

  - Asymmetrical CC BW: 40M-5M

3. Conclusion

Observation 1: For FDD bands, the REFSENS level over different DL CBWs are usually not equal PSD duel to interference from self-band uplink transmission

Observation 2: Equal PSD may apply for BS Tx perspective for single band DL transmission non-contiguous carriers

Proposal 1: We propose option 5 : The study starts from existing test configurations

Proposal 2: The evaluation assumptions on UL Tx shall be consistent with previous RAN4 assumptions. For ex: PC3 PA ACPR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dBc

Proposal 3: In order to evaluate impacts on UE DL performance for the study, company may select preferred example band to evaluate impacts on UE RF requirements and DL performance

Observation 3: Not only SCC is impacted by self-band UL Tx, PCC may also be impacted by it due to channel selection filter would be adjusted to wider total bandwidth in fully-shared Rx chain architecture

Proposal 4: Impacts on DL performance shall also consider degradation on PCC due to wider channel selection filter configured under fully-shared Rx chain architecture

Reference:

- RP-241669 New SID: Study on NR FR1 DL Fragmented Carriers, RANP#104

- RP-233374, “Fragmented carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular

- R4-2402309: “Study on effective utilization of fragmented FR1 carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular