3GPP TSG-RAN WG4 Meeting #112	R4-2411557

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	BT

Title:	Scenarios and requirements for spatial channel modelling

Agenda item:	8.14.1

Document for:	Decision

Introduction

The study item on Spatial Channel Model (SCM) for demodulation performance requirements was agreed in RAN#102 [1]. 3GPP technologies with a significant reliance on the spatial properties of radio propagation environments, such as R16 e-MIMO, and R17 fe-MIMO increasingly get implemented in the features in mobile network infrastructure delivering connectivity services, as well as in consumer devices utilising the infrastructure.

Beyond the justification for [1], we believe that now is the appropriate time to prepare a solid basis for performance specifications in upcoming Releases 20 and 21 [2] to support the design and introduction of 5G-A and 6G technologies.

This contribution outlines our perspective on the general approach to the study item, proposes scenarios for evaluation of the SCM channel models, and provides initial basic requirements for the SCM based on a set of sample measurements from typical MIMO deployments.

Discussion

Approach

While it is understood that channel models are not expected to serve as a complete and universally usable tool for evaluation of radio systems, we feel it is important to ensure that models utilised in 3GPP continue to provide a reasonable reflection of operating conditions. Benefits from such versatility include the enhanced ability to focus the effort of experts in 3GPP on development of standardised features. This SI also has a unique opportunity to contribute significantly to the success of 6G by laying robust and long-term foundation for further evolution and optimisation of MIMO technologies. Our view therefore is that this study item should aim for the following principles:

- Ability of the model to reflect current scenarios: it is essential for the outcome of this study to be able to reflect key current MIMO technologies deployed in the field with reasonable accuracy.

- Flexibility of the model to address future use cases should be achieved through reflection of physical MIMO processes, so that the model is naturally reflective of future technology developments.

- Performance scaling of features with should be aimed. i.e. the SCM should be naturally reflective of current and foreseeable operating scenarios and associated features.

Both 5G-Advanced and 6G technologies will require a trusted, versatile and flexible channel model.

The SCM shall (a) address current deployment scenarios, (b) possess flexibility to address future use cases, (c) ensure performance scaling of features.

Scenarios for evaluation and preliminary observations

This section outlines an initial set of deployment scenarios that are deemed to be important to be used in the design and evaluation of the SCM and can be considered as a starting point for further discussions. These scenarios are intended to reflect the typical current and anticipated future deployment scenarios for systems utilising 3GPP technologies.

3GPP technologies for SU-MIMO, MU-MIMO and Multi-cell MIMO are widely deployed in mobile networks and continue to evolve.

RAN4 shall focus on SU-MIMO, MU-MIMO and Multi-Cell MU-MIMO scenarios in priority order.

Single-user MIMO (SU-MIMO)

SU-MIMO techniques remain central for operation of networks in the field and can serve as a baseline for more sophisticated high efficiency or reliability features. It is therefore important to ensure that the channel model reflects the properties of radio propagation channels with sufficient detail to ensure versatility in setting the performance requirements for a wide range of current and future use cases.

Table 1 outlines key characteristics for the corresponding channel model evaluation scenario, including notes on the rationale. For the values in bold in the table, Section 3 provides properties for measured MIMO channels from sample field measurements from typical deployments. While it is not expected that models will match the specific samples, we do expect the model to be versatile enough to capture the key properties observed in the sample measurements.

Table 1. Scenario parameters and characteristics

| Scenario property | Characteristic/value | Notes |
| --- | --- | --- |
| Number of transmitter ports for DL | 4,8,32 [64,128,256] | Numbers in bold – sample measurement results provided in Section 3. While current specifications do not support CSI-RS port numbers over 32, we believe it is important to ensure long-term applicability of the developed mode, and therefore encourage to account for systems with a larger transceiver port count |
| Number of receiver ports for DL | 2,4,8 |  |
| Antenna elements | Cross-polarised dipoles | Required to model the polarisation-dependent propagation properties of RAN deployments. Other options are not precluded. Different antenna spacing, should be explored |
| Port-to-to polarisation mapping | First Ntx/2 ports to one polarisation, remaining Ntx/2 ports to the orthogonal polarisation | Assumption arising from RAN1 precoder codebook creation/definition |
| Receive port imbalance | Several dB | To preserve the variability of relative receive power levels at the receiver – due to the combined effect of transmitter antenna design, propagation characteristics as well as the receive antenna design. |
| Long-term stability & consistency | As dictated by scenario, e.g. several frames | If a UE doesn’t move then signals are received from long-term stable directions with stable power differences and delays (consistent with clusters) |

Table 1 provides a set of important system characteristics necessary for assessment of the SCM

RAN4 shall ensure the following properties of the channel model are reflected as a minimum:

| Scenario property | Characteristic/value |
| --- | --- |
| Number of transmitter ports for DL | 4,8,32 [64,128,256] |
| Number of receiver ports for DL | 2,4,8 |
| Antenna elements | Cross-polarised dipoles |
| Port-to-to polarisation mapping | First Ntx/2 ports to one polarisation, remaining Ntx/2 ports to the orthogonal polarisation |
| Receive port imbalance | Several dB |
| Long-term stability & consistency | As dictated by scenario, e.g. several frames |

Multi-user MIMO (MU-MIMO)

A similar set of characteristics as in Table X is applicable to multiple users. We would expect the MU-MIMO scenarios with multiple users to experience conditions:

- Low inter-user interference originating from significant spatial separation and/or environment (including the MIMO transmitter properties such as transceiver port count) conducive to spatial separability of users.

- High inter-user interference originating from close proximity of users and/or the properties of operating environment.

Multi-cell MIMO

At least two adjacent cells with partially overlapping coverage should be modelled, to support specification of 3GPP features involving inter-site coordination, as well as cases requiring interoperability.

Sample behaviour

This section provides several characteristics of measured MIMO channels from typical field deployments. The purpose of these samples is to illustrate the key properties for the SCM to capture in order to provide a versatile tool to support specification of relevant 3GPP features. Results presented in this section are an extension of the results reported in [3].

Measurement conditions

The MIMO channel measurements were conducted using a typical commercial MIMO deployment using a base station with 4 transceiver ports and receiving equipment with 4 ports. Measurements were conducted in a 15MHz channel at 2162.2MHz centre frequency, over several locations in a campus-type environment shown in Figure 1. Reported samples were collected in static positions during office hours, with some movement of people and vehicles in the coverage sector, yet not in the immediate vicinity of the measurement setup. More detailed information on the measurement setup, similar to the one utilized in results presented here, can be found in [4].

The output of the measurements were complex channel gains per transmitter and receiver port pair (the H matrices), including all system gains and losses. Noise power was estimated at the receiving equipment, while the transmitter power setting was 15 dBm per resource element (15 kHz subcarrier spacing).

Figure 1. Measurement point locations for the sample MIMO channels from a typical 4T4R base station.

Singular value distributions

Histograms in Figure 2 illustrate the distribution of singular values of measured MIMO channels in locations B and D, in conditions outlined above. Remaining sample measurements are provided in Appendix for information. The histograms demonstrate that the measured MIMO channels produce eigenmodes delivering varying qualities of signal transmission. While appropriate processing for a specific communication system aiming a high spectral efficiency, coverage (or some other KPI) would be required, examples below demonstrate the conditions where such systems would need to operate in.

It is therefore important to ensure a similar variability in the spatial structure of the modelled channels is utilised for demodulation performance requirements and other assessment types.

Figure 2. Histograms of the SINR assuming SVD precoding and combining for measurement locations with a strong line of sight (B), and for non-line of sight position (D)

Table 2 below summarises the relative channel “eigenmode” gains for the set of sample measurements. While the figures in the table are a summary of measurements for a specific environment, the SCM in question should be able to produce similar results for test conditions.

Table 2. Average eigenmode gains relative to the strongest spatial preference per measurement location

| Location | Average gain relative strongest eigenmode, dB |  |  |
| --- | --- | --- | --- |
|  | Eigenmode 2 | Eigenmode 3 | Eigenmode 4 |
| A | -4.85 | -20.70 | -33.18 |
| B | -5.84 | -12.94 | -24.38 |
| C | -7.48 | -15.71 | -27.86 |
| D | -12.74 | -18.65 | -30.67 |
| E | -7.92 | -14.89 | -24.89 |
| F | -7.84 | -15.17 | -27.00 |
| G | -7.91 | -18.58 | -31.92 |
| Average | -7.80 | -16.66 | -28.56 |

These sample results lead to the following observation and a corresponding proposal.

Eigenmodes of measured MIMO channels in field deployments deliver varying qualities of signal transmission. Averaged across the specific observation set presented here, the MIMO channels’ eigenmodes exhibit gain offsets of approximately 8, 17 and 29 dB for modes 2, 3 and 4 relative to the strongest eigenmode.

Similar effects must be reflected in the SCM.

the channel generated by the SCM shall exhibit variations in channel eigenmodes similar to that in the sample measurements presented. In particular, there should be difference between eigenmode gains, for example 8, 17 and 29 dB for modes 2, 3 and 4 relative to the strongest eigenmode on average across model realisations.

Long-term stability and consistency

Consistency of generated MIMO channel properties would be an important characteristic for a channel model to reflect the nature of operating environment correctly, and to support the specification of 3GPP technologies that operate across a considerable time horizon.

Figure 3 demonstrates the expected behaviour for the eigenmodes of the measured MIMO channels across the measurement duration, while Figure 4 shows the evolution of the measured gains for the transmitter and receiver links over time. In both cases, a long-term stability of the levels is observed, with minor variations at specific time instants. Such stability and consistency are expected from a channel model both for the basic properties of the channel demonstrated in, but also for other properties such as spatial preferences of the channel, AoA, AoD, power spectrum, correlation.

Figure 3 Stability of spatial preferences in the measured channels, consistent with the expected behavior for a static measurement scenario.

Figure 4. Evolution of magnitudes relative power losses, average across RBs, for the transmit-receive port pairs over the measurement duration

Long-term stability and consistency with expected behaviour in a measurement scenario is observed.

the channel generated by the spatial channel model shall exhibit explainable, and consistent spatial domain preferences.

Basic properties

Several properties of signal propagation important for MIMO performance are highlighted in this section, in particular power imbalance at the receiver ports, and the amount of leakage from one polarisation to another (including the sensitivity to the specific source of the leakage). As MIMO technologies continue to evolve towards 6G, it is important to ensure the basic properties of the operating environment are captured to ensure a solid basis for development and specification of high-performance systems.

Power imbalance at the receiver port due to the properties of propagation environments, as well as the transmitter antenna structures is expected, and Figure 4 provides an illustration. Specifically, distinct levels of channel gains are observed for all receiver pairs, with clear relative offsets for the duration of the measurement. It is important to reflect such basic effects both for SU-MIMO and MU-MIMO scenarios in order to ensure reliable and physically-consistent context for relevant specification work.

Further, correlation and energy leakage should be reflected in a reliable and robust manner as they play an important role in modern MIMO systems. Figure 5 shows the calculated cross-polarisation discrimination (XPD) coefficients for two transmitter ports with orthogonal polarisation, and several receiver ports. Detailed explanation of the figures is provided in Appendix, Section 6.1.3.

Figure 5 shows that receiver ports experience imbalance in received power due to propagation environment. Energy leakage between polarisations is asymmetric across polarisations, frequency-selective, and variable across receiving ports with same polarity. In particular, leakage of energy from polarisation 1 is not equivalent to leakage of energy from polarisation 2, most likely due to the propagation environment.

Figure 5. XPD values ρ_kl from transmitters 1 (Polarisation 1) and 4 (Polarisation 2) towards receiver with index k with matching polarisation to that of transmitter, relative to receiver l with polarisation, orthogonal to polarisation of receiver k.

Receiver ports experience imbalance in received power due to propagation environment. Energy leakage between polarisations is asymmetric across polarisations, frequency-selective, and variable across receiving ports with same polarity.

Correlation and cross-polarisation discrimination properties for the modelled channel need to reflect the variability observed in the field, e.g. through appropriate adjustment of κn,m-1 factors in Equation 7.2-9 in TR38.827.

Conclusion

Observations and proposals for the SCM are summarised below.

- Both 5G-Advanced and potential 6G technologies will require a trusted, versatile and flexible channel model.

- The SCM shall (a) address current deployment scenarios, (b) possess flexibility to address future use cases, (c) ensure performance scaling of features.

3GPP technologies for SU-MIMO, MU-MIMO and Multi-cell MIMO are widely deployed in mobile networks and continue to evolve.

- RAN4 shall focus on SU-MIMO, MU-MIMO and Multi-Cell MU-MIMO scenarios in priority order.

Table 1 provides a set of important system characteristics necessary for assessment of the SCM

RAN4 shall ensure the following properties of the channel model are reflected as a minimum:

| Scenario property | Characteristic/value |
| --- | --- |
| Number of transmitter ports for DL | 4,8,32 [64,128,256] |
| Number of receiver ports for DL | 2,4,8 |
| Antenna elements | Cross-polarised dipoles |
| Port-to-to polarisation mapping | First Ntx/2 ports to one polarisation, remaining Ntx/2 ports to the orthogonal polarisation |
| Receive port imbalance | Several dB |
| Long-term stability & consistency | As dictated by scenario, e.g. several frames |

Eigenmodes of measured MIMO channels in field deployments deliver varying qualities of signal transmission. Averaged across the specific observation set presented here, the MIMO channels’ eigenmodes exhibit gain offsets of approximately 8, 17 and 29 dB for modes 2, 3 and 4 relative to the strongest eigenmode.

the channel generated by the SCM shall exhibit variations in channel eigenmodes similar to that in the sample measurements presented. In particular, there should be difference between eigenmode gains, for example 8, 17 and 29 dB for modes 2, 3 and 4 relative to the strongest eigenmode on average across model realisations.

Long-term stability and consistency with expected behaviour in a measurement scenario is observed.

the channel generated by the spatial channel model shall exhibit explainable, and consistent spatial domain preferences.

Receiver ports experience imbalance in received power due to propagation environment. Energy leakage between polarisations is asymmetric across polarisations, frequency-selective, and variable across receiving ports with same polarity.

Correlation and cross-polarisation discrimination properties for the modelled channel shall reflect the variability observed in the field, e.g. through appropriate adjustment of κn,m-1 factors in Equation 7.2-9 in TR38.827.

References

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- RP-233985, High-level Considerations for 6G Timeline, RAN chair (Qualcomm), CT chair (Huawei), SA chair (Intel)

- R4-2402277, “Discussion on 8Rx general demodulation aspects and spatial channel models”, Nokia, Nokia Shanghai Bell, BT

- A. Tukmanov, J. Weng and R. Husbands, "Polarization and Correlation in MIMO Channels," WSA & SCC 2023; 26th International ITG Workshop on Smart Antennas and 13th Conference on Systems, Communications, and Coding, Braunschweig, Germany, 2023

- M. Coldrey, "Modeling and Capacity of Polarized MIMO Channels," VTC Spring 2008 - IEEE Vehicular Technology Conference, Marina Bay, Singapore, 2008

Appendix

Results for remaining locations in the set

This section provides results for locations A,C,E,F and G, for information. The narrative is similar to the one captured in respective sub-sections of Section 3.

Singular value distributions

Figure 6. Histograms of the SINR assuming SVD precoding and combining for measurement locations

Long-term stability and consistency

Figure 7. Stability of spatial preferences in the measured channels, consistent with the expected behavior for a static measurement scenario.

Figure 8. Evolution of magnitudes relative power losses, average across RBs, for the transmit-receive port pairs over the measurement duration

Basic properties

The XPD for a given transmitter port is calculated based on Eq.(2.3) in [5], so that an XPD ρ_kl in Figure 5 for transmitters 1 (Polarisation 1) and 4 (Polarisation 2) represent leakage of power between polarisations, where receiver with index k is one with matching polarisation to that of transmitter (i.e. either Tx1 or Tx4), relative to receiver l with polarisation, orthogonal to polarisation of receiver k. For example, ρ_13 Transmitter 1 (Polarisation 1) reflects the ratio of power received by receiver port 1 with matching polarisation, relative to the power received by port 3 with orthogonal polarisation, in logarithmic scale. Note that first Ntx/2 ports are assumed to map to one polarisation, remaining Ntx/2 ports to the orthogonal polarisation.

Figure 9. XPD values ρ_kl from transmitters 1 (Polarisation 1) and 4 (Polarisation 2) towards receiver with index k with matching polarisation to that of transmitter, relative to receiver l with polarisation, orthogonal to polarisation of receiver k.