3GPP TSG-RAN WG4 Meeting #112	R4- 2411560

Maastricht, Netherland, Aug 19-23, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance on FR2 SCell activation delay reduction

Agenda item:	5.14.1

Document for:	Discussion

- Introduction

At RAN4#111 meeting, the L3 reporting after SCell activation command was further discussed in multiple SCell activation scenario and the big draft CR was endorsed in [1]. In this contribution, we will continue the discussion on the open issues according to the WF [2].

- Multiple SCell activation with L3 reporting on FR1 band

Application conditions

At RAN4#111 meeting, the applicability conditions for multiple SCell activation delay requirement with L3 reporting were agreed as below and the following texts were added to TS38.133 clause 8.3.18.

Issue 1-1: Applicability of multiple SCell activation with L3 reporting on FR1 and FR2 band

Agreement:

  - For FR1, L3 reporting based multiple SCell activation requirements are applicable to unknown target SCell activation when there is no contiguous active serving cell or there is no contiguous known SCell(s) to the unknown to-be-activated SCell on the FR1 band.

    - This condition will be added section 8.3.18.

  - For FR2, L3 reporting based multiple SCell activation requirements are applicable to unknown target SCell activation when there is no active serving cell or there is no known SCell(s) on the same band.

    - This condition will be added section 8.3.18.

TS 38.133 clause 8.3.18:

In two CGs of NR-DC, the requirements in this clause shall apply when the following conditions are met:

-	UE receives one MAC command per CG for multiple SCell activation within the activation period defined in this clause, and

-	UE supports per-FR measurement gap capability, and

-	all to-be-activated SCells are unknown on the same FR2 band, and there is neither active serving cell(s) nor known SCell(s) on the same band, or,

-	all to-be-activated SCells are unknown on the same FR1 band, and there is neither active serving cell contiguous to the SCell nor known SCell(s) contiguous to the to-be-activated SCell on the same band, and

-	the UE reports valid L3 measurement results after receiving the SCell activation command for the to-be-activated SCell in FR1, or at least one unknown SCell in the same FR2 band

In our view, the condition “all to-be-activated SCells are unknown” is well applicable to FR2 band but is too restricted for FR1 band. In one example, assuming two SCells are to be activated where one is known and the other is unknown, and they are non-contiguous on the same band, the UE would need cell detection in order to activate the unknown SCell hence L3 reporting is still expected. In other words, L3 reporting is expected as long as there is at least one unknown to-be-activated SCell and there is no active serving cell or known to-be-activated SCell contiguous to the unknown SCell on the same band. Therefore, the applicability condition “all to-be-activated SCells are unknown” shall be changed to “at least one to-be-activated SCell is unknown” for the FR1 band.

Observation #1: If all to-be-activated SCells are on the same FR1 band, the L3 reporting is expected as long as “there is at least one unknown to-be-activated SCell” and there is no active serving cell or known to-be-activated SCell contiguous to the unknown SCell on the same band.

Proposal 1: For multiple SCell activation on the same FR1 band, the applicability condition “all to-be-activated SCells are unknown” shall be changed to “at least one to-be-activated SCell is unknown”.

Multiple SCell activation delay with L3 reporting

With the L3 reporting, the SCell activation delay for activating multiple SCells on same band is now defined based on TL3 report without considering N1 as cited below. It assumes no cell detection for any of the to-be-activated SCells on the same band. For FR1 band, this implies all the unknown SCells requiring cell detection are reported in L3 reporting.

TS 38.133 clause 8.3.18:

Upon receiving SCell activation command in slot n for more than one SCell, for each of the to-be-activated SCell, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot  n+THARQ+Tactivation_time_multiple_scells+TCSI_ReportingNR slot length, where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

Tactivation_time_multiple_scells is the target SCell activation delay in millisecond in multiple SCell activation scenario.

Tactivation_time_multiple_scells is:

-	10ms + THARQ + TL3 report   + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if the semi-persistent CSI-RS is used for CSI reporting

-	7ms + TL3 report   + max (THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if the periodic CSI-RS is used for CSI reporting

However, the UE may not have the valid measurement results for all the to-be-activated unknown SCells. If there are multiple non-contiguous unknown SCells on the same FR1 band, and the UE reports only one of them, the unknown SCells which are not reported in L3 reporting needs to be activated in legacy manner with cell detection. RAN4 to discuss if to consider the case where not all the unknown SCells are reported in the L3 reporting and cell detection is still needed on some of the unknown SCells.

Observation #2: When receiving SCell activation command, UE may not have the valid measurement results for all the to-be-activated unknown SCells on the same FR1 band.

Observation #3: For non-contiguous unknown SCells on the same FR1 band, the unknown SCell which is not reported in L3 reporting needs to be activated in legacy manner with cell detection.

Proposal 2: RAN4 to discuss if to consider the case where not all the unknown to-be-activated SCells are reported in the L3 reporting and cell detection is still needed on some of the unknown SCells on the same FR1 band.

If this case is to be studied, N1 needs to be counted when activating multiple non-contiguous unknown SCells on FR1 band, wherein N1 is the number of parallel cell detection on the to-be-activated unknown SCells which were not reported and non-contiguous to the reported SCells in L3 reporting.

Otherwise, the assumption needs to be clarified in the applicability conditions to indicate that all the unknown SCells requiring cell detection are reported in L3 reporting.

Proposal 3: If the case in P2 is agreed to be discussed, N1 needs to be counted for the cell detection on the unknown SCells which were not reported and non-contiguous to any of the reported unknown SCells. Otherwise, it shall be clarified in applicability conditions that all the unknown SCells requiring cell detection are reported in L3 reporting.

In accordance with the proposals, the companion CR is provided in [3].

Other issues with FR1 SCell activation enhancement

In last meeting, another issue was raised about the impact from measurement period. In our understanding, if the measurement period of the SCell is larger than 2400ms, the UE would need additional AGC refinement when activating an FR1 known SCell. While L3 reporting helps activating the unknown SCell as if it is known, the measurement period can also be considered as one side condition in determining the SCell activation delay with L3 reporting.

Issue 1-4: Other issues related with FR1 SCell activation enhancement

- FFS following proposals:

  - Proposal 1:

    - RAN4 to discuss if measurement period shall be considered as a condition to differentiate the requirements for FR1 SCell activation enhancement with L3 report.

    - Like the legacy FR1 known SCell activation, SCell activation delay requirement with L3 report shall be differentiated according to measurement period below or above 2400ms, and decide if AGC refinement or T/F tracking is needed.

  - Proposal 2:

    - RAN4 also consider applying enhancements of L3 reporting during SCell activation in FR1 to the case when only one SSB is transmitted in ssb-PositionInBurst.

    - RAN4 further extend the requirement applicability of 8.3.17 and 8.3.18 to the scenarios in FR1 where only one SSB is considered. If only one SSB is considered, Tuncertainty_MAC, Tuncertainty_SP, Tuncertainty_RRC and TRRC_delay are counted as zero, and the 3ms MACE CE decoding delay for TCI state activation is removed, i.e. the overall delay Tactivation_time is 7ms + TL3,report+ THARQ + TFineTiming + 2ms.

Proposal 4: The measurement period can be considered as one side condition in determining the SCell activation delay with L3 reporting.

Following the principle in legacy FR1 known SCell activation, additional time period for AGC refinement would be needed if the measurement period of SCell is larger than 2400ms. However, a time period of TL3,report is introduced when L3 reporting is transmitted, which indicates a delay of acquiring the first available UL resource for reporting from 7ms +THARQ after receiving SCell activation command. The AGC refinement can be performed in parallel while the UE is waiting for the uplink resources for L3 reporting. That is, the AGC refinement will cause additional delay only if it is performed after L3 reporting. Therefore, the SCell activation delay with L3 reporting shall be extended only if the measurement period is larger than 2400ms and the AGC refinement is performed after L3 reporting.

Observation #4: The AGC refinement can be performed in parallel while the UE is waiting for the uplink resources for L3 reporting.

Proposal 5: The SCell activation delay with L3 reporting shall be extended only if the measurement period is larger than 2400ms and the AGC refinement is performed after L3 reporting.

- Conclusion

This contribution discusses the open issues for the enhanced FR2 SCell activation. The observations and proposals are summarized as below:

Observation #1: If all to-be-activated SCells are on the same FR1 band, the L3 reporting is expected as long as “there is at least one unknown to-be-activated SCell” and there is no active serving cell or known to-be-activated SCell contiguous to the unknown SCell on the same band.

Proposal 1: For multiple SCell activation on the same FR1 band, the applicability condition “all to-be-activated SCells are unknown” shall be changed to “at least one to-be-activated SCell is unknown”.

Observation #2: When receiving SCell activation command, UE may not have the valid measurement results for all the to-be-activated unknown SCells on the same FR1 band.

Observation #3: For non-contiguous unknown SCells on the same FR1 band, the unknown SCell which is not reported in L3 reporting needs to be activated in legacy manner with cell detection.

Proposal 2: RAN4 to discuss if to consider the case where not all the unknown to-be-activated SCells are reported in the L3 reporting and cell detection is still needed on some of the unknown SCells on the same FR1 band.

Proposal 3: If the case in P2 is agreed to be discussed, N1 needs to be counted for the cell detection on the unknown SCells which were not reported and non-contiguous to any of the reported unknown SCells. Otherwise, it shall be clarified in applicability conditions that all the unknown SCells requiring cell detection are reported in L3 reporting.

Proposal 4: The measurement period can be considered as one side condition in determining the SCell activation delay with L3 reporting.

Observation #4: The AGC refinement can be performed in parallel while the UE is waiting for the uplink resources for L3 reporting.

Proposal 5: The SCell activation delay with L3 reporting shall be extended only if the measurement period is larger than 2400ms and the AGC refinement is performed after L3 reporting.

- References

- R4-2410397, Draft Big CR to TS 38.133 on core requirement maintenance for Even Further RRM enhancement for NR and MR-DC, Apple

- R4-2410259, WF for [111][205] NR_RRM_enh3_part1, Apple

- R4-2411561, CR on multiple SCell activation with L3 reporting, Nokia, Nokia Shanghai Bell