3GPP TSG-RAN WG4 Meeting #112	R4-2411562

Maastricht, Netherland, Aug 19-23, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Performance aspects for FR2 SCell activation delay reduction

Agenda item:	5.14.2

Document for:	Discussion

- Introduction

At RAN4#111 meeting, the test cases were endorsed based on the principles as captured in the WF [1]. In this contribution, we are discussing the leftover issues on test cases.

- Test cases

At RAN4#110 meeting, it was agreed to specify the test cases as listed in the table below [2]. In particular, for each of the test cases, it shall be defined for the corresponding CA/DC modes respectively and the UE is required to pass the test in one of the CA/DC modes.

Issue 2-6-1: Candidate test case list based on proposals from companies

Agreement: Following TCs shall be introduced.

| Feature | Index | Test case | CA/DC mode | Other configurations | Test applicability | Volunteer Company to lead CR |
| --- | --- | --- | --- | --- | --- | --- |
| FG31-1: L3 report based enhancement | 1 | FR2 unknown SCell activation with L3 report | - FR1 PCell+FR2 target SCell - FR2 PCell+FR2 inter-band target SCell - LTE PCell + FR1 PSCell + FR2 target SCell |  | Define test applicability and UE is required to perform one test for each FR. UE only needs to pass test in one CA/DC mode. | QC |
|  | 2 | FR2 PUCCH SCell activation delay with L3 report |  |  |  | Huawei |
|  | 5 | FR1 unknown SCell activation with L3 report | - FR1 PCell+FR1 inter-band target SCell - LTE PCell + FR1 PSCell + FR1 inter-band target SCell |  | Define test applicability and UE is required to perform one test for each FR. UE only needs to pass test in one CA/DC mode. | vivo |
|  | 7 | Multiple SCell activation delay with FR1 unknown SCell with L3 report |  |  |  | ZTE |
| FG31-2: Beam sweeping factors reduction | 9 | FR2 unknown SCell activation with FG31-2 and FG31-3 | - FR1 PCell+FR2 target SCell - FR2 PCell+FR2 inter-band target SCell - LTE PCell + FR1 PSCell + FR2 target SCell | DRX.8: DRX cycle = 320 ms and TAT = Infinity | Verify UE supporting FG 31-2 and/or FG 31-3 for FR2, depending the capabilities supported by the UE. FFS:UE only needs to pass test in one CA/DC mode. | Nokia |
| FG31-3: (1)Use SSB periodicity instead of SMTC periodicity” (2)“Performing L1-RSRP measurement in non-DRX mode even DRX is configured” | 17 | FR1 unknown SCell activation with FG31-3 | - FR1 PCell+FR1 inter-band target SCell - LTE PCell + FR1 PSCell + FR1 inter-band target SCell | DRX.8: DRX cycle = 320 ms and TAT = Infinity | Verify UE supporting FG 31-3 for FR1. UE only needs to pass test in one CA/DC mode. | MTK |

The BigCR on the performance requirements was endorsed in RAN4#111 meeting in [3]. However, for TC#1:FR2 unknown SCell activation with L3 report, this BigCR only includes the test case in EN-DC scenario in A.5.5.3.x i.e. (3) LTE PCell + FR1 PSCell + FR2 target SCell in above table. There is no test case for TC#1 in FR1+FR2 and FR2+FR2 scenarios. Hence the test cases shall be specified for the other agreed CA/EN-DC modes: (1) FR1 PCell+FR2 target SCell and (2) FR2 PCell+FR2 inter-band target SCell. We provide these test cases in our companion CR [4].

Observation #1: The endorsed BigCR only includes the test case in EN-DC scenario i.e. (3) LTE PCell + FR1 PSCell + FR2 target SCell in above table.

Proposal 1: To specify the test cases for TC#1 in the other agreed CA/EN-DC modes:

(1) FR1 PCell+FR2 target SCell and

(2) FR2 PCell+FR2 inter-band target SCell.

- Conclusion

This contribution discusses the performance aspects of the R18 SCell activation enhancement solution. The observations and proposals are summarized as below:

Observation #1: The endorsed BigCR only includes the test case in EN-DC scenario i.e. (3) LTE PCell + FR1 PSCell + FR2 target SCell in above table.

Proposal 1: To specify the test cases for TC#1 in the other agreed CA/EN-DC modes:

(1) FR1 PCell+FR2 target SCell and

(2) FR2 PCell+FR2 inter-band target SCell.

- References

- R4-2406339, WF for [110bis][207] NR_RRM_enh3_part1, Apple

- R4-2403466, WF for [110][207] NR_RRM_enh3_part1, Apple

- R4-2410398, Big CR to TS 38.133 on performance requirements for Even Further RRM enhancement for NR and MR-DC, Apple

- R4-2411563, CR to TS 38.133 on TC#1 in FR1+FR2 and FR2+FR2 scenarios, Nokia, Nokia Shanghai Bell