3GPP TSG-RAN WG4 Meeting #112	R4-2411563

Maastricht, Netherland, Aug 19-23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4690 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on FR2 unknown SCell activation with FG31-1 in FR1+FR2 and FR2+FR2 scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh3-Perf |  |  |  |  |  | Date: |  |  | 2024-8-9 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It was agreed to define the test cases for FR2 unknown SCell activation with L3 report in the following three CA/DC scenarios. - FR1 PCell+FR2 target SCell - FR2 PCell+FR2 inter-band target SCell - LTE PCell + FR1 PSCell + FR2 target SCell However, the latest spec only includes the TC for (3) while TC in (1) and (2) are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding two new test cases based on the A.5.5.3, but for different scenarious. One (1) is for FR1 PCell + FR2 target SCell and one (2) is for FR2 PCell + FR2 inter-band target SCell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TC covers only the scenario of EN-DC LTE PCell + FR1 PSCell + FR2 target SCell |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.5.3.x, A7.5.3.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of changes 1>>

#### A.7.5.3.x	SCell Activation and deactivation of FR2 unknown SCell with FR1 PCell in non-DRX with L3 reporting during activation

##### A.7.5.3.x.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3.17, when the SCell is unknown in FR2 by the UE at the time of activation. In this test, UE shall perform two sub-tests where two different UL resource locations are configured.

The supported test configurations are shown in table A.7.5.3.x.1-1 below. The test parameters are the same as in clause A.4.5.3.3.1 except those described in the following clause. The general test parameters are given in Table A.7.5.3.x.1-2 and cell-specific test parameters in Table A.7.5.3.x.1-3 below. In this case, OTA related test parameters are shown in table A.7.5.3.x.1-4 below.

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, PCell (Cell 1) in FR1 and SCell (Cell 2) in FR2. Cell 1 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) but is not aware of Cell 2 (SCell). The UE is monitoring the PCell. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. In the measurement control information for Cell 2, it is indicated to the UE that event-triggered reporting with Event A2 and reportOnActivation is used. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the SCell activation.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n (where n mode 20=1), defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3.

In sub-test1, TE shall transmit DCI 0-1 to PCell at slot n+THARQ+7ms NR slot length.  The UE shall be able to send L3 measurements report of the SCell at slot n+THARQ+7ms+0.125ms NR slot length for sub-test 1.
In sub-test2, TE shall transmit DCI 0-1 to PCell at slot n+THARQ+3ms+M-0.125ms NR slot length. The UE shall be able to send L3 measurements report of the SCell at slot n+THARQ+7ms+ M NR slot length for sub-test 2. TE will send TCI activation command after receiving L3 measurement report of the SCell.

The UE shall start reporting CSI in PCell after at least one CSI-RS transmission occasion for channel measurement and reporting after slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Time period T3 starts when a MAC message for deactivation of the SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell and PSCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CSI reporting for SCell is discontinued.

Table A.7.5.3.x.1-1: Supported test configurations for FR2 SCell activation case

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | PCell: 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 3 | PCell: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations |  |

Table A.7.5.3.x.1-2: General test parameters for FR2 SCell activation case with FR1 PSCell

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| T1 | s | 16 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| Hysteresis | dB | 0 |  |
| a2-Threshold | dBm | -100 |  |
| THARQ | ms | k1×NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] depends on UE’s capability |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.7.5.3.x.1-3: Cell specific test parameters for FR2 SCell activation case with FR1 active PCell

| Parameter |  | Unit | Cell 1 |  |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 |  | T3 |  | T1 |  | T2 |  | T3 |
| SSB ARFCN |  |  | Freq1 |  |  |  |  |  | Freq2 |  |  |  |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  | FDD |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  |  | Not Applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP Configuration | Config 1,2,3 |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP Configuration | Config 1,2,3 |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration | Config 1,2,3 |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
| TRS configuration | Config 1,2,3 |  | N/A |  |  |  |  |  | N/A |  |  |  |  |
| TCI state | Config 1,2,3 |  | TCI.State.0 |  |  |  |  |  |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |  |  |  | 10: NRB,c = 52 |  |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  | 40: NRB,c = 106 |  |  |  |  |
| Data RBs allocated | Config 1,2 |  | 52 | 52 |  | 52 |  |  | 52 | 52 |  | 52 |  |
|  | Config 3 |  | 106 | 106 |  | 106 |  |  | 106 | 106 |  | 106 |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  |  |  | SR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  | SR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |  |  | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET Parameters | Config 1 |  | CR.1.1 FDD |  |  |  |  |  | CR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  | CR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |  |  | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET Parameters | Config 1 |  | CCR.1.1 FDD |  |  |  |  |  | CCR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |  |  | CCR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |  |  |  | CCR.2.1 TDD |  |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  | SSB.1 FR1 |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  | SSB.2 FR1 |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Propagation conditions |  |  | N/A Link only, see clause A.3.7A |  |  |  |  | N/A Link only, see clause A.3.7A |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.7.5.3.x.1-4: OTA related test parameters for FR2 SCell activation case with FR1 PCell

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration |  |  | N/A |  |  | According to clause A.3.15.1 |  |  |
| Assumption for UE beams Note 7 |  |  | N/A |  |  | Rough |  |  |
| Note 1 | Config 1,2,3 | dBm/15kHz | Link only, see clause A.3.7A |  |  | -104.7 |  |  |
| Note 1 | Config 1,2,3 | dBm/SCS |  |  |  | -95.7 |  |  |
|  | Config 1,2,3 | dB |  |  |  | -∞ | 7 | 7 |
|  | Config 1,2,3 | dB |  |  |  | -∞ | 7 | 7 |
| SSB_RPNote 2, Note 4 | Config 1,2,3 | dBm/SCS |  |  |  | -∞ | -88.7 | -88.7 |
| IoNote 2, Note 4 | Config 1,2,3 | dBm/95.04 MHz |  |  |  | -66.68 | -58.92 | -58.92 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Void Note 6: Void Note 7: Information about types of UE beam is given in B.2.1.3 and does not imit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |

##### A.7.5.3.x.2	Test Requirements

During T2, the UE shall be able to send a valid L3-RSRP report for the SCell in the configured slots for CSI reporting at slot n+THARQ+7ms+0.125msNR slot length for sub-test 1. For sub-test2, the UE shall be able to send a valid L3-RSRP for the SCell at slot n+THARQ+7ms+MNR slot length . The UE is not required to send L3-RSRP report after slot n+THARQ+3ms+MNR slot length, where M is defined in 8.3.17.

During T2, the UE shall start sending CSI reports for the SCell with non-zero CQI index in the configured slots for CSI reporting no later than slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, where

-		THARQ and TCSI_Reporting are defined in Table A.7.5.3.16.1-2.

-		In this case, TSSB=TSMTC = 20ms and TL1-RSRP,report = 5ms.

-		For sub-test1, Tactivation_time = 7ms + 0.125ms + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay).

- 	For sub-test2, Tactivation_time = 3ms + M  + max (THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay)

-		NR slot length is 0.125ms for this test case.

During T3 the UE shall stop sending CSI reports for SCell no later than slot n+THARQ+3msNR slot length, as defined in clause 8.3.

During T2 interruption of PCell / PSCell during SCell activation shall not happen outside the slot m+1+THARQNR slot length  to m+1+THARQ+3ms+TXNR slot length, as defined in clause 8.3, where TX =20ms.

During T3 interruption of PCell / PSCell during SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3.

The interruption on any activated serving cell shall not be more than the values specified for SA in clause 8.2.2.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

## << End of changes 1>>

## << Start of changes 2>>

#### A.7.5.3.y	SCell Activation and deactivation of FR2 unknown SCell with FR2 PCell in non-DRX with L3 reporting during activation

##### A.7.5.3.y.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3.17, when the SCell is unknown in FR2 by the UE at the time of activation. In this test, UE shall perform two sub-tests where two different UL resource locations are configured.

The supported test configurations are shown in table A.7.5.3.y.1-1 below. The test parameters are the same as in clause A.4.5.3.3.1 except those described in the following clause. The general test parameters are given in Table A.7.5.3.y.1-2 and cell-specific test parameters in Table A.7.5.3.y.1-3 below. In this case, OTA related test parameters are shown in table A.7.5.3.y.1-4 below.

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, PCell (Cell 1) in FR2 and SCell (Cell 2) in FR2. Cell 1 has constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) but is not aware of Cell 2 (SCell). The UE is monitoring the PCell. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. In the measurement control information for Cell 2, it is indicated to the UE that event-triggered reporting with Event A2 and reportOnActivation is used. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the SCell activation.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n (where n mode 20=1), defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3.

In sub-test1, TE shall transmit DCI 0-1 to PCell at slot n+THARQ+7ms NR slot length.  The UE shall be able to send L3 measurements report of the SCell at slot n+THARQ+7ms+0.125ms NR slot length for sub-test 1.
In sub-test2, TE shall transmit DCI 0-1 to PCell at slot n+THARQ+3ms+M-0.125ms NR slot length. The UE shall be able to send L3 measurements report of the SCell at slot n+THARQ+7ms+ M NR slot length for sub-test 2. TE will send TCI activation command after receiving L3 measurement report of the SCell.

The UE shall start reporting CSI in PCell after at least one CSI-RS transmission occasion for channel measurement and reporting after slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Time period T3 starts when a MAC message for deactivation of the SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell and PSCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CSI reporting for SCell is discontinued.

Table A.7.5.3.y.1-1: Supported test configurations for FR2 SCell activation case

| Configuration | Description |
| --- | --- |
| 1 | PCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |

Table A.7.5.3.y.1-2: General test parameters for FR2 SCell activation case with FR2 PCell

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| T1 | s | 16 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| Hysteresis | dB | 0 |  |
| a2-Threshold | dBm | -100 |  |
| THARQ | ms | k1×NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] depends on UE’s capability |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.7.5.3.y.1-3: Cell specific test parameters for FR2 SCell activation case with FR2 active PCell

| Parameter Note 5 |  | Unit | Cell 1 |  |  | Cell2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| SSB ARFCN |  |  | Freq1 |  |  | Freq2 |  |  |
| Duplex mode | Config 1 |  | TDD |  |  | TDD |  |  |
| TDD configuration | Config 1 |  | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| Downlink initial BWP Configuration | Config 1 |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Downlink dedicated BWP Configuration | Config 1 |  | DLBWP.1.1 |  |  | DLBWP.1.1 |  |  |
| Uplink initial BWP configuration | Config 1 |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Uplink dedicated BWP configuration | Config 1 |  | ULBWP.1.1 |  |  | ULBWP.1.1 |  |  |
| TRS configuration | Config 1 |  | TRS.2.1 TDD |  |  | TRS.2.1 TDD |  |  |
| TCI state | Config 1 |  | TCI.State.0 |  |  | TCI.State.0 |  |  |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  |
| Data RBs allocated | Config 1 |  | 66 |  |  | 66 |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET Parameters | Config 1 |  | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET Parameters | Config 1 |  | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| OCNG Patterns | Config 1 |  | OP.1 |  |  | OP.1 |  |  |
| SSB configuration | Config 1 |  | SSB.1 FR2 |  |  | SSB.1 FR2 |  |  |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |  |  | CSI-RS.3.1 TDD |  |  |
| reportConfigType for CSI reporting | Config 1 |  | periodic |  |  | periodic |  |  |
| reportQuantity for CSI reporting | Config 1 |  | cri-RI-PMI-CQI |  |  | cri-RI-PMI-CQI |  |  |
| CSI reporting periodicity | Config 1 | slot | 40 |  |  | 40 |  |  |
| CSI reporting offset | Config 1 | slot | 4 |  |  | 4 |  |  |
| SMTC configuration | Config 1 |  | SMTC.1 |  |  | SMTC.1 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| Propagation conditions |  |  | AWGN |  |  | WGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |  |  |  |

Table A.7.5.3.y.1-4: OTA related test parameters for FR2 SCell activation case with FR2 PCell

Table A.7.5.3.y.1-4: OTA related test parameters for FR2 SCell with FR2 PCell

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration |  |  | According to clause A.3.15.1 |  |  | According to clause A.3.15.1 |  |  |
| Assumption for UE beams Note 7 |  |  | Rough |  |  | Rough |  |  |
| Note 1 | Config 1 | dBm/15kHz | -104.7 |  |  | -104.7 |  |  |
| Note 1 | Config 1 | dBm/SCS | -95.7 |  |  | -95.7 |  |  |
|  | Config 1 | dB | 7 | 7 | 7 | -∞ | 7 | 7 |
|  | Config 1 | dB | 7 | 7 | 7 | -∞ | 7 | 7 |
| SSB_RPNote 2, Note 4 | Config 1 | dBm/SCS | -88.7 | -88.7 | -88.7 | -∞ | -88.7 | -88.7 |
| IoNote 2, Note 4 | Config 1 | dBm/95.04 MHz | -58.92 | -58.92 | -58.92 | -66.68 | -58.92 | -58.92 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Void Note 6: Void Note 7: Information about types of UE beam is given in B.2.1.3 and does not imit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |

##### A.7.5.3.y.2	Test Requirements

During T2, the UE shall be able to send a valid L3-RSRP report for the SCell in the configured slots for CSI reporting at slot n+THARQ+7ms+0.125msNR slot length for sub-test 1. For sub-test2, the UE shall be able to send a valid L3-RSRP for the SCell at slot n+THARQ+7ms+MNR slot length . The UE is not required to send L3-RSRP report after slot n+THARQ+3ms+MNR slot length, where M is defined in 8.3.17.

During T2, the UE shall start sending CSI reports for the SCell with non-zero CQI index in the configured slots for CSI reporting no later than slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, where

-		THARQ and TCSI_Reporting are defined in Table A.7.5.3.16.1-2.

-		In this case, TSSB=TSMTC = 20ms and TL1-RSRP,report = 5ms.

-		For sub-test1, Tactivation_time = 7ms + 0.125ms + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay).

- 	For sub-test2, Tactivation_time = 3ms + M  + max (THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay)

-		NR slot length is 0.125ms for this test case.

During T3 the UE shall stop sending CSI reports for SCell no later than slot n+THARQ+3msNR slot length, as defined in clause 8.3.

During T2 interruption of PCell / PSCell during SCell activation shall not happen outside the slot m+1+THARQNR slot length  to m+1+THARQ+3ms+TXNR slot length, as defined in clause 8.3, where TX =20ms.

During T3 interruption of PCell / PSCell during SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3.

The interruption on any activated serving cell shall not be more than the values specified for SA in clause 8.2.2.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

## << End of changes 2>>