3GPP TSG-RAN WG4 Meeting #112	R4-2411564

Maastricht, Netherland, Aug 19-23, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements maintenance for network energy saving

Agenda item:	5.29.1

Document for:	Discussion

Introduction

At RAN4#111 meeting, the core part of the WI was further discussed but there are still some open issues as listed in the WF [1]. In this paper, we will continue the discussion on the leftover issues.

SSB-less operation

EPRE difference condition

In last meeting, it was clarified the EPRE difference is normalized by SCSs of SSB in reference Cell and A-TRS/P-TRS of SSB-less Cell. But there were different views on if a UE is able to do AGC compensation to filter out the impact from carrier frequency difference which may lead to e.g. 12dB in some band combination. The following options are now listed for further discussion.

Issue 1-1-1: Power difference condition Agreement: Clarify that EPRE difference is based on the EPRE normalized by SCSs of SSB of reference Cell and A-TRS/P-TRS of SSB-less Cell. Only change [9] dB to [12] dB in this meeting. Further discuss whether and how to support larger EPRE difference in next meeting: - Option 1: Clarify that EPRE difference is smaller or equal to [12] dB + ¦20*log (f1/f2)¦ - Margin, where f1 and f2 is the center frequency of reference Cell and SSB-less Cell and the value of Margin is FFS. - Option 2: Further increase [12] dB and the value is FFS - Option 3: Update the requirements with one more P-TRS sample for larger EPRE difference - Other options are no precluded.

In our views, we are defining the requirements allowing all potential UE implementations. But if AGC compensation is not considered, the EPRE difference could be much larger than [12]dB. Some higher value of the maximum EPRE difference is expected otherwise the SSB-less operation scenarios would be quite restricted. Hence either Option 1 or Option 2 can be the starting point for the discussion. Meanwhile we would not like to further relax the SSB-less SCell activation delay with one more P-TRS sample, which would make SSB-less operation less attractive.

A value higher than 12dB is expected for EPRE difference condition hence either Option 1 or Option 2 can be the starting point for the discussion.

Multiple SSB-less SCells activation

When multiple SSB-less SCells are activated via a single SCell activation command, a UE is expected to monitor the P-TRSs on each of the SCells based on the timing of corresponding reference cell. As TRSs are going through the Rx chains same as data reception, the UE is able to receive TRSs simultaneously from multiple SSB-less SCells without interference between each other. Hence the SCell activation is performed separately based on the TRS periodicity on each of the SSB-less SCells.

Meanwhile, we agreed to define the requirements when all to-be-activated SCells are SSB-less on the same band and SCells are contiguous. Once one of the SCells with the shortest periodicity is activated, all the other to-be-activated SCells can be considered as activated. Therefore, the SCell activation delay requirement for activating multiple SSB-less SCells can be defined based on TRS with the shortest periodicity, i.e. Option 1 is preferred.

Issue 1-1-3: Multiple SSB-less SCells activation Agreements: - Also define requirements for following scenario: - When all to-be-activated SCells are SSBless on the same band and SCells are contiguous. - All to-be-activated SCells have same QCL source cell. - P-TRS is used. - Option 1: The multiple SCell activation delay requirements are based on TRS with the shortest periodicity. The detailed requirements can be discussed in the CR. - Option 2: UE activate each to-be-activated SCell based on the TRS on the SCell, and the requirements to be defined accordingly (i.e. single CC delay) - FFS A-TRS based requirements.

The SCell activation delay requirement for activating multiple SSB-less SCells is defined based on P-TRS with the shortest periodicity, i.e. Option 1 is preferred.

Relation to R15 intra-band SSB-less

Another issue from last meeting is the expected UE behaviour when both R18 inter-band SSB-less and R15 intra-band contiguous SSB-less are available. From network point of view, if there is intra-band SCell contiguous to the SSB-less SCell, network can simply assume the SSB-less SCell as activated within 3ms with R15 approach. We do not see it a reasonable scenario to configure both R18 reference cell indication while R15 SSB-less operation is feasible. Hence it is not expected that the to-be-activated SCell is configured with QCL source to both intra-band contiguous and inter-band Cells.

Issue 1-1-5: Relation to R15 intra-band SSB-less Agreement: From RAN4 understanding, for UE supports both R18 inter-band SSB-less and R15 intra-band contiguous SSB-less, - It is not expected that the to-be-activated SCell is configured with QCL source to both intra-band contiguous and inter-band Cells [when the Rel-18 reference cell indication is not configured]. - When R18 reference cell indication is configured, network configure with QCL source to both intra-band contiguous and inter-band Cells, there is no RAN4 agreement on whether Rel-15 or Rel-18 requirement is applied. - The above RAN4 understanding does not intend to trigger any further RAN2 discussion.

It is not expected that the to-be-activated SCell is configured with QCL source to both intra-band contiguous and inter-band Cells.

Intra-band non-contiguous CA

In RAN4#110 meeting, there was a proposal to reuse the SSB-less SCell activation delay requirement in FR1 inter-band CA for FR1 intra-band non-contiguous CA scenario. But there were some concerns on the RTD side condition i.e. RTD within CP.

Issue 1-3-1: Intra-band NCCA Agreement: Regarding FR1 intra-band NCCA with SSB-less operation, further study whether we can reuse or not the SSB-less activation delay requirement specified for FR1 inter-band CA. The study does not impact the completion timeline of the WI.

In previous discussion, RTD has been derived from the TAE and the propagation delay difference from the inter-band carries. We understood 260ns TAE is required only for intra-band contiguous carriers. But for intra-band non-contiguous carriers, we still follow TAE within 3us which is the same as inter-band CA. In addition, in TS38.133 clause 7.6.4.1, it already specifies the UE shall be able to handle up to 3us receive timing difference for intra-band non-contiguous carriers which is also derived from the 3us TAE. Therefore, we believe for FR1 intra-band non-contiguous CA, the SSB-less SCell activation delay requirement defined for FR1 co-located inter-band CA can be applied with the same RTD side condition.

TS 38.133 clause 7.6.4.1:

For intra-band contiguous CA, only co-located deployment is applied. For intra-band non-contiguous NR carrier aggregation, the UE not capable of [intraBandNRCA-NonCollocated-r18] shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in Table 7.6.4-1 below.

Table 7.6.4-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation

| Frequency Range | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 31 |
| FR2-1 | 0.26 |
| Note 1: In the case of different SCS on different CCs, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot. |  |

Observation 1: 260ns TAE is assumed only for intra-band contiguous CA.

Observation 2: For FR1 intra-band non-contiguous CA, UE shall be able to handle up to 3us receive timing difference which is derived from 3us TAE.

For FR1 intra-band non-contiguous CA, reuse the SSB-less SCell activation delay requirement defined for FR1 co-located inter-band CA with the same RTD side condition i.e. RTD within CP.

NES-specific CHO

Measurement behaviour

In current specification, the measurement time delay for NES-based conditional handover is defined based on if the DCI 2-X command comes before or after TEvent_DU + Tidentify_intra_with_index.

TS 38.133:

##### 6.1.4.2.2	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

For conditional inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4 or clause 9.3.9.

For NES-based conditional intra-frequency handover:

- If UE successfully decodes DCI 2-9 command earlier than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equal to Tidentify_intra_with_index or Tidentify_intra_without_index

- If UE successfully decodes DCI 2-9 command later than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until UE successfully decodes DCI 2-9 command.

For the case DCI 2-X command comes after TEvent_DU + Tidentify_intra_with_index, the assumption behind existing NES-based condition handover delay is that the CHO condition remains fulfilled/met from the end of Tevent_DU until UE successfully decodes DCI 2-X command. However, the CHO condition may vary from time to time and the NES-based CHO condition may or may not be met when receiving the DCI 2-9 command. As the arrival of DCI 2-9 command is uncertain relevant to the end of Tevent_DU, the possibility of channel variation needs to be considered when defining the NES-based conditional handover delay.

Observation 3: Existing NES-based condition handover delay assumes that the CHO condition remains fulfilled/met from the end of Tevent_DU until UE successfully decodes DCI 2-9 command.

Observation 4: For the case DCI 2-9 command comes after TEvent_DU + Tidentify_intra_with_index, the NES-based CHO condition may or may not be met when receiving the DCI 2-9 command.

The NES-based conditional handover delay shall be defined considering the possible channel variation when DCI 2-9 command comes after TEvent_DU + Tidentify_intra_with_index.

Based on the latest RAN1/2 agreement, the NES-based CHO shall be executed only if the condition of NES-based CHO is met when receiving the DCI 2-9 command. As shown in Fig.1-(a), the measurement time delay equals to the time from the end of Tevent_DU until UE successfully decodes DCI 2-9 command. This is the same as the formulation in existing spec.

Otherwise, if the CHO condition has been met but is not fulfilled when receiving the DCI 2-9 command, the UE shall not execute NES-based conditional handover immediately. Instead, the UE keeps the measurement until the condition exists after DCI 2-9 command which will trigger the NES-based conditional handover. Afterwards, the UE will measure till the NES-based CHO is met i.e. another measurement time delay is needed. As the cell has been detected, the measurement time delay is equal to TSSB_measurement_period_intra or TSSB_measurement_period_inter..

The NES-based CHO shall be executed only if the condition of NES-based CHO is met when receiving the DCI 2-X command after TEvent_DU + Tidentify_intra_with_index.

      - NES-based CHO condition is met when receiving DCI 2-9

      - NES-based CHO condition is not met when receiving DCI 2-9

Fig.1 NES-based handover delay

While there was consensus understanding on the expected UE behaviour, there was no conclusion how to formulate the CHO delay. Based on the discussion last meeting, if the condition of NES-based CHO is not met when receiving the DCI 2-X command as in Fig.1(b), we may consider defining Tevent_DU as the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists after receiving DCI 2-9 which will trigger the NES-based conditional handover. And the measurement time delay Tmeasure equals to TSSB_measurement_period_intra or TSSB_measurement_period_inter. We provide a tentative CR to clarify the conditions for measurement time delay in R4-2407741[2].

If the condition of NES-based CHO is not met when receiving the DCI 2-9 command after TEvent_DU + Tidentify_intra_with_index.,

- Tevent_DU is defined as the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until “a condition exists at the measurement reference point after receiving DCI 2-9 which will trigger the NES-based conditional handover”, and

- Tmeasure equals to TSSB_measurement_period_intra or TSSB_measurement_period_inter.

Conclusion

In this paper we have made the following proposals and observations:

- A value higher than 12dB is expected for EPRE difference condition hence either Option 1 or Option 2 can be the starting point for the discussion.

- The SCell activation delay requirement for activating multiple SSB-less SCells is defined based on P-TRS with the shortest periodicity, i.e. Option 1 is preferred.

- It is not expected that the to-be-activated SCell is configured with QCL source to both intra-band contiguous and inter-band Cells.

Observation 1: 260ns TAE is assumed only for intra-band contiguous CA.

Observation 2: For FR1 intra-band non-contiguous CA, UE shall be able to handle up to 3us receive timing difference which is derived from 3us TAE.

For FR1 intra-band non-contiguous CA, reuse the SSB-less SCell activation delay requirement defined for FR1 co-located inter-band CA with the same RTD side condition i.e. RTD within CP.

Observation 3: Existing NES-based condition handover delay assumes that the CHO condition remains fulfilled/met from the end of Tevent_DU until UE successfully decodes DCI 2-9 command.

Observation 4: For the case DCI 2-9 command comes after TEvent_DU + Tidentify_intra_with_index, the NES-based CHO condition may or may not be met when receiving the DCI 2-9 command.

The NES-based conditional handover delay shall be defined considering the possible channel variation when DCI 2-9 command comes after TEvent_DU + Tidentify_intra_with_index.

The NES-based CHO shall be executed only if the condition of NES-based CHO is met when receiving the DCI 2-X command after TEvent_DU + Tidentify_intra_with_index.

If the condition of NES-based CHO is not met when receiving the DCI 2-9 command after TEvent_DU + Tidentify_intra_with_index.,

- Tevent_DU is defined as the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until “a condition exists at the measurement reference point after receiving DCI 2-9 which will trigger the NES-based conditional handover”, and

- Tmeasure equals to TSSB_measurement_period_intra or TSSB_measurement_period_inter.

References

[1] R4-2410269, WF on RRM requirements for NR network energy saving, Huawei

[2] R4-2411565, 38133CR on handover delay for NES-based handover, Nokia, Nokia Shanghai Bell