3GPP TSG-RAN WG4 Meeting #112	R4-2411568

Maastricht, Netherlands, Aug 19 – 23, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for NR FR1/FR2 and EN-DC Phase 4

Agenda item:	8.1.2

Document for:	Discussion

Introduction

At RAN#103 meeting, a new R19 WI was approved to further investigate the UE RF enhancements for NR FR1 and FR2 Phase 4 [1].  This WI includes the study on several working areas wherein the 6Rx for handheld and FWA UE may have impacts on the RRM requirements.

### 4.1	Objective of Core part WI

…

6Rx for handheld and FWA UE

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this contribution, we are discussing the potential impact on RRM requirements from 6Rx capable UEs.

Discussion

According to the WID, one objective is to specify the requirements to support SRS antenna switching for 6Rx capable UEs. In RAN4#111, an LS R4-2406718 was sent to RAN1 to start the work on enabling 3T6R and 4T6R SRS antenna switching. It is understood 3T6R and 4T6R SRS antenna switching has not been specified in RAN1.

Observation #1: In RAN1, 3T6R and 4T6R SRS antenna switching has not been specified.

In R17, the RRM requirements for SRS antenna switching were specified in the RRM enhancement WI. During the discussion, the SRS antenna switching before R17 i.e. up to 4Rx was considered hence SRS resource(s) is assumed being configured within the last 6 symbols of a slot as cited below.

8.2.2.2.16	Interruptions at NR SRS antenna port switching

The requirements in this clause are applicable to SRS antenna port switching on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot. For interruption caused by SRS antenna port switching, the victim cell is based on the entry number of the band indicated by txSwitchImpactToRx and/or txSwitchWithAnotherBand regardless of per-FR MG capability. An UL interruption is allowed on any of the serving cells as indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells as indicated in txSwitchImpactToRx.

Observation #2: In R17, the interruption requirements for SRS antenna switching were specified considering up to 4Rx and SRS resource(s) is assumed being configured within the last 6 symbols of a slot.

When 6Rx UEs are considered in R19, it is not clear if SRS resources are always configured within the last 6 symbols of  a slot which depends on the possible SRS switching patterns to be defined in RAN1. If SRS resources are not able to be fit within the last 6 symbols of a slot, we shall define new interruption requirements for SRS antenna switching with 6Rx UEs. As the antenna switching patterns are up to RAN1 discussion, we shall wait for RAN1 conclusion on the 6Rx relevant SRS antenna switching before defining the RRM requirements.

Observation #3: In R19, it is not clear SRS resources are always configured within the last 6 symbols of a slot pending on RAN1 discussion.

Proposal 1: To wait for RAN1 conclusion on the 6Rx relevant SRS antenna switching before defining the RRM requirements in RAN4.

Conclusion

In this paper we have made the following proposals and observations related to the NR FR1/FR2 EN-DC phase 4:

Observation #1: In RAN1, 3T6R and 4T6R SRS antenna switching has not been specified.

Observation #2: In R17, the interruption requirements for SRS antenna switching were specified considering up to 4Rx and SRS resource(s) is assumed being configured within the last 6 symbols of a slot.

Observation #3: In R19, it is not clear SRS resources are always configured within the last 6 symbols of a slot pending on RAN1 discussion.

Proposal 1: To wait for RAN1 conclusion on the 6Rx relevant SRS antenna switching before defining the RRM requirements in RAN4.

References

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei, RAN#103