3GPP TSG-RAN WG4 Meeting #112	R4-2411569

Maastricht, Netherlands, Aug 19 – 23, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for intra-band non-collocated scenarios Phase2

Agenda item:	8.5.3

Document for:	Discussion

Introduction

In R18, the support of intra-band non-collocated EN-DC/NR-CA deployment was discussed and the requirements for 2-layer case i.e. Type 2 UE were specified. At RAN#103 meeting, a R19 WI for intra-band non-collocated EN-DC/NR-CA phase 2 is approved [1] to further specify the requirements for Type 4 UE. The objectives are cited as below:

The core part of the work item includes:

- Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed.

  - RF power imbalance requirements of up to 25dB

  - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption.

  - Work is limited to EN-DC/NR-CA for bands 42, n77/n78

- Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

There have been some discussions in RF session on this topic and the agreements were captured in [2]. In this contribution, we will start discussing the potential impact on RRM requirements.

Discussion

Table 1 shows the different UE architecture types being discussed in RF session. It is understood Type 4 is different from Type 2 in that a Type 4 capable UE is able to support up to 4-layer DL MIMO per CC while Type 2 UE can support up to 2-layer. In the WID, it also indicates the RF power imbalance requirements for Type 4 are the same as Type 2.

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

Table 1: UE architectures

From RRM point of view, MRTD/MTTD requirements are specified for intra-band and inter-band CA respectively. In particular, a tight MRTD/MTTD requirement is defined for intra-band CA due to collocation assumption. When intra-band non-collocated scenarios are considered in R18, the MRTD/MTTD follows the same requirements as in legacy inter-band CA. We can see the MRTD/MTTD requirements are impacted by the collocation or non-collocation deployment but have nothing to do with the number of MIMO layers. For a Type 4 capable UE in non-collocated scenarios, the MRTD/MTTD requirements are the same as that for Type 2 UE.

Observation #1: MRTD/MTTD requirements have nothing to do with the number of MIMO layers.

Proposal #1: For a Type 4 capable UE in non-collocated scenarios, the MRTD/MTTD requirements are the same as that for Type 2 UE.

Meanwhile, it is being discussed in RF session on the potential BS signaling to enable the transition between single CC, Type 1, Type 2 and Type 4 operation as below. Similar as the UE type transition between Type 1 and Type 2 in R18, we believe the MRTD/MTTD and the other RRM requirements need to be adapted based on the eventual UE type in operation which is indicated by the BS signaling. We need wait for RF conclusion on the UE type transition before changing the RRM requirements accordingly.

< Issue 2-2-3: New BS Signalling to switch between Type 4a/4b and Type 1(collocated) > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting. < Issue 2-2-4: New BS Signalling to switch between Type 4a/4b and Type 2 > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting. < Issue 2-2-5: When to inform RAN2 the demand on new UE capability(s) and new BS signalling(s) > Way Forward: - Given the discussions on UE capabilities and network signalling, it is not hurry to send this. < Issue 2-2-6: UE behavior between Type 1, Type 2 and Type 4a/4b with new BS signalling > Way Forward: - First, conclude discussions of whether to add new UE capability(s) and BS signalling for Type 4. - And then, continue further discussion on BS signalling/UE behavior in the next meeting.

Proposal #2: The MRTD/MTTD and the other RRM requirements need to be adapted based on the eventual UE type in operation which is indicated by the BS signaling.

Proposal #3: To wait for RF conclusion on the UE type transition before specifying the RRM requirements.

Conclusion

In this paper we have made the following proposals and observations related to the intra-band non-collocated scenarios for Type 4 capable UEs:

Observation #1: MRTD/MTTD requirements have nothing to do with the number of MIMO layers.

Proposal #1: For a Type 4 capable UE in non-collocated scenarios, the MRTD/MTTD requirements are the same as that for Type 2 UE.

Proposal #2: The MRTD/MTTD and the other RRM requirements need to be adapted based on the eventual UE type in operation which is indicated by the BS signaling.

Proposal #3: To wait for RF conclusion on the UE type transition before specifying the RRM requirements.

References

[1] RP-240838 New WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), Apple, RAN#103

[2] R4-2410686 WF on Rel-19 non-collocated UE RF requirements, KDDI, RAN4#111