3GPP TSG-RAN WG4 Meeting #112	R4- 2411570

Maastricht, Netherlands, Aug 19 – 23, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for R19 network energy saving

Agenda item:	8.21.2

Document for:	Discussion

Introduction

At RAN#102 meeting, a Rel-19 WID [1] was approved to further enhance network energy savings for NR. The objectives of the work item are the following:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3]

  - Triggering method by uplink wake-up-signal using an existing signal/channel.

  - Wake-up-signal configuration provisioning to UE

    - Note: No modification of SSB will be discussed under this objective

  - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary.

  - Checkpoint for normative work in RAN#105

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial

    - This study is to be done in 2Q’2024 only

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

- Specify the corresponding core requirements, for the above features [RAN4].

In the WID, RAN4 study is expected at least for the support of on-demand SSB and adaptation of common signal/ channel transmissions. In this contribution, we are discussing the scenarios on these two topics based on RAN1/2 progress and present our views on the potential RAN4 impact.

RRM requirements for on-demand SSB

Scenarios for on-demand SSB

The on-demand SSB scenarios have been extensively discussed in RAN1. Based on the latest agreement, the on-demand SSB SCell operation triggered by gNB is supported. Moreover, RAN1 have identified 2 cases and several scenarios and reached the following agreements:

Agreement (RAN1#116): Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission Agreement (RAN1#116bis) For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

RAN1 first considers two cases based on if always-on SSB is present in the SCell in addition to the on-demand SSBs. As shown in Fig.1, in Case #1 UE assumes there is no any other SSB transmission before OD-SSB is triggered for the UE, while in Case #2 UE is aware of some periodic always-on SSB transmission besides OD-SSB. In RAN1 there seems to be different understanding on Case #1 regarding “no always-on SSB” with respect to the SSB-less operation. In our view, this is independent from the legacy SSB-less operation. For instance, the UE may be configured with SSB-less operation on the SCell e.g. in collocated inter-band CA but OD-SSB may be triggered to help with the side conditions. Alternatively, the UE may not be configured with SSB-less operation on the SCell e.g. in non-collocated inter-band CA where SSB-less operation is considered as infeasible. In any case, the triggering of OD-SSB is up to network and Case #1 includes all the cases where no always-on SSB is assumed.

Fig. 1 Cases and Scenarios for OD-SSB transmission

Observation #1:  Case #1 is independent from SSB-less operation and includes all the cases where no always-on SSB is assumed.

On top of the two cases, RAN1 further identified different scenarios based on when the OD-SSB transmission is triggered. Based on the RAN1 agreements, the Scenario#2, Scenario#2A and Scenario#3A are depicted in Fig.1, where:

- Scenario#2: the on-demand SSB transmission is triggered by gNB at the time instance T1 (i.e. at the time instance when UE receives SCell activation via RRC message) or during the time period between T1 and T2.

- Scenario#2A: the on-demand SSB transmission is triggered by gNB at the time instance T2 (i.e. at the time instance when UE receives SCell activation via MAC CE command).

- Scenario#3A: the on-demand SSB transmission is triggered by gNB after the time instance T2 until SCell activation procedure is completed.

- Scenario#3B: the on-demand SSB transmission is triggered by gNB at the time instance T3 and after the time instance T3, i.e. when and after the SCell activation procedure is completed.

According to RAN1, the Scenario#2 and Scenario#2A has been agreed for both Case #1 and Case #2 while the other scenarios are still FFS.  In RAN4 we also believe the two scenarios are most sensible and we can prioritize the OD-SSB discussion in Scenario#2 and Scenario#2A for both Case #1 and Case #2.

Proposal 1: Prioritize the OD-SSB discussion in Scenario#2 and Scenario#2A for both Case #1 and Case #2.

Starting from the RAN1 agreed cases/scenarios, we will discuss the potential RRM impacts in the four combination scenarios respectively.

Case #1 & Scenario #2:

In scenario #2, the OD-SSB transmission is triggered when or after UE receives SCell configuration and before receiving SCell activation command. Especially if the SCell configuration or addition indicates the SCell status to active i.e. direct SCell activation, the OD-SSB transmission needs to be triggered immediately. We should at least specify the OD-SSB based direct SCell activation delay when OD-SSB is triggered at the time instance T1 i.e. by SCell addition message.

It is also possible the SCell configuration message does not indicate cell activation, and the OD-SSB may be triggered sometime after SCell configuration. While this is up to network implementation, the OD-SSBs may help UE with the cell detection and SSB measurements before SCell is activated so that SCell activation delay may be reduced. In any case, when OD-SSB is triggered after SCell addition, the OD-SSB based measurement before receiving SCell activation command shall be considered to reduce the SCell activation delay. But this case can be down-prioritized comparing to triggering OD-SSB at the time instance in scenario 2.

Proposal 2: RAN4 shall specify the OD-SSB based direct SCell activation delay when OD-SSB is triggered at the time instance T1 i.e. by SCell addition message in Case #1.

Proposal 3: When OD-SSB is triggered after SCell addition in Case 1, the OD-SSB based measurement before receiving SCell activation command shall be considered to reduce the SCell activation delay.

Proposal 4: In scenario #2, RAN4 can prioritize the case where on-demand SSB is triggered at the time instance T1 over the case where OD-SSB is triggered after T1.

Case #1 & Scenario #2A:

In this combination scenario, the OD-SSB is not triggered until UE receives the SCell activation command. As there is no SSB to be measured before SCell activation command, the SCell to be activated is always considered as unknown. The unknown SCell activation delay shall be specified based on OD-SSB. In order to ensure the activation of the cell, the OD-SSB once being triggered shall be transmitted at least until the end of the SCell activation procedure.

Proposal 5: RAN4 shall specify the OD-SSB based unknown SCell activation delay when OD-SSB is triggered at the time instance T2 i.e. by SCell activation command in Case #1.

Proposal 6: When OD-SSB is triggered at the time instance T2 in Case #1, the OD-SSB shall be transmitted at least until the end of the SCell activation procedure.

In legacy measurement behavior, a UE is required to perform the SSB based measurements on a deactivated SCell following the measurement requirements in TS 38.133 clause 9.2.5. But in these two combinations of Case #1, there is no always-on SSBs while the SCell is in deactivated state. The UE is not expected to perform any measurement on the deactivated SCell before OD-SSB is triggered hence existing measurement requirements for a deactivated SCell does not apply.

Observation #2: In legacy behavior, a UE is required to perform the SSB based measurements on a deactivated SCell following the measurement requirements in TS 38.133 clause 9.2.5.

Proposal 7: In Case#1 scenarios, a UE is not expected to perform any measurement on the deactivated SCell before OD-SSB is triggered hence existing measurement requirements for a deactivated SCell does not apply.

Case #2 & Scenario #2:

In Case #2, it is different from Case #1 that there is always-on SSB available in addition to OD-SSB. When OD-SSB in triggered by SCell configuration in Scenario #2, the UE may use both always-on SSB and OD-SSB for activating the SCell. We understood always-on SSB is likely transmitted with a longer periodicity for network energy saving, the SCell is preferred to be activated based on OD-SSB to speed up the SCell activation. RAN4 shall specify the direct SCell activation delay based on OD-SSB when OD-SSB is triggered at the time instance T1 i.e. by SCell addition message in Case #2.

When OD-SSB is triggered after SCell addition message, the UE is assumed to perform SSB-based measurements on the always-on SSBs before OD-SSB is triggered. But after OD-SSB is triggered, both always-on SSBs and OD-SSBs are available for the UE measurements. It needs to be discussed how to handle the measurement on the always-on SSBs and OD-SSBs when OD-SSB is triggered after SCell addition message in Case#2.

Proposal 8: RAN4 shall specify the direct SCell activation delay based on OD-SSB when OD-SSB is triggered at the time instance T1 i.e. by SCell addition message in Case #2.

Proposal 9: When OD-SSB is triggered after SCell addition message in Case#2, RAN4 to discuss how to handle the measurement on the always-on SSBs and OD-SSBs.

Case #2 & Scenario #2A:

In this combination scenario, the OD-SSB is triggered in addition to always-on SSBs when UE receives the SCell activation command. Similarly as the OD-SSB is assumed with a shorter periodicity, we prefer specifying the SCell activation delay based on OD-SSB when OD-SSB is triggered at the time instance T2 i.e. by SCell activation command in Case #2.

In addition, the UE may have been performing measurements on SCell based on always-on SSBs and the SCell may be considered as known if it has transmitted a valid measurement reporting. Hence the UE measurement based on always-on SSBs in the SCell shall be considered to reduce the SCell activation delay.

Proposal 10: RAN4 shall specify the SCell activation delay based on OD-SSB when OD-SSB is triggered at the time instance T2 i.e. by SCell activation command in Case #2.

Proposal 11: When OD-SSB is triggered at the time instance T2 in Case #2, the UE measurement based on always-on SSBs in the SCell shall be considered to reduce the SCell activation delay.

In Case #2, there is at least always-on SSB and potentially additional OD-SSB before receiving the SCell activation command. That is, while the SCell is in deactivated state, there may be both always-on SSB and OD-SSB available. We shall discuss the UE measurement behavior and specify the measurement requirements considering both always-on SSB and OD-SSB in Case #2.

Proposal 12: In Case #2 scenarios, RAN4 shall discuss the UE measurement behavior and specify the measurement requirements considering both always-on SSB and OD-SSB.

RRM requirements for SSB adaptation

The Rel-19 WID [1] specifies for the objective “adaptation of common signal/channel transmissions”, among other enhancements, SSB adaptation in time domain, e.g. adapting periodicity. RAN1 has discussed this enhancement in their last meetings. At RAN1#117 meeting, the following agreement was reached:

Agreement (RAN1#117) For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g., two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

In our understanding the agreed Option 1 is related to adaptation of SSB burst periodicity for a configured single SSB pattern by defining a set of SSB burst periodicities for it. Option 2 based on definition of two SSB configurations, that can be both active, is not precluded, with the open issue that the two SSB configurations may differ in the SSB burst periodicity and suitable other parameters, if any, the latter requiring further study in RAN1.. For Option 2 it is unclear, whether this related to single active SSB pattern or two concurrent active SSB patterns. As RAN1 is discussing the change of SSB burst periodicity before paging to improve the UE synchronization when being paged, it is assumed that both Options 1 and 2 are related to alternating between SSB burst periodicities or between two SSB configurations with different SSB burst periodicity.

Observation #3: Options 1 and 2 are discussing the alternating between 2 SSB periodicities or SSB configurations.

Observation #4: Only one SSB pattern is active at a time, and this may have different SSB burst periodicity.

Until RAN1 discuss Option 2 in view of applicable scenarios, e.g. if this is related to CD-SSB and NCD-SSB configurations, RAN4 should focus on Option 1. Moreover, RAN1 should also discuss applicable scenarios for Option 1 triggering SSB adaptation in greater detail.

Proposal 13: For RRM impact, RAN4 shall focus on Option 1 as starting point for SSB adaptation.

Based on the above RAN1 agreements, RAN4 can start to investigate RRM impacts for SSB adaptation in the time domain, i.e. usage of different SSB burst periodicities.

Proposal 14: RAN4 to start investigation of RRM impacts for SSB adaptation in time domain for a single SSB pattern, due to usage of different SSB burst periodicities.

Although RAN1 has not yet got agreement on the scenarios for SSB adaptation, RAN 1 has defined the scope of scenarios for SSB adaptation at their RAN1#116bis meeting:

Agreement (RAN1#116bis) Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode Note: Impact to idle/inactive UEs shall be minimized

SSB adaptation in time domain is being studied in RAN1 for Rel-19 NES capable UEs in a PCell and/or an SCell in connected mode.

- In a PCell scenario,, adaptation of SSB burst periodicity can improve energy efficiency. For instance, SSB transmissions can be reduced in PCell for higher NES gain (more sleep opportunities can be achieved with fewer SSB transmissions), e.g., longer SSB periodicity in the SCell when procedures related to SCell addition are not needed.

- In an SCell scenario, SSB transmissions can be more frequent to facilitate UE measurements, e.g., for the activation of the SCell. In this case, a trade-off between benefiting the UE with faster measurements vs. benefiting the network for NES gain may be considered.

There against, RAN1 has not got agreement to investigate SSB adaptation in time domain for UE’s in idle / inactive mode so far.

Observation #5: RAN1 has been discussing SSB adaptation only for UEs in connected mode so far.

Proposal 15: RAN4 to prioritize the discussion for UEs in connected mode.

Regarding the range of SSB periodicity, we observe that 160ms is the longest periodicity in current spec. RAN1 is proposing not extending the existing SSB periodicity. Thus, RAN4 should maintain the existing specified range for SSB burst periodicity.

Proposal 16: RAN4 to maintain the specified range for SSB burst periodicity i.e. 5ms to 160ms.

The UE needs to process the signaling message, depending on the signaling method, which will be defined by RAN1 and RAN2 (e.g., based on DCI, MAC-CE or RRC). Hence some processing time needs to be defined, until the UE is ready to receive SSBs with the new adjusted burst periodicity.

Proposal 17: RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity.

Furthermore, due to the change in SSB burst periodicity, SSB measurements consisting of a number of measurement samples will be faster or slower for shorter or longer SSB burst periodicity, respectively. Thus, there is an impact on ongoing measurements due to SSB burst periodicity adaptation, e.g. whether or not to combine measurement samples stemming from different SSB burst periodicities to generate a L1/L3 measurement. RAN4 needs to consider this L1/L3 measurement impact.

Proposal 18: RAN4 to consider the L1/L3 measurement impact due to a change of the SSB burst periodicity.

In addition, RAN4 needs to investigate whether a transition period is needed when changing SSB burst periodicity, where no or relaxed RRM requirements are specified. Assuming such transition period can be a starting point for defining the RRM impact.

Proposal 19: RAN4 to investigate whether a transition period with no or relaxed RRM requirements can serve as starting point when changing SSB burst periodicity for defining the RRM impact.

Conclusion

In this contribution, we discussed the scenarios for the support of on-demand SSB and SSB adaptation of common signal/ channel transmissions, being objectives of the Rel-19 WID [1] to further enhance network energy savings for NR, based on RAN1/2 progress and presented our views on the potential RAN4 impact.

Following observations are made.

Observation #1:  Case #1 is independent from SSB-less operation and includes all the cases where no always-on SSB is assumed.

Observation #2: In legacy behavior, a UE is required to perform the SSB based measurements on a deactivated SCell following the measurement requirements in TS 38.133 clause 9.2.5.

Observation #3: Options 1 and 2 are discussing the alternating between 2 SSB periodicities or SSB configurations.

Observation #4: Only one SSB pattern is active at a time, and this may have different SSB burst periodicity.

Observation #5: RAN1 has been discussing SSB adaptation only for UEs in connected mode so far.

And following proposals are made.

Proposal 1: Prioritize the OD-SSB discussion in Scenario#2 and Scenario#2A for both Case #1 and Case #2.

Proposal 2: RAN4 shall specify the OD-SSB based direct SCell activation delay when OD-SSB is triggered at the time instance T1 i.e. by SCell addition message in Case #1.

Proposal 3: When OD-SSB is triggered after SCell addition in Case 1, the OD-SSB based measurement before receiving SCell activation command shall be considered to reduce the SCell activation delay.

Proposal 4: In scenario #2, RAN4 can prioritize the case where on-demand SSB is triggered at the time instance T1 over the case where OD-SSB is triggered after T1.

Proposal 5: RAN4 shall specify the OD-SSB based unknown SCell activation delay when OD-SSB is triggered at the time instance T2 i.e. by SCell activation command in Case #1.

Proposal 6: When OD-SSB is triggered at the time instance T2 in Case #1, the OD-SSB shall be transmitted at least until the end of the SCell activation procedure.

Proposal 7: In Case#1 scenarios, a UE is not expected to perform any measurement on the deactivated SCell before OD-SSB is triggered hence existing measurement requirements for a deactivated SCell does not apply.

Proposal 8: RAN4 shall specify the direct SCell activation delay based on OD-SSB when OD-SSB is triggered at the time instance T1 i.e. by SCell addition message in Case #2.

Proposal 9: When OD-SSB is triggered after SCell addition message in Case#2, RAN4 to discuss how to handle the measurement on the always-on SSBs and OD-SSBs.

Proposal 10: RAN4 shall specify the SCell activation delay based on OD-SSB when OD-SSB is triggered at the time instance T2 i.e. by SCell activation command in Case #2.

Proposal 11: When OD-SSB is triggered at the time instance T2 in Case #2, the UE measurement based on always-on SSBs in the SCell shall be considered to reduce the SCell activation delay.

Proposal 12: In Case #2 scenarios, RAN4 shall discuss the UE measurement behavior and specify the measurement requirements considering both always-on SSB and OD-SSB.

Proposal 13: For RRM impact, RAN4 shall focus on Option 1 as starting point.

Proposal 14: RAN4 to start investigation of RRM impacts for SSB adaptation in time domain for a single SSB pattern, due to usage of different SSB burst periodicities.

Proposal 15: RAN4 to prioritize the discussion for UEs in connected mode.

Proposal 16: RAN4 to maintain the specified range for SSB burst periodicity i.e. 5ms to 160ms.

Proposal 17: RAN4 to consider the impact of processing time for decoding the updated SSB burst periodicity.

Proposal 18: RAN4 to consider the L1/L3 measurement impact due to a change of the SSB burst periodicity.

Proposal 19: RAN4 to investigate whether a transition period with no or relaxed RRM requirements can serve as starting point when changing SSB burst periodicity for defining the RRM impact.

References

[1] RP-234065, New WID: Enhancements of network energy savings for NR, Ericsson