3GPP TSG RAN WG4 #112	R4-2411571

Maastricht, Netherlands, Aug 19 – 23, 2024

Agenda item:		8.19.3

Source:	Nokia

Title:	RRM requirements for evolution of NR duplex operation

Document for:		Approval

# Introduction

At RAN#103 meeting, a new WID [1] was approved regarding evolution of NR duplex operation. The following objectives were listed indicating the potential impacts on the RRM core requirements in RAN4:

- Specify applicable RRM core requirements for co-channel CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

From August meeting, we will start the discussion on RRM requirements in RAN4. This contribution will provide our initial views on the potential RRM impacts due to SBFD operation and co-channel CLI handling mechanisms based on the progress in other WGs.

# RRM requirements for SBFD operation

SBFD is being specified in Rel-19 as a new mode of operation in unpaired spectrum where separate set of RBs within a TDD carrier are used for UL and DL transmission simultaneously. The intention is to allow more UL transmission opportunities compared to the typical DL-heavy TDD configurations used in today’s TDD deployments. This bring benefits in terms of UL coverage and UL latency especially in wide area macro networks where many UEs are in power-limited conditions in the UL direction.

A study item on SBFD took place in Rel-18 in RAN1 and RAN4 working groups, while the specification work started in RAN1 in February 2024, followed by BS core requirement specification work that started in RAN4#110-bis in April 2024.

Before discussing the RRM requirements, it is worth noting the following high-level principles on SBFD operation as agreed by RAN1 in the SI phase (TR 38.858):

- Simultaneous transmission and reception (on non-overlapping set of RBs) is assumed at the gNB side, while half-duplex operation is assumed the UE side. RAN1 is currently specifying UE behavior for handling collision and priorities between UL and DL channels. - Only one UL subband, while one or at most two DL subbands within a TDD carrier. These are commonly referred as DU (or UD) or DUD operation, respectively – see Figure 1. - A SBFD subband is defined as 1 RB or more consecutive RBs which are used for the same transmission direction (either UL or DL). - UL transmissions take place within UL subband only, while DL receptions take place within DL subband(s) only. - SBFD-aware UEs is used to refer to UEs supporting (at least some) SBFD-related features to be standardized in Rel-19. - As baseline, it is assumed that both time and frequency locations of subbands for SBFD operation are known to SBFD-aware UEs, while this information is not available for legacy (pre Rel-19) UEs. - Legacy UEs are not aware of SBFD operation and thus behave in similar manner as in today’s TDD system. The Rel-18 study concluded that non-SBFD aware UEs, including legacy UEs, and SBFD aware UEs can coexist in cells with SBFD operation at gNB side from RAN1 specification point of view.

Figure 1 illustrates an example arrangement of UL and DL resources for both DU/UD (left) and DUD (right) SBFD operation. SBFD symbols refer to those symbols (or slots) where both a UL and DL subband(s) are simultaneously available, and non-SBFD symbols refer to (legacy) unidirectional symbols. Different time-domain sequences were discussed by RAN1 in the TR 38.858, e.g. DXXXU, XXXXX, or XXXXU where D, U, X refers to downlink, uplink and SBFD slots, respectively. One or more RBs may be used as guard band between the UL and DL subbands to facilitate rejection of cross-link interference; Similarly, a guard time or period of one or more OFDM symbols may take place for the transition between different symbol/slot types.

Figure 1: SBFD resources in time and frequency domain

## Coexistence between SBFD-aware UE and non-SBFD UEs

As noted earlier, RAN1 has concluded that non-SBFD aware UEs, e.g. legacy UEs, and SBFD aware UEs can coexist in cells with SBFD operation at gNB side at least from RAN1 specification point of view. According to current RAN1 agreements, SBFD subbands are configured only in DL and/or flexible symbols indicated by TDD-UL-DL-ConfigCommon – for instance, a TDD pattern DDDDU provided via TDD-UL-DL-ConfigCommon may be complemented with additional (new) signalling that converts it to DXXXU or XXXXU for SBFD-aware UEs, where X refers to a slot where SBFD subbands are present. Legacy UEs are only aware of the legacy DDDDU TDD pattern thus the the gNB (e.g. by implementation) would need to ensure that UL transmission and DL receptions are confined within the corresponding subbands in the X slots.

As the legacy non-SBFD aware UEs would see the slot with SBFD as legacy downlink slot, there is no impact on the UE behaviour. From RRM requirements point of view, the legacy UEs shall perform and operate according to the legacy RRM requirements, despite being served by a gNB operating with SBFD.

Proposal 1: The legacy UEs shall perform and operate according to the legacy RRM requirements, despite being served by a gNB operating with SBFD.

## UL/DL Collision handling and SSB reception

The fact that the gNB has full-duplex capabilities (i.e. can transmit and receive at the same time), while the UE remains half-duplex, introduces new challenges in terms of handling UL-DL transmissions at the UE-side, as compared to existing UE operation in legacy TDD bands. This topic has been discussed extensively in RAN1 under the term Collision handling. For unpaired TDD spectrum, several rules were specified since Rel-15 primarily targeting dynamic or flexible TDD operation where some OFDM symbols are assumed as flexible by the higher layers and can be more dynamically switched between UL and DL at the lower layers. Those rules are primarily defined in TS 38.213 and generally differentiate between a dynamically scheduled signal (e.g. UL or DL transmission triggered by a downlink control information (DCI) reception) and semi-static or higher-layer configured signal (UL or DL transmission not triggered by DCI, e.g. PRACH, SRS or periodic configured-grant PUSCH). More specifically, the following cases have been studied in RAN1:

Agreement: For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

Some agreements have been reached by RAN1 for these different collision cases which are noted in Table 2 below. Overall, RAN1 has agreed to keep same behaviour as legacy, that is, a dynamically scheduled signal is prioritized over a semi-static (RRC-configured signal), and overlapping of semi-static UL and DL signals, or dynamic UL and DL signals is not expected to happen at the UE side (thus is to be avoided at the gNB side).

Table 2: RAN1 agreed operation for UL-DL collision cases in SBFD operation

| Collision case | RAN1 agreed operation |
| --- | --- |
| Case 1: Dynamic DL vs semi-static UL | Same as legacy (if link direction is not supported): UL transmission is cancelled if cancellation timeline is met. FFS on dynamically scheduled DL reception with repetition |
| Case 2: Dynamic UL vs semi-static DL | Same as legacy (if link direction is not supported): UE does not receive DL channel/signal. FFS on dynamically scheduled UL transmission with repetition |
| Case 3: Semi-static DL vs Semi-static UL | Same as legacy (if link direction is not supported): An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol |
| Case 4: Dynamic UL vs dynamic DL | Same as legacy (if link direction is not supported): it is considered as an error case if a dynamically scheduled DL reception in DL subband(s) without repetitions overlaps with a dynamically scheduled UL transmission in UL subband without repetitions. FFS DL reception with repetitions and/or UL transmission with repetitions |
| Case 5: Dynamic/semi-static UL vs. SSB reception | Same as legacy: Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. |

For RRM point of view, Case 5 is the most relevant one as it handles the priority between SSB reception vs potential UL transmissions. According to RAN1 discussions, SSB resources can be configured or take place in both SBFD and DL-only symbols. For SSBs taking place in SBFD symbols, the SSB resources shall be within the DL subbands; note this is not expected to be a major challenge considering that the UL subband is expected to occupy only ~20% of the total bandwidth, while remaining ~80% of the resources are expected to be available for the DL direction. One of the motivations for not restricting SBFD resources to only full-DL symbols or slots is to allow SBFD operation as much as possible in time-domain (up to the gNB based on the configured TDD pattern in TDD-UL-DL-ConfigCommon).

Observation #1: From RAN1 point of view, SSB resources can be configured in both SBFD and DL-only symbols.

Observation #2: For SSB resources on SBFD symbols, the SSB resources shall be contiguous and contained within the DL resource blocks. Puncturing of the SSB resources is not expected since SBFD operation is mainly used with large carrier bandwidths where up to 80% of the frequency resources are used for downlink direction.

For the detailed handling of SSB receptions in SSB slots or symbols, the following was agreed in RAN1#117:

Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

The main agreement specifies that SSB reception is prioritized over UL transmissions. In our understanding, this would mean the UE would ignore or drop an UL transmission if it overlaps with a SSB that needs to be measured by the UE. Two additional possible behaviours are still under discussion as captured in the FFS above resulting in a total of three different possibilities for handling SSB reception as illustrated in Figure 2:

- First, RAN1 is considering whether a slot consisting of SSB symbols is considered as a full DL slot. This would mean that the UL subband is not expected to be active in that slot, thus the SSB may be placed on any part of the bandwidth without any SBFD-related restriction.

- Alternatively, RAN1 is also considering whether SSB symbols configured on SBFD symbols are still to be regarded as “SBFD symbols” but only DL receptions within the DL PRBs are allowed, i.e. no UL transmissions on the UL PRBs are expected by the UE.

Figure 2. Possibilities for handling of SSB receptions based on RAN1 agreements and RAN1’s FFS

For the SSB on the left-most side in Figure 3, the SSB reception is prioritized over UL transmissions, although in principle the UE may still transmit in UL in case the UE does not need to measure the SSB. This for example means that the gNB may schedule the UE for UL transmission for SSBs that occur outside the configured SMTC or not included in SSB-toMeasure. Nevertheless, in practice, the UE may still measure SSBs outside the SMTC window, which means that the scheduling of UL transmissions overlapping with SSB transmissions may not be used much in practice given the overall uncertainty on the UE SSB measurement behaviour.

In contrast, the behaviours according to the RAN1 FFS (centre and right-most case in Figure 3) provide a more restrictive but deterministic UE behaviour since no UL transmissions are expected on SSB symbols. The centre case in Figure 2 would furthermore allow the SSB to be placed in on any part of the bandwidth as SBFD operation is not assumed in that slot. In any case, for all the RAN1-agreed cases (incl. FFS), the SSB reception is always prioritized irrespective if the symbol is considered as a full DL-slot or SBFD-slot. For SBFD-aware UE, the SSB-based measurement requirement will not be impacted due to SBFD configuration.

Observation #3: For all the RAN1-agreed cases (incl. FFS), SSB is always prioritized and not expected to be impacted by SBFD operation.

Proposal 2: For SBFD-aware UE, the SSB-based measurement requirement will not be impacted due to SBFD configuration.

## Idle/Inactive mode aspects

RAN1 recently agreed to support random access for SBFD-aware UEs in RRC_IDLE and RRC_INACTIVE mode:

Agreement Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

A pre-requisite for the above to work is that the SBFD time and frequency configuration needs to be signalled to the UEs as part of the SIB messages. Regarding the SSB reception, we do not expect any impact for RRC_IDLE/RRC_INACTIVE UEs. Therefore, the existing cell detection requirements can still be valid for SBFD:

Proposal 3: Existing RRM requirements for idle/inactive mode can still apply because SSB transmissions are not impacted by SBFD configuration.

# RRM requirements for co-channel CLI handling

According to the WID, one of the objectives is to specify RRM requirements for co-channel CLI handling mechanisms. At RAN#104, the WID was further improved elaborating the details on CLI handling and the relevant WGs as below. In RRM, we should at least specify the UE requirements for the L1 based UE-to-UE CLI measurement and reporting.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement Note: Without dedicated optimization for dynamic/flexible TDD.

Proposal 4: In RAN4 RRM, we should at least specify the UE requirements for the L1 based UE-to-UE CLI measurement and reporting.

CLI measurement scenarios

In R16, the CLI measurements have been introduced to mitigate the cross-link interference for dynamic TDD. As cited below, the victim UE may be configured with SRS-RSRP and/or CLI-RSSI measurements to measure the interference on its DL reception due to the UL transmission from aggressor UE.

TS 38.300 ## 17.2 Cross-Link Interference Management When different TDD DL/UL patterns are used between neighbouring cells, UL transmission in one cell may interfere with DL reception in another cell: this is referred to as Cross Link Interference (CLI). To mitigate CLI, gNBs can exchange and coordinate their intended TDD DL-UL configurations over Xn and F1 interfaces; and the victim UEs can be configured to perform CLI measurements. There are two types of CLI measurements: - SRS-RSRP measurement in which the UE measures SRS-RSRP over SRS resources of aggressor UE(s); - CLI-RSSI measurement in which the UE measures the total received power observed over RSSI resources. Layer 3 filtering applies to CLI measurement results and both event triggered and periodic reporting are supported

.

Observation #4: In R16, the CLI measurements was introduced for dynamic TDD to mitigate the interference on DL reception due to UL transmission from aggressor UE.

In R19, the CLI measurements are mostly discussed in SBFD i.e. the victim UE is configured with SBFD. During RAN1#116, four methods were identified for how to measure the co-channel UE-to-UE CLI in SBFD as depicted in Figure 3:

Agreement For SBFD aware UEs, CLI measurements is performed within the active DL BWP and the following can be considered - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Method#3: UE measures RSSI within UL subband - Method#4: UE measures RSSI within guard band, if guard band exists Note: If DL subband, UL subband or guard band is outside the active DL BWP, the above methods does not apply. Note: Method#4 does not imply that guard band is explicitly configured.

Figure 3. Methods for UE-to-UE CLI measurements in SBFD

The main difference between Method#1, Method#2 and Method#3 is whether the UE measures the CLI over the DL subband(s) or over the UL subband of the victim UE where SBFD is configured. In other words, the UE measures the leakage interference in Method#1 and the fundamental UL signal in Method#2 and Method#3. Method#4 performs measurements over the guard bands. We can see in R19 the CLI measurements may be configured over DL subbands and/or UL subband of the victim UE, which is different from the legacy CLI measurements.

As the SBFD introduces new scenarios to cross link interference, we propose discussing the CLI measurements in SBFD without considering dynamic TDD in R19. While the WID indicates “without dedicated optimization for dynamic/flexible TDD”, we can further revisit if and which of the CLI measurement methods in SBFD can be applied to dynamic TDD when the conclusion is ready.

Observation #5: In R19, the CLI measurements are being discussed in SBFD and may be configured over DL subbands and/or UL subband of the victim UE, which is different from legacy CLI measurements.Proposal 5: To start discussing the CLI measurements in SBFD without considering dynamic TDD in R19.

While the discussion on the CLI measurement methods is still ongoing in RAN1, we would foresee some potential impact on the measurement requirements. For example, depending on if CLI-RSSI is measured over DL subbands, UL subbands or the guardbands, the side conditions for the CLI measurements, scheduling restriction and the measurement performance may be different. To simplify the RAN4 discussion, we suggest prioritizing specifying the measurement requirements for the CLI measurements in SBFD on UL subbands i.e. Method#2 and #3 as these are the fundamental interference to be mitigated in SBFD.

Observation #6: For the different CLI measurements methods in SBFD, the CLI measurement requirements may be different in RAN4.

Proposal 6: RAN4 to prioritize specifying the measurement requirements for the CLI measurements in SBFD on UL subbands i.e. Method#2 and #3.

Current UE-to-UE CLI measurements for FR2 UEs assume that the Rx beam used for the measurements is the same as the latest beam used for the reception of the PDCCH/PDSCH (as cited below). There is an on-going discussion on RAN1 about allowing the gNB to provide an indication of the QCL-type D assumptions as part of the L1 UE-to-UE CLI measurement configuration. Therefore, the Rel-16 assumptions for Rx beam used for CLI measurement may not be applicable. It is expected that the CSI measurement resource configuration is used to indicate the TCI state of the CLI measurement resources. In principle, it is applicable to CLI-RSSI and SRS-RSRP measurement resources. Given the differences with existing Rel-16 assumptions on Rx beam in FR2, RAN4 should discuss the CLI measurement requirements considering the potential different Rx beams based on RAN1 conclusion.

TS 38.133 clause 9.7.1: For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET.

Observation #7: The Rel-16 assumptions for Rx beam used for CLI measurement in FR2 may not be applicable provided the latest RAN1 discussion.

Proposal 7: RAN4 to discuss the CLI measurement requirement considering the potential different Rx beams based on RAN1 conclusion.

CLI measurement periods

In R16, the CLI measurement periods for SRS-RSRP and CLI-RSSI are defined in the way of L3 measurements for dynamic TDD. For instance, 3 samples are assumed for deriving the SRS-RSRP measurement period and 1 sample RSSI is assumed for the CLI-RSSI measurement period.

TS 38.133 clause 9.7 Table 9.7.2.5-1 Measurement period TSRS_RSRP_measurement_period Configuration; TSRS_measurement_period (ms). No DRX; Max(60, 3 X TSRS). DRX cycle ≤ 320ms; Max(60, Ceil(1.5 X 3) X max(TSRS, TDRX)). DRX cycle > 320ms; 3 X TDRX. Note: TSRS is SRS measurement periodicity configured SRS-PeriodicityAndOffset, and TDRX is the DRX cycle length..

Observation #8: In R16, the CLI measurement periods are defined for SRS-RSRP and CLI-RSSI in the way of L3 measurements for dynamic TDD.

According to the RAN1 discussion in R19 duplex evolution, it was agreed to define L1-SRS-RSRP and L1-CLI-RSSI for CLI measurements. These reporting quantities are different from legacy CLI measurement hence existing CLI measurement requirements do not apply. RAN4 should specify the measurement periods for L1-SRS-RSRP and L1-CLI-RSSI measurements and the same requirements shall apply irrespective of different CLI measurement methods.

Observation #9: In R19 duplex discussion, new reporting quantities L1-SRS-RSRP and L1-CLI-RSSI are agreed for CLI measurements hence legacy CLI measurement requirements do not apply.

Proposal 8: RAN4 should specify the measurement periods for L1-SRS-RSRP and L1-CLI-RSSI measurements irrespective of different CLI measurement methods.

As the WID has indicated L1 based UE-to-UE CLI measurement and reporting is based on existing CSI framework, the CLI measurement requirement can be specified following L1-RSRP measurement and same assumption can be applied.

Proposal 9: The CLI measurement requirement can be specified following L1-RSRP measurement and same assumption can be applied.

CLI measurement reporting

With the updated WID, it informs the L1 based CLI measurement and reporting are based on existing CSI framework and at least aperiodic reporting is supported. This means the CLI measurement reporting will be handled similar as L1-RSRP reporting, hence at least the L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the L1 based CLI measurement reporting.

Proposal 10: At least the L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the L1 based CLI measurement reporting.

CLI subband for CLI-RSSI measurements

Due to the frequency dependency of the CLI, RAN1 is currently discussing whether finer reporting granularity rather than wideband reporting should be supported. This approach allows that multiple CLI-RSSI values are reported for a single CLI-RSSI measurement resource. This is similar to the already existing subband reporting supported in CSI report for CQI and PMI. However, now the interest is on L1-CLI-RSSI measurements. Moreover, the subband sizes for CQI and PMI derivation might be too larger for the CLI-RSSI measurements and, therefore, new subband sizes might need to be introduced. If this feature is supported by RAN1, RAN4 should define new requirements for the mini-band CLI-RSSI reporting accordingly.

Proposal 11: RAN4 should define new requirements for the mini-band CLI-RSSI reporting if this feature is supported by RAN1.

Measurement/scheduling restriction

As discussed in CLI measurement scenarios, different CLI measurement methods are being discussed in RAN1 based on if the CLI measurements are configured on DL subbands or UL subband. In particular, if the UE is configured with CLI measurements on UL subband i.e Method#2 and Method#3, no further DL reference signals are expected to be measured other than the CLI measurement resources hence there will not be measurement restriction. Otherwise, the measurement restriction shall be defined.

Similarly, when the UE is performing CLI measurements on serving cell, the scheduling availability also depends on if the CLI measurements are configured on DL subbands or UL subband i.e. the respective CLI measurement methods. RAN4 shall wait for RAN1 conclusion on the CLI measurement methods before discussing the measurement and scheduling restriction.

Proposal 12: RAN4 shall wait for RAN1 conclusion on the CLI measurement methods before discussing the measurement and scheduling restriction.

# Conclusion

In this contribution, our views on the RRM requirements are presented. The following proposals were listed.

Proposal 1: The legacy UEs shall perform and operate according to the legacy RRM requirements, despite being served by a gNB operating with SBFD.

Observation #1: From RAN1 point of view, SSB resources can be configured in both SBFD and DL-only symbols.

Observation #2: For SSB resources on SBFD symbols, the SSB resources shall be contiguous and contained within the DL resource blocks. Puncturing of the SSB resources is not expected since SBFD operation is mainly used with large carrier bandwidths where up to 80% of the frequency resources are used for downlink direction.

Observation #3: For all the RAN1-agreed cases (incl. FFS), SSB is always prioritized and not expected to be impacted by SBFD operation.

Proposal 2: For SBFD-aware UE, the SSB-based measurement requirement will not be impacted due to SBFD configuration.

Proposal 3: Existing RRM requirements for idle/inactive mode can still apply because SSB transmissions are not impacted by SBFD configuration.

Proposal 4: In RAN4 RRM, we should at least specify the UE requirements for the L1 based UE-to-UE CLI measurement and reporting.

Observation #4: In R16, the CLI measurements was introduced for dynamic TDD to mitigate the interference on DL reception due to UL transmission from aggressor UE.

Observation #5: In R19, the CLI measurements are being discussed in SBFD and may be configured over DL subbands and/or UL subband of the victim UE, which is different from legacy CLI measurements.

Proposal 5: To start discussing the CLI measurements in SBFD without considering dynamic TDD in R19.

Observation #6: For the different CLI measurements methods in SBFD,  the CLI measurement requirements may be different in RAN4.

Proposal 6: RAN4 to prioritize specifying the measurement requirements for the CLI measurements in SBFD on UL subbands of victim UEs i.e. Method#2 and #3.

Observation #7: The Rel-16 assumptions for Rx beam used for CLI measurement in FR2 may not be applicable provided the latest RAN1 discussion.

Proposal 7: RAN4 to discuss the CLI measurement requirement considering the potential different Rx beams based on RAN1 conclusion.

Observation #8: In R16, the CLI measurement periods are defined for SRS-RSRP and CLI-RSSI in the way of L3 measurements for dynamic TDD.

Observation #9: In R19 duplex discussion, new reporting quantities L1-SRS-RSRP and L1-CLI-RSSI are agreed for CLI measurements hence legacy CLI measurement requirements do not apply.

Proposal 8: RAN4 should specify the measurement periods for L1-SRS-RSRP and L1-CLI-RSSI measurements irrespective of different CLI measurement methods.

Proposal 9: The CLI measurement requirement can be specified following L1-RSRP measurement and same assumption can be applied.

Proposal 10: At least the L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the L1 based CLI measurement reporting.

Proposal 11: RAN4 should define new requirements for the mini-band CLI-RSSI reporting if this feature is supported by RAN1.

Proposal 12: RAN4 shall wait for RAN1 conclusion on the CLI measurement methods before discussing the measurement and scheduling restriction.

# References

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei