3GPP TSG-RAN WG4 Meeting #112	R4-2411572
Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.12.2.3

Source:	CAICT, SAICT

Title:	On FR1 dynamic MIMO OTA

Document for:	Approval

1	Introduction

The FR1 dynamic MIMO OTA test methodology was discussed at the last two meetings. RAN4 reached some preliminary agreements as captured in the WF [1].

In this paper, we share our further views on FR1 dynamic MIMO OTA test methodology.

2	Discussion

Environmental condition is an essential aspect of MIMO OTA test methodology. The debate on this issue went on for many years. A good review was provided in R4-2404142 [2] and quoted as below:

It is highly desirable to harmonize the FR1 dynamic MIMO OTA methodologies in industry to avoid the fragmentation experienced with LTE MIMO and the differences in test configurations selected by 3GPP [2], [3] and CTIA [4]. The LS from GSMA [5] provided a good overview of the concerns from European operators based on empirical data with the LTE MIMO OTA test plan in CTIA utilizing the SIR-controlled approach. In summary, the main concern was that “it was not possible to distinguish between good and bad performing devices due to low variance of the results (the deviations were within measurement uncertainty)” while the 3GPP UE noise limited approach “saw at least 10 dB difference between devices” and one “could observe improvements of devices from 2013 to 2016.” The LS from CTIA [6] on the other hand highlighted that “an SIR-based test is more appropriate as antenna efficiency will not play a significant role in MIMO performance when operating in real deployment scenarios.” The main concerns raised over years for these two test plans that resulted in the industry fragmentation is that the CTIA test plan is not able to rank device performance and that the 3GPP test plan is primarily ranking device performance based on antenna efficiency. Reference: - TR 37.977, Verification of radiated multi-antenna reception performance of User Equipment (UE) - TS 37.544, Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) Over The Air (OTA) performance; Conformance testing - CTIA Certification™: “CTIA Certification Test Plan for Wireless Device Over-the-Air Performance, CTIA 01.40: Test Methodology, MIMO, Static Channel Model, Multi-Probe Anechoic Chamber”, latest active version available at: https://ctiacertification.org/test-plans/ - R5-176033, MIMO OTA Testing, GSMA TSG, 3GPP TSG RAN WG5 Meeting #77, November 2017 - R5-176863, LS Response to GSMA TSG30_007 (LS Regarding MIMO OTA), CTIA MOSG, 3GPP TSG RAN WG5 Meeting #77, November 2017

In 3GPP, noise-limited condition has been adopted in LTE MIMO OTA test methodology [3] and NR static MIMO OTA test methodology [4]. In the WID [5], it was agreed to adopt noise-limited environmental condition as the baseline. In the last meeting, the agreements were captured in the WF [1]:

Issue 4-2-4: Environmental condition for RAN4 FR1 dynamic MIMO OTA Agreements - RAN4 adopt the full path loss model from TR 38.901 for the UE Noise-limited environment condition. - With understanding of noise-limited is baseline for dynamic MIMO OTA in RAN4, the study of Interference-limited environment condition for RAN4 FR1 Dynamic MIMO OTA is not precluded.

It would be helpful and valuable to investigate both of the environmental conditions on their effectiveness of distinguishing between different UE MIMO OTA performance. A measurement campaign can be performed to compare the two environmental conditions. CAICT could provide test resource to support such comparison measurement campaign.

Proposal 1: Perform a measurement campaign on both environmental conditions, to compare their effectiveness of distinguishing between different UE MIMO OTA performance.

Regarding UE Performance metric, the following agreements were summarized in the WF [1]:

Issue 4-3-1: UE measurement campaign to decide final performance metric Agreements - To study/verify the UE performance metrics, RAN4 could use the UE throughput data collected during the measurement campaign Issue 4-3-3: UE orientations for dynamic MIMO OTA testing Agreements - RAN4 further study and check whether the four device orientations DMP, DML (both left and right tilt), and DMSU and four rotations (0°, 90°, 180°, 270°) per orientation can be used.

To further study and check the candidate device orientations and rotations, measurement data from different UEs collected during the measurement campaign can be used.

Proposal 2: Collect measurement throughput data from different UEs during the measurement campaign to study and check the candidate device orientations (DMP, DML, DMSU) and rotations (0°, 90°, 180°, 270°).

3	Conclusion

In this paper, we share our views on FR1 dynamic MIMO OTA test methodology.

Proposal 1: Perform a measurement campaign on both environmental conditions, to compare their effectiveness of distinguishing between different UE MIMO OTA performance.

Proposal 2: Collect measurement throughput data from different UEs during the measurement campaign to study and check the candidate device orientations (DMP, DML, DMSU) and rotations (0°, 90°, 180°, 270°).

References

- R4-2409931, WF for [111][338] TRP_TRS_MIMO_OTA, RAN4#111, Apr 2024.

- R4-2404142, On FR1 dynamic MIMO OTA methodology, Keysight, RAN4#110-bis, Apr 2024.

- TR 37.977, Verification of radiated multi-antenna reception performance of User Equipment (UE)

- TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)

- RP‑240841, New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, vivo, CAICT, RAN meeting #103, Mar 2024.