3GPP TSG- RAN WG4 Meeting #112	R4-2411586

# Maastricht, Netherlands, 19th – 23rd August 2024

Title:	Discussion on performance requirements for RTK/PPP positioning for NR

Source:	Rohde & Schwarz

Agenda item:	5.35

Document for:	Approval

# Introduction

In RAN4#110bis meeting, [1] was presented and shortly discussed. The discussion paper made the following proposals:

Proposal 1: Add missing RTK/PPP performance requirement as part of a new TEI for Rel-19.

Proposal 2: Define RTK/PPP performance requirement only for NR and skip LTE.

Proposal 3: Add the requirements in a new section in TS 38.171.

Proposal 4: Complete the specification work by 3GPP RAN4#113 (Nov 2024)

Following comments were received in the online sessions, as documented in the RRM chairman notes:

E///: We are not against. Try to understand the overall impact by this proposal. The test configuration needs to be updated, which needs more work. Integrity support is in the scope or not.

Nokia: Share view as E/// that the work scope needs to be clarified in more details. posSIB is not mentioned in the proposals although it is included in the title. Further discuss whether for NR only.

QC: Need more time to evaluate the proposals in detail. TEI is expected to complete in one quarter, which may not be sufficient for this work. Maybe by a new WI, as we have a chance to consider new items in Dec in general. Will consider further.

MTK: In general support the intention. Have similar question on the details.

The current contribution intends to elaborate more the idea of RTK performance requirement and provide feedback to the above stated questions.

# Discussion

As depicted is section 1, the proposal in [1] was mostly received positively in RAN4. However, more details about the scope of work and the impact on the RAN4 specification and work were asked. In the following we try to provide more details about the intended requirements, which hopefully help to answer the questions and move forward.

## 2.1 Type of requirements

RTK (Real Time Kinematic) / PPP (Precise Point Positioning): The main purpose is to test the higher accuracy positioning performance of the UE. RTK & PPP are two different techniques (which can also be combined), which the UE might use for the high precision position, based on its capabilities. Both techniques need a similar A-GNSS environment and target similarly increased accuracy. Based on this, we need only one set of high accuracy requirements and test cases definition for RTK/PPP.

posSIB: The purpose of the posSIB is to broadcast the additional assistance data in the network, which is required for RTK/PPP positioning. However, there are other ways how to provide this information to the UE, using the Control plane or User plane. Since the scope of RAN4 in to test the positioning performance, the way how the assistance data is provided, should be left to RAN5 during the test case definition. We could test the posSIB reception performance as part of a protocol test, which is not in the scope of RAN4 minimum requirement. Thus, we propose not to explicitly consider posSIB in the requirements definition.

Integrity: This is an additional functionality, which intends to check the integrity of the calculated position. It is currently not clear, whether the integrity is related to the UE performance requirement or not. In any case, our intention is to focus first on the initial RTK/PPP performance requirements and to potentially consider additional features at a second stage (if at all).

## 2.2 RAT

LTE/NR: Are both supported in terms of WI in [3]. However, given the load of RAN4, and the market relevance, our intention is to define requirements (at least) for NR.

## 2.3 Impact on RAN4 spec

From RAN4 perspective, only improved accuracy requirements and test case configuration in TS 38.171 are required.  A draft proposal to illustrate the intended changes has been attached to this contribution 
(38171-i01_RTK_PPP_Proposal_R&S.docx, [2]).

In summary the proposed changes are:

- To keep the specification clearly structured we would propose a new section (7) for A-GNSS for RTK/PPP

- TCs are proposed similarly to section 6, with exception of

  - Fine time assistance, as a UE already possesses fine time, when it starts RTK/PPP

  - Multi-path scenario, as a very challenging in combination with an RTK/PPP receiver / testing environment

- In general, the powers are increased compared to the legacy case, as required for the improved performance.

- In case of mixed GNSS, the total number of satellites is not kept constant, but results from the sum of satellites per GNSS. This is closer to real deployments and mostly required for the increased RTK/PPP accuracy.

- The proposed RTK/PPP accuracy requirements have been defined according to measurements with real RTK/PPP receivers, including reasonable margin to accommodate a broader variety of UE performance.

- All the deltas to the legacy requirements are highlighted in the attached proposal [2] in yellow.

## 2.4 Impact on RAN4 work

In our view, if we concentrate only on the definition of the above performance requirements, the work is manageable within a TEI (Rel-19). R&S could volunteer to propose and drive the TEI.

The Time Plan for a TEI could be:

- RAN#105: Propose and agree the TEI (if required to be submitted in the plenary).

- RAN4#112bis: Draft proposal of requirements and limited simulations.

- RAN4#113: Confirmation of simulation results and approval of the requirements.

The TC list (TS38.171), as drafted also in [2}, would be as following:

- 7	A-GNSS minimum performance requirements for high accuracy positioning 
(UE supports RTK/PPP)

  - 7.0	Introduction

    - 7.1	Sensitivity

  - 7.1.1	Coarse time assistance

    - 7.1.1.1	Minimum requirements (Coarse time assistance)

  - 7.2	Nominal accuracy

    - 7.2.1	Minimum requirements (nominal accuracy)

  - 7.3	Dynamic range

    - 7.3.1	Minimum requirements (dynamic range)

  - 7.4	Moving scenario and periodic update

    - 7.4.1	Minimum requirements (moving scenario and periodic update)

We think, that the conclusion on the requirements is feasible without many controversial discussions, since UEs supporting RTK/PPP are already performing well, and the proposed requirements allow sufficiently margin for several implementations, still ensuring a reasonable advantage towards the legacy A-GNSS.

However, if more aspects and more simulations need to be considered, a new Rel-19 WI would probably be the best way. In that case, it would be more preferred that UE / Infra vendors propose and own the WI (certainly R&S would be happy to contribute and support).

## 2.5 Impact on RAN5

RAN5 would need to adopt requirements from TS 38.171 to test cases in TS 37.571-1. The topics expected currently to be handled in RAN5 would be:

- Definition of the channel for assistance data over posSIB, or C-plane, or U-plane.

- SV IDs and respective assistance data.

- Test case procedure in terms of point of time and means how to trigger and receive the RTK/PPP improved positioning. This needs discussion since even UEs supporting RTK/PPP, mostly execute first the legacy A-GNSS procedure, before updating the results with more accurate RTK/PPP measurements.

Other topics are not precluded.

# Proposal for the WF

Based on section 2, following high-level proposals are done:

Proposal 1: RAN4 to introduce RTK/PPP performance requirements for NR in TS 38.171.

Proposal 2: Do it as a part of a Rel-19 TEI (2 RAN4 meeting cycles), or introduce a Rel-19 WI for more comprehensive requirements and work.

Proposal 3: Consider the drafted requirements and test cases attached in R4-2405618, as an initial reference for facilitating the specification and simulation work.

# References

[1] R4-2405618: New release 19 TEI on performance requirements for RTK/PPP positioning and posSIB for NR, Rohde & Schwarz

[2] 38171-i01_RTK_PPP_Proposal_R&S.docx (attached)

[3]

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 750263 | LCS_LTE_acc_enh-Perf | WI introducing RTK/PPP and posSIB for LTE |
| 830177 | NR_pos | Extending posSIB for NR, adding RTK-PPP |