3GPP TSG RAN WG4 Meeting #112		R4-2411587

Maastricht, Netherlands, 19th – 23rd August 2024

Source: 		Rohde & Schwarz

Title: 			Discussion on test setup for AI/ML based beam management

Agenda Item: 	8.17.2

Document for: 	Approval

# Introduction

The Rel-18 Study Item on Artificial Intelligence (AI) / Machine Learning (ML) for NR Air Interface is described in [1], with an objective to study the framework for each of the concluded use cases: CSI feedback enhancement, beam management and positioning accuracy enhancements for different scenarios. The testability aspects for beam management are of specific importance, where two use cases are considered in [2]:

- Spatial domain beam prediction

- Temporal domain beam prediction

In this contribution we focus on the spatial beam prediction (as illustrated in Figure 1) without loss of generality.

Figure 1 - Beam management use case. UE prediction for spatial beams.

In the spatial beam prediction case, the Base Station (BS) sweeps through its beam-codebook, transmitting only a subset of the available beams (Set B as in [2]), while the UE is expected to predict (all) other beams (Set A as in [2]) based on a given KPI (e.g. L1-RSRP measurement).

During RAN4#111 meeting, companies started the discussion on the potential OTA test methodology, which should be deployed for testing this use case.

In this contribution, we discuss several potential scenarios, the resulting channel model which should be emulated OTA, and the feasibility of the available test methodologies to support such scenarios.

Furthermore, we suggest a way forward on how to approach the high complexity of such test scenarios and the next steps to be discussed.

# Discussion

## Assessment of the current FR2 NR MIMO OTA system layout for AI/ML BM usage

One of the discussed topics in RAN4#111 was the reuse of the methodology established for FR2 NR MIMO OTA performance testing. This is the 3D MPAC test method as referenced and described in [3]. The concrete system layout was the result of several decisions made during the respective Study Item:

The channel models considered for testing in [3] were restricted to two channel models, CDL-A InO and CDL-C UMi. Nevertheless, the Work Item to define performance requirements as captured in [4] ‎focused only on one channel model, CDL-C UMi.

The CDL channel model tables in [3] are the fast fading radio channel realizations generated from the reference in [5] applying several simplifications, among others:

  - Fixed coupling of ZoD/ZoA/AoD/AoA rays within one cluster.

  - Fixed Delay Spread, set for 28GHz.

  - Fixed Angular Spread, set for 28GHz.

PAS similarity percentage (PSP) was defined as the metric defined to characterize the FR2 channel model within the quiet zone, which assumes a 4x4 DUT antenna array for the calculation of the Power Angular Spectrum (PAS).

On top of the channel model tables, filtering effect is applied to account for the beamforming characteristics of the Base Station.

  - Single panel with 8x16 antenna array is considered:

The gNB beam pattern including the assumptions for gNB antenna for definitions and symbols of subclause 7.3 of TR38.901 for FR1 and FR2 are summarized in Table 7.2-7.

Table 7.2-7: BS Antenna Parameters

| Parameter description | Symbol | Parameter value |  |  |
| --- | --- | --- | --- | --- |
|  |  | FR1 ≤2.5GHz | FR1 >2.5GHz | FR2 |
| Antenna panels in vertical dimension | Mg | 1 | 1 | 1 |
| Antenna panels in horizontal dimension | Ng | 1 | 1 | 1 |
| Elements per panel in vertical dimension | Me | 4 | 8 | 8 |
| Elements per panel in horizontal dimension | Ne | 8 | 8 | 16 |
| Number of polarizations per panel | P | 2 | 2 | 2 |
| Element spacing in horizontal dimension () | dH | 0.5 | 0.5 | 0.5 |
| Element spacing in vertical dimension () | dV | 0.5 | 0.5 | 0.5 |

  - The codebook is composed of a total of 128 beams:

-	For FR2: A code book of 128 fixed beams is constructed to a grid of eight elevation angles from –25 to +25 with ~7.1 step size and 16 azimuth angles from –60 to +60 with 8 step size；

  - Only the strongest beam towards the strongest cluster is simulated:

For NR FR2 MIMO OTA, 1 strongest transmitting beam is generated from BS, the direction of this beam towards the strongest cluster of each FR2 channel model. In detail, the directions in CDL-A InO and CDL-C UMi models are (-4.0°, 93.6°) and (-12.0°, 100.7°), respectively.

The following excerpt from [3] shows the fundamental definition of the 3D MPAC system layout, including the exact probe locations which results in a sector of 55º by 10º:

### 6.2.3	3D Multi-Probe Anechoic Chamber (MPAC) for FR2

The 3D MPAC test method is the reference methodology for FR2 NR MIMO OTA testing. By arranging an array of antennas around the Equipment Under Test (EUT), a spatial distribution of angles of arrival in the 3D MPAC system may be simulated to expose the EUT to a near field environment that appears to have originated from a complex multipath far field environment.

As illustrated schematically in Figure 6.2.3-1, signals propagate from the base station/communication tester to the EUT through a simulated multipath environment known as a spatial channel model, where appropriate channel impairments such as Doppler and fading are applied to each path prior to injecting all of the directional signals into the chamber simultaneously through the probe array. The resulting field distribution in the test zone is then integrated by the EUT antenna(s) and processed by the receiver(s) just as it would do so in any non-simulated multipath environment. The 3D MPAC system with 6 dual-polarized probes (illustrated with black dots in Figure 6.2.3-1) placed on a sector with minimum radius of 0.75m from the centre of the test zone is permitted for NR FR2 MIMO OTA testing.

Figure 6.2.3-1: 3D MPAC system layout for NR FR2 MIMO OTA testing

The exact probe locations with respect to the OTA test system coordinate system are tabulated in Table 6.2.3-1.

Table 6.2.3-1. FR2 3D MPAC Probe Locations in OTA test system coordinate system

| Probe Number | Theta [deg] | Phi [deg] |
| --- | --- | --- |
| 1 | 0.0 | 0.0 |
| 2 | 11.2 | 116.7 |
| 3 | 20.6 | -104.3 |
| 4 | 20.6 | 104.3 |
| 5 | 20.6 | 75.7 |
| 6 | 30.0 | 90.0 |

The 3D MPAC probes in Table 6.2.3-1 can be implemented using conventional millimetre-wave probes as well as IFF-based probes as long as the same probe configuration and same number of probes is used.

Observation 1: The current FR2 NR MIMO OTA system layout is limited to 6 probes, within a 55º by 10º sector, to simulate the resulting AoA/ZoA after applying the BS filtering (i.e. strongest beam towards the strongest cluster) for one, from two possible simplified CDL channel modes (i.e. CDL-A InO and CDL-C UMi).

Observation 2: The near field distribution in the test zone, simulating a complex multipath far field environment for the FR2 NR MIMO OTA system, is the result of combining the simultaneous signals transmitted by all probes.

On the other hand, as agreed during last meeting in the WF [6], the number of Tx beams to be emulated for AI/ML based beam management testing was bounded to:

- maximum number of set B Tx beams that test system should be able to emulate: [8-16]

- maximum number of set A Tx beams that test system should be able to emulate: [64-128]

Therefore, it is clear that the current FR2 NR MIMO OTA system layout, is neither capable to simulate such number of different BS beams (at least 8 for set B, and 64 for set A), nor implement the multi-path fading CDL model conditions at the same time.

Observation 3: The current FR2 NR MIMO OTA system defined in TR 38.827 cannot be used for testing AI/ML based beam management scenarios.

## Upscaling 3D MPAC system layout for AI/ML based beam management

Under the assumption that the channel model simplifications resulting in the channel models in [3], as listed in previous section, are still valid for AI/ML based beam management, the modifications required to upscale a 3D MPAC environment are analyzed hereby.

For such analysis, we considered a few scenarios for the intended test case, where the BS is sweeping its beam towards different directions in a CDL-C UMi channel model from [3] and the UE is observing the resulting channel at its end.

We present here the CDL-C UMi FR2 channel model table as defined in [3] to facilitate the clarification of the simulation results shown later in this document:

Table 7.2.2-3: Channel model parameters for UMi CDL-C at 28 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -30.4353 | -134.4434 | 98.9242 | 83.3318 |
| 2 | 12.594 | -1.25 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 3 | 13.314 | -3.4684 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 4 | 13.974 | -5.2294 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 5 | 13.056 | -2.5215 | -28.0782 | -152.8206 | 99.5732 | 71.1282 |
| 6 | 38.196 | 0 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 7 | 38.688 | -2.2185 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 8 | 39.36 | -3.9794 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 9 | 39.504 | -7.4215 | 17.3861 | 84.3647 | 100.4513 | 69.2454 |
| 10 | 47.61 | -7.1215 | -37.5865 | 92.0623 | 98.5616 | 66.7349 |
| 11 | 49.278 | -10.7215 | 20.2226 | -97.7585 | 100.6231 | 72.0348 |
| 12 | 56.016 | -11.1215 | -50.6106 | 78.4702 | 98.218 | 64.4337 |
| 13 | 73.71 | -5.1215 | -33.911 | 93.1719 | 100.165 | 85.4238 |
| 14 | 78.498 | -6.8215 | -37.5066 | -112.0441 | 100.2604 | 64.1548 |
| 15 | 130.224 | -8.7215 | -43.1797 | 102.4645 | 98.1225 | 64.7824 |
| 16 | 162.63 | -13.2215 | 29.2116 | 67.2359 | 100.2604 | 92.467 |
| 17 | 255.534 | -13.9215 | 27.8133 | 34.5731 | 98.4852 | 65.6889 |
| 18 | 276.018 | -13.9215 | 23.6584 | 48.5813 | 98.1416 | 68.7572 |
| 19 | 329.412 | -15.8215 | -52.5282 | 36.4455 | 97.9698 | 59.1339 |
| 20 | 336.462 | -17.1215 | 25.0168 | 52.6729 | 100.7376 | 65.3402 |
| 21 | 378.39 | -16.0215 | 25.4562 | 49.8296 | 98.1225 | 58.4365 |
| 22 | 398.244 | -15.7215 | 30.7697 | 46.4316 | 98.1034 | 65.2705 |
| 23 | 422.562 | -21.6215 | 35.9234 | 30.759 | 100.4513 | 62.6903 |
| 24 | 519.138 | -22.8215 | -61.2775 | 69.2469 | 100.9476 | 61.993 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 4.8814 | 7 |  |

The BS antenna and beamforming assumptions are the same as described in [3], resulting in the following simplified representation of the codebook as in Figure 2:

Figure 2 - BS beamforming code book as defined in [3]. 16 beams in azimuth [-60°,60]. 
8 beams in elevation [-25°,25°]. Each beam is labelled with B and its index.

We labelled the beams from left to right and top to bottom with ‘B’ and a beam index ‘[1,2,…128]’. The beam azimuth and elevation angles are shown on the x and y axes respectively.

Four BS beam sweeping cases are simulated and analyzed in the following section. In each of the cases, the BS is using and sweeping over 8 Tx beams (i.e. lower bound of the maximum number of set B agreed in [6]) and the UE is predicting the probability of the rest of the available beams (i.e. Set A).

In addition, we show the spatial channel that should be emulated in the test zone for each case at the UE end.

In the following figures (Figure 3 to Figure 6) the applied beams for the beam sweeping process are highlighted with red and the cluster departure angles of the discussed channel model (Table 7.2.2.-3 from [3] presented above) are marked with black circles and labeled with ‘C’ & ‘Cluster Index’, with the indices corresponding to the channel model table above.

- Case 1: 8 BS beams w/o knowing the channel

- The BS has no knowledge about the channel and is sweeping its beam for the initial phase to determine the range of the departure angles it should target. In this case, we assume the BS is selecting a range of beams around the center and around the edges of its codebook to cover as much angular spread as possible, in both azimuth and elevation. Furthermore, we assume that the BS is slightly biasing its sweep towards negative elevation angles, as for typical UMi scenario the UE is commonly located lower than the BS antenna.

The applied beams during the beam sweep process are shown in Figure 3, where the active beams are marked in red as mentioned above. In this case B99, B35, B87, B55, B90, B58, B110 and B46.

Figure 3 - BS beams for case 1

- Case 2: 8 BS beams with partial posteriori information about the channel, i.e. elevation AoDs

- The BS has information about the elevation angle of the UE and is sweeping its beams over all its azimuth range with a subset of beams, i.e. 8 out of 16 beams available. This is shown in Figure 4.

Figure 4 - BS beams for case 2

- Case 3: 8 BS beams towards the 8 best clusters in the channel

- The BS has some posteriori knowledge about the channel and knows the departure angles of the channel clusters. Thus, it sweeps its beam towards the strongest clusters sequentially. The BS selects the closest beam to the intended departure angles from its codebook. The target 8 beam directions are the 8 strongest cluster that results in 8 different codebook beams. The active beams for this scenario are shown in Figure 5.

Figure 5 - BS beams for case 3

- Case 4: 8 BS beams around/close to the direction of strongest cluster

- The BS has some posteriori knowledge about the channel and is sweeping its beam around the departure angle of the strongest cluster. In this case the BS is sweeping its beam and selecting the closest beam set to the departure angle of strongest cluster from its codebook as shown in Figure 6. In this case, we assume that the BS is rather targeting negative elevations.

Figure 6 - BS beams for case 4

Our simulation results for the above cases are presented in the following (Figure 7 to Figure 10), depicting the Spherical Power Spectrum (SPS) for each of the scenarios and the emulated BS beams, i.e. CDL-C UMi for a carrier frequency of 28GHz. The results have been normalized for each BS beam individually to illustrate the relative power between the observed clusters for each beam. This gives a hint on the angular spread in both azimuth and elevation of the channel for each BS beam that needs to be emulated.

Figure 7 - Case 1: SPS at the UE end for 8 BS beams corresponding to Figure 3

Figure 8 - Case 2: SPS at the UE end for 8 BS beams, corresponding to Figure 4

Figure 9 - Case 3: SPS at the UE end for 8 BS beams corresponding to Figure 5

Figure 10 - Case 4: SPS at the UE end for 8 BS beams corresponding to Figure 6

As it can be seen in the figures (Figure 7 to Figure 10), the different BS beams manipulate the channel significantly, resulting into a large extent of different channel models to be emulated. In other words, the beam sweeping process introduces a large variation in the relative angles of arrivals and the total angular spread (AoA and ZoA) that need to be emulated.

As a next step (Figure 11 to Figure 14), we try to make a rough estimation of the number of probes that would be required, following the currently agreed approach for FR2 MIMO OTA, i.e. 3D MPAC [3].

Using Case 2 as an example, the simplest BS beam is #87, which is also used to determine the probe layout in [3]. This results in 6 probes: 5 of them are used for the strongest cluster and one for the weaker cluster, covering an angular range of roughly [130°, 190°] and [-25°, 5°] in azimuth and elevation respectively as shown in Figure 11.

Figure 11 - Probe locations for BS beam #87 PSP

Within Case 2, BS beam #85 requires additional 6 to 8 probes to cover the range [180°, 250°] and [-30°, 10°] in azimuth and elevation respectively, and another 4-6 probes would be needed to cover the range of [90°, 150°] and [-30°, 0°]. In total, 10-14 probes to cover a completely different angular range as presented in Figure 12.

Figure 12 - Probe locations for BS beam #85 PSP

To emulate BS beam #83, 4-6 probes would be needed for each of the dominant clusters, and 1-2 for the weaker ones. The angular range of the probes would extend further to [60°, 270°] and [-45°, 0°]. This would require roughly 19 probes as shown in Figure 13.

Figure 13 - Probe locations for BS beam #83 PSP

To emulate BS Beam #81, following the same rule of thumb, around 24 probes would be needed, again having different locations to cover the angular range of [10°, 270°] and [-45°,10°] in azimuth and elevation respectively, as the resulting PSP “illuminates” different angular regions. This is shown in Figure 14.

Figure 14 - Probe locations for BS beam #81 PSP

When targeting to emulate multiple beams, obviously most of the probes used to reconstruct one beam cannot be reused for another. This is because the PSP is different per beam, and the intended beam management scenario does not allow the rotation of the UE in the OTA chamber between BS beams (which would enable the adjustment of the dominant illuminated angular range). In other words, most of the probes required for different beams, do not overlap. This can be seen in Figure 11 through Figure 14. This means that for only the four beams considered we would need at least 50 probes. Considering the remaining four beams for Case 2, we would roughly reach a range of 100 probes.

Another approach to estimate the number of probes required, is to consider the AoA/ZoA for all clusters. Out of 24 clusters, there are 20 different angles of departure and arrival. This means that, if the BS is illuminating the range of the channel model departure angles, the UE will observe the full angular range (i.e. 20 clusters with different angles of arrival at its end throughout the BS beam sweep). If we consider the agreed approach for FR2 MIMO OTA, i.e. 3D MPAC, where 4-6 probe are used to emulate each strong cluster, the 20 different angles of arrival to be emulated result (again) in approximately 100 probes.

With both approaches we conclude for the current 3D MPAC to approx. >100 probes i.e. also  >200 RF channels (to count for dual polarized probes) for emulating CDL-C for AI/ML based BM testing. This would be simply unfeasible. Thus, further simplifications of the test scenarios are necessary.

Observation 4: Upscaling the FR2 NR MIMO OTA methodology (based on 3D MPAC chamber) to emulate CDL-C UMi requires a test system with >100 probes and >200 RF channels which is unfeasible.

Observation 5: For feasible implementation of an OTA test setup for AI/ML based beam management, further simplifications are inevitable.

## Sparse probe layout approach

As shown in the previous section, simulating a complex scenario with multiple BS beams towards a complex CDL model is not feasible following the MIMO OTA methodologies defined in [3], because of the need of a massive number of probes and RF channels in the channel emulator. In addition, such a massive test setup, would significantly increase the TE MU and the complexity of the calibration.

Indeed, other contributions ([7] and [8]) propose an intermediate approach, where the response of the CDL model as seen by the UE (incl. BS beam, AoD/ZoD and AoA/ZoA) can be simulated with a time-varying input power to a sparse probe layout resulting from the complex CDL model.

This leads to an interesting approach, where the main objective is not to create a (close to) perfect representation of the resulting CDL channel model in a static case (i.e. FR2 MIMO OTA as analyzed in previous sections), but to emulate a time varying power environment with a limited resolution, avoiding the need to simulate physically (cluster- and total resulting) angular spread.

In order to assess the testability requirements for such a simplified approach, i.e. to simulate the time-varying input power to a sparse probe layout resulting from the CDL model, the following points need to be discussed:

- Set of CDL models to be considered.

- Minimum number of clusters to be emulated for the CDL model(s), that would not bias the results (e.g. by power truncation). Ideally, only the 1 or 2 strongest clusters in the channel model should be considered.

- Neglection of the angular spread of the clusters in the spatial domain (i.e. single AoA/ZoA per cluster).

- Limitation of the angular range (azimuth and elevation).

The conclusion on the above points, would help to reduce and probably conclude to a feasible test setup for testing AI/ML base BM.

Proposal 1: RAN4 to consider a sparse probe layout as baseline for AI/ML based beam management testing and discuss the testability requirements for:

- Set of CDL models to be considered.

- Minimum number of clusters to be emulated for the CDL model(s), that would not bias the results (e.g. by power truncation).

- Neglection of the angular spread of the clusters in the spatial domain (i.e. single AoA/ZoA per cluster).

- Limitation of the angular range (azimuth and elevation).

On the other side, the key metric used for all AI/ML based beam management decisions, is a reliable and accurate (L1-)RSRP measurement. As such, a crucial requirement for the test setup, becomes the accuracy of the DL power, which results as combination of signals from various impinging angles.

Observation 6: Key requirement for AI/ML based beam management testing is not the CDL channel as such, but a time varying and accurate DL power, resulting as combination of signals from various impinging angles, which is suitable for accurate and reliable (L1-)RSRP measurements.

With respect of power accuracy, the most established methodology for FR2 testing, including multi- AoA, has been proven to be IFF (as also re-iterated in [9]). In comparison to multiprobe systems based on DFF, multiAoA systems based in IFF (e.g. Enhanced IFF), enable an accurate control of the DL power at the quiet zone with a lower MU, which is a key prerequisite for a test case based on (L1-)RSRP reporting.

Observation 7: Multi-AoA systems-based on IFF (e.g. Enhanced IFF) provide better DL power accuracy and lower MU compared to DFF based systems and thus, represent the best candidate to address the power accuracy based requirements for AI/ML based beam management testing.

Proposal 2: RAN4 to consider multi-AoA IFF based system as baseline for AI/ML based beam management testing.

# Conclusions

In this contribution, the following observations and proposals were made:

Observation 1: The current FR2 NR MIMO OTA system layout is limited to 6 probes, within a 55º by 10º sector, to simulate the resulting AoA/ZoA after applying the BS filtering (i.e. strongest beam towards the strongest cluster) for one, from two possible simplified CDL channel modes (i.e. CDL-A InO and CDL-C UMi).

Observation 2: The near field distribution in the test zone, simulating a complex multipath far field environment for the FR2 NR MIMO OTA system, is the result of combining the simultaneous signals transmitted by all probes.

Observation 3: The current FR2 NR MIMO OTA system defined in TR 38.827 cannot be used for testing AI/ML based beam management scenarios.

Observation 4: Upscaling the FR2 NR MIMO OTA methodology (based on 3D MPAC chamber) to emulate CDL-C UMi requires a test system with >100 probes and >200 RF channels which is unfeasible.

Observation 5: For feasible implementation of an OTA test setup for AI/ML based beam management, further simplifications are inevitable.

Observation 6: Key requirement for AI/ML based beam management testing is not the CDL channel as such, but a time varying and accurate DL power, resulting as combination of signals from various impinging angles, which is suitable for accurate and reliable (L1-)RSRP measurements.

Observation 7: Multi-AoA systems-based on IFF (e.g. Enhanced IFF) provide better DL power accuracy and lower MU compared to DFF based systems and thus, represent the best candidate to address the power accuracy based requirements for AI/ML based beam management testing.

Proposal 1: RAN4 to consider a sparse probe layout as baseline for AI/ML based beam management testing and discuss the testability requirements for:

- Set of CDL models to be considered.

- Minimum number of clusters to be emulated for the CDL model(s), that would not bias the results (e.g. by power truncation).

- Neglection of the angular spread of the clusters in the spatial domain (i.e. single AoA/ZoA per cluster).

- Limitation of the angular range (azimuth and elevation).

Proposal 2: RAN4 to consider multi-AoA IFF based system as baseline for AI/ML based beam management testing.

# References

- RP-240774, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface“, 3GPP TSG RAN Meeting #103, March 2024

- 3GPP TR 38.843 V18.0.0, “Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”

- 3GPP TR 38.827 V16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”

- 3GPP TS 38.151 V18.1.0, “NR; User Equipment (UE) Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements”

- 3GPP TR 38.901 V18.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”

- R4-2401570, “WF on AI/ML”, Qualcomm Incorporated, 3GPP TSG RAN WG4 Meeting #111, May 2024

- R4-2407333, “On AI/ML RAN4 Use Cases: Beam Management”, Qualcomm Incorporated, 3GPP TSG RAN WG4 Meeting #111, May 2024

- R4-2407234, “AI/ML Testability and interoperability issues for beam management”, Apple, 3GPP TSG RAN WG4 Meeting #111, May 2024

- R4-2409739, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #111, May 2024