# 3GPP TSG-RAN WG4 Meeting #112	R4-2411602

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.8.2.1

Source: 	Xiaomi

Title: 	Discussion on co-existence of NTN HPUE

Document for:	Discussion

Introduction

In the RAN#103 meeting, a new WID on enhanced requirements and test methodology for NR and IoT NTN was approved [1] and it was revised in RAN #104 meeting [2]. The WI objectives for NTN HPUE are excerpted here as the reference:

High power UE (HPUE) for NTN

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes

  - Evaluate the impact on the uplink performance of TN (Terrestrial network)

    - Example bands include bands n256 and 256, n255 and 255

    - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS

    - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

This contribution discuss the study of co-existence for NTN HPUE.

Discussion

In the R-19, the purpose of NTN HPUE is to introduce PC1, PC1.5 and PC2 for NTN bands and

- PC2 will apply to both handheld and non-handheld UE

- PC1 and PC1.5 are prioritized for non-handheld UE

In the WID, PC1.5 is only for NR-NTN, therefore, the co-existence study should be done for the power classes for NR-NTN and IOT-NTN as below table:

Table 2-1 PCs and frequency bands for co-existence study

|  | PCs | Frequency bands |
| --- | --- | --- |
| NR-NTN | PC1, PC1.5, PC2 | n256, n255 |
| IOT-NTN | PC1, PC2 | 256, 255 |

Whether PC1 and PC1.5 supported by handheld UEs can further evaluate based on the ACLR values.

Proposal 1: The co-existence study should be done for PC1, PC1.5 and PC2 of NR-NTN and PC1 and PC2 of IoT-NTN, whether PC1 and PC1.5 supported by handheld UEs can further evaluate based on the ACLR values.

Proposal 2: The scenarios for NTN HPUE co-existence study should keep align with R-17 NR-NTN co-existence as listed in below table:

Table 2-2: Scenarios for NTN HPUE co-existence study

| FR1: 2GHz |  | NTN |  |  |
| --- | --- | --- | --- | --- |
|  |  | Set 1 |  |  |
|  |  | GEO | LEO 600km | LEO 1200km |
| NR / NB-IoT | Rural | X | X | X |
|  | Urban macro | X | X | X |

The co-existence study for NTN HPUE including NR-NTN and IOT-NTN is to evaluate the impact on the uplink performance of TN, therefore, only the impact of NTN UL to NR TN UL should consider for HPUE co-existence study based on the Rel-17 NTN co-existence study:

Table 2-3: Aggressor and victim

| Combination | Aggressor | Victim |
| --- | --- | --- |
| TN with NTN | NTN UL | NR TN UL |

Proposal 3: Only the impact of NTN UL to NR TN UL should consider for NTN HPUE co-existence study.

In Rel-17, the co-existence of NR-NTN and NR-TN was comprehensively studied, and the simulation assumptions has been fully discussed, therefore, the coexistence of NR-NTN HPUE and NR TN could re-use the simulation assumptions in TR 38.863 as the starting point.

In Rel-18, the co-existence of IoT-NTN and NR-TN was comprehensively studied, the simulation assumptions has been fully discussed and approved in WF [1], therefore, the coexistence of IoT-NTN HPUE and NR TN could re-use the simulation assumptions in WF as the starting point.

Proposal 4: The simulation assumptions can use TR 38.863 and WF R4-2217473 as the starting point for NR-NTN and IoT-NTN separately.

Conclusion

In this contribution, we discussed the co-existence study for NTN HPUE and proposed:

Proposal 1: The co-existence study should be done for PC1, PC1.5 and PC2 of NR-NTN and PC1 and PC2 of IoT NTN, whether PC1 and PC1.5 supported by handheld UEs can further evaluate based on the ACLR values.

Proposal 2: The scenarios for NTN HPUE co-existence study should keep align with R-17 NR-NTN co-existence as listed in below table:

Table 2-2: Scenarios for NTN HPUE co-existence study

| FR1: 2GHz |  | NTN |  |  |
| --- | --- | --- | --- | --- |
|  |  | Set 1 |  |  |
|  |  | GEO | LEO 600km | LEO 1200km |
| NR / NB-IoT | Rural | X | X | X |
|  | Urban macro | X | X | X |

Proposal 3: Only the impact of NTN UL to NR TN UL should consider for NTN HPUE co-existence study.

Table 2-3: Aggressor and victim

| Combination | Aggressor | Victim |
| --- | --- | --- |
| TN with NTN | NTN UL | NR TN UL |

Proposal 4: The simulation assumptions can use TR 38.863 and WF R4-2217473 as the starting point for NR-NTN and IoT-NTN separately.

- Reference

[1] RP-240857, New WID on Enhanced requirements and test methodology for NR and IoT NTN, Samsung, Xiaomi, RAN #103.

[2] RP-241281, Revised WID on Enhanced requirements and test methodology for NR and IoT NTN, Samsung, Xiaomi, RAN #104.

[3] R4-2217473, Way Forward on coexistence study for IoT over NTN