# 3GPP TSG-RAN WG4 Meeting #112	R4-2411603

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.8.2.2

Source: 	Xiaomi

Title: 	Discussion on Tx requirements for NTN HPUE

Document for:	Discussion

Introduction

In the RAN#103 meeting, a new WID on enhanced requirements and test methodology for NR and IoT NTN was approved [1] and it was revised in RAN #104 meeting [2]. The WI objectives for NTN HPUE are excerpted here as the reference:

High power UE (HPUE) for NTN

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes

  - Evaluate the impact on the uplink performance of TN (Terrestrial network)

    - Example bands include bands n256 and 256, n255 and 255

    - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS

    - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

This contribution discuss the Tx requirements for NTN HPUE.

Discussion

The UE Tx requirements may be impacted to introduce the NTN HPUE, i.e., MOP, MOP tolerance, MPR, A-MPR, ACLR, SAR issue.

For A-MPR, the Mapping of network signalling label with NTN bands as listed in below table:

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| n254 | NS_01 | NS_03N | NS_04N | NS_05N |  |  |  |  |

We can see NS_24 for band n256, NS_02N for band n255 and NS_03N, NS_04N and NS_05N all request that the power of any UE emission shall not exceed the levels specified in the Spec. and NS-100 for band n256 are related with UTRA ACLR and power classes. Since the emission may be impacted when the MOP is increased, the A-MPR need re-evaluate based on different power classes.

And the purpose of co-existence study is to define the ACLR for higher power classes, therefore, MPR and ACLR will depend on the simulation results of co-existence.

For the SAR issue, it can use P-MPR as the starting point.

Proposal 1: Using +2/-3 tolerance for PC1, PC1.5 and PC2 for NTN HPUE.

Proposal 2: MPR, A-MPR and ACLR need further evaluate based on the simulation results of co-existence study.

Proposal 3: For the SAR issue, using P-MPR as the starting point.

Conclusion

In this contribution, we discussed the Tx requirements for NTN HPUE and proposed:

Proposal 1: Using +2/-3 tolerance for PC1, PC1.5 and PC2 for NTN HPUE including NR-NTN and IoT-NTN.

Proposal 2: MPR, A-MPR and ACLR for NTN HPUE need further evaluate based on the simulation results of co-existence study.

Proposal 3: For the SAR issue, using P-MPR as the starting point.

- Reference

[1] RP-240857, New WID on Enhanced requirements and test methodology for NR and IoT NTN, Samsung, Xiaomi, RAN #103.

[2] RP-241281, Revised WID on Enhanced requirements and test methodology for NR and IoT NTN, Samsung, Xiaomi, RAN #104.