# 3GPP TSG-RAN WG4 Meeting #112	R4-2411604

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.8.3.1

Source: 	Xiaomi

Title: 	Discussion on system parameters of less than 5MHz for NTN

Document for:	Discussion

Introduction

In the RAN#103 meeting, a new WID on enhanced requirements and test methodology for NR and IoT NTN was approved [1] and it was revised in RAN #104 meeting [2]. The WI objectives are excerpted here as the reference:

Less than 5MHz for NTN

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signaling to support the above objectives

This contribution discuss the system parameter of less than 5MHz for NTN FR1 bands based on the work done in rel-18 for TN.

Discussion

In Rel-18 WID “NR support for dedicated spectrum less than 5 MHz for FR1”, 3MHz CBW with 15kHz SCS was introduced for NR TN FR1 bands. Therefore, to enable more efficient deployment of the FR1 NTN bands, 3MHz CBW with 15kHz SCS should be introduced for NTN FR1 bands.

To introduce 3MHz CBW, it will impact the system parameters in specifications of TS 38.101-5 and TS 38.108 as below table list:

Table 2-1 The impact on system parameters for UE and SAN specification

| NTN system parameters | Impact to introduce 3MHz for NTN FR1 bands |  |
| --- | --- | --- |
|  | TS 38.101-5 for UE | TS 38.108 for SAN |
| 5.2.2 Operating bands with conducted requirements | No specification impact | No specification impact |
| 5.3.2 Maximum transmission bandwidth configuration/ Transmission bandwidth configuration | √ New define for 3MHz CBW | √ New define for 3MHz CBW |
| 5.3.3 Minimum guardband and transmission bandwidth configuration | √ New define for 3MHz CBW | √ New define for 3MHz CBW |
| 5.3.4 RB alignment | No specification impact | No specification impact |
| 5.3.5 UE/SAN channel bandwidth per operating band | √ Need introduce for bands n256,n255,n254 | √ Need introduce for bands n256,n255,n254 |
| 5.3.6 Asymmetric channel bandwidths | √ Need Operators whether there is a demand | X |
| 5.4.1 Channel spacing | No specification impact | No specification impact |
| 5.4.2 channel raster | √ Reuse channel raster of other channel bandwidths for NTN FR1 UE | √ Reuse channel raster of other channel bandwidths for NTN FR1 BS |
| 5.4.3 Synchronization raster | √ New define | √ New define |
| 5.4.4 TX–RX frequency separation | No specification impact | No specification impact |

Based on current NTN Spec for both of UE and SAN, the transmission bandwidth configuration and minimum guardband for channel bandwidth larger than 3MHz of NTN FR1 UE and SAN are the same with NR TN FR1. Hence, the transmission bandwidth configuration and minimum guardband for 3MHz CBW of NTN FR1 UE and SAN should keep the same with 3MHz CBW of NR TN FR1 UE.

Proposal 1: The transmission bandwidth configuration, minimum guardband and transmission bandwidth configuration for 3MHz CBW of NTN FR1 UE and SAN should reuse the configuration of 3MHz CBW of NR TN FR1 UE as below table.

Table 2-2 The max transmission bandwidth configuration and minimum guardband for 3MHz

| SCS (kHz) | 3 MHz |  |
| --- | --- | --- |
|  | NRB | min GB(kHz) |
| 15 | 15 | 142.5 |

Proposal 2: Introduce 3MHz CBW with 15kHz SCS for NTN FR1 bands n256, n255, n254.

Proposal 3: Adopting 100kHz and 10kHz channel raster for 3MHz CBW for NTN FR1 bands.

Form above table, it shows that asymmetric channel bandwidth only for NTN UE, and need operators confirm whether there is a demand for less than 5MHz for band n254.

Proposal 4: Operators should confirm whether there is a demand for 3MHz CBW in asymmetric channel bandwidth for NTN UE operating in band n254.

About the synchronization raster, finer synchronization raster was defined for NR TN FR1 3MHz CBW in Rel-18, therefore, finer synchronization raster should be defined for NTN FR1 3MHz CBW.

Proposal 5: The finer synchronization raster of TN 3MHz CBW should be considered and need be extended to 3000MHz for NTN FR1 3MHz CBW.

Conclusion

In this contribution, we discussed the power domain enhancement for NR single carrier proposed:

FR1 UE as below table.

Table 2-2 The max transmission bandwidth configuration and minimum guardband for 3MHz

| SCS (kHz) | 3 MHz |  |
| --- | --- | --- |
|  | NRB | min GB(kHz) |
| 15 | 15 | 142.5 |

Proposal 2: Introduce 3MHz CBW with 15kHz SCS for NTN FR1 bands n256, n255, n254.

Proposal 3: Adopting 100kHz and 10kHz channel raster for 3MHz CBW for NTN FR1 bands.

Proposal 4: Operators should confirm whether there is a demand for 3MHz CBW in asymmetric channel bandwidth for NTN UE operating in band n254.

Proposal 5: The finer synchronization raster of TN 3MHz CBW should be considered and need be extended to 3000MHz for NTN FR1 3MHz CBW.

- Reference

[1] RP-240857, New WID on Enhanced requirements and test methodology for NR and IoT NTN, Samsung, Xiaomi, RAN #103.

[2] RP-241281, Revised WID on Enhanced requirements and test methodology for NR and IoT NTN, Samsung, Xiaomi, RAN #104.