# 3GPP TSG-RAN WG4 Meeting #112	R4-2411605

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.8.3.2

Source: 	Xiaomi

Title: 	Discussion on UE RF requirements of less than 5MHz for NTN

Document for:	Discussion

Introduction

In the RAN#103 meeting, a new WID on enhanced requirements and test methodology for NR and IoT NTN was approved [1] and it was revised in RAN #104 meeting [2]. The WI objectives are excerpted here as the reference:

Less than 5MHz for NTN

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signaling to support the above objectives

This contribution discuss the UE RF requirements of less than 5MHz for NTN FR1bands based on the work done in rel-18 for TN.

Discussion

In Rel-18 WID “NR support for dedicated spectrum less than 5 MHz for FR1”, 3MHz channel bandwidth with 15kHz SCS was introduced for NR TN FR1 bands. Therefore, to enable more efficient deployment of the FR1 NTN bands, 3MHz channel bandwidth with 15kHz SCS should be introduced for NTN FR1 bands.

To introduce 3MHz channel bandwidth with 15kHz SCS for NTN FR1 bands, the NTN UE RF requirements should be discussed and re-defined as below tables list:

Table 2-1 NTN UE Tx RF requirements

| NTN UE Tx RF requirements | Impact to introduce 3MHz for NTN FR1 bands |
| --- | --- |
| 6.2.1 UE maximum output power | √ Need Operators confirm whether need introduce 3MHz CBW for n254 in Table 6.2.1-2: Additional requirements for transmit power density |
| 6.2.2 UE maximum output power reduction | √ Reuse NR TN FR1 UE requirement for 3MHz CBW |
| 6.2.3 UE additional maximum output power reduction | √ Figure out which NS values need define A-MPR for 3 MHz CBW |
| 6.2.4 Configured transmitted power | No specification impact |
| 6.3.1 Minimum output power | √ Reuse NR TN FR1 UE requirement for 3MHz CBW |
| 6.3.2 Transmit OFF power | √ Reuse NR TN FR1 UE requirement for 3MHz CBW |
| 6.3.3 Transmit ON/OFF time mask | No specification impact |
| 6.3.4 Power control | No specification impact |
| 6.4.1 Frequency error | No specification impact |
| 6.4.2 Transmit modulation quality | No specification impact |
| 6.5.1 Occupied bandwidth | √ Reuse NR TN FR1 UE requirement for 3MHz CBW |
| 6.5.2.2 Spectrum emission mask | √ Reuse NR TN FR1 UE requirement for 3MHz CBW |
| 6.5.2.3 Additional spectrum emission mask | No specification impact |
| 6.5.2.4.1 NR ACLR | √ Reuse NR TN FR1 UE requirement for 3MHz CBW |
| 6.5.2.4.2 UTRA ACLR | No specification impact |
| 6.5.3.1 General spurious emissions | No specification impact |
| 6.5.3.2 Spurious emissions for UE co-existence | No specification impact |
| 6.5.3.3 Additional spurious emissions | √ New requirements need define for NS_02N, 03N, 04N, 05N |

Since only considering to introduce 3MHz CBW for PC3 NTN FR1 UE, and the MPR of other channel bandwidths for NTN FR1 PC3 UE reused the requirements of NR TN FR1 UE. The MPR of 3MHz CBW for PC3 NTN FR1 UE should reuse the requirements of NR TN FR1 UE.

Observation 1: For NTN FR1 UE Tx RF requirements, there are no specification impact for the requirements of Configured transmitted power, Transmit ON/OFF time mask, Power control, Frequency error, Transmit modulation quality, Additional spectrum emission mask, UTRA ACLR, General spurious emissions and Spurious emissions for UE co-existence

Proposal 1: For NTN FR1 UE Tx RF requirements, MPR, Minimum output power, Transmit OFF power, Spectrum emission mask and NR ACLR for NTN 3MHz CBW could reuse the 3MHz requirements of NR TN FR1 UE.

Proposal 2: Operators need figure out which NS values need introduce 3MHz CBW, then RAN4 discuss how to define transmit power density, A-MPR and Additional spurious emissions for 3MHz CBW.

Table 2-2 NTN UE Rx RF requirements

| NTN UE Rx RF requirements | Impact to introduce 3MHz for NTN FR1 bands |
| --- | --- |
| 7.3.2 Reference sensitivity power level | √ New requirements need define for 3MHz CBW |
| 7.4 Maximum input level | √ Whether can reuse the requirements for 5MHz, 10MHz, 15MHz and 20MHz of NTN FR1 UE |
| 7.5 Adjacent channel selectivity | √ Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE |
| 7.6.2 In-band blocking | √ Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE |
| 7.6.3 Out-of-band blocking | √ Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE |
| 7.6.4 Narrow band blocking | √ Reuse NR TN FR1 UE requirement for 3MHz CBW |
| 7.7 Spurious response | √ Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE |
| 7.8 Intermodulation characteristics | No specification impact |
| 7.9 Spurious emissions | No specification impact |

Observation 2: For NTN FR1 UE Rx RF requirements, there are no specification impact for the requirements of

Intermodulation characteristics and Spurious emissions.

Proposal 3: For NTN FR1 UE Rx RF requirements, Maximum input level, ACS, In-band blocking, Out-of-band blocking and Spurious response can reuse the requirements for 5MHz, 10MHz of NTN FR1 UE.

Proposal 4: For NTN FR1 UE Rx RF requirements, Narrow band blocking can reuse the NR TN FR1 UE requirement for 3MHz CBW.

Proposal 5: The reference sensitivity need new define for 3MHz CBW of Bands n256, n255, n254.

Conclusion

In this contribution, we discussed NTN FR1 UE RF requiremetns for 3MHz CBW and proposed:

Observation 1: For NTN FR1 UE Tx RF requirements, there are no specification impact for the requirements of Configured transmitted power, Transmit ON/OFF time mask, Power control, Frequency error, Transmit modulation quality, Additional spectrum emission mask, UTRA ACLR, General spurious emissions and Spurious emissions for UE co-existence

Proposal 1: For NTN FR1 UE Tx RF requirements, MPR, Minimum output power, Transmit OFF power, Spectrum emission mask and NR ACLR for NTN 3MHz CBW could reuse the 3MHz requirements of NR TN FR1 UE.

Proposal 2: Operators need figure out which NS values need introduce 3MHz CBW, then RAN4 discuss how to define transmit power density, A-MPR and Additional spurious emissions for 3MHz CBW.

Observation 2: For NTN FR1 UE Rx RF requirements, there are no specification impact for the requirements of

Intermodulation characteristics and Spurious emissions.

Proposal 3: For NTN FR1 UE Rx RF requirements, Maximum input level, ACS, In-band blocking, Out-of-band blocking and Spurious response can reuse the requirements for 5MHz, 10MHz of NTN FR1 UE.

Proposal 4: For NTN FR1 UE Rx RF requirements, Narrow band blocking can reuse the NR TN FR1 UE requirement for 3MHz CBW.

Proposal 5: The reference sensitivity need new define for 3MHz CBW of Bands n256, n255, n254.

- Reference

[1] RP-240857, New WID on Enhanced requirements and test methodology for NR and IoT NTN, Samsung, Xiaomi, RAN #103.

[2] RP-241281, Revised WID on Enhanced requirements and test methodology for NR and IoT NTN, Samsung, Xiaomi, RAN #104.