# 3GPP TSG-RAN WG4 Meeting #112				                       R4-2411617

# Maastricht, Netherlands, 19th - 23rd August, 2024

Source: 	Xiaomi

Title: 	Discussion on RRM core requirements for LP-WUS/WUR

Agenda Item:	8.22.4.1

Document for:	Discussion

# 1Introduction

In RAN#103 meeting, the latest revised WID on low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved [1]. The related RRM work was initially discussed in last meeting. A WF was achieved and approved [2]. In this contribution, we provide our further analysis on RRM impacts of LP-WUS/WUR features.

- Discussion

- LP-WUR requirements at RRC_IDLE/INACTIVE state

- General aspects

According to the WID scope, LP-SS based measurement via LR can only be used for RRM for serving cell. Following this basic principle, we think one of the conditions for LR exiting measurement is whether UE needs to conduct neighbor cell measurements, which can only be performed in MR.

Observation 1: Measurements on neighbor cell can only be conducted in MR, irrespective of normal measurements or relaxed measurements.

In accordance with the legacy measurement procedure for MR only UE, measurements for neighbor cell could be divided into two parts. One is for intra-frequency measurements and NR inter-frequency or inter-RAT frequencies measurements with equal or lower priority, and the decision for MR to conduct these measurements hinges on the results from the serving cell measurements. When the signal quality of the serving cell reduce to certain threshold, the intra-frequency measurements and NR inter-frequency or inter-RAT frequencies measurements with equal or lower priority will be triggered. The other one is for NR inter-frequency or inter-RAT frequencies with higher priority, and the decision for MR to conduct the measurements depend on NW configuration. In this case, as long as NW configuration indicates the priority information, UE needs to perform measurements of higher priority NR inter-frequency or inter-RAT frequencies.

Observation 2: According to existing cell selection/reselection procedure:

- The initiation of intra-frequency measurements, as well as NR inter-frequency or inter-RAT frequency measurements with equal or lower priority, depends on the serving cell measurement results from the MR;

- The initiation of NR inter-frequency or inter-RAT frequencies measurements with higher priority depends on NW configuration.

Issue 1-1-2: Cases/states to be considered for RRM relaxation Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON. FFS on other topics listed in [1] for sub-topic 1-1 Issue 1-1-3: Core requirements to be specified for MR RRM relaxation - Proposals - P1: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR. (Apple xiaomi oppo LG CT ZTE) - RAN4 can investigate the mobility performance to quantify the relaxation, e.g., scaling factor for measurement period. If both LP-WUR and MR are ON, RAN4 to discuss whether UE uses LP-WUR measurement to decide MR relaxation or UE uses both LP-WUR and MR measurement to decide MR relaxation (Apple) - P1-1: RAN4 to specify MR RRM relaxation requirements for serving cell measurements in Idle/Inactive mode when (MTK) - LP-SS based LP-WUR RRM measurement is activated, or - PSS/SSS based LP-WUR RRM measurement is activated Not to consider RRM relaxation based on the hybrid measurements of both (LP-SS and SSS).

Based on the preliminary discussion in last meeting, 4 cases of RRM measurement relaxation are listed.

In case 1, i.e., fully offloading case, MR stops perform any RRM measurement and LP-WUR performs serving cell measurement. RAN4 has already agreed to support case1 and define requirements. There is still FFS on the condition and details for the MR neighboring cell measurement. From our perspective, this is the case where UE could perform the greatest level of relaxation measurement. We prefer not to consider any neighbor cell measurement in this case.

Related to case 1, we think case 4 is the further relaxed case after case 1, where MR performs neighbour cell measurement and LP-WUR performs serving cell measurement. Based on the previous observations, i.e. the initiation of neighbor cell measurements depend on either MR serving cell measurements or NW configuration, we think the case 4 is the case NW configures NR inter-frequency or inter-RAT frequencies measurements with higher priority, while LR is still doing serving cell measurements. However, we think whether UE capable of LP-WUR still follow the legacy procedure should depend on RAN2 decision. So we prefer to waiting for RAN2 progress in this case.

In case 3, MR has RRM relaxation on serving cell and neighbour cell measurement and LP-WUR performs serving cell measurement. We think this is a reasonable case, and also clearly included in the objective. The relaxed neighbor cell measurements can include both two kinds of neighbor cells.

In case 2, MR has RRM relaxation on serving cell measurement (no neighbour cell measurement is needed) and LP-WUR performs serving cell measurement. We think this is the case UE in the cell center, when UE does not need to measure neighbor cell, as well as no higher priority inter-f or inter-RAT configured. From our perspective, this is the typical case for starting LP-WUR monitoring and no need to start MR measurement. So, we prefer to de-prioritize this case.

Proposal 1: For RRM relaxation case #2 to #4, it is suggested to prioritize case 3.

Considering the core requirements to be specified for MR RRM relaxation, based on the analysis above, we support to discuss MR measurement with relaxation for both serving cell and neighbor cell.

Proposal 2: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR.

Issue 1-1-4: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation - Proposals - P1-1: RAN4 specifies the following for entry/exit criteria evaluation for WUS paging monitoring: (Qualcomm) - The number of measurement samples needed for filtering. - The number of consecutive times the UE needs to meet the entry/exit criteria - P1-2: Define requirements on entry/exit criteria for LP-WUR measurement/ MR RRM measurement relaxation based on RAN1 and RAN2 design (CT) - P1-3: RAN4 to discuss the requirement about exit the LR by serving cell evaluation based on the LP-RSRP/LP-RSRQ simulation result and the requirement about entry the LR by serving cell evaluation based on the SSB RSRP/RSRQ. (Ericsson) Recommendations: Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR measurement - Proposals - P1: RAN2 is be the main group for criteria (entry/exit conditions) design (Apple oppo vivo) - P2: RAN4 shall lead the discussion regarding criterion or scenario (entry/exit condition) for LP-WUR measurement and MR RRM measurement relaxation with the involvement of RAN1 and RAN2 (Samsung) - RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package - It’s not precluded multiple states can be introduced with different assumption on side condition - P3: RAN4 to firstly discuss the differences between entry/exit conditions for LP-WUS monitoring defined in RAN1/RAN2 and conditions for LP-WUR measurement. RAN4 should at least consider LP-SS and PSS/SSS when discussing the entry/exit conditions for LP-WUR measurement. RAN4 to wait for RAN1/RAN2, if the entry/exit conditions for LP-WUS monitoring are introduced by RAN1/RAN2, the thresholds on serving cell measurement result can be further discussed in RAN4. (CATT) - P4: Both serving cell quality and UE mobility should be considered for the criteria(entry/exit conditions) for MR RRM measurement relaxation &LP-WUR measurement (State2) and only LP-WUR measurement (State 3). Whether network configure the high priority frequency carrier measurement or not should also be considered when defining the criteria (entry/exit conditions). (CMCC) - P5: When UE enters the State 3 (Measurement only based on LP-WUR), The serving cell quality should at least be Srxlev > SIntraSearchP and Squal > SIntraSearchQ (CMCC) - P6: Study other conditions for the entry/exit condition for LP-WUS monitoring other than the measurement based condition from the RAN4 pespective (LG) - P7: RAN4 to discuss serving cell evaluation for entry/exit conditions for PSS/SSS and LP-SS based LR. PSS/SSS based entry/exit evaluation can be prioritised. (Nokia) Recommendations: Issue 1-1-6: Criteria (entry/exit conditions) for MR RRM measurement relaxation - Proposals - P1: RAN2 is be the main group for criteria (entry/exit conditions) design (oppo vivo Apple) - RAN4 can work on the RRM measurement relaxations (e.g., Scaling factor) and offloading mechanisms based on the criteria defined by RAN2. (Apple) - P2: RAN4 should firstly discuss the meaning of MR RRM measurement relaxation on which phases and what purposes are included for RRM relaxation in the entire procedure, i.e. for cell selection/reselection or handover, for serving cell measurement or neighbor cell measurement, etc. (CATT) - P3: RAN4 to discuss and provide input to RAN2 such as possible serving/neighbour measurement relaxzation conditions or which receiver (i.e., MR or LP-WUR) to base it on for MR based RRM measurement relaxation (LG) - P3a: RAN4 shall lead the discussion regarding criterion or scenario (entry/exit condition) for LP-WUR measurement and MR RRM measurement relaxation with the involvement of RAN1 and RAN2 (Samsung) - RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package - It’s not precluded multiple states can be introduced with different assumption on side condition Recommendations: Issue 1-1-7: Criteria (entry/exit conditions) for LP-WUS monitoring - Proposals - P1: RAN4’s involvement on entry/exit conditions for LP-WUS monitoring can be triggered by other groups if necessary (Apple) - P2: For Entry/exit condition for LP-WUS monitoring, the performance of miss detection rate on LP-WUS can be considered as side condition in addition to RSRP and RSRQ (Samsung) - P3-1: The entry/exit conditions for LP-WUR measurement should be consistent with the entry/exit conditions LP-WUS monitoring (xiaomi) - P3-2: The entry/exit condition for LP-WUS monitoring and the entry/exit condition for LP-WUR measurement should be considered jointly.(CMCC) - P3-3: Same criteria and same/different entry/exit conditions can be used for LP-WUS monitoring and LP-WUR RRM measurement. (vivo) - P3-4: RAN4 to confirm that same entry/exit condition for LP-WUS monitoring applies for serving cell RRM measurement offloading (from MR to LP-WUR). (HW) Notes: MR measurements results shall be larger than the threshold and satisfy the related MR relaxation conditions. (ZTE) - P4: If there are other conditions that should be considered such as UE mobility besides serving cell RRM measurement for measurement offloading from MR to LP-WUR, RAN4 needs to provide input to RAN2 (LG) - P5: For the entry/exit conditions for LP-WUS monitoring, the threshold for the serving cell measurement performed by MR or LR shall be decided by RAN4 (ZTE) Recommendations: Suggest to check whether the following top level description: LP-WUR monitoring is triggered no later than one of cases identified in issue 1-1-2

In last meeting, companies showed their opinions on the entry/exist conditions for LP-WUR related operations, including LP-WUR measurement, LP-WUS monitoring and MR RRM measurement relaxation.

For the entry/exist conditions for LP-WUS monitoring, we understand that this issue is under discussion in RAN1. We think RAN4 should wait for RAN1 progress. RAN4’s discussion on entry/exit conditions for LP-WUS monitoring can be triggered RAN1 if needed.

Proposal 3: RAN4’s discussion on entry/exit conditions for LP-WUS monitoring can be triggered by RAN1 if needed.

As we understand that the LP-WUR measurement is initially introduced to guarantee the performance of LP-WUS monitoring, we think the measurement is not expected to start later than the LP-WUS monitoring. Also, we see little advantage in early starting of the LR measurement. If UE still use MR for paging reception, it is reasonable to use MR for RRM measurements as MR also need measurement for T/F synchronization. In this case, the LR measuremnt results seems useless. So, we prefer ro define the same entry/exit condition of LP-WUR measurement and LP-WUS monitoring.

Proposal 4: It is suggested to define the same entry/exit conditions for LP-WUR serving cell measurement and LP-WUS monitoring.

RAN4 also discuss the entry/exit conditions for MR RRM measurement relaxation. Based on our analysis in the previous section, we think it is more reasonable that the MR RRM relaxation is executed when LR is used for serving cell measurement. If the LR is not doing measurement, the evaluation of measurement relaxation can only depend on serving cell measurement in MR, which is aligned with Rel-16 power saving functionality. If the LR is doing RRM measurement while MR is also doing normal RRM measurement, there seems little benefit but more power consumption. So, from our perspective, the entry/exit conditions for MR RRM measurement relaxation should also be the same as LP-WUR serving cell measurement. However, we understand RAN2 is currently working on the criteria design, it is suggest RAN4 to focus on the relaxation cases.

Proposal 5: It's suggested that RAN4 concentrate on discussing relaxation cases, while the entry/exit criteria should be based on RAN2 design.

With the feature of LP-WUS/WUR, the UE MR could enter ultra-deep sleep state in RRC_IDLE/INACTIVE mode. UE MR is supposed to wake-up based on LP-WUS triggering. In this case, the MR may need to do re-synchronization on time/frequency domain in the first place and then paging reception and RRM measurements. We think that RAN4 should also consider the corresponding requirements for MR to guarantee the paging reception and RRM measurements.

Proposal 6: RAN4 should define the requirements to guarantee the paging reception and RRM measurements when UE MR existing sleep mode.

- MR RRM relaxation

Based on the conclusion reached in study phrase, UE power saving gain cannot be observed if the existing Rel-18 MR RRM measurement periodicity for serving and neighbor cells are applied and UE MR enters in ultra-deep sleep during LP-WUS monitoring. To obtain enough power saving gain, one of the objectives in the WI is to “specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR”. In Rel-16 power saving WI, relaxed RRM measurement requirements for neighbor cell in RRC_IDLE/INACTIVE mode are introduced. UE meeting the RRM relaxation criteria is allowed to conduct measurements with longer intervals. Generally, we agree that the Rel-16 relaxation method could be taken as baseline when considering the MR RRM relaxation for UE supporting LP-WUR.

Proposal 7: The Rel-16 relaxation method, i.e., extending measurement interval, could be taken as baseline when considering the MR RRM relaxation for UE supporting LP-WUR. The detailed scaling factor can be discussed after RAN4 have concrete agreements on the MR RRM relaxation cases.

- Accuracy

During discussion in previous meeting, it was agreed to wait RAN1 progress in target SNR/SINR and use the RAN1 agreed target SNR/SINR as baseline for RRM simulation assumption. Meanwhile, companies still have different understanding on whether/how to define the accuracy. From our perspective, we support the idea that the accuracy discussed here is for simulation assumption and no need to define dedicated accuracy requirement in performance section. As per the existing serving cell measurement requirements defined in RRC_IDLE/INACTIVE state, no corresponding measurement accuracy requirement introduced. Since we are discussing RRM requirements for LP-WUR in RRC_IDLE/INACTIVE state, we prefer to follow the same logic as for MR.

Proposal 8: RAN4 to determine the measurement accuracy in RRC_IDLE/INACTIVE state for simulation purpose, and NO need to define dedicated accuracy requirement in the performance section.

Following up on RAN1#117 agreement, RAN1 has agrees several candidate SNR values for LP-WUS and LP-SS design:

Agreement For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB

These candidate SNR values can be used for RRM simulation evaluation. We provide our initial simulation results at SNR=-3 dB in the accompany contribution [3].

The requirements of measurement accuracy, side condition of SNR and measurement delay are to be decided in RAN4, and they are interconnected. Considering the serving cell RRM measurement results play a significant role on the mobility schemes, we prefer that RAN4 start the work from the measurement accuracy requirements. Based on the desired accuracy, the number of the reference signal samples could be decided. Then, the measurement period requirements are derived the number of reference signal samples within a given time.

The LP-SS is used for serving cell measurement. In our view, the accuracy target for LP-WUR RRM measurement should be comparable to the SSB based RRM measurement for the main receiver, in order to avoid UE performance degradation. Based on the existing performance requirements, RAN4 has defined intra-frequency RSRP/RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements in clause 10.1.2B and10.1.7B for UE capable of reporting RSRP and RSRQ measurements of the serving cell to higher layers. As the LP-WUR is only expected to perform RRM serving cell measurement in RRC idle mode, we prefer to take the accuracy requirements as for CA/DC Idle Mode Measurements, i.e., ±6dB RSRP measurement accuracy and ±4dB RSRQ measurement accuracy, as baseline.

Proposal 9: It is suggested to take the accuracy requirement defined for CA/DC Idle Mode Measurements, i.e., ±6dB RSRP measurement accuracy and ±4dB RSRQ measurement accuracy, as the starting point when determining the measurement accuracy in RRC_IDLE/INACTIVE state for LP-WUR serving cell measurement.

- Other requirements

In our understanding, the measurement delay requirements are defined on top of the desired accuracy as well as SINR. Based on measurement accuracy and SINR, the number of the reference signal samples could be decided. Then, the measurement period requirements are derived from the number of reference signal samples within a given time. According to the outcome of last RAN4 meeting, it was agreed to introduce two sets of measurement requirements for LP-WUR serving cell measurement, based on LP-SS and PSS/SSS respectively. From our side, it is preferred to consider the same set of measurement accuracy no matter what RS used for measurement.

Proposal 10: RAN4 to consider the same target accuracy when defining LP-SS based and PSS/SSS based RRM delay requirements for LP-WUR.

- LP-WUR requirements at RRC_CONNECTED mode

In RRC_CONNECTED mode, as clearly described in the objectives, UE RRM/RLM/BFD/CSI measurements are performed by MR. So, we think there should be no RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode expected. However, we are fine not to jumps to conclusions in this early stage.

Proposal 11: No RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode is expected.

- Conclusion

Observation 1: Measurements on neighbor cell can only be conducted in MR, irrespective of normal measurements or relaxed measurements.

Observation 2: According to existing cell selection/reselection procedure:

- The initiation of intra-frequency measurements, as well as NR inter-frequency or inter-RAT frequency measurements with equal or lower priority, depends on the serving cell measurement results from the MR;

- The initiation of NR inter-frequency or inter-RAT frequencies measurements with higher priority depends on NW configuration.

Proposal 1: For RRM relaxation case #2 to #4, it is suggested to prioritize case 3.

Proposal 2: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR.

Proposal 3: RAN4’s discussion on entry/exit conditions for LP-WUS monitoring can be triggered by RAN1 if needed.

Proposal 4: It is suggested to define the same entry/exit conditions for LP-WUR serving cell measurement and LP-WUS monitoring.

Proposal 5: It's suggested that RAN4 concentrate on discussing relaxation cases, while the entry/exit criteria should be based on RAN2 design.

Proposal 6: RAN4 should define the requirements to guarantee the paging reception and RRM measurements when UE MR existing sleep mode.

Proposal 7: The Rel-16 relaxation method, i.e., extending measurement interval, could be taken as baseline when considering the MR RRM relaxation for UE supporting LP-WUR. The detailed scaling factor can be discussed after RAN4 have concrete agreements on the MR RRM relaxation cases.

Proposal 8: RAN4 to determine the measurement accuracy in RRC_IDLE/INACTIVE state for simulation purpose, and NO need to define dedicated accuracy requirement in the performance section.

Proposal 9: It is suggested to take the accuracy requirement defined for CA/DC Idle Mode Measurements, i.e., ±6dB RSRP measurement accuracy and ±4dB RSRQ measurement accuracy, as the starting point when determining the measurement accuracy in RRC_IDLE/INACTIVE state for LP-WUR serving cell measurement.

Proposal 10: RAN4 to consider the same target accuracy when defining LP-SS based and PSS/SSS based RRM delay requirements for LP-WUR.

Proposal 11: No RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode is expected.

- Reference

- R4-2410296, WF for RRM core requirements for LP-WUS/WUR, Vivo