# 3GPP TSG-RAN WG4 Meeting #112				                       R4-2411618

# Maastricht, Netherlands, 19th - 23rd August, 2024

Source: 	Xiaomi

Title: 	Initial discussion on RRM core requirements in NTN for IoT Phase 3

Agenda Item:	8.25.4

Document for:	Discussion

# 1Introduction

In RAN#104 plenary meeting, the revised WID of “Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” has been approved in [1] with the following objectives. The corresponding UE RRM requirements shall be defined in RAN4.

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

Based on current progress in RAN2, we see no RRM impact on the objective of support of Store & Forward based on regenerative payload. In this contribution, we would like to focus on the uplink capacity enhancements and provide some initial discussion on the potential RRM impact.

- Discussion

For uplink capacity enhancements for IoT NTN, one of the motivations is OCC enhancements. RAN1 has had discussion on the OCC enhancements, but currently only some initial conclusions for further evaluation and study were made. RAN4 can wait for more progress on the OCC enhancements in RAN1 to see if there is some RRM impact.

Proposal 1: RAN4 to wait for more progress on the OCC enhancements in RAN1 to see if there is RRM impact.

Another motivation of the uplink capacity enhancements for IoT NTN is to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction. The EDT feature was introduced in Rel-15 for both LTE-M and NB-IoT, enabling uplink and downlink data transmissions in Msg3 and Msg4 during the RA procedure. In Rel-16, the earlier transmission of UL data payload has further been enhanced by introducing UL transmission using Pre-configured Uplink Resources (PUR). With PUR, both the random access preamble transmission (Msg1) and the random access response (Msg2) can be omitted, and the data transmission can be completed in only two messages (i.e., Msg3 and Msg4). According to the sub-bullets of the objective of the uplink capacity enhancements, the EDT enhancement targeting at supporting Msg3 transmission without msg1/Msg2 is quite similar with the PUR feature. In last RAN2 meeting, a discussion on the EDT enhancement was took place and a LS was sent [2]:

In RAN2#126 meeting, the following agreements related to Msg3 transmission for uplink capacity enhancement for R19 IoT NTN have been achieved: Agreements: => RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). => If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.. Based on the above agreements, RAN2 kindly asks RAN4 and RAN1: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case?

Since the Msg3 transmission without msg1/Msg2 concept is quite similar as PUR feature, we recall the discussion in Rel-18 IoT NTN in RAN4 on the PUR requirements. It was agreed in RAN4#105 that not to define requirements for the legacy RSRP-based TA validation for PUR for both GEO and LEO considering the NTN characteristic, but whether or not to define other validation conditions is still FFS. Below we capture the current related requirements specified in latest TS36.133:

### 4.7A.4 Requirements for transmission using preconfigured uplink resources for UE category M1 for Satellite Access #### 4.7A.4.1 Introduction The requirements in this clause are applicable when the UE is configured with timing alignment (TA) validation for transmitting in uplink using preconfigured uplink resources (PUR) as specified in [TS 36.331]. #### 4.7A.4.2 Requirements on UE synchronization for transmission using PUR The requirements in this clause are applicable for the UE in normal coverage or in enhanced coverage area of the serving cell served by a satellite access node. The UE is allowed to transmit using the preconfigured uplink resources provided that the UE is synchronized towards the serving cell prior to transmission. If the UE is not able to obtain the synchronization towards the serving cell then the UE shall drop the PUR transmission. The UE determines the PUR transmission occasion according to the received PUR configuration [TS 36.331]. #### 4.7A.4.3 Requirements on TA validation for transmission using PUR The UE is allowed to transmit using PUR provided that TA is valid. [ FFS for the validation conditions] The UE shall determine the uplink timing, TTA, for transmitting on PUR using the valid values of NTA, NTA,offset, NTA,adjcommon and NTA,adjUE where TTA is defined in [16] which are the latest acquired values before PUR transmission.

Per TS 36.133, the initial timing accuracy requirement was specified as timing error limit (Te). The UE initial transmission timing error shall be less than or equal to ±Te.

According to TS 36.211, transmission of the uplink radio frame number i from the UE shall start TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs seconds before the start of the corresponding downlink radio frame at the UE. The parameter NTA and NTA,offset are legacy TA as in TN scenario, and in the case of RA, the NTA should be 0. So, the main point for UE to satisfy the timing requirement is the common TA information.

The common TA information, i.e., NTA,adjcommon, is derived from NW configuration, while the UE specific TA, i.e.,NTA,adjUE, is computed by the UE based on UE location and serving-satellite-ephemeris related parameters. Based on current spec, NW configures the satellite assistance information (SAI) for the serving cell to UE via SIB31. As per in TS 36.331, the validity duration of the satellite ephemeris data and common TA parameters is indicated by ul-SyncValidityDuration. With this indication, UE can acknowledge the maximum time duration that the satellite ephemeris is applicable.

From our perspective, an RRC Idle UE with a pre-compensated TA can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

Proposal 2: An RRC Idle UE with a pre-compensated TA can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2 if SIB31 is acquired before the transmission.

- Conclusion

Proposal 1: RAN4 to wait for more progress on the OCC enhancements in RAN1 to see if there is RRM impact.

Proposal 2: An RRC Idle UE with a pre-compensated TA can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2 if SIB31 is acquired before the transmission.

- Reference

- RP-241624, Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc.

- R4-2411009, LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, RAN2