# 3GPP TSG-RAN WG4 Meeting #112				                       R4-2411619

# Maastricht, Netherlands, 19th - 23rd August, 2024

Source: 	Xiaomi

Title: 	Initial discussion on RRM core requirements in NTN for NR Phase 3

Agenda Item:	8.26.3

Document for:	Discussion

# 1Introduction

In RAN#104 plenary meeting, the revised WID of “Non-Terrestrial Networks (NTN) for NR Phase 3” has been approved in [1]. The corresponding UE RRM requirements shall be defined in RAN4. In this contribution, we would like to provide some initial discussion on RRM impact in NTN for NR Phase 3 and provide our proposals.

- Discussion

NR-NTN downlink coverage enhancement

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

In Rel-19, satellite may be unable to have all its beams active with the nominal EIRP density per beam at a given time, in order to balance the DL coverage and satellite payload. To ensure that all UEs can be served across the satellite foot print while maximizing the overall satellite throughput, the nominal EIRP density may separate by several beams as multiple beams are activated simultaneously.

Based on current progress, RAN1 is conduction evaluation considering three different beam states:

Agreement For system level study based on analytical evaluation: - N1 beam footprints are in state “off” - These beam footprints are not served by any signal (no satellite service in this area) - N2 beam footprints are in state “common messages only” - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access. - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation - N3 beam footprints are in state “active traffic” - These beam footprints have X active (e.g. VoNR) users each. - These beam footprints are also served the necessary information for cell discovery and initial access - N1 + N2 + N3 = “Total number of beam footprints “ - N1, N2, N3, X are to be reported by companies. - Resource utilization obtained under the assumptions above is to be reported by companies. - Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc.

The scheme of beams activation/deactivation is still under discussion in RAN1 and RAN2. For example, one of the methods currently considering is the cell DTX/DRX like mechanism.

Observation 1: Satellite beam activation/deactivation scheme is under discussion in RAN1/2.

We understand that the beams activation/deactivation would have RRM impact. Currently, measurement requirements in both IDLE/INACTIVE mode and CONNECTED mode are basically based on SSB or DRX configuration. If cell DTX/DRX like mechanism is used to indicate beam status, the cell DTX/DRX configuration shall also be considered in the measurement delay requirements. Also, we think that the satellite beam transition between different states may have impact on mobility requirements and the corresponding UE behavior should be further studied. However, since the conclusions from RAN1 is not sufficient yet, we think those RRM impact can be FFS.

Proposal 1: RAN4 to study the RRM impact of satellite beams activation/deactivation scheme after RAN1/2 reaching more concrete conclusions.

Based on the latest revised WID, SSB channel enhancement focusing on SSB periodicity extension will be considered in RAN1, while the final decision will be given by RAN1 at the latest by RAN#106. The concrete method for SSB periodicity extension depends on the subsequent RAN1 discussion.

Observation 2: The SSB periodicity enhancement will be considered in RAN1.

The reception of SSB is the basics of measurement. UE conducting SSB measurement to get timing/frequency/AGC information of the serving cell as well as the target cell. The SSB periodicity extension clearly has RRM impact. However, the RAN4 study is pending on RAN1 finial decision.

Proposal 2: RAN4 to study the RRM impact of SSB periodicity enhancement once further progress is made in RAN1.

Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

Based on the objective captured above, Rel-19 NTN is also targeting at supporting RedCap devices operating in FR1 NTN.

- Reduced UE complexity

Based on Rel-17 and Rel-18 progress, the main characteristic for RedCap UE is the reduced UE complexity. RedCap UE supports 1 or 2 UE Rx branches and HD-FDD or FD-FDD operation. Besides some additional clarification on available samples for HD-FDD operation, the same RRM requirements as for normal NR UE are reused for Rel-17 RedCap with 2Rx, while the relaxed measurement requirements are defined for Rel-17 RedCap with 1Rx. The relaxation method including extending the number of measurement samples and relaxing the measurement accuracy. From our perspective, when defining RRM requirements for RedCap and eRedCap UEs with NR NTN, similar principle could be taken.

Proposal 3: RAN4 to define RRM requirements for (e)RedCap UEs with NR NTN on reduced number of UE Rx branches:

- For 2RX RedCap

  - Take measurement requirements for FR1-NTN as baseline；

- For 1RX RedCap

  - Relax measurement requirements from the aspects of extending the number of measurement samples or relaxing the measurement accuracy.

Currently, RAN1 is focusing on evaluating the applicability of the existing handling rules for HD collisions cases, considering the TA mismatch between actual TA used by UE and assumed TA at the gNB. RAN4 can waiting RAN1’s final decision to check whether there is new RRM impact.

Proposal 4: RAN4 to check whether existing HD-FDD related requirements can be reused or not after RAN1 reaching further conclusions.

- eDRX

The Rel-17 RedCap WI introduced extended DRX cycles for RRC Idle state (up to 10485.76 seconds) and RRC Inactive state (up to 10.24 seconds). Furthermore, the Rel-18 eRedCap WI introduced enhanced eDRX cycle length in RRC Inactive state beyond 10.24 seconds, up to the same eDRX cycle length as for RRC Idle state (up to 10485.76 seconds). During discussion in the (e)RedCap WI, the eDRX feature is introduced as an optional feature for both RedCap and non-RedCap UEs. As the eDRX feature is not specific for (e)RedCap UE, we think RAN4 should discuss whether to support eDRX feature for NR NTN.

Proposal 5: RAN4 to discuss whether to define measurement requirements for eDRX feature for NR NTN.

- RRM relaxation

RRM measurement relaxation for neighbour cells is introduced as an optional feature for RedCap UEs that can be enabled by the network in both RRC idle/inactive states and RRC connected state. From RRM perspective, the spec impact only exists in RRC idle/inactive states. The UE is allowed to further relax neighbour-cell RRM measurements (compared to existing Rel-16 relaxation functionality) when an RSRP/RSRQ-based stationarity criterion is met for a period of time, or when both the stationarity criterion and a not-at-cell-edge criterion are met.

Based on the current TS 38.133, NTN has already supported Rel-16 relaxation functionality.

#### 4.2C.2.7 Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion The requirements in clause 4.2.2.9 apply provided that target cell’s satellite is GEO. #### 4.2C.2.8 Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion The requirements in clause 4.2.2.10 apply provided that target cell’s satellite is GEO.

If we follow the same logic as in Rel-17 NTN discussion, the requirements in clause 4.2B.2.9 and 4.2B.2.10 for RRM measurement relaxation for Redcap could apply provided that target cell’s satellite is GEO. We can firstly check if RAN4 can agree to define Rel-17 relaxation measurement requirements for RedCap UE with NR NTN.

Proposal 6: RAN4 to discuss whether to define relaxed measurement requirements for NR NTN when RedCap UE configured with Rel-17 relaxation criteria.

- Conclusion

Observation 1: Satellite beam activation/deactivation scheme is under discussion in RAN1/2.

Proposal 1: RAN4 to study the RRM impact of satellite beams activation/deactivation scheme after RAN1/2 reaching more concrete conclusions.

Observation 2: The SSB periodicity enhancement will be considered in RAN1.

Proposal 2: RAN4 to study the RRM impact of SSB periodicity enhancement once further progress is made in RAN1.

Proposal 3: RAN4 to define RRM requirements for (e)RedCap UEs with NR NTN on reduced number of UE Rx branches:

- For 2RX RedCap

  - Take measurement requirements for FR1-NTN as baseline；

- For 1RX RedCap

  - Relax measurement requirements from the aspects of extending the number of measurement samples or relaxing the measurement accuracy.

Proposal 4: RAN4 to check whether existing HD-FDD related requirements can be reused or not after RAN1 reaching further conclusions.

Proposal 5: RAN4 to discuss whether to define measurement requirements for eDRX feature for NR NTN.

Proposal 6: RAN4 to discuss whether to define relaxed measurement requirements for NR NTN when RedCap UE configured with Rel-17 relaxation criteria.

- Reference

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT