3GPP TSG-RAN WG4 Meeting #112				                                        R4-2411621

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.21.2

Title: 	Discussion on NES RRM requirements

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1,RP-240170] for R19 NES was agreed and the objectives are self-contained as below.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this discussion, we provided some preliminary analysis on the potential RRM impacts due to these aspects in Rel19 NES.

- Discussion

  - On-demand SSB SCell operation

According to previous RAN1 discussion below, the on-demand SSB SCell operation scenarios are categorized according to both SCell activation status and whether SSB is transmitted before SCell activation.

R1#116 agreements: - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell

And in RAN1#116bis the following scenarios were agreed also.

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

In the figures below, the agreed four scenarios are illustrated and potential RRM requirements impacts are analyzed.

- Scenario #2 and Case #1

Figure 1-1.

- Scenario #2 and Case #2

Figure 1-2.

- Scenario #2A and Case #1

Figure 1-3

- Scenario #2A and Case #2

Figure 1-4.

It seems there is some ambiguity on these RAN1 agreed scenarios. For an example, for the case 2 it is not clear whether the always-on SSB which is periodically transmitted on the cell is co-existed with the legacy SSBs within SMTC configuration. In other words, if the OD SSB was not trigged whether there is any other SSBs available.

Observation 1: It seems there is some ambiguity on the definition on agreements from RAN1 of on-demanding SSB scenarios and cases (e.g. whether there are co-existed OD SSB and legacy SSBs).

In order to avoid any possible misalignment on these RAN1 agreed scenarios to be studied in this WI, it is better that RAN4 can confirm the same understanding on these scenarios definition. Thus, we can propose that:

Proposal 1: RAN4 can firstly align the understanding of the definition on the scenarios of on-demanding SSB.

In WID, the on-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation. It is obviously that if there is no always-on SSB the SCell measurement requirements on the deactivated SCell (e.g. 9.2.5.1 in TS38.133) will be huge impacted because UE needs synchronized to SCell to be activated by SSB detection.

Table 9.2.5.1-4: Time period for PSS/SSS detection, deactivated SCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Table 9.2.5.1-5: Time period for PSS/SSS detection, deactivated SCell (FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Additionally, all Scell activation/deactivation delay requirements in current spec [2, TS38.133] needs to be re-specified. But since there are quite complicated SCell activation/deactivation enhancements from Rel15 to Rel18 (e.g. the multiple Scell activation in Rel16, fast SCell activation in Rel17), in order to simplify RAN4 discussion we can take single SCell activation as the baseline to redefine the SCell activation delay requirements with OD SSB.

Proposal 2: RAN4 can start to redefine the RRM requirements below firstly when OD-SSB is introduced in Rel19

Table 1. RRM impacts summary due to OD-SSB

| RRM requirements in TS38.133 | Updates needed |
| --- | --- |
| 8.3. SCell Activation Delay Requirement for Deactivated SCell | FFS |
| 8.3.3 SCell Deactivation Delay Requirement for Activated SCell | FFS |
| 9.2.5 Intra-frequency measurements without measurement gaps | FFS |
| 9.2.7 Intra-frequency measurements with NCSG | FFS |

For the detailed RRM requirements updates, in our view it needs more discussion under the different conditions and scenarios. For an example, as illustrated in Figure 2 below, for the case of “Scenario #2 and Case #1“ and known Scell activation case, if the first SSB transmitted after OD SSB trigger command can located before the time “n + THARQ+5ms+TfirstSSB +margin”, UE can follow the current behavior for Scell activation and corresponding requirements defined in TS38.133 for known cases can be reused. Otherwise, the requirements in current spec needs to be updated. However, for the unknown case since the more than one SSB needed for the SCell timing and frequency synchronization, the current requirements could not be reused with the condition above for known cases indeed.

Observation 2: When leveraging the current requirements in TS38.133 to the SCell activation/deactivation requirement with on-demanding SSB, whether SCell is known or unknown needs to be differentiated.

Figure 2.

Also, if there are more than one SSB within the time window of [ t0,t0 + n + THARQ+5ms+TfirstSSB +margin] , in order to save more power up to NW implementation, gNB needs only to forward the SSB of Scell which is closer to the end of this window.

Observation 3: RAN4 can also consider whether further power saving optimization method needs to be accounted when specifying SCell activation/deactivation requirements with on-demanding SSB.

Nevertheless, beside the scenarios, RAN1 also discussed the always-on SSB is cell based or not.

Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB

That is it is possible UE can receive both non-cell-defined on-demand SSB and cell based legacy SSB together. If this is implemented by gNB, from UE perspective, it is also up to UE to perform the measurement based either of them.

Figure 3.

Observation 4: In order to simplify RAN4 discussion, it can assume that from the requirement perspective, we need only to define the scenario when there is only one cell-defining OD SSB available.

In summary, we can suggest that:

Proposal 3: RAN4 can further discuss the SCell activation/deactivation requirements with OD SSB upon

- UE known or unknow conditions

- the relationship between on-demanding trigger time and Scell activation command receiving

- legacy SSB or on-demanding SSB if both of them are activated

- further power saving

  - On-demand SIB1

According to the current WID objectives below [1], there is no RAN4 works was expected.

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3]

Proposal 4: Deprioritize RAN4 RRM works for OD SIB unless there is any explicit concerns from other WGs.

  - Adaptation of common signal/channel transmissions

Below the agreements made in RAN1 on adaptation mechanism of SSB and scenarios was summarized.

R1#116 Agreement Agreement For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study - Adaptation of SSB burst periodicity - Adaptation based on two SSB configurations where up to two configurations can be active - Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration - Adapting the transmitted number of SSBs within a SSB burst - Cell DTX for SSB adaptation - Whether to support new SSB burst periodicity value(s) - Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst) - New compact SSB burst(s) - Adapting the position of SSBs within a SSB burst - Other mechanisms/combinations are not precluded Agreement For the adaptation mechanisms of SSB in time-domain, study further following mechanisms: - Adaptation mechanism indicated or configured by gNB without UE trigger - Adaptation triggered by UE (if any) FFS: Details of associated signaling/indication/configuration R1#116bis Agreement Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized R1#117 Agreement Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

Observation 5: For the SSB adaptation, the more than one SSB burst periodicity will be used. But how to indicate the one NW used for UE measurement are still FFS.

Beyond Rel18, SSBs are always-on transmitted and their periodicity can be configured by NW RRC message. And for the initial access on a cell may assume a default SSB periodicity of 20ms. In order to save NW energy, SSB adaptation periodicity or skipping SSB transmissions on certain occasions can save NW energy.

Figure 4.

Thus, the corresponding requirements impacts in RAN4 RRM are also expected generally. However, from RAN4 RRM perspective, in legacy RRM requirement, both intra and inter-frequency measurements requirement in TS38.133 section 9.2 and 9.3 can be summarized as:

Intra-frequency measurements: Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1) DRX cycle; T SSB_measurement_period_intra. No DRX; max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil( 5 x Kp ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Inter-frequency measurements: Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Observation 6: In current spec, the total L3 measurement delay requirements depend on the SMTC periodicity which is configured by NW RRC message.

Thus UE can follow the indicated SSB adaptation was indicated by NW RRC message or other SIB message as well.

Observation 7: UE’s measurement requirements can be little impacted because SMTC period are still decided by NW.

Thus we can suggest that:
Proposal 4: RAN4 to wait RAN1’s progress on SSB adaptation to define the necessary requirements or update the current spec.

Up to now, in our view, one of potential impacts is if the SSB adaption happened within a measurement reporting duration (e.g. 4 measurement samples) as shown in the figure below, usually UE can

- Alt1: continue the measurements with the new SMTC/SSB period

- Alt2: discard the previous measurement samples with the obsolete SMTC period but restart a new measurement reporting procedure.

Figure 5.

Observation 8: the adaptation SSB configuration procedure happened within a successful measurement report period may result in the measurement requirements updates.

In order to simplify RAN4’s work, as the method we used before, we can assume the requirements applicability is no measurement period (SMTC) was not updated within a measurement reporting interval

Therefore, we can propose that:

Proposal 5: When defining the requirement with adaptation SSB, only the case in which there is no any SSB adaptation period changed within one reporting interval.

Secondly, the discussion in RAN1 regarding adaptation mechanism of PRACH and scenarios was quite premature.

R1#116bis Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211 Agreement For adaptation of PRACH in time-domain, support the following: - SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any) - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources - FFS: SSB-RO mapping for the additional PRACH resources Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode Agreement For adaptation of PRACH in spatial domain, - Study possibility of scenarios with non-uniform distribution of UEs in different beams - Note 6: Companies are encouraged to provide details on how they map UEs to different beams - Study network energy savings gain achieved by non-uniform PRACH resource allocation across SSBs for scenarios with non-uniform distribution of UEs in different beams (if any), - Assume the following framework for network energy evaluation in FR1 and companies to report at least the below settings used in the evaluation/simulation - 20ms SSB period - 30kHz SCS, DDDSU TDD pattern - Setting A: SIB1 period (20ms/40ms/160ms) - Setting B1: Cell load (Empty/low/medium) - Setting B2: Traffic model - Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2 - Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11 - Setting E1: PRACH configurations - (legacy) PRACH resources according to the following PRACH configuration for all transmitted SSBs - Case A1-1: PRACH configuration #5 (20ms) - Case A1-2: PRACH configuration #17 (10ms) - Case A2-1: PRACH configuration #0 (160ms) - (time-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration for all transmitted SSBs - Case B1: PRACH configuration #17 (10ms) - Case B2: PRACH configuration #0 (160ms) - Companies to report details of assumed time domain adaptation mechanism - (spatial-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration - Case C1: PRACH configuration #17 (10ms) - Case C2: PRACH configuration #0 (160ms) - Companies to report details of assumed spatial domain adaptation mechanism, including details of non-uniform PRACH resource allocation across SSBs - Setting F: Cat 1/Cat 2 BS as defined in TR38.864 - Setting G1: Number of SSB beams: 4,8 SSBs in a SSB burst with SSB pattern case C - Note 1: Baseline to compare is Case C1 vs Case B1/A1-1/A1-2, Case C2 vs Case B2/A2-1 - Note 2: It is up to company to report the SSB-RO mapping ratio and FDMed RO number, etc - Note 3: Other PRACH configuration index with different PRACH format other than format 0 is not precluded - Note 4: Other SSB/SIB1/RACH periodicity/PRACH resource/configuration assumptions are not precluded (up to companies to report) - Other frameworks for network energy evaluation are not precluded, e.g. including for FR2 Conclusion There is no consensus in RAN1 on the support of PRACH adaptation in spatial domain Agreement For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed - Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources Agreement At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. - Note: This mapping is not impacted by time domain PRACH adaptation - Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

In our view, the main purpose of adaptation PRACH is to add the PRACH resource in the time domain and spatial domain more dynamically. As a result, the random access occasion may be unknown to UE before the random access procedure initiated. But how to mapping and indicating them to UE is still under RAN1 discussion. Hence, we suggest that:

Proposal 6: RAN4 can wait RAN1’s progress on PRACH adaptation to define the necessary requirements or update the current spec.

For the adaptation paging, according to RAN2 agreements below, from RAN4 perspective, we observed that:

Agreements on paging adaptation in RAN2#125bis:

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

Observation 9: There is little impacts on cell selection/reselection requirements in RRC_Idle TS38.133 （e.g. 4.2.2.6	 Maximum interruption in paging reception）.

- Conclusion

In this contribution, serval issues related to RRM in Rel19 NES are discussed. The observations and proposals can be summarized as:

Observation 1: It seems there is some ambiguity on agreements from RAN1 of on-demanding SSB scenarios and cases (e.g. whether there are co-existed OD SSB and legacy SSBs).

Proposal 1: RAN4 can firstly align the understanding of the definition of the scenarios for on-demanding SSB.

Proposal 2: RAN4 can start to redefine the RRM requirements below firstly when OD-SSB is introduced in Rel19

Table 1. RRM impacts summary due to OD-SSB

| RRM requirements in TS38.133 | Updates needed |
| --- | --- |
| 8.3. SCell Activation Delay Requirement for Deactivated SCell | FFS |
| 8.3.3 SCell Deactivation Delay Requirement for Activated SCell | FFS |
| 9.2.5 Intra-frequency measurements without measurement gaps | FFS |
| 9.2.7 Intra-frequency measurements with NCSG | FFS |

Observation 2: When leveraging the current requirements in TS38.133 to the SCell activation/deactivation requirement with on-demanding SSB, whether SCell is known or unknown needs to be differentiated.

Observation 3: RAN4 can also consider whether further power saving optimization method needs to be accounted when specifying SCell activation/deactivation requirements with on-demanding SSB.

Observation 4: In order to simplify RAN4 discussion, it can assume that from the requirement perspective, we need only to define the scenario when there is only one cell-defining OD SSB available.

Proposal 3: RAN4 can further discuss the SCell activation/deactivation requirements with OD SSB upon

- UE known or unknow conditions

- the relationship between on-demanding trigger time and Scell activation command receiving

- legacy SSB or on-demanding SSB if both of them are activated

- further power saving

Proposal 4: Deprioritize RAN4 RRM works for OD SIB unless there is any explicit concerns from other WGs.

Observation 5: For the SSB adaptation, the more than one SSB burst periodicity will be used. But how to indicate the one NW used for UE measurement are still FFS.

Observation 6: In current spec, the total L3 measurement delay requirements depend on the SMTC periodicity which is configured by NW RRC message.

Observation 7: UE’s measurement requirements can be little impacted because SMTC period are still decided by NW.

Proposal 4: RAN4 can wait RAN1’s progress on SSB adaptation to define the necessary requirements or update the current spec.

Observation 8: the adaptation SSB configuration procedure happened within a successful measurement report period may result in the measurement requirements updates.

Proposal 5: When defining the requirement with adaptation SSB, only the case in which there is no any SSB adaptation period changed within one reporting interval.

Proposal 6: RAN4 can wait RAN1’s progress on PRACH adaptation to define the necessary requirements or update the current spec.

Observation 9: There is little impacts on cell selection/reselection requirements in RRC_Idle TS38.133 （e.g. 4.2.2.6	 Maximum interruption in paging reception）.

- References

- RP-240830

- TS38.133 v18.5.0

- TS38.331 v17.1.0