3GPP TSG-RAN WG4 Meeting # 112	R4-2411625

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Xiaomi

Title: 	Discussion on general aspects of AI/ML testability and interoperability

Agenda Item:	8.17.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss about general aspects of AI testability and interoperability.

- Discussion

      - 2.1 Post deployment

Issue 1-1: Post deployment testing - Proposals - Option 1: Option 1 is feasible only if model updates are infrequent - Option 2: Option 2 is feasible only if the validation can be performed by UE vendors individually (no need to test in a certified lab) - FFS whether some validation information needs to be shared with gNB - Option 3: Add an option in which the conformance test inputs are stored and used for post-deployment testing/validation - Option 4: Drop Option 1, only discuss post deployment handling as part of LCM - Option 5: RAN4 should discuss a framework to ensure that at least a validated model exists at the UE - Option 6: RAN4 should discuss a framework to ensure that non-validated/non-tested models are not used by the UE - Option 7: Others

There is a tensive discussion for post deployment of model update. From our observation, the reason why there are so many options is due to the fact that there are too many cases and scenarios for model update, which may lead to different solutions.

Considerations include:

- Quantity:

  - Large number of models

  - Small number of models

- Frequency:

  - Infrequent updates

  - Frequent updates

- Complexity and Size:

  - New model setup or transfer (e.g., model transfer)

  - Minor updates or fine-tuning of existing models

- Test Data Set Availability:

  - Availability of test data sets

  - Lack of test data sets

- New Scenario/Condition:

  - Updates for existing scenarios

  - Updates for enhanced scenarios or conditions

For instance, if the number of models to be updated is large, but the updates themselves are minor and the scenario/condition remains unchanged, performance monitoring could be considered sufficient for post-deployment evaluation, provided that the performance monitoring is well-designed. However, for significant updates, such as model type change, model transfer or enhanced scenarios where no prior conformance testing has been conducted, RAN4 may need to further assess whether performance monitoring alone is adequate.

Observation 1: The multitude of options for post-deployment arises from the variety of cases and scenarios associated with model updates.

Proposal 1: RAN4 may categorize model updates based on different cases and evaluate whether a unified approach or multiple solutions are required for post-deployment.

For option 2, whether performance monitoring after deployment is sufficient to guarantee the model performance will depend on how performance monitoring is defined and what’s the criteria for model activation, activation or fall back. There is some progress regarding to performance monitoring of BM:

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

For Option 1 of performance monitoring, the UE sends a report to the network, which includes L1-RSRP and/or RS index. The network then compares the predicted L1-RSRP and/or RS index with the measured L1-RSRP and/or RS index. The performance monitoring metric is similar to conformance test discussed in RAN4, such as comparing the L1-RSRP delta.

The key difference is that for post-deployment, the UE performs measurements for Set A under relatively lower SNR conditions. The L1-RSRP ground truth will have more errors compared to conformance tests conducted in a lab environment, where high SNR is assumed for measuring Set A. Therefore, when defining conformance test requirements, it is advisable to consider performance monitoring performance under more realistic conditions encountered after post-deployment. This approach will facilitate the application of performance monitoring for model updates after deployment.

Proposal 2: RAN4 to identify difference between conformance test and performance monitoring. It is advisable for RAN4 to consider performance monitoring under conditions encountered after post-deployment when defining conformance test requirements.

- Conclusion

In this contribution, we provide the following proposals:

Observation 1: The multitude of options for post-deployment arises from the variety of cases and scenarios associated with model updates.

Proposal 1: RAN4 may categorize model updates based on different cases and evaluate whether a unified approach or multiple solutions are required for post-deployment.

Proposal 2: RAN4 to identify difference between conformance test and performance monitoring. It is advisable for RAN4 to consider performance monitoring under conditions encountered after post-deployment when defining conformance test requirements.

- Reference

- TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interfa