3GPP TSG-RAN WG4 Meeting # 112	R4-2411626

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Xiaomi

Title: 	Discussion on testability and interoperability issues for beam management

Agenda Item:	8.17.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss testability issues for beam management.

- Ground truth - Test metric for AI based BM - TCI state activation

- Discussion

      - 2.1 Ground truth

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

In last meeting, it’s agreed that the ground truth of the predicted RSRP is the ideal measurement of RSRP on the predicted TX beam.

Test procedure

Fig.1 test procedure

As shown in Fig.1, the main steps for test beam or L1-RSRP accuracy are as below:

- Step 1: TE configures resource sets for measurement of set B and prediction report for set A

- Step 2: UE measures for beams in set B in T1

- Step 3: UE performs beam prediction for set A

- Step 4: UE report predicted best beam(s) and/or L1-RSRP(s)

- Step 5: TE configures resource set for measurement of set A

- Step 6: UE measures for beams in set A in T2 under high SNR

- Step 7: UE report ground truth of L1-RSRP of set A and/or best beam index

- Step 8: TE compares predicted beam with ground truth based on different metric

We identified that there are several issues that will have impact on the test metric accuracy.

### 2.1.1 RX beam consistency impact

The L1-RSRP for a given TX beam can vary depending on the selected RX beam. Consequently, the optimal TX beam index is determined based on the L1-RSRP values. The assumption made about the RX beam affects both the value of the RSRP measurement and the precision of the beam selection metric.

Step 1- Step 3

For step 1, TE configures resource sets for measurement of set B and prediction report for set A.

For step 2 and 3, UE will measurement L1-RSRP of set B and predict L1-RSRP and/or best TX beam index of set A. L1-RSRSP and/or best TX beam index will be derived based on certain RX beam assumption. Since in TR, two kinds of RX assumptions are supported:

Top-1 genie-aided Tx beam considers the following definitions: - Option A (baseline), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B (optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s)

For Option A, the output of AI model can be predicted L1-RSRP with full RX beam sweeping. For option B, the output of AI model can be predicted L1-RSRP with specific RX beams. Similarly, the input of AI model can also be measured L1-RSRP based on specific RX beams or full RX beam sweeping.

Observation 1: From TR, predicted best TX beam index and/or L1-RSRP can be based on full RX beam sweeping or specific RX beam.

Therefore, there are 2 kinds of possible UE implementation for RX beam assumption for the input and output of AI model, shown as below:

Fig.2 different RX beam assumption for prediction

For Case 1, when L1-RSRP measurements of Set B based on a specific RX beam are used as input, the output for predicted L1-RSRP of Set A is also based on the same specific RX beam. For case 2, when L1-RSRP measurements of Set B based on full RX beam sweeping are used as input, the output for predicted L1-RSRP of Set A can also be based on full RX beam sweeping. During full RX beam sweeping, the L1-RSRP for a given TX beam is determined based on the best RX beams. The best TX beam index will then be chosen based on the value of L1-RSRP of all TX beams.

As a result, the predicted L1-RSRP for a TX beam will vary depending on whether specific RX beams or full RX beam sweeping is assumed. For best TX beam index, it may also vary depending on whether specific RX beams or full RX beam sweeping is assumed.

Observation 2: Whether specific RX beams or full RX beam sweeping is assumed may lead to different best predicted TX beam index and predicted L1-RSRP value.

Step 5

For T2 in step 5, UE will measure set A to get L1-RSRP ground truth. For this step, it’s just legacy non-AI based L1-RSRP measurement. If the L1-RSRP configuration and reporting can re-use legacy rule, UE will measure for beams in set A based on RX beam sweeping if SSB or CSI with repetition ON is configured.

In legacy non-AI L1-RSRP measurement, there are 8 RX beams in RX beam sweeping set and the selection of these 8 beams depends on the UE's implementation. For instance, the UE might divide the full spatial scope into 8 equal parts, as shown in Fig.3(a) or narrow down the spatial scope based on L3 measurements, as illustrated in Fig. 3(a). In fact, even RX beams occupy full spatial scope, there may still be angle offset between two RX beam sets. For example, the angle of 8 RX beams in RX beam set 1 may be {0, 45, 90, 135, 180, 225, 270, 315} degree. The angle of 8 RX beams in RX beam set 2 may be {15, 60, 105, 150, 195, 240, 285, 330} degree.

- RX beams occupy full spatial scope         (b)  RX beams occupy partial spatial scope

Fig.3 RX beam spatial scope

Observation 3: In legacy non-AI based L1-RSRP measurement, when RX beam sweeping is applied, it’s up to UE implementation to choose RX beam angle.

RX beam mismatch for prediction and measurement for set A

In summary, without RX beam information indication for measurement of set A in step 5, it’s possible that UE choose different RX beams for measurement and prediction. There may be mismatch for predicted L1-RSRP/best beam index and measured L1-RSRP/best beam index for set A:

- Case 1: For prediction, specific RX beam is assumed while for measurement, RX beam sweeping is assumed.

- Case 2: For both prediction and measurement, RX sweeping beam is assumed. However, there may still be angle offset between two RX beam sweeping sets.

RAN4 needs to discuss how to keep the RX beam Consistency between prediction and measurement for set A to guarantee correct comparation.

Observation 4: If different RX beam is assumed for prediction and measurement, the L1-RSRP and/or best TX beam index may be different in two steps, which will lead to wrong performance evaluation.

There are several possible solutions. For example, NW/TE can also configure UE to report RX beam index for beam prediction of set A in step 4. Then NW/TE configures RX beam index for the measurement of set A in step 5. TE can also configure a RX beam association relationship between beam report and beam measurement for set A, e.g. indicate report ID. or NW/TE just send one configuration command including both step 1 and step 5, then UE will apply the same RX assumption for different steps.

Proposal 1: RAN4 to discuss how to keep the RX beam Consistency between prediction and measurement for set A to correctly evaluate prediction performance.

### 2.1.2 SNR impact

The ground truth for L1-RSRP in testing is defined by measuring L1-RSRP with a specified SNR threshold. To reduce measurement errors, a high SNR is essential. A higher SNR helps minimize errors in the measurement process. RAN4 should discuss how to select an appropriate SNR to ensure measurement quality. This SNR level can be determined through simulation. We recommend selecting the appropriate SNR level based on the L1-RSRP delta CDF curve, similar to the method used in Rel-15 to establish the accuracy requirements for measured L1-RSRP.

RSRP delta is defined as below:

RSRP delta= measured RSRP in Time duration 2 – ideal RSRP in Time duration 2

The procedure to determine the SNR level is outlined below:

- Set the RSRP delta threshold to XdB (e.g., 0.5 dB).

- Calculate and plot the RSRP delta CDF for different SNR levels and configurations. Configurations may include:

  - Channel type

  - Doppler

  - Number of reference signals (RS) in the time domain

  - Density of RS in the frequency domain

- Select the SNR level so that the maximum RSRP delta values (corresponding to the 5% and 95% CDF points) are below the XdB threshold.

Proposal 2: Suggest to use RSRP delta CDF-based simulation to determine the appropriate SNR level for deriving ground truth.

### 2.1.3 Channel time variation impact

Since the beam prediction will be based on the measurement of set B at T1. Ground truth L1-RSRP will be measured at T2. There will be a time gap between T1 and T2. Due to doppler, L1-RSRP will also vary in time domain. L1-RSRP time domain variation will also have impact on ground truth.

For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation. While for BM case-2, UE will be assumed to move. Therefore, channel doppler can’t be set for 0. Doppler will depend on UE speed and UE trajectory model. The RSRP variation can be evaluated by different doppler and time gap. RSRP variation impact to BM case-2 can be considered in RSRP accuracy requirement.

Proposal 3: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 4: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

### 2.1.4 Multiple-AoA test setup impact

For multiple-AOA test setup, as pointed early by TE vendor, multiple directions might be addressed by the DUT with different antennas, so that RF baseline system assumptions are not valid and FF-QZ-MU configuration needs further investigation. it’s more challenging to obtain accurate RSRP measurement as ground truth. it’s FFS whether extra margin needs to considered in RSRP accuracy requirement.

Proposal 5: It’s more challenging to obtain accurate RSRP measurement in multiple-AOA test setup. it’s FFS whether extra margin needs to considered in RSRP accuracy requirement.

### 2.1.5 How to solve UE cheating issue

In legacy L1-RSRP test, ground truth is set by TE. Therefore, UE can honestly report the measured L1-RSRP results.

In current AI based BM test, UE will report both best predicted beam and ground truth value to TE. TE can only rely on the two reports from UE to decide whether UE can pass the test. It’s easy for UE to cheat if the configuration for set A at T1 and T2 are the same. For example, suppose that for both T1 and T2, the same resource is configured and beam is mapping to these resources with the same order. Then in T1, UE report the best predicted RS index is 7. And in T2, after measurement for all RSs, UE find out the best beam is in RS 6. However, UE can just report the best measured RS index with 7 to TE. since TE didn’t know which beam is the best, it will only compare two indexes from UE reported at T1 and T2. If the two index is the same, TE will assume that predicted best beam is correct. Therefore, RAN4 needs to discuss how to avoid the issue.

Observation 5: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it’s easy for UE to pass the test by cheating.

The issue can be solved by several possible options.

- Option 1: Configure different resource for set A in T1 and T2.

- Option 2: Configure same resource for set A in T1 and T2 while the mapping relation of RS index and beam index is different. For example, suppose the beam index and RS index mapping relation for T1 is shown in Fig.4(a). For T2, TE will re-transmit all beams in set A on all RSs. TE will change the mapping relationship between RS index and beam index. In this way, UE can only honestly report the best measured beam in T2.

Fig.4 (a) beam index and RS index mapping relation for set A in T1

Fig.4 (b) beam index and RS index mapping relation for set A for T2

## 2.2 Test metric

Issue 2-1: Metrics/KPIs for Beam Management requirements/tests Agreement: Companies are invited to provide inputs/proposals to refine the definition of RSRP accuracy Hold on the discussions for concrete test metrics until RAN1 had conclusions on the schemes.

In RAN4#111 WF, it’s suggested to discuss the refinement of RSRP accuracy definition for option 1. Besides, we also see some ambiguity for option 3.

### 2.2.1 Definition refinement for Option 1

For option 1, RSRP prediction accuracy metric is defined in clause 6.3.1 in TR 38.843[1]:

For AI/ML models, which provide L1-RSRP as the model output, the accuracy of predicted L1-RSRP is to be evaluated. Companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as the difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the same beam.

In last meeting, it’s agreed that ideal L1-RSRP is measured RSRP under high SNR. Therefore, RSRP prediction accuracy focus about the RSRP difference between predicted and measured RSRP for the same beam.

Option 1 is similar as legacy absolute RSRP measurement accuracy definition. In legacy L1-RSRP test metric, the absolute RSRP difference is the delta between the measured L1-RSRP and ideal RSRP for the same beam index. During Rel-15 L1-RSRP accuracy performance discussion, the simulation results is also provided by assuming that RSRP difference is corresponding to the same beam index. No best beam selection functionality is tested.

Besides, Relative L1-RSRP accuracy is also defined for differential L1-RSRP reports in legacy. In RAN1 #117, it agrees about differential L1-RSRP reporting. RAN4 needs to discuss whether relative L1-RSRP accuracy is needed or not for option 1.

Agreement At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support - Support differential L1-RSRP reporting with legacy quantization step and range - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range

Proposal 6: For test metric option 1, refine the definition:

- Option 1: Absolute RSRP prediction accuracy is the RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and measured L1-RSRP of the same beam.

  - FFS to define relative accuracy requirement.

### 2.2.2 Definition refinement for Option 3

For option 3, the current wording is a little ambiguous. it’s not clear what does “maximum RSRP among top-K predicted beams” refers to.

- Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

There are two interpretations for “maximum RSRP among top-K predicted beams” for option 3:

- Interpretation 1: RSRP difference between Measured RSRP of best predicted beam and Measured RSRP of the strongest beam

- Interpretation 2: RSRP difference between Predicted RSRP of best predicted beam and Measured RSRP of the strongest beam

Different interpretation of RSRP may lead to different test purpose.

For Interpretation 1, option 3 will only test beam prediction accuracy in terms of RSRP difference. It will not check RSRP prediction performance and it only check whether beam selection is correct. measured RSRP for best predicted beam and best measured beam will be used for comparison. If that is the motivation, then the upper bound is not necessary. However, the current wording needs to be further refined.

For Interpretation 2, option 3 will try to verify both beam prediction and RSRP prediction together. However, it currently defines only the lower bound of the difference between predicted RSRP and measured RSRP, without addressing the upper bound. Therefore, we recommend modifying Option 3 to account for both the upper and lower bounds as detailed in [2].

From our understanding, option 3 is more similar as the test metric for beam prediction defined in 38.843 as below:

- Beam prediction accuracy (%) with 1dB margin for Top-1 beam - The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam "whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam"

In above definition, measured L1-RSRP for both best predicted beam and best measured beam is compared. Then the main motivation is just to verify beam prediction accuracy. Therefore, we suggest to refine the wording of option 3 to indicate that the motivation is for beam prediction accuracy.

Proposal 7: For test metric option 3, refine the definition:

- Option 3: successful rate for the correct beam prediction which is considered as measured L1-RSRP of top-1 predicted beams is larger than the measured L1-RSRP of the Top-1 measured beam – x dB.

### 2.2.3 Test metric relation

In RAN1, it agrees to support to include both predicted beam index and L1-RSRP for L1-RSRP reporting at least for BM-Case 1:

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

Therefore, both predicted beam index and RSRP can be included in L1 report at least for BM-case 1, similar as legacy. RAN4 needs to discuss how to select test metric between these options.

Observation 6: From RAN1 agreement, both predicted beam index and predicted RSRP can be included in L1 report at least for BM case-1, similar as legacy.

For option 1, RSRP prediction accuracy focus about the RSRP difference for the same beam.

While both option 2 and 3 can be classified into beam prediction accuracy.

In RAN1, the beam prediction accuracy can be defined from two different aspects:

- Type 1: Based on RSRP difference between predicted Top-1 beam and Genie Top-1 beam

- Type 2: Based on beam index difference between predicted Top-1/K beam and Genie Top-1/K beam

Therefore, Option 2 refers to beam index difference while option 3 refers to RSRP difference between predicted best beam and ideal best beam.

Observation 7: For option 1, RSRP prediction accuracy focus about the RSRP difference for the same beam.

Observation 8: Both option 2 and 3 can be classified into beam prediction accuracy. Option 2 refers to beam index difference while option 3 refers to RSRP difference between predicted best beam and ideal best beam.

Next, we will discuss the relationship between these options.

Some company mention that beam prediction accuracy requirement can be skipped if RSRP prediction accuracy (Option 1) requirement is defined. From our understanding, whether RSRP prediction accuracy can verify both beam prediction and RSRP accuracy or only RSRP accuracy depends on how accurate the RSRP prediction requirement is and the RSRP difference between two ideal best TX beams.

Suppose that the ideal best two TX beam RSRP difference is 2dB. If RSRP prediction for each beam index can be accurate enough, for example, the difference is within 1dB. Then it’s easy for UE to choose the best beam index according to predicted L1-RSRPs. In this case, RSRP prediction accuracy requirement can verify both beam index prediction and RSRP accuracy performance. On the other side, if the RSRP prediction accuracy is not so accurate, e.g. 5dB, then it can’t verify the best beam prediction performance. However, the RSRP difference between two best TX beam may change depends on different scenarios. It can’t guarantee that beam prediction functionality can always been tested. Therefore, we suggest not to skip beam prediction accuracy test.

Since both option 2 and option 3 are defining beam prediction requirement from two different aspects, RAN4 needs to discuss whether to choose one or both of them to define requirement.

Observation 9: RSRP prediction accuracy requirement can’t replace beam prediction accuracy.

Absolute RSRP prediction is regression and beam prediction is classification. RSRP prediction is more difficult than beam prediction, especially for BM-case 2 where RSRP prediction involves both time-domain and spatial domain prediction, there is only limited performance evaluation in 38.843.

If UE can correctly predict beam, even if RSRP is not so accurate, NW can send extra one dedicated RS for UE to measure the RSRP. If UE can’t predict correct beam, NW needs to configure many RSs for UE to measure different beams and resource overhead can’t be reduced. From our understanding, beam prediction performance is more important. RAN4 shall at least define beam prediction accuracy requirement. It’s FFS whether RSRP prediction accuracy needs to be defined.

Proposal 8: RAN4 needs to at least define beam prediction accuracy requirement and discuss which test metric to be chosen, e.g. based on beam index difference or RSRP difference.

      - 2.3 TCI state activation

UE will measure for set B first and then predict and report best TX beam index in set A, then NW can configure TCI state activation. There are two possibilities for the reported best predicted TX beam index:

- Case 1: best predicted TX beam is in set B.

- Case 2: best predicted TX beam is in set A but not in set B.

For case 1, UE knows the corresponding best RX beam since UE performs RX beam sweeping for all TX beams in set B. For case 2, UE may know or not know the best RX beam. It will depend on UE implementation. If the input of AI model and output of AI model are based on specific RX beam, UE can know the corresponding RX beam. If the input of AI model and output of AI model are based on RX beam sweeping, UE can first predict the L1-RSRP based on different TX-RX pairs, then UE sort L1-RSRP and report the best beam index. Then it’s possible for UE to know the corresponding best RX beam. If UE directly predict best TX beam index, it’s possible that UE didn’t know the RX beam index.

Observation 10: When best predicted TX beam is in set B, UE knows the best RX beam. When best predicted TX beam is in set A but not in set B, it’s still possible for UE to know the RX beam corresponding to the best TX beam.

Whether UE knows the RX beam or not will have impact on TCI state activation delay. Current known condition only considers whether UE has sent the report within a period and it didn’t consider whether UE has known the RX beam or not. We prefer to define UE capability to indicate whether UE can know the RX beam or not. Then the L1 reporting part in known condition for TCI state will be updated to:

The UE has sent at least 1 L1-RSRP report for the target downlink TCI state before the downlink TCI state switch command. Where, - the target TCI state in L1-RSRP report has been measured, or - the target TCI state in L1-RSRP report is predicted and UE support [RX beam known] capability

Proposal 9: RAN4 to discuss known TCI state condition for AI based beam indication.

- Conclusion

In this contribution, we provide the following proposals:

Observation 1: From TR, predicted best TX beam index and/or L1-RSRP can be based on full RX beam sweeping or specific RX beam.

Observation 2: Whether specific RX beams or full RX beam sweeping is assumed may lead to different best predicted TX beam index and predicted L1-RSRP value.

Observation 3: In legacy non-AI based L1-RSRP measurement, when RX beam sweeping is applied, it’s up to UE implementation to choose RX beam angle.

Observation 4: If different RX beam is assumed for prediction and measurement, the L1-RSRP and/or best TX beam index may be different in two steps, which will lead to wrong performance evaluation.

Proposal 1: RAN4 to discuss how to keep the RX beam Consistency between prediction and measurement for set A to correctly evaluate prediction performance.

Proposal 2: Suggest to use RSRP delta CDF-based simulation to determine the appropriate SNR level for deriving ground truth.

Proposal 3: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 4: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

Proposal 5: It’s more challenging to obtain accurate RSRP measurement in multiple-AOA test setup. it’s FFS whether extra margin needs to considered in RSRP accuracy requirement.

Observation 5: UE will report predicted RSRP/beam at T1 and measured best beam as ground truth at T2 to TE, it’s easy for UE to pass the test by cheating.

Proposal 6: For test metric option 1, refine the definition:

- Option 1: Absolute RSRP prediction accuracy is the RSRP difference between the predicted L1-RSRP of Top-1 predicted beam and measured L1-RSRP of the same beam.

  - FFS to define relative accuracy requirement.

Proposal 7: For test metric option 3, refine the definition:

- Option 3: successful rate for the correct beam prediction which is considered as measured L1-RSRP of top-1 predicted beams is larger than the measured L1-RSRP of the Top-1 measured beam – x dB.

Observation 6: From RAN1 agreement, both predicted beam index and predicted RSRP can be included in L1 report at least for BM case-1, similar as legacy.

Observation 7: For option 1, RSRP prediction accuracy focus about the RSRP difference for the same beam.

Observation 8: Both option 2 and 3 can be classified into beam prediction accuracy. Option 2 refers to beam index difference while option 3 refers to RSRP difference between predicted best beam and ideal best beam.

Observation 9: RSRP prediction accuracy requirement can’t replace beam prediction accuracy.

Proposal 8: RAN4 needs to at least define beam prediction accuracy requirement and discuss which test metric to be chosen, e.g. based on beam index difference or RSRP difference.

Observation 10: When best predicted TX beam is in set B, UE knows the best RX beam. When best predicted TX beam is in set A but not in set B, it’s still possible for UE to know the RX beam corresponding to the best TX beam.

Proposal 9: RAN4 to discuss known TCI state condition for AI based beam indication.

- Reference

- TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[2]  R4-2404573, Discussion on testability and interoperability issues for beam management, Xiaomi