3GPP TSG-RAN WG4 Meeting # 112	   R4-2411627

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Discussion on testability and interoperability for AI positioning

Source: 	Xiaomi

Agenda item:	8.17.3

Document for:	Discussion

Introduction

In Rel-18 RAN4 studied the testability and interoperability of NR AI/ML and the conclusions are included in TR 38.843 [1]. A new WI in Rel19 on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RAN #102 meeting [2] with the following RAN4 objectives :

- Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: - … - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - … - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. - Objective of Performance part WI - For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

And in last RAN4 meeting the discussion on this objective were initialized and WF was approved in [3].  In this contribution we will provide some considerations on the open issues under AI positioning scenario.

- Measurements for model input

- AI Model monitoring hypothesis

Discussion

Measurements for model input

In RAN1#117, the definition of sample-based measurement and path-based measurements are agreed.

RAN1#117 Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed). RAN1#116 Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation.

From RAN4 perspective, the legacy requirements defined in Rel16/17 are dependent on path-based measurements. For an example, PRS RSRPP accuracy below in existing spec [5] is based on the measurement reporting of first path and additional paths.

10.1.38.2.1 Absolute PRS RSRPP accuracy The absolute accuracy requirements for PRS-RSRPP measurement defined in Table 10.1.38.2.1-3 and Table 10.1.38.2.1-4 apply for the UE supporting supportedDL-PRS-ProcessingSamples [34]. Note: The requirements in this clause are derived based on two-tap channel defined in 38.101-4 Annex B.2.4 (a = 1, τd=0.45 µs and fD=5 Hz). Note: The requirements in this clause are derived based on the difference between the estimated PRS-RSRPP compared to the ideal PRS-RSRPP defined as RSRPPp∝kHkexpj2πDpkNIFFT2 Where: Hk is the effective channel frequency response (over REs occupied by PRS) measured without receiver noise. Dp is the exact delay of the p-th path in the channel model.

Observation 1: The legacy PRS measurement requirements up to Rel18[5] are based on the path-based measurement.

That is if the sample-based measurement as model input, from RAN4 perspective, the existing requirements for Pos shall be revisited.

Proposal 1: If RAN1 using sample-based measurement as model input for AI Pos, the existing accuracy requirements (e.g. PRS RSRPP) shall be restudied at least.

Model monitoring

RAN1 triggered batch of discussion on AI monitoring for positioning. One of important issues for AI monitoring is the necessary performance hypothesis which needs RAN4’s investigation.

R1#116bis Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation

For all AI positioning cases, no matter whether the target UE or LMF performing monitoring metric calculation (Option A or Option B), the evaluation on the validity of current AI models is dependent with AI pos performance threshold. For an instance, if the output error based the known data with label is larger than a specific value (e.g. 10%) the monitoring entity could trigger one report of performance outrage. Particularly, such works related to performance could be studied and decided by RAN4.

Observation 2: For AI model performance monitoring, the proper threshold used to justify AI model validity can be studied by RAN4.

For Case 1, if the monitoring performed by UE, RAN4 can further discuss the acceptable evaluation period to avoid the long time performance outrage because of invalid AI modelling. And such requirements can be also defined in TS38.133.

Observation 3: For Case 1 if AI model performance monitoring handled by UE, the requirements on model evaluation period can be defined in RAN4.

Therefore, we can propose that:

Proposal 2: RAN4 shall study the following necessary requirements for AI pos model monitoring. The specification impacts on TS38.133 or other 3GPP spec can be FFS also.

- Monitoring hypothesis and threshold

- Performance evaluation period

Conclusion

This contribution provided some initial analysis on the testability for AI positioning in Rel19

Observation 1: The legacy PRS measurement requirements up to Rel18[5] are based on the path based measurement.

Proposal 1: If RAN1 using sample-based measurement as model input for AI Pos, the existing accuracy requirements (e.g. PRS RSRPP) shall be restudied at least.

Observation 2: For AI model performance monitoring, the proper threshold used to justify AI model validity can be studied by RAN4.

Observation 3: For Case 1 if AI model performance monitoring handled by UE, the requirements on model evaluation period can be defined in RAN4.

Proposal 2: RAN4 shall study the following necessary requirements for AI pos model monitoring. The specification impacts on TS38.133 or other 3GPP spec can be FFS also.

- Monitoring hypothesis and threshold

- Performance evaluation period

References

- 3GPP TR 38.843 Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface

- RP-221348 SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #96, June 2022

- R4-2410570

- R4-2403712

- 3GPP TS 38.133 v18.5.0