3GPP TSG-RAN WG4 Meeting # 112	R4-2411628

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Xiaomi

Title: 	Discussion on testability and interoperability issues for CSI compression

Agenda Item:	8.17.4

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss testability and interoperability issues for CSI compression.

- Discussion

      - 2.1 Feasibility of deriving a test decoder for Option 3

Option 1: companies bring encoder + decoder set based on agreed parameters. RAN4 chooses one of the decoders and interested companies further check if an encoder can be trained with this decoder to obtain similar performance/complexity (or other evaluation criteria) Option 2 companies bring training data for encoder + decoder pair, interested companies train an encoder + decoder pair based on the aggregated dataset from all companies to check decoder derivation feasibility and performance/complexity (or other evaluation criteria) RAN4 agrees to work on option 1. Interested companies are invited to bring further feasibility analysis for Option 2

From our understanding, option 1 and option 2 listed above are both NW-first training method, where decoder training will be finished first, and then other company can train encoder accordingly later.

After completing training about decoder, specific information will be indicated to UE side. This information is crucial for enabling the UE side to train its CSI encoder.

For option 1, the necessary information includes a fully specified decoder and a training dataset, such as channel eigen vectors. For option 2, the necessary information includes latent information and a training dataset. The primary distinction between the two options lies in option 2 providing latent information whereas option 1 provides a fully specified decoder.

Observation 1: The main difference between options is that option 2 provide latent information while option 1 provide fully decoder information.

Latent information size is smaller than that of fully specified decoder. However, option 2 provide more limitation for performance generalization. Therefore, we prefer option 1.

Observation 2: Option 2 provide more limitation for performance generalization compared with option 1.

Issue 4-5: Option 3 standardization process - Proposals - Option 1: Flowchart below better represents the steps needed in RAN4, should be considered as baseline: - Option 2: Following changes should be made to the chart from R4-2405653(discussed in RAN4#110Bis) - Add a step, “identify target cases with specific test conditions” before step 1. - For step 1, suggest to limit the scope of potential model types. Transformer-based could be prioritized. - Add a step, “Define evaluation methodology” before step 4. - Suggest to align the understanding of step 5 on, whether only a single model architecture would be determined for 2-sided case at least for this release. - Discuss the necessity of step 8 and 9. - Option 3: other changes

      - 2.2 Standardization process for option 3

In last meeting, companies discussed standardization process for option 3. The proposed procedure is as below:

Fig.1 procedure for standardization process of option 3

Step-1

In post meeting email discussion, frameworks of CNN based encoder and decoder are provided. then RAN4 has finished step 1.

Step-2~4

The next coming step is step-2, e.g. database generation used for reference model training. Detail of step-2 will also have impact on step-3/step4.

In last meeting, there is discussion about whether training data set for deriving reference decoder is generated based on SLS or LLS. to make AI functionality more practical, we prefer to adopt SLS channel model for training. While for inference, CDL model can be applied. From our understanding, CDL is just a specific implementation of SLS channel model defined in 38.901. the channel characteristic didn’t change a lot when CDL model is applied for inference. There is no channel model mismatch issue. Therefore, we suggest to adopt SLS based simulation assumption to generate training data set.

Proposal 1: For step 2, Adopt SLS based simulation assumption to generate training data set.

For step-3 and step-4, UE will train the model to get model parameter according to training data set. Even if SLS based simulation assumption is applied, it can’t guarantee that data set generated by each company is aligned. If each company train the model based on the training data generated by each company itself, it’s better to align the training data set first. It’s FFS how to check training data set between companies is aligned or not. If it’s not easy to align data set, there are some other options.

- Option 1: company can first train the model based on its own data set and get the parameter. Then each company upload training data set to server respectively and download data from other companies. Company can further evaluate the performance by applying data set from other companies with trained model to check whether performance degrade or not in step 4.

- Option 2: the training data is mixed by data set from multiple companies. Then each company will train their own decoder based on the mixed data set.

Proposal 2: For step 2, RAN4 needs to discuss whether each company train reference model by data set generated individually or mixed data set from multiple companies.

No matter cross-check performance by different data set or mix data set from multiple companies, training data set format needs to be aligned first. RAN4 needs to discuss the training data format of AI model.

In last meeting, it’s agreed that training data set will be in terms of channel eigenvectors. Data format will consider the following aspect:

- Number of drops

- Number of UE

- number of TX port

- number of subband

- number of IQ

- Number of slot

For example, the data set format will be in the form of matrix with the following dimension:

Number of drops*Number of UE * number of TX port * number of subband *number of IQ*Number of slot

Proposal 3: For step 2, RAN4 needs to align the input training data format for data mixing or performance cross-checking.

Step-5

Suppose that the performance results have been provided, RAN4 needs to discuss how to align the results and decide the model parameter. Since there are too many detail parameters in a model, it’s challenging that all the parameters for all the companies are the same. In this case, if one decoder is chosen as a candidate to be fully specified, the decoder also needs to be accommodated to multiple encoders provided by different companies.

As shown below in Fig.2, each company can provide its preferred encoder. Different encoder-decoder pair will be used to derive different performance. Then the performance degradation for multiple encoder-decoder pair needs to be evaluated. The candidate fully specified decoder should satisfy the condition that performance degradation due to different encoder-decoder pair will be smaller than a threshold.

Fig.2 different encoder-decoder pair

Proposal 4: For step 5, the candidate fully specified decoder should satisfy the condition that performance degradation due to different encoder-decoder pair will smaller than a threshold.

- Conclusion

In this contribution, we provide the following proposals:

Observation 1: The main difference between options is that option 2 provide latent information while option 1 provide fully decoder information.

Observation 2: Option 2 provide more limitation for performance generalization compared with option 1.

Proposal 1: For step 2, Adopt SLS based simulation assumption to generate training data set.

Proposal 2: For step 2, RAN4 needs to discuss whether each company train reference model by data set generated individually or mixed data set from multiple companies.

Proposal 3: For step 2, RAN4 needs to align the input training data format for data mixing or performance cross-checking.

Proposal 4: For step 5, the candidate fully specified decoder should satisfy the condition that performance degradation due to different encoder-decoder pair will smaller than a threshold.

- Reference

- TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface