3GPP TSG-RAN WG4 Meeting # 112	R4-2411629

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Xiaomi

Title: 	Discussion on RRM impact on Rel-19 FeMIMO

Agenda Item:	8.18.3

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the working scope for Rel-19 FeMIMO.

- Discussion

The latest WID for Rel-19 FeMIMO is as below[1]:

The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

For objective 1, in legacy, RAN4 only defines L1-RSRP requirement for Periodic Reporting, Semi-Persistent Reporting and Aperiodic Reporting. In Rel-19, RAN1 will define UE-initiated/event-driven beam reporting. RAN4 need to add UE-initiated/event-driven beam reporting type in Measurement Reporting Requirements. The reporting delay part needs to be updated. The measurement period, scheduling restriction and measurement restriction requirement will be the same for both event-triggered reporting and other reporting type.

Proposal 1: UE-initiated/event-driven beam management will have impact on RAN4 RRM requirement.

For objective 2, RAN1 will Specify CSI support for up to 128 CSI-RS ports, which will have no impact on RAN4 RRM.

For objective 3, for non-ideal synchronization and backhaul, there may be time and frequency offset for multiple TRPs, which will degrade CJT performance. UE reporting enhancement for CJT deployments is related to improve data reception performance. Therefore, there is no RAN4 RRM impact.

For objective 4, RAN1 will specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, there is no RAN4 RRM impact.

Proposal 2: CSI codebook design will not have impact on RAN4 RRM.

Proposal 3: UE reporting enhancement for CJT deployments will not have impact on RAN4 RRM.

Proposal 4: Non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions will not have impact on RAN4 RRM.

For objective 5, for asymmetric DL sTRP/UL mTRP deployment scenarios, there will be one anchor DL TRP and multiple UL-only TRPs and they are not co-located. In legacy, each TRP has both downlink and uplink transmission. While for UL-only TRP, there is no downlink. The scenario is as below:

Fig.1 asymmetric DL sTRP/UL mTRP deployment scenarios

From our understanding, in legacy RAN4 has defined following requirement for uplink transmission for mTRP:

- UL timing requirement

- UL TCI state activation, including PL-RS calculation

Currently, RAN1 mainly discuss TX power control, which is related to PL-RS in UL TCI state activation. TCI state activation scheme will re-use Rel-17 and Rel-18 based unified TCI state switch, which is shown below:

Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

Observation 1: For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework,

Since UL-only TRP didn’t have downlink, pathloss can only be calculated by PL-RS from anchor DL TRP and extra pathloss offset will be considered. In RAN4, the current applicability for PL-RS for unified UL TCI state switch is defined as below:

- 8.16 Active uplink TCI state switching delay for unified TCI ### 8.16.1 Introduction …… PL-RS may be associated with or included in UL TCI state or joint TCI state. The requirements in this clause shall apply if the following conditions are met: - PL-RS is identical to source RS in UL TCI state or joint TCI state - PL-RS and source RS in UL TCI state or joint TCI state are QCL-Type D

As shown above, in legacy, PL-RS and source RS in UL TCI state are the same or QCL-D. while for UL TCI activation for UL-only TRP, PL-RS and source RS in UL TCI state may not satisfy the condition anymore. The reason is that UL-only TRP and DL anchor TRP are not co-located, the spatial relationship of RS in UL TCI state and PL-RS may not be kept the same. The applicability of PL-RS may need some modification.

Observation 2: For the asymmetric DL sTRP/UL mTRP deployment scenario, PL-RS and source RS in UL TCI state may not be the same or QCL-typeD.

The legacy UL TCI activation procedure is tied to DL beam management in advance, specifically through L1-RSRP measurement and reporting. Different delay requirements are specified for known and unknown TCI states. In the case of a known TCI, the measurement report is based on DL-RS. However, for UL-only TRP, DL-RS is not available. One potential approach is to use SRS for UL beam management. However, RAN4 has not yet defined any requirements related to SRS. It’s FFS how to consider SRS as reference signal when defining UL TCI state activation requirement. Additionally, it remains to be decided whether DL-RS from the anchor DL RS can be utilized to define UL TCI state activation requirements.

Observation 3: Legacy UL TCI state activation requirement is define based on DL-RS. While for UL-only TRP, there is no DL-RS. It’s FFS how to consider SRS as reference signal when defining UL TCI state activation requirement. It’s FFS whether DL-RS from anchor DL RS can be used to define UL TCI state activation requirement.

Another crucial aspect involves defining UL timing. In the legacy sDCI scenario, a single timing advance (TA) is typically considered. However, in the context of UL-only TRP and non-co-located DL anchor TRP, ensuring timing offset within CP is challenging.

In RAN1 #117 meeting chairman note, RAN1 is currently exploring the feasibility of extending the single TA approach to accommodate two TAs to enable this functionality.

Proposal 3.1: To fulfil the asymmetric DL sTRP/UL mTRP deployment scenarios, support two TAs for single DCI based multi-TRP/panel and single TRP. - Reuse Rel-18 specification of two TA for multi-DCI based multi-TRP/panel and remove the restriction that coresetPoolIndex needs to be configured. Companies are encouraged to consider above for further discussion in RAN1#118.

Observation 4: RAN1 is currently exploring the feasibility of extending single TA to two TAs to enable this functionality.

When two TA will be considered, the DL reference timing for UL-only TRP in sDCI scenario may need to be updated. RAN4 can discuss whether reference timing for mDCI based multi-TRP operation with two TAs can be re-used or any update is needed.

Proposal 5: Asymmetric DL sTRP/UL mTRP will have impact on RAN4 RRM.

- Conclusion

In this contribution, we provide the following proposals：

Proposal 1: UE-initiated/event-driven beam management will have impact on RAN4 RRM requirement.

Proposal 2: CSI codebook design will not have impact on RAN4 RRM.

Proposal 3: UE reporting enhancement for CJT deployments will not have impact on RAN4 RRM.

Proposal 4: Non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions will not have impact on RAN4 RRM.

Observation 1: For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework,

Observation 2: For the asymmetric DL sTRP/UL mTRP deployment scenario, PL-RS and source RS in UL TCI state may not be the same or QCL-typeD.

Observation 3: Legacy UL TCI state activation requirement is define based on DL-RS. While for UL-only TRP, there is no DL-RS. It’s FFS how to consider SRS as reference signal when defining UL TCI state activation requirement. It’s FFS whether DL-RS from anchor DL RS can be used to define UL TCI state activation requirement.

Observation 4: RAN1 is currently exploring the feasibility of extending single TA to two TAs to enable this functionality.

Proposal 5: Asymmetric DL sTRP/UL mTRP will have impact on RAN4 RRM.

- Reference

[1] RP-234007, New WID: NR MIMO Phase 5, Samsung