3GPP TSG-RAN WG4 Meeting #112	R4-2411632

Maastricht, Netherlands, 19th -23rd August, 2024

Source:	Qualcomm Incorporated

Title:	MPR applicability for FR1 intra-band UL CA

Agenda item:	8.1.1.2.2

Document for:	Approval

# 1.	Introduction

In RAN4#111 there was discussion on which MPR tables apply for FR1 intra-band ULCA when intra-band contiguous UL CA is configured but only 1 CC is activated. In this paper we further present our views on this topic.

# 2. 	Discussion

In RAN4#111 a WF was created that stated the following [1]:

## 2.1. MPR applicability for FR1 intra-band contiguous UL CA

Way forward:

- For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated.

  - FFS: MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C.

  - FFS: MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated.

  - FFS: MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

  - FFS whether spurious emissions/ACLR/SEM are kept for aggregated CBW for CA

In RAN4#111 it was discussed that for intra-band contiguous UL CA if only 1 CC is activated then for PC3 and PC2 scenarios the single CC MPR tables should apply. This issue could not be resolved fully at the last meeting and was left for further discussion. We see no reason why the single CC MPR tables cannot be used in the above scenarios. The single CC MPR tables have lower MPR values compared to the intra-band contiguous UL CA MPRs. The use of the single CC MPR values will enable the UL to transmit more power there by increasing its throughput. The point we believe that warranted further discussion was that the MPR in tables 6.2.2-1, 6,2D.2-1 and 6.2.2-2 have the following note:

- Applicable for a UE indicating support for UE capability [powerBoostRel18] or [powerBoostRel18TS] and if the IE [powerBoostPi2BPSKRel18] is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass]

This note indicates that the power boosting is possible for UE indicating [powerBoostRel18] or [powerBoostRel18TS] capability and the corresponding MPR tables shows that power boosting is possible for PC3 and PC2 QPSK and Pi/2 BPSK modulations. To ensure that power boosting is allowed the corresponding configured power requirements have to be referenced along with each MPR table. Omission of the configured power requirements will leave the use of the power boosting feature ambiguous as certain Tx UL configurations such as carrier aggregation does not have the power boosting feature. Therefore, to remove this ambiguity we propose that the following be agreed:

- For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated.

  - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C, along with configured Tx power requirements for 6.2.4

  - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated, along with configured Tx power requirements for 6.2D.4

  - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent, along with configured Tx power requirements for 6.2.4

Proposal 1: For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated and refer to the following MPR tables and corresponding configured Tx power requirements.

  - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C, along with configured Tx power requirements for 6.2.4

  - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated, along with configured Tx power requirements for 6.2D.4

  - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent, along with configured Tx power requirements for 6.2.4

We believe that though only 1 CC is activated the aggregated BW for CA should be used for evaluating metrics such as spurious emissions, SEM and ACLR as this is in keeping with what is done at present.

Proposal2: When only 1 CC is activated the aggregated BW for CA should be used for evaluating metrics such as spurious emissions, SEM and ACLR.

For intra-band non-contiguous UL CA the scheme to be used when only 1 CC is activated was left for further discussion [1]. From [2] we see that according to section 6.2A.2.2.0 the following is mentioned for UEs indicating and not indicating dualPA-Architecture.

#### 6.2A.2.2	UE maximum output power reduction for Intra-band non-contiguous CA

##### 6.2A.2.2.0	General

For intra-band non-contiguous CA, the allowed Maximum Power Reduction (MPR) for the maximum output power is specified into 2 types: MPR to meet -30dBm/MHz and -13dBm/MHz. The UE determins the MPR type as follows:

For UE indicating dualPA-Architecture supported

If OR (LCRB1 = 0, LCRB2 = 0)

MPR defined in Table 6.2.2-1 and Table 6.2.2-2 for PC3 and PC2 UE respectively

Else If AND( FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPR defined in Clause 6.2A.2.2.2.1 and Clause 6.2A.2.2.2.2 for PC3 and PC2 UE respectively.

Else

MPR defined in Clause 6.2A.2.2.1.1 and Clause 6.2A.2.2.1.2 for PC3 and PC2 UE respectively.

For UE without indicating dualPA-Architecture supported

If OR( LCRB1 = 0, LCRB2 = 0 )

For PC3 UE, MPR defined in Table 6.2.2-1, except for B < 9 MHz where 5.5 dB MPR is used;

For PC2 UE without indicating TxD, MPR defined in Table 6.2.2-2 is used, except for B < 11.52 MHz where 6.5 dB MPR is used;

For PC2 UE indicating TxD, MPR defined in Table 6.2D.2-1 is used, except for B < 11.52 MHz where the maximum value between 6.5 dB and MPR defined in Table 6.2D.2-1 is used.

According to [2] for a UE indicating or not indicating dualPA-Architecture for intra-band non-contiguous CA when either LCRB1=0 or LCRB2=0 the single CC MPR tables are used. As the RB determination occurs at scheduling it is seen that for the non-contiguous CA the standard [2] already accounts for the scenario where only 1 CC is scheduled. In the above text the exception stated for the cases when B< 9MHz or 11.52MHz and MPR is set to either 5.5 dB or 6.5 dB for  PC3 and PC2 respectively is to meet the -30 dBm/MHz emissions requirement. These exceptions were added based on measurement work done in [3]

Observation 1: For PC3 and PC2 intra-band non-contiguous CA the standard already accounts for using the single CC MPR tables when only one CC is scheduled.

Proposal 3: For PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required.

Observation2: The exceptions stated in section 6.2A.2.2.0 for the cases when B< 9MHz or 11.52MHz and MPR is set to either 5.5 dB or 6.5 dB for PC3 and PC2 respectively is to meet the -30 dBm/MHz emissions requirement.

# Conclusion

In this paper we present further views on MPR applicability for FR1 intra-band UL CA and make the following observations and proposals:

Proposal 1: For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated and refer to the following MPR and corresponding configured Tx power requirements.

  - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C, along with configured Tx power requirements for 6.2.4

  - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated, along with configured Tx power requirements for 6.2D.4

  - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent, along with configured Tx power requirements for 6.2.4

Proposal 2: When only 1 CC is activated the aggregated BW for CA should be used for evaluating metrics such as spurious emissions, SEM and ACLR.

Observation 1: For PC3 and PC2 intra-band non-contiguous CA the standard already accounts for using the single CC MPR tables when only one CC is scheduled.

Proposal 3: For PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required.

Observation2: The exceptions stated in section 6.2A.2.2.0 for the cases when B< 9MHz or 11.52MHz and MPR is set to either 5.5 dB or 6.5 dB for PC3 and PC2 respectively is to meet the -30 dBm/MHz emissions requirement.

# References

[1] R4-2410589, WF on power domain enhancements, May 20th – May 24th , 2024

[2] TS38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V18.6.0

[3] R4-2204225, 1CC fall-back MPR for NC UL CA with 1LO architecture, February 21st – March 3rd, 2022