3GPP TSG-RAN WG4 Meeting #112	R4-2411633

Maastricht, Netherlands, 19th -23rd August, 2024

Source:	Qualcomm Incorporated

Title:	Introduction of NR band n68

Agenda item:	7.16.2

Document for:	Approval

# 1.	Introduction

In RAN#104 a new WI was created to introduce band n68 into NR. In this paper we present our views on the work that needs to be done to introduce NR band n68.

# 2. 	Discussion

In RAN#104 there was a new WI [1] created to introduce band n68 to NR. The objectives are detailed in [1] were:

## 1.1 Objective

The core objectives of this WI are:

- Specify a new NR FDD operating band n68 (the uplink band for this new band is 30 MHz, 698-728 MHz; the downlink band is 30 MHz, 753-783 MHz; the duplex gap is 25 MHz, 728-753 MHz) to support subcarrier spacing of 15 kHz for 5 MHz channel bandwidth, and subcarrier spacing of 15 kHz and 30 kHz for 10 and 15 MHz channel bandwidths.

  - Only symmetric bandwidths are supported.

- Specify system parameters and RF characteristics of the new band.

- Address potential BS and UE co-existence issues, if any.

- Update the related technical specifications to include support for the new band

- Specify the UE RF requirements for the new bands based on 2 Rx operation.

Note: This new NR band will be introduced in a REL-independent way starting from REL-15.

B68 has already been introduced in LTE and the following specifications are in [2]. Currently B68 has to meet NS_26 and NS_36 the specifications for which are outlined below:

The above specifications were derived for LTE. To introduce band 68 to NR conformance to NS_26 and NS_36 specifications would have to be verified either by measurements and/or simulations for the additional NR specifications of SCS=30KHz and CPOFDM waveforms along with the LTE/NR specific values of SCS=15kHz and DFTS waveforms.

In-order to facilitate uniformity in results we suggest using the following parameters for simulations.

  - PA model calibration

    - DFT-s-OFDM QPSK 20MHz

    - 100RB0

    - 4dB post PA loss

    - 1dB MPR

  - Carrier Leakage: 28dBc

  - IQ Image: 28dBc

  - CIM3: 60dBc

  - EVM: 17.5%

  - SCS: 15kHz, 30kHz

  - Waveform type: DFT-s-OFDM, CPOFDM

  - ACLR: 30dB for PC3, 31dB for PC2

Proposal 1: To introduce band 68 to NR NS_26 and NS_36 specifications must be verified not only with SCS=15kHz and DFTS waveforms but also with the NR specific metrics of SCS=30kHz and CPOFDM waveforms.

Proposal2: For simulations use the following parameters:

  - PA model calibration

    - DFT-s-OFDM QPSK 20MHz

    - 100RB0

    - 4dB post PA loss

    - 1dB MPR

  - Carrier Leakage: 28dBc

  - IQ Image: 28dBc

  - CIM3: 60dBc

  - EVM: 17.5%

  - SCS: 15kHz, 30kHz

  - Waveform type: DFT-s-OFDM, CPOFDM

  - ACLR: 30dB for PC3, 31dB for PC2

# Conclusion

In this paper we further discuss what NS cases need to be re-verified to introduce band 68 into NR. Our   proposals are as follows:

Proposal 1: To introduce band 68 to NR NS_26 and NS_36 specifications must be verified not only with SCS=15kHz and DFTS waveforms but also with the NR specific metrics of SCS=30kHz and CPOFDM waveforms.

Proposal2: For simulations use the following parameters:

  - PA model calibration

    - DFT-s-OFDM QPSK 20MHz

    - 100RB0

    - 4dB post PA loss

    - 1dB MPR

  - Carrier Leakage: 28dBc

  - IQ Image: 28dBc

  - CIM3: 60dBc

  - EVM: 17.5%

  - SCS: 15kHz, 30kHz

  - Waveform type: DFT-s-OFDM, CPOFDM

  - ACLR: 30dB for PC3, 31dB for PC2

# References

[1] RP-241664, New WID on introduction of NR band n68, RAN#104, June 17th – June 20th, 2024

[2] TS36.101, User Equipment (UE) radio transmission and reception; V18.6.0