3GPP TSG-RAN WG4 Meeting #112	R4-2411634

Maastricht, Netherlands, 19th -23rd August, 2024

Source:	Qualcomm Incorporated

Title:	Introduction of 3Tx to NR specifications

Agenda item:	8.18.2

Document for:	Approval

# 1.	Introduction

In RAN#104 a new WI created to introduce specifications for single CC 3Tx [1]. In this paper we present our preliminary views on the work that needs to be done to introduce 3Tx into the NR specifications

# 2. 	Discussion

The 3Tx WI was started in RAN1#116 and to date the following 1-layer(1L-1, 1L-2, 1L-3), 2-layer(2L-1, 2L-2, 2L-3) and 3-layer(3L-1) TPMIs have been defined for 3Tx:

Also, it has been stated in the WI [1] that UL full power transmission mode1 and mode2 will not be supported. Therefore, only UL full power mode 0 will be supported in 3Tx. Furthermore, it has been agreed that the above TPMIs will support both Rel-15 power scaling and UL full power mode. For power scaling the factor 1/3  will be used while for full power mode the scaling factor will be set to 1.0. Also, it has been stated in the WI [1] that these 3Tx TPMIs will support only non-coherent operation.

To introduce 3Tx to the RAN4 standards the ULFPTx mode 0 section of table 6.2.D.1-3 will have to be updated once the indices for the above TPMI’s are known as shown below.

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 NOTE1 | 2 |
|  |  |  |  |  | 4 NOTE4 | 13 |
|  |  |  | CP-OFDM NOTE3 | 2 | 4 | 6 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 NOTE1 | 0 or 1NOTE2 |
|  |  |  |  |  | 4 NOTE4 | 4, 5, 6 ,7 or 4, 5, 6 ,7, 8, 9, 10, 11NOTE2 |
|  |  |  | CP-OFDM | 2 | 4 | 1 or 0, 1, 2, 3, 4 ,5NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 NOTE1 | 0,1 |
|  |  |  | DFT-s-OFDM | 1 | 3 | 1L-1, 1L-2, 1L-3 |
|  |  |  | CP-OFDM | 2 | 3 | 2L-1, 2L-2, 2L-3 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. NOTE 3: For PUSCH configured with ul-FullPowerTransmission set to fullpowerMode1, all the transmitter requirement for CP-OFDM based modulation does not need to be verified if the requirements for 2-layer or 4-layer UL MIMO according to Table 6.2D.1-2 has been verified. NOTE 4: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 4. |  |  |  |  |  |  |

Proposal 1: Update table 6.2.D.1-3 with the 3 Tx mode-full power TPMIs as given below once TPMIs numbers are allocated to them

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 NOTE1 | 2 |
|  |  |  |  |  | 4 NOTE4 | 13 |
|  |  |  | CP-OFDM NOTE3 | 2 | 4 | 6 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 NOTE1 | 0 or 1NOTE2 |
|  |  |  |  |  | 4 NOTE4 | 4, 5, 6 ,7 or 4, 5, 6 ,7, 8, 9, 10, 11NOTE2 |
|  |  |  | CP-OFDM | 2 | 4 | 1 or 0, 1, 2, 3, 4 ,5NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 NOTE1 | 0,1 |
|  |  |  | DFT-s-OFDM | 1 | 3 | 1L-1, 1L-2, 1L-3 |
|  |  |  | CP-OFDM | 2 | 3 | 2L-1, 2L-2, 2L-3 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. NOTE 3: For PUSCH configured with ul-FullPowerTransmission set to fullpowerMode1, all the transmitter requirement for CP-OFDM based modulation does not need to be verified if the requirements for 2-layer or 4-layer UL MIMO according to Table 6.2D.1-2 has been verified. NOTE 4: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 4. |  |  |  |  |  |  |

To enable 3Tx feature to be used in a broad range of applications it should be capable of supporting both PC3 and PC2. Also, MPR tables need to be defined for 3Tx operation for handheld UEs as well as CPE/FWA devices either using measurements or simulations.

Proposal 2: Allow 3Tx feature to support PC3 and PC2.

Proposal 3: Create MPR tables for 3Tx for both UE handheld and CPE/FWA devices. Use 10 dB antenna-antenna isolation for UE handhelds and 20 dB antenna-antenna isolation for CPE/FWA devices.

# Conclusion

In this paper present our initial views on 3 Tx and what specifications need to be established to introduce it into the RAN4 standard. Based on this we make the following proposals:

Proposal 1: Update table 6.2.D.1-3 with the 3Tx mode-full power TPMIs as given below once TPMIs numbers are allocated to them

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 NOTE1 | 2 |
|  |  |  |  |  | 4 NOTE4 | 13 |
|  |  |  | CP-OFDM NOTE3 | 2 | 4 | 6 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 NOTE1 | 0 or 1NOTE2 |
|  |  |  |  |  | 4 NOTE4 | 4, 5, 6 ,7 or 4, 5, 6 ,7, 8, 9, 10, 11NOTE2 |
|  |  |  | CP-OFDM | 2 | 4 | 1 or 0, 1, 2, 3, 4 ,5NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 NOTE1 | 0,1 |
|  |  |  | DFT-s-OFDM | 1 | 3 | 1L-1, 1L-2, 1L-3 |
|  |  |  | CP-OFDM | 2 | 3 | 2L-1, 2L-2, 2L-3 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. NOTE 3: For PUSCH configured with ul-FullPowerTransmission set to fullpowerMode1, all the transmitter requirement for CP-OFDM based modulation does not need to be verified if the requirements for 2-layer or 4-layer UL MIMO according to Table 6.2D.1-2 has been verified. NOTE 4: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 4. |  |  |  |  |  |  |

Proposal 2: Allow 3Tx feature to support PC3 and PC2.

Proposal 3: Create MPR tables for 3Tx for both UE handheld and CPE/FWA devices. Use 10 dB antenna-antenna isolation for UE handhelds and 20 dB antenna-antenna isolation for CPE/FWA devices.

# References

[1] RP-234007, New WID: NR MIMO Phase 5, RAN#102, December 11th – December 15th , 2023