3GPP TSG-RAN WG4 Meeting #112                                                                R4-2411635

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:	Further discussion on testability and interoperability issues for beam management

Agenda item:			8.17.2

Document for:	Approval

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two confirmed use cases (i.e., beam management and positioning accuracy enhancements) [2].

Particularly for the use case of beam management, it is required to provide specification support for DL TX beam prediction for both UE-sided and NW-sided model, and correspondingly necessary RAN4 core requirements for this use case and LCM procedure including performance monitoring [2], as follows:

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In RAN4#110, there were discussions on core requirements for AI beam management including, measurement accuracy for beam measurement reporting, metrics/KPIs and other testability issues for AI-BM requirements and tests, while there is no agreement achieved yet [4]. In RAN4#110bis, for the metrics/KPIs for Beam Management requirements/tests, the following agreement is achieved:

Issue 2-1: Metrics/KPIs for Beam Management requirements/tests Agreement: Companies are invited to provide inputs/proposals to refine the definition of RSRP accuracy Hold on the discussions for concrete test metrics until RAN1 had conclusions on the schemes.

In RAN4#111, for reported measurements and ground truth, test setup and datasets for training/testing, the following agreements are achieved:

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded Issue 2-5: Test setup needs Agreement: - maximum number of set B Tx beams that test system should be able to emulate: [8-16] - maximum number of set A Tx beams that test system should be able to emulate: [64-128] - FFS on AoAs - UE rotation during the test: FFS - UE rotation/repositioning between different tests: Yes Issue 2-6: Datasets for training/testing Agreement: - Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training - FFS on proposals to augment training data to avoid overfitting of UE models to the test environment - RAN4 to strive to make the test conditions similar to field deployment conditions

Based on the outcome of the study objectives captured in TR 38.843 [1] and other RAN1 progress, we would like to discuss on core requirement and testability issues for beam management in this contribution.

# 2. General handling for AI-BM core requirement

After several meeting’s discussion, some progresses has been achieved on AI-beam management, but most of the progresses are related to the ground truth of predicted RSRP and the testing environment for the beam prediction performance.

Our understanding is we need to discuss the general scope for the expected RAN4 impact for AI-BM. Based on our contribution [9], we suggest to use a similar table to firstly settle down the expected RAN4 requirement impact for AI-BM, especially by concluding that some features will not have RAN4 impact.

Proposal 1: RAN4 shall firstly agree on the expected RAN4 requirement impacts for AI-BM, by separating the discussion by (1) NW and UE-sided model; (2) different sub-features introduced in RAN1/2, including: Data collection, Inference, Perf. monitoring, LCM and additional assistance information (for UE-sided model only).

Proposal 2: The following table can be used as the template for collecting views from companies.

|  | New Sub-Feature introduced in RAN1/2 | Whether/How RAN4 requirement impact is expected? (e.g., clause # in TS 38.133 impacted/new requirement needed/no impact) |
| --- | --- | --- |
| NW-sided model | Data collection |  |
|  | Inference |  |
|  | Perf. monitoring |  |
|  | LCM |  |
| UE- sided model | Data collection |  |
|  | Additional assistance information |  |
|  | Inference |  |
|  | Perf. monitoring |  |
|  | LCM |  |

# 3. RAN4 Core Requirement Impact for NW-Sided Model

For NW-sided model for AI-BM, we provide our analysis on RAN4 core requirement impact for NW-sided model, for the procedure of data collection, inference, performance monitoring and LCM.

## 3.1 Data collection for NW-sided model (for training and inference)

In RAN1#116bis, RAN1 agree to using existing CSI framework for configuration of Set A and B as starting point. From L1-RSRP reporting content perspective, as provided in TR 38.843 [1], regarding data collection for NW-side AI/ML model, options for the contents of collected data are provided by considering the different reported contents and larger number of beams.

Particularly for model inference, for data collection for NW-sided model inference, the enhancement on UE beam reporting (i.e., the report of more than 4 beam related information) in L1 signaling could be introduced for Rel-19 AI-BM.

On the other hand, for data collection for NW-side AI/ML model of BM-Case1 and BM-Case2, the potential approaches for overhead reduction are provided in TR 38.843 [1], while the following agreement in RAN1#117 could be considered as the way for overhead reduction: since the beam indexes are given accordingly to the configured measurement report, only the differential RSRP are needed.

Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information - Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

Furthermore, we see the possibilities of more works for overhead reduction, which could be specified in the normative work.

From signaling perspective, both L1 signaling and higher-layer signaling to report the collected data are considered in TR 38.843.

Regarding data collection for NW-side AI/ML model of BM-Case1 and BM-Case2, the following reporting signalling for beam-specific aspects maybe applicable: - L1 signalling to report the collected data - Higher-layer signalling to report the collected data - At least not applicable to AI/ML model inference - Note1: higher layer signalling design is up to RAN2 - Note2: Whether each signalling applicable to each LCM purpose is a separate discussion - Note3: The legacy signalling principle (e.g. RSRP reporting for L1) can be re-used

And from RAN2#125bis, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW-side AI/ML model inference, and RRC-based measurement and reporting for NW-sided training:

Agreements from RAN2#125bis:

- RAN2 to consider an RRC configuration to configure radio measurements and the related reporting to enable data collection for NW-side training

- For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference.  FFS if further enhancements are needed

And for NW-sided data collection for training, RAN2 is still considering how to enhance MDT for data collection framework:

Agreements

1	For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches

2	We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

3	RAN2 supports enhancements to MDT for data collection framework for training.  FSS Whether to enhance logged or immediate MDT

We can observe that for data collection for NW-side AI/ML model (for BM-Case1 & 2), L1 measurement framework will be used for model inference, and MDT-based measurement/reporting will be used for training.

Based on the above analysis, RAN4 shall introduce the necessary core requirement on supporting the data collection for NW-sided model.

Proposal 3: RAN4 shall introduce the necessary L1-RSRP reporting requirement on supporting data collection for NW-side AI/ML model inference/training (for both BM-Case1 & 2), by considering:

- L1 measurement/reporting for inference: e.g., beam reporting for more than 4 beam in L1 signaling, and other new reporting format (including potential granularity step);

- Potential enhancement on MDT-based measurement/reporting for training.

Another issue related is whether or not measurement accuracy needs to be improved to enable better AI/ML performance. In RAN4#110, the following options were given:

Issue 2-2: Measurement accuracy - Proposals - Option 1: RAN4 should consider tighter accuracy requirements for existing measurements (e.g. RSRP) if they are used/reported for AI/ML (e.g. training, inference, etc) - Option 2: RAN4 should study the impact of measurement accuracy on performance before discussing any possible tightening - Option 3: Accuracy requirements cannot be tightened - Option 4: Others

From system design perspective for L1 beam reporting accuracy for NW-sided model, the tradeoff between higher achievable system performance (i.e., the necessity of introducing higher measurement accuracy requirement) and implementation feasibility shall be balanced, while RAN1 may focus on the former aspect and the latter part shall be within RAN4 scope/expertise.

In existing RAN4 specification, FR2 RSRP measurement absolute/relative accuracy requirements differ under normal/extreme conditions, and under different input level conditions: e.g., 6dB is assumed for SS-RSRP intra-frequency accuracy requirement for normal condition without Noc. Additionally, the absolute accuracy test requirement is further relaxed by adding Gmin and Gmax (i.e,. the minimum and maximum UE gain values from Table B.2.1.5.1-1 in TS38.133), which is Gmin = -10dB and Gmax = +20dB for FR2 PC3 UE. Therefore, the following proposal is given:

Proposal 4: For tightening measurement accuracy requirement for AI-BM NW-sided model:

- RAN4 shall only focus on the feasibility part of tightened measurement accuracy;

- Without any particular request, RAN4 shall reuse the accuracy requirement from Rel-15 L1-RSRP for both absolute and relative RSRP reporting.

## 3.2 Inference for NW-sided model

For the model inference for BM-Case1 and BM-Case2 with a NW-side AI/ML model, the following potential specification impacts from physical layer aspects are given in TR 38.843 [1]:

For BM-Case1 and BM-Case2 with a NW-side AI/ML model: - L1 beam reporting enhancement for AI/ML model inference: - UE to report the measurement results of more than 4 beams in one reporting instance - Other L1 reporting enhancements can be considered For BM-Case 2: - Reporting information about measurements of multiple past time instances in one reporting instance. Notes: Only applicable to NW-side AI/ML model. The potential performance gains of measurement reporting should be justified by considering UCI payload overhead.

Accordingly, we see no RAN4 impact expected for model inference (except the relevant data collection as provided in Section 2.1.2), because the model inference is implemented in gNB, in which the AI/ML inference results are also used by gNB to conduct beam management.

From the below RAN2 agreement, it can be observed that there is no new signaling introduced for gNB-sided model inference (depending on further RAN1 input).

Agreements from RAN2#125bis:

- There is no specification impact associated to gNB-side model inference, depending on further RAN1 input.

It should be noted that the potential enhancement of “measurements of multiple past time instances in one reporting instance” shall also be categorized as data collection for inference, and even if this mechanism is introduced in Rel-19, it should also not introduce any new RAN4 requirement for model inference.

Proposal 5: No RAN4 impact is expected for NW-side AI/ML model inference for BM-Case1 & 2, except data collection for model inference.

## 3.3 Performance monitoring for NW-sided model

Similar to model inference with a NW-side AI/ML model, the performance monitoring shall also be gNB implementation-based, i.e., the monitoring can be configured by the NW based on the adopted performance metric. Till now, there is no clear conclusion that there will be RAN2 specification impact for gNB-side model monitoring, i.e., the following agreement from RAN2#126:

Agreements for NW-sided model for Beam Management in RAN2#126:

1	For the network-side model, required network side additional condition is left up to the network implementation

2	RAN2 will wait for RAN1 for any required UE side additional conditions.

3	For network-sided model for BM use case, RAN2 confirms that UE inputs for inference at network-sided model will rely on L1 signaling, RAN2 will not further spend time on this aspect.

4	The gNB is responsible for monitoring its own performance.  RAN2 will work on RAN2 specifications enhancements associated to gNB-side model monitoring, only based on RAN1 inputs, if any

Proposal 6: FFS RAN4 impact from NW-side AI/ML model performance monitoring for BM-Case1 & 2 (depending on RAN1/2 input).

## 3.4 LCM for NW-sided model

In WID [2], it is expected that RAN2 will introduce the necessary LCM related aspects, enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback. However, as given in RAN2#125bis, it has confirmed the roles of UE in the NW-sided LCM procedures: i.e., For NW-sided model for AI-BM, UE will neither be informed about any LCM decision, nor be involved in any LCM decision making, except being configured for providing the required measurement/data.

Agreements from RAN2#125bis

1	RAN2 confirms that UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

2	RAN2 confirms that UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

3	RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management).

Considering RAN4 requirement shall focus on UE-relevant signaling procedure, there will be no RAN4 impact expected for LCM for NW-sided model.

Proposal 7: No RAN4 impact is expected for NW-side AI/ML model LCM for BM-Case1 & 2.

# 4. RAN4 Core Requirement for Supporting UE-Sided Model

For UE-sided model for AI-BM, we provide our analysis on RAN4 core requirement impact for UE-sided model, for the procedure of data collection, additional assistance information, inference, performance monitoring and LCM.

## 4.1 Data collection for UE-sided model

For data collection for UE-sided model, the following solutions are provided [1]. (Note: OTT server is also called as “server for UE-side data collection” based on RAN2 discussion.)

| 1a) | 1b) |
| --- | --- |
| 2. Data collection via CN | 3. Data collection via OAM |

Based on the above contents from TR 38.843, the potential interaction(s) between UE and NW for data collection for UE-sided model could be: (1) UE reporting of the supported/preferred DL RS configuration; (2) trigger/initiating data collection; (3) assistance information from NW to UE for data collection.

According to the below RAN4 agreement, the requirements for data collection for training could only be introduced if the training procedure is defined in 3GPP specifications. Since RAN1/2 is still discussing the solution for data collection for UE-sided model training, then we can further study this until clear RAN1/2 solution is obtained.

Proposal 8: FFS RAN4 impact from data collection for UE-sided model training (depending on RAN1/2 input).

Regarding the data collection mechanisms for inference and performance monitoring, we treat them as the part of procedures for model inference and performance monitoring, which will be analyzed in the following sections.

## 4.2 Additional assistance information for UE sided model

Different from NW-sided model, AI/ML operation at UE side could require the additional assistance information, since UE-sided model have no information of the NW-side beambook configuration which is essential to AI/ML operation. Based on RAN1#116bis discussion, RAN1 is still discussing possible options for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2:

Agreement from RAN1#116bis Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded.

And the following agreement from RAN2:

Agreements from RAN2#126

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

Accordingly, from RAN4 perspective on testing requirement, we see the consistency/association of Set B beams and Set A beams across training and inference shall be guaranteed in RAN4 core requirement as the NW-side additional assistance information for inference at UE.

Proposal 9: For the testing of UE-sided model inference and monitoring (if any), the consistency of NW-side additional condition across training and inference shall be guaranteed as side condition.

## 4.3 Inference for UE-sided model

As illustrated in the Figure-1 (right figure) with UE-sided model for AI-BM, UE shall report the prediction result to NW based on the output of a UE-side AI/ML model. In RAN1#116-bis and RAN1#117, the following agreement is achieved for content in the report of inference results, for UE-sided AI/ML model inference for BM-Case1 and Case2:

Agreement from RAN1#116-bis For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. Agreement from RAN1#116-bis For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report - wherein information of inference results of one time instance is as in one report for BM-Case 1 - Note: overhead reduction is not precluded - FFS on details Agreement from RAN1#117 For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output

And RAN1 is still discussing how the CSI-ReportConfig is used for the configuration of inference results reporting:

Agreement from RAN1#116-bis For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting - FFS on the details in the CSI-ReportConfig, at least considering: - Alt 1: one CSI-ResourceConfigId is configured for Set B - FFS: how UE can determine the information about set A - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId - FFS: how to configure/indicate separate resource set(s) for Set A - Note: separate CSI-ReportConfig for Set A and Set B are not precluded. - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig - FFS on the association between Set A and Set B with or without additional IE - Other necessary configuration are not precluded.

Accordingly, RAN4 requirement for model inference with UE-side AI/ML model needs to be specified for both AI-BM Case1 and Case 2, in which the above RAN1 conclusion and future progress shall be considered.

Proposal 10: RAN4 requirement of UE-side AI/ML model inference needs to be specified.

Proposal 11: RAN4 shall specify RRM requirement for UE-side AI/ML model inference: to guarantee (1) accuracy of inference results (2) the delay of reporting inference results (containing the inference latency).

Based on RAN4 agreement from RAN4#111, the ground truth of predicted RSRP is given, while for the detailed testing issues for model inference, we will discuss in the Section 5 of this contribution.

## 4.4 Performance monitoring for UE-sided model

Regarding the performance monitoring for UE-sided model, key issues discussed in RAN1 include: performance metrics, benchmark/reference for the performance comparison, and monitoring type. Corresponding analysis for potential specification impact for performance metrics and benchmark/reference for the performance comparison is provided below from physical layer perspective:

For the performance monitoring of BM-Case1 and BM-Case2: - Performance metric(s) with the following alternatives: - Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy - Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER - Alt.3: Performance metric based on input/output data distribution of AI/ML - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP - Benchmark/reference for the performance comparison, including: - Alt.1: The best beam(s) obtained by measuring beams of a set indicated by gNB (e.g., Beams from Set A) - Alt.4: Measurements of the predicted best beam(s) corresponding to model output (e.g., Comparison between actual L1-RSRP and predicted RSRP of predicted Top-1/K Beams) - Signalling/configuration/measurement/report for model monitoring, e.g., signalling aspects related to assistance information (if supported), Reference signals

In the normative phase, based on RAN1’s down-selection on performance metric(s) and benchmark/reference for the performance comparison, RAN4 requirement on performance monitoring can be specified accordingly. Specifically, RAN1 has provided two types of performance monitoring, i.e.,

For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Type 1 performance monitoring: - Configuration/Signalling from gNB to UE for measurement and/or reporting - UE may have different operations - Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW) - Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - Indication from NW for UE to do LCM operations - Note: At least the performance and reporting overhead of model monitoring mechanism should be considered - Type 2 performance monitoring: - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signalling from gNB to UE for performance monitoring measurement and/or reporting - If it is for UE side model monitoring, UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Mechanism that facilitates the UE to detect whether the functionality/model is suitable or no longer suitable

Particularly, Type 1 performance monitoring is performed in the NW-side, for which Option 1 and Option 2 differ in which entity shall perform the calculation of the performance metric(s). On the other hand, Type 2 performance monitoring is performed in the UE-side, while no feedback is required, thus making no RAN4 requirement being needed.

In RAN1#117, the following agreement is achieved, particular for Type-1 performance monitoring:

For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

Based on the above understanding on Type 1 and Type 2 performance monitoring, the following proposal is obtained.

Proposal 12: For different types of performance monitoring for UE-sided model, the necessity of RAN4 requirement is provided as:

- Type 1, Option 1 (NW-side performance monitoring): The necessity of RAN4 requirement on data collection for monitoring is not significant, because it is similar to data collection for other purposes.

- Type 1, Option 2 (UE-assisted performance monitoring): RAN4 requirement on data collection for monitoring can be specified to test the accuracy of performance metrics calculated by UE.

- Type 2 (UE-side performance monitoring): No RAN4 requirement is needed because no UE feedback will be performed.

## 4.5 LCM for UE-sided model

Based on the above analysis for different types of performance monitoring for UE-sided model (NW-side or UE-side performance monitoring), the model/functionality control can be performed by the UE or gNB:

- Management: - For UE-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the UE when the monitoring resides within the UE. - For UE-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the gNB when the monitoring resides within the gNB or UE.

Based on the agreement obtained in RAN2#125bis, the “network decision, network-initiated” AI/ML management is supported as a baseline:

Agreements from RAN2#125bis:

1	For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

2	“UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

In RAN2#126, the following agreements are achieved on LCM for UE-sided model:

Agreements from RAN2#126

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

5	For BM use case, As a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

Proposal 13: For UE-sided model LCM, the necessity of RAN4 requirement:

- “Network decision, network-initiated” LCM: RAN4 requirement is needed.

- FFS the detailed RAN4 requirement (depending on RAN1/2 input).

Here we would like to provide the summary on the expected RAN4 requirement impact and correspondingly our proposals for Rel-19 AI-BM Scope.

Proposal 14: Based on corresponding proposals, the expected RAN4 requirement impact from AI-BM is summarized as:

|  | New Sub-Feature introduced in RAN1/2 | Whether/How RAN4 requirement impact is expected? (e.g., clause # in TS 38.133 impacted/new requirement needed/no impact/FFS) | Related Proposal(s) |
| --- | --- | --- | --- |
| NW-sided model | Data collection | - Clause 9.5 L1-RSRP measurements for reporting (or similar new requirement for L1-RSRP measurement for reporting for AI-BM) - FFS new clause for MDT-based measurement/reporting for training - FFS Clause 10.1.20 for L1-RSRP accuracy requirements for FR2 | Proposal 3/4 |
|  | Inference | - No RAN4 impact expected | Proposal 5 |
|  | Perf. monitoring | - FFS new clause for performance monitoring | Proposal 6 |
|  | LCM | - No RAN4 impact expected | Proposal 7 |
| UE- sided model | Data collection | - FFS new clause for data collection for UE-sided model training | Proposal 8 |
|  | Additional assistance information | - Added as new side condition: consistency/association shall be guaranteed for inference and perf. monitoring requirement testing | Proposal 9 |
|  | Inference | - New clause for UE-side AI/ML model inference | Proposal 10/11 |
|  | Perf. monitoring | - FFS new clause is needed for Type 1 - Option 1 - New clause for Type 1 - Option 2 - No RAN4 impact expected for Type 2 | Proposal 12 |
|  | LCM | - New clause for RAN4 RRM requirement for “Network decision, network-initiated” LCM | Proposal 13 |

# 5. Testability Issues for AI-Beam Management

## 5.1 Testing Procedure/Metrics for UE-sided Model

In RAN4#111, for reported measurements and ground truth, it is agreed that the ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam:

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

For the metrics/KPI discussion, the following options are given in the Rel-18 study item and captured in TR [1]:

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

Here, RAN4 requirement shall be set up by considering the expected behavior from the good UE implementation. For beam prediction accuracy, it is straightforward that NW will directly follow the UE’s prediction results by TCI switching to the Top-1 predicted beam. From that perspective, we propose the new option by combining Option 2 (Top-1/K) and 3:

Proposal 15: For metrics for beam management requirements/tests, RAN4 adopt the following combined option of Option 2 and 3 for compromise:

- Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB”

  - FFS the values of K and x

## 5.2 Procedure to obtain channel model for UE-sided AI-BM model inference requirement testing

If RAN4 requirements will be tested based on one of the FR2 OTA chamber systems, the following steps shall be considered to emulate the required channel model in OTA chamber:

Step-1: Determine the required evaluation scenario:

- RAN1 SLS assumption can be used as baseline:

Certain channel model, e.g., UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901 can be used as baseline.

FFS UE mobility included in this evaluation scenario

BS TX Beambook can be proposed based on gNB vendors’ proposals for alignment

UE RX Antenna modeling shall be included for DFF/IFF chamber, and precluded for 3D MPAC OTA chamber.

Step-2: SLS for channel model for all TX beams:

- Based on the targeted evaluation scenario, run SLS to get channel model for training/testing

To obtain the data for different UE location, multiple drops will be simulated in SLS.

For each drop, the resultant channel models are obtained for all beams in Set-A/Set-B

Step-3: Test signal is mapped over probe(s) in OTA chamber

- The signals from tester (to be over certain TX beam-i) + corresponding channel model for TX beam-i (obtained in Step-2), to be mapped over the probe(s) in OTA chamber.

- Depends on DFF or 3D MPAC OTA chamber is decided to be used, the mapping can be different.

Figure 1: Procedure of emulating the required channel model in OTA chamber

Proposal 16: The following steps can be followed to emulate the required channel model in OTA chamber for UE-sided AI-BM model inference requirement testing:

Step-1: Determine the required evaluation scenario;

Step-2: SLS for channel model for all TX beams;

Step-3: Test signal is mapped over probe(s) in OTA chamber.

Proposal 17: RAN4 shall firstly discuss the required evaluation scenario, which is used to perform SLS for generating the UE received signals from different beams, in order to obtain the dataset for AI model training/evaluation.

- The evaluation assumption from Table 6.3.1-1 in TR38.843 could be used as the baseline for RAN4 to perform SLS for generating the UE received signals from different beams.

Here we try to simplify the parameters based on the baseline SLS assumptions for AI/ML in beam management evaluations from Table 6.3.1-1 in TR38.843

Table 1: Simplified Baseline System Level Simulation assumptions for RAN4 to generate the UE received signals from different beams

| Parameter | Value |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) Other deployment assumption is not precluded |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | For spatial domain beam prediction: 3km/h For time domain beam prediction: 30km/h (baseline), 60km/h (optional) 90km/h (optional), 120km/h (optional) Other values are not precluded |
| UE distribution | 10 UEs per sector/cell for system performance related KPI (if supported) [e.g., throughput] for full buffer traffic (if supported) evaluation (model inference). X UEs per sector/cell for system performance related KPI for FTP traffic (if supported) evaluation (model inference). Other values are not precluded. Number of UEs per sector/cell during data collection (training/testing) is reported by companies if relevant. For spatial domain beam prediction (optional to compare different UE distributions assumptions): - Option 1: 80% indoor ,20% outdoor as in TR 38.901 - Option 2: 100% outdoor For time domain prediction: 100% outdoor |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Other assumptions are not precluded. Companies to explain TXRU weights mapping. Companies to explain beam selection. Number of BS beams: 32 or 64 downlink Tx beams (max number of available beams) at NW side. Other values, e.g., 256 not precluded. |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Other assumptions are not precluded Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Number of UE beams: 4 or 8 downlink Rx beams (max number of available beams) per UE panel at UE side. Other values, e.g., 16 not precluded. |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Beam correspondence | Companies to explain beam correspondence assumptions (in accordance to the two types agreed in RAN4) |
| Link adaptation | Based on CSI-RS |
| Traffic Model | For system performance related KPI (if supported) evaluation (model inference), companies report either of the following traffic model: Option 1: Full buffer Option 2: FTP model with detail assumptions (e.g., FTP model 1, FTP model 3) |
| Inter-panel calibration for UE | Ideal, non-ideal following 38.802 (optional) – Explain any errors |
| Control and RS overhead | Companies report details of the assumptions |
| Control channel decoding | Ideal or Non-ideal (Companies explain how it is modelled) |
| UE receiver type | MMSE-IRC as the baseline, other advanced receiver is not precluded |
| BF scheme | Companies to explain what scheme is used |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| Other simulation assumptions | Companies to explain serving TRP selection Companies to explain scheduling algorithm |
| Other potential impairments | Not modelled (assumed ideal). If impairments are included, companies will report the details of the assumed impairments |
| BS Tx Power | 40 dBm (baseline) Other values (e.g., 34 dBm) not precluded |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200 m |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |
| UE measurements/reports | At least for Temporal Downlink beam prediction: - Periodicity of time instance for each measurement/report in T1: 20ms, 40ms, 80ms, [100ms], 160ms, [960ms]. Other values can be reported. - Number of time instances for measurement/report in T1 can be reported. Time instance(s) for prediction can be reported. |
| Scenario | Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation. Other scenarios are not precluded. |
| Spatial consistency | At least for BM-Case1, companies report the one of spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 |
| UE trajectory model | UE trajectory model is defined at least for temporal beam prediction in initial phase of the evaluation. Further details below. UE trajectory model is not necessarily to be defined at least for spatial-domain beam prediction in initial phase of the evaluation. |
| UE rotation | UE speed to be reported. Note: UE rotation speed = 0, i.e., no UE rotation, is not precluded |
| Baseline for performance evaluation | For temporal beam prediction: - Option 1: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources or all possible beams from Set A of beams at the time instants within T2 - Option 2: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources from Set B of beams at the time instants within T1 - Companies to explain the detail on how to select the best beam for T2 from Set A based on the measurements in T1. where T2 is the time duration for the best beam selection, and T1 is a time duration to obtain the measurements of all the RS resource from Set B of beams. T1 and T2 are aligned with those for AI/ML based methods. Whether Set A and Set B are the same or different depend on the sub-use case. Other options are not precluded. For spatial-domain beam prediction: - Option 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping) - Option 2: Select the best beam within Set A of beams based on the measurement of RS resources from Set B of beams - Other options are not precluded. |

## 5.3 OTA testing for AI-BM model inference

For the selection of OTA chamber, we suggest this work can be studied in different OTA testability item.

From technical aspects, for DFF chamber, RAN4 has studied the method to use one probe to simulate multi-path fading condition, as provided in Section 8.2 in TR 38.810, in which two options are given. However, it should be noted that in the corresponding single probe channel modelling, the UE RX beamforming can be included in the channel model, while since there is only one AoA, it is impossible to test actual RX beam management procedure.

#### 8.2.1.1 Channel model Option 1 Channel model option 1 is based on the TDL methodology in the TR 38.901 [10]: - The multi-path propagation conditions model consists of several parts: - A power delay profile in the form of a "tapped delay-line" (TDL), characterized by a number of taps with certain power at fixed positions on a sampling grid. - The channel model parameters include the Delay spread scaling factor and the maximum Doppler frequency. The test system shall allow flexible control of the respective parameters. - Each tap is modeled based on the Jakes fading model. - Generation of TDL channel models and power delay profiles from CDL channel models by including spatial filters to capture Tx and Rx antenna patterns is not precluded and based on the procedure described in the TR 38.901 [10].

For 3D MPAC on the other hand, it should be FFS existing 6 probes (with different AoAs) are enough to simulate the targeted scenario for AI/ML-Beam management use case.

Proposal 18: FFS the feasibility of DFF chamber and 3D MPAC in a separate OTA testability study item, by considering:

-  The necessity/feasibility of evaluate RX beam management in OTA chamber;

-  The feasibility to generate the required channel model (for certain TX beambook) with limited number of probes.

By comparing two kinds of existing FR2 OTA chambers, we have observed the pros/cons, the solution for test signal mapping and expected issues/challenges are provided:

|  | Pros | Cons | Test Signal Mapping | Issues/Challenges |
| --- | --- | --- | --- | --- |
| Option 1 (FR2 SISO chamber) | Already used for FR2 RRM testing | Only 1 AoA emulated, RX beam management impact can’t be tested | Mapping the test signal into tapped delay line signal, which is to be applied on 1 probe | RX beams (at UE) needs to be modeled in the test signal generation, and no possible to test UE RX beam selection/design |
| Option 2 (FR2 MIMO chamber, i.e., 3D MPAC) | Possibility for multiple AoAs | More expensive setup, and exiting 6 AoAs still come from limited spherical area | Mapping requested test signal on 6 probes by TE vendor’s algorithm | Existing 6 AoAs in limited range, which are enough or not for channel model like UMi or CDL needs FFS |

# 6. Conclusion

In this contribution, we provided our viewpoints on testability and interoperability issues for beam management in this contribution, accordingly the following observations and proposals are obtained:

General handling for AI-BM core requirement

Proposal 1: RAN4 shall firstly agree on the expected RAN4 requirement impacts for AI-BM, by separating the discussion by (1) NW and UE-sided model; (2) different sub-features introduced in RAN1/2, including: Data collection, Inference, Perf. monitoring, LCM and additional assistance information (for UE-sided model only).

Proposal 2: The following table can be used as the template for collecting views from companies.

|  | New Sub-Feature introduced in RAN1/2 | Whether/How RAN4 requirement impact is expected? (e.g., clause # in TS 38.133 impacted/new requirement needed/no impact) |
| --- | --- | --- |
| NW-sided model | Data collection |  |
|  | Inference |  |
|  | Perf. monitoring |  |
|  | LCM |  |
| UE- sided model | Data collection |  |
|  | Additional assistance information |  |
|  | Inference |  |
|  | Perf. monitoring |  |
|  | LCM |  |

RAN4 Core Requirements for Supporting NW-sided Model

Proposal 3: RAN4 shall introduce the necessary L1-RSRP reporting requirement on supporting data collection for NW-side AI/ML model inference/training (for both BM-Case1 & 2), by considering:

- L1 measurement/reporting for inference: e.g., beam reporting for more than 4 beam in L1 signaling, and other new reporting format (including potential granularity step);

- Potential enhancement on MDT-based measurement/reporting for training.

Proposal 4: For tightening measurement accuracy requirement for AI-BM NW-sided model:

- RAN4 shall only focus on the feasibility part of tightened measurement accuracy;

- Without any particular request, RAN4 shall reuse the accuracy requirement from Rel-15 L1-RSRP for both absolute and relative RSRP reporting.

Proposal 5: No RAN4 impact is expected for NW-side AI/ML model inference for BM-Case1 & 2, except data collection for model inference.

Proposal 6: FFS RAN4 impact from NW-side AI/ML model performance monitoring for BM-Case1 & 2 (depending on RAN1/2 input).

Proposal 7: No RAN4 impact is expected for NW-side AI/ML model LCM for BM-Case1 & 2.

RAN4 Core Requirements for Supporting UE-sided Model

Proposal 8: FFS RAN4 impact from data collection for UE-sided model training (depending on RAN1/2 input).

Proposal 9: For the testing of UE-sided model inference and monitoring (if any), the consistency of NW-side additional condition across training and inference shall be guaranteed as side condition.

Proposal 10: RAN4 requirement of UE-side AI/ML model inference needs to be specified.

Proposal 11: RAN4 shall specify RRM requirement for UE-side AI/ML model inference: to guarantee (1) accuracy of inference results (2) the delay of reporting inference results (containing the inference latency).

Proposal 12: For different types of performance monitoring for UE-sided model, the necessity of RAN4 requirement is provided as:

- Type 1, Option 1 (NW-side performance monitoring): The necessity of RAN4 requirement on data collection for monitoring is not significant, because it is similar to data collection for other purposes.

- Type 1, Option 2 (UE-assisted performance monitoring): RAN4 requirement on data collection for monitoring can be specified to test the accuracy of performance metrics calculated by UE.

- Type 2 (UE-side performance monitoring): No RAN4 requirement is needed because no UE feedback will be performed.

Proposal 13: For UE-sided model LCM, the necessity of RAN4 requirement:

- “Network decision, network-initiated” LCM: RAN4 requirement is needed.

- FFS the detailed RAN4 requirement (depending on RAN1/2 input).

Summary of Proposals for AI-BM Core Requirement

Proposal 14: Based on corresponding proposals, the expected RAN4 requirement impact from AI-BM is summarized as:

|  | New Sub-Feature introduced in RAN1/2 | Whether/How RAN4 requirement impact is expected? (e.g., clause # in TS 38.133 impacted/new requirement needed/no impact/FFS) | Related Proposal(s) |
| --- | --- | --- | --- |
| NW-sided model | Data collection | - Clause 9.5 L1-RSRP measurements for reporting (or similar new requirement for L1-RSRP measurement for reporting for AI-BM) - FFS new clause for MDT-based measurement/reporting for training - FFS Clause 10.1.20 for L1-RSRP accuracy requirements for FR2 | Proposal 3/4 |
|  | Inference | - No RAN4 impact expected | Proposal 5 |
|  | Perf. monitoring | - FFS new clause for performance monitoring | Proposal 6 |
|  | LCM | - No RAN4 impact expected | Proposal 7 |
| UE- sided model | Data collection | - FFS new clause for data collection for UE-sided model training | Proposal 8 |
|  | Additional assistance information | - Added as new side condition: consistency/association shall be guaranteed for inference and perf. monitoring requirement testing | Proposal 9 |
|  | Inference | - New clause for UE-side AI/ML model inference | Proposal 10/11 |
|  | Perf. monitoring | - FFS new clause is needed for Type 1 - Option 1 - New clause for Type 1 - Option 2 - No RAN4 impact expected for Type 2 | Proposal 12 |
|  | LCM | - New clause for RAN4 RRM requirement for “Network decision, network-initiated” LCM | Proposal 13 |

Testability Issues for AI-BM

Proposal 15: For metrics for beam management requirements/tests, RAN4 adopt the following combined option of Option 2 and 3 for compromise:

- Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB”

  - FFS the values of K and x

Proposal 16: The following steps can be followed to emulate the required channel model in OTA chamber for UE-sided AI-BM model inference requirement testing:

Step-1: Determine the required evaluation scenario;

Step-2: SLS for channel model for all TX beams;

Step-3: Test signal is mapped over probe(s) in OTA chamber.

Proposal 17: RAN4 shall firstly discuss the required evaluation scenario, which is used to perform SLS for generating the UE received signals from different beams, in order to obtain the dataset for AI model training/evaluation.

- The evaluation assumption from Table 6.3.1-1 in TR38.843 could be used as the baseline for RAN4 to perform SLS for generating the UE received signals from different beams.

Proposal 18: FFS the feasibility of DFF chamber and 3D MPAC in a separate OTA testability study item, by considering:

-  The necessity/feasibility of evaluate RX beam management in OTA chamber;

-  The feasibility to generate the required channel model (for certain TX beambook) with limited number of probes.

# 7. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-240774 (revised from RP-234039), “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402389, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.

[5] 3GPP TR 38.810, v16.7.0, “NR; Study on test methods”.

[6] 3GPP TR 38.827, v16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”.

[7] R4-2405652, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110bis, April 2024.

[8] R4-2406617, “WF on AI/ML for NR Air Interface”, Qualcomm, RAN#110bis, April 2024.

[9] R4-2409761, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#111, May 2024.

[10] R4-2410570, “WF on AI/ML”, Qualcomm, RAN#111, May 2024.