3GPP TSG-RAN WG4 Meeting #112                                                                R4-2411637

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:	Discussion on SBFD general aspects

Agenda item:			8.19.1

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly from RAN4 perspective, it is tasked to specify BS RF, necessary RRM and demodulation requirements.

In this contribution, we would like to provide our analysis and viewpoints on the general aspects including RAN4 aspects for SBFD system parameters.

# 2. General aspects for SBFD operation

### 2.1 BS RF specification structure for SBFD requirements

In previous RAN4 discussion, two options, i.e., creating new sub-clauses and embedding the SBFD RF requirement in corresponding section, were listed on how to incorporate SBFD requirements in the BS RF core specification (i.e., TS 38.104). Particularly, in RAN4#111, the following agreements are achieved:

<From RAN4#111> Issue 1-4-1: BS RF Specification structure for SBFD requirements - Agreement: - How to introduce BS RF new requirements for SBFD-capable BS: - FFS firstly the feasibility of creating new and standalone sub-clauses (e.g., with a suffix) in TS 38.104 for SBFD-specific existing or new gNB RF requirements - FFS detailed how to implement such approach in the later phase of WI. - FFS secondly the new specification for SBFD-capable BS

By considering the readability of the specification for SBFD-capable BS, we think the way of creating new and standalone sub-clauses (e.g., with a suffix) in TS 38.104 can be followed for SBFD-specific existing or new gNB RF requirements. As mentioned in previous RAN4 discussion by some companies, one comparable precedent is using suffix-B for Mobile IAB-node in the core and conformance specification for IAB nodes (TS 38.174, TS 38.176-1 and TS 38.176-2). One different issue is that RAN4 could introduce the new BS RF requirement dedicated for SBFD symbols, and the below proposal also takes that into account:

Proposal 1: For introducing BS RF requirements for SBFD-capable BS, RAN4 shall adopt the way of creating new and standalone sub-clauses in TS 38.104 by setting up new sub-clauses with suffix-B.

In other words, for both cases (modifying existing BS RF requirements or specifying new BS RF requirements for SBFD-capable BS), setting up new sub-clauses with suffix-B shall be used to guarantee specification readability:

- For the SBFD BS RF requirements specified by modifying existing requirements:

  - Set up new sub-clause with suffix-B

- For the new BF RF requirements dedicated to SBFD BS

  - Set up new sub-clause with suffix-B

### 2.2 SBFD feature support for particular band/bandwidth

Based on the discussion in RAN4#111, SBFD is recognized as “a feature with requirements of which can potentially be applied to all TDD band”, and two options are considered for particular band and channel bandwidth supported for SBFD operation. Particularly, the following agreements are provided with the two options given for further study:

<From RAN4#111> Issue 1-1-1: SBFD as band specific or general feature to all TDD bands - Agreement: - SBFD is a feature with requirements of which can potentially be applied to all TDD band, under the following considerations: - Option 1: Declaration based method - The band supported for SBFD shall be manufacturer declaration based - The channel bandwidth supported for SBFD shall be manufacturer declaration based - The supported channel bandwidth can be impacted by the subband/guard band size discussion - Option 2: General principle but with consideration(s) for a specific band - General principle: The channel bandwidth shall be larger than X MHz - FFS the value of X - X can be different for high and middle TDD bands - For a specific band, the following further restriction(s) can be considered: - Certain restriction can be provided by operator(s) for certain band - Other restrictions are not precluded.

The option 2 above, such as the restriction on channel bandwidth for SBFD operation is proposed by some companies, while we think it should not be considered as the general principle of SBFD operation and the declaration-based method match the existing BS RF requirement framework and gives the manufacturer and operator the flexibility to operating the SBFD feature in any band. Therefore, the option 1 is preferred by us:

Proposal 2: SBFD is a feature with requirements of which can potentially be applied to all TDD band, with Option 1 (declaration-based method) being adopted, i.e.,

- Option 1: Declaration based method

  - The band supported for SBFD shall be manufacturer declaration based

  - The channel bandwidth supported for SBFD shall be manufacturer declaration based

# 3. RAN4 aspects for SBFD system parameters

### 3.1 SBFD time-domain terminology

In the existing RAN4 study (including the conclusions captured in TR 38.858 [2]) and subsequential RAN4 discussion in the work item phase, the term “SBFD and non-SBFD slots/symbols” are used. Based on the RAN1 agreements, one time slot can consist of SBFD symbols and non-SBFD symbols:

<From RAN1#116> Agreement A slot can consist of SBFD symbols and non-SBFD symbols.

Accordingly, we suggest to use SBFD and non-SBFD symbols when specifying RAN4 SBFD and non-SBFD requirements for SBFD-capable BS.

Proposal 3: When specifying RAN4 requirements for SBFD-capable BS, SBFD symbols (instead of SBFD slots/symbols) shall be used to designate the time domain duration in which SBFD related requirements is applied.

### 3.2 SBFD frequency-domain configuration

As requested in WID, to support SBFD operation at gNB side within a TDD carrier, the mechanism for semi-static indication of frequency location of SBFD subbands to UE shall be specified. Based upon previous RAN4 discussion, it is “within RAN4 scope to study/specify the limitation or restriction on the size of subband/guardband”. Firstly, the RAN1 agreements achieved in RAN1#117 is provided below, in which we can observed that RAN1 agree to support the flexible frequency domain configuration with the cell-specific configuration, by explicitly configuring frequency locations of UL subband and DL subband(s) thus making the guardband(s) if any can be derived implicitly.

<From RAN1#116> Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands Agreement The subband frequency-domain resources are same across different SBFD symbols within a TDD carrier. Frequency location of cell specific UL subband, and DL subband(s) if explicitly indicated, are indicated with reference to CRB grid. - RB-level granularity is supported for semi-static indication of SBFD subband frequency location. - Subject to RAN4 guidance on the size of subband/guardband, if any - FFS reference starting RB and reference SCS Agreement For discussion purpose, UL subband frequency resources within active UL BWP are called UL usable PRBs and DL subband(s) frequency resources within active DL BWP are called DL usable PRBs. For determining UL/DL usable PRBs, consider the following options. - Option 1: UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols. - Option 2: UL/DL usable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols. <From RAN1#116bis> Agreement For cell-specific configuration of frequency locations of SBFD subbands, - Option 1: Cell-specific frequency locations of SBFD subbands are separately configured for each SCS configuration in SCS-SpecificCarrierList. - For each SCS configuration, the reference starting PRB is the PRB determined by the SCS configuration and offsetToCarrier corresponding to this subcarrier spacing. <From RAN1#117> Agreement For cell-specific indication of SBFD subband frequency location, adopt the following option. - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

Based on the discussion in RAN4#111, the following agreements are achieved:

Issue 1-2-2: How to handle guard band and subband configurations in specification - Agreement: - Add clarification that only DUD and DU patterns are specified for the sub-band configuration. - FFS which channel bandwidth(s) or all channel bandwidths shall be defined for RF requirements - For a certain channel bandwidth which RAN4 agree to introduce RF requirements: - FFS RAN4 only define the UL/DL subbands configuration(s) for RF requirements - FFS which UL/DL subbands configuration(s) will be defined in RAN4 - FFS Guard band size is declaration based and can be different for different BS classes - FFS the limitation on the maximum guard band - FFS possible range for UL/DL subband sizes - From RAN4 perspective, FFS restriction or no restriction to RAN1 definition for UL/DL subband sizes within the transmission configuration for this channel bandwidth, except: - 1RB granularity (already introduced in RAN1)

Firstly, RAN4 agreed to further study whether the channel bandwidth(s) or all channel bandwidths shall be defined for RF requirements. The discussion is triggered by the fact that the overhead of guardband(s) between UL and DL subbands could occupy the larger portion of channel/transmission bandwidth when the channel/transmission bandwidth is relatively small. However, based on RAN1 study, the benefits of SBFD comes not only allocating more UL resources flexibility but also reducing the UL transmission delay. Furthermore, the supported channel bandwidth for a particular BS implementation is declared by manufacturer, and the discussion of supporting all channel bandwidths or not will only impact the standardization work. From that perspective, we see no reason to exclude any channel bandwidth(s) for SBFD operation.

Proposal 4: All channel bandwidths shall be defined for RF requirements

For the sizes of subband and guardband, different feasible BS/UE implementations shall be considered.

Especially in the field of RF, direct DL transmission can pose challenges due to channel filtering and potential saturation of the RX receiver. However, passive SIC isolation can help mitigate this issue by incorporating an additional UL subband RF filter. To address interference from the DL and ensure accurate reception of the desired UL signal, an analog filter can be employed prior to the ADC. This filter serves to attenuate DL interference and protect the dynamic range of the ADC. An active approach can also involve an analog interference canceller, although in mMIMO systems, the complexity of the analog canceller increases as the number of Tx and Rx chains grows. All these factors could potentially impact the determination of the size of subband and guardband.

In the digital domain, various techniques can be utilized to reduce the impact on the sensitivity of the BS receiver, aiming to keep it below 1 dB. One such technique is digital interference cancellation (IC), which aims to eliminate interference from the desired UL signal and reduce any remaining self-interference (SI) to an acceptable level. Digital IC involves synthesizing interference from baseband Tx samples with accurate modeling of Tx chain non-linearity or capturing interference from the RF chain on the transmit side and converting it to baseband. This process requires estimating the SI channel taps for each Tx chain to each Rx chain. Additionally, other digital techniques can help mitigate interference. For example, Digital Pre-distortion (DPD) can reduce leakage into the UL subband. Other techniques, such as beam nullification to lessen the impact of clutter reflections or optimizing digital Tx beamforming to decrease SI, can also be employed.

Therefore, for determine the limitation/restriction of the size of subband and guardband, the BS SBFD implementations including the possibility of adding UL subband RF filter, analog filter prior to the ADC or other RF and digital domain techniques related to subband bandwidth shall be considered. From that perspective, we propose that RAN4 can consider the following restriction/limitation for the size of subband and guardband, at least as baseline for further RAN4 study.

Proposal 5: The following limitation/restriction of the size of subband and guardband shall be considered as baseline:

- Subband (including both UL/DL subband):

  - Granularity: 1 RB

  - Possible Subband size (expressed in RBs): {Xmin, Xmin+1,..., Xmax}

    - Xmin = 1

    - Xmax = NRB (i.e., Transmission bandwidth configuration)

- Guardband:

  - Granularity: 1 RB

  - Possible guardband size (expressed in RBs): {0, 1, 2, ..., Ymax}

    - Where Ymax is the maximum allowed number of RBs for guardband

      - FFS the value of Ymax

From the approved WF [3], the group shall further study how RAN4 specification captures the subband configurations. Specifically, the following text proposal with the specification structure for SBFD-capable BS could be considered as the baseline for RAN4 discussion.

Proposal 6: A new 5.3B clause is added to capture the BS channel bandwidth for SBFD (including Transmission bandwidth configuration, guardband and subband configuration for SBFD), with the following text proposal to be considered as baseline skeleton for triggering discussion:

=================== Start of Text Proposal ===================

## 5.3B	BS channel bandwidth for SBFD

### 5.3B.1	General

<To add the illustrative definition of channel bandwidth, transmission bandwidth configuration, uplink and downlink subbands and guardband within one NR channel for SBFD operation>

### 5.3B.2	Transmission bandwidth configuration for SBFD

<To be added>

### 5.3B.3	Guardband and Subband configuration for SBFD

<To be added>

=================== End of Text Proposal ===================

# 3. Conclusion

In this contribution, we would like to provide our analysis and viewpoints on the general aspects including RAN4 aspects for SBFD system parameters, accordingly with the following observations and proposals obtained:

Proposal 1: For introducing BS RF requirements for SBFD-capable BS, RAN4 shall adopt the way of creating new and standalone sub-clauses in TS 38.104 by setting up new sub-clauses with suffix-B.

Proposal 2: SBFD is a feature with requirements of which can potentially be applied to all TDD band, with Option 1 (declaration-based method) being adopted, i.e.,

- Option 1: Declaration based method

  - The band supported for SBFD shall be manufacturer declaration based

  - The channel bandwidth supported for SBFD shall be manufacturer declaration based

Proposal 3: When specifying RAN4 requirements for SBFD-capable BS, SBFD symbols (instead of SBFD slots/symbols) shall be used to designate the time domain duration in which SBFD related requirements is applied.

Proposal 4: All channel bandwidths shall be defined for RF requirements

Proposal 5: The following limitation/restriction of the size of subband and guardband shall be considered as baseline:

- Subband (including both UL/DL subband):

  - Granularity: 1 RB

  - Possible Subband size (expressed in RBs): {Xmin, Xmin+1,..., Xmax}

    - Xmin = 1

    - Xmax = NRB (i.e., Transmission bandwidth configuration)

- Guardband:

  - Granularity: 1 RB

  - Possible guardband size (expressed in RBs): {0, 1, 2, ..., Ymax}

    - Where Ymax is the maximum allowed number of RBs for guardband

      - FFS the value of Ymax

Proposal 6: A new 5.3B clause is added to capture the BS channel bandwidth for SBFD (including Transmission bandwidth configuration, guardband and subband configuration for SBFD), with the following text proposal to be considered as baseline skeleton for triggering discussion:

=================== Start of Text Proposal ===================

## 5.3B	BS channel bandwidth for SBFD

### 5.3B.1	General

<To add the illustrative definition of channel bandwidth, transmission bandwidth configuration, uplink and downlink subbands and guardband within one NR channel for SBFD operation>

### 5.3B.2	Transmission bandwidth configuration for SBFD

<To be added>

### 5.3B.3	Guardband and Subband configuration for SBFD

<To be added>

=================== End of Text Proposal ===================

# 4. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[4] R4-2409763, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#111.

[5] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.