3GPP TSG-RAN WG4 Meeting #112                                                                R4-2411638

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:	On the modification of existing TX requirements for SBFD-capable BS

Agenda item:			8.19.2.2

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1.

Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”, and this work shall be based on the good outcome from the study, i.e., Section 10.1, “Impact on BS RF requirements” in TR 38.858. Furthermore, the way forward encourages companies to continue contributing to ongoing discussions on (1) the modification of existing requirements for SBFD-capable BS and (2) potentially new requirement for SBFD operation, and address issues that have been raised during meetings [4]. In the RAN4#111, further agreements are achieved on the modification of existing requirements for SBFD-capable BS [6]. In this contribution, accordingly, we would like to provide our analysis and viewpoints on the modification of existing TX requirements for FR1 and FR2-1 SBFD-capable BS.

# 2. Modification on Existing TX requirement for SBFD

### 2.1  Base Station output power and radiated transmit power

For the requirement of BS output power and radiated transmit power, the following agreements is achieved in the study phase [2]:

Since configuration (e.g. antenna, power configuration etc) between SBFD and non-SBFD symbols/slots might be different, RAN4 reached the following consensus for the BS RF requirement of BS output power for both conducted and OTA output power: - It is allowed to have the different conducted declaration for normal DL symbols/slots and SBFD DL symbols/slots. - It is allowed to have different EIRP/TRP declaration (for level and direction) for normal DL symbols/slots and SBFD DL symbols/slots. - Accuracy requirement for TRP/EIRP and conducted power shall be the same for normal DL symbols/slots and SBFD DL symbols/slots.

And the following agreement is achieved in RAN4#111 [6]:

Issue 2-1-1: PSD scaling for normal and SBFD slots/symbols - Agreement: - PSD scaling for normal and SBFD slots/symbols: - No need to introduce the restriction on PSD scaling for normal and SBFD slots/symbols - Vendors can declare different TX power values for normal and SBFD slots/symbols

Based on the above agreements achieved in study item and RAN4#111, we expect the manufacturer declaration-based method will be used for base station output power, radiated transmit power and OTA base station output power, and further discussion could be expected on the specification implementation by allowing different declarations of output power for normal DL symbols and SBFD DL symbols. Based on our accompanying contribution on general aspects for SBFD BS RF requirement, we suggest that RAN4 shall adopt the way of creating new and standalone sub-clauses in TS 38.104 by setting up new sub-clauses with suffix-B. Accordingly, the following proposal is given.

Proposal 1: For conducted output power and radiated transmit power (radiated transmit power and OTA base station output power), the following new sub-clauses with suffix-B shall be adopted to capture the corresponding declaration for SBFD symbol:

- New clause 6.2B, titled as “Base station output power for SBFD”

- New clause 9.2B, titled as “Radiated transmit power for SBFD”

- New clause 9.3B, titled as “OTA base station output power for SBFD”

Another aspect is the required output limits for different BS classes, by which Wide Area, Medium Range and Local Area BS classes are defined. Take the example of BS type 1-C rated output power limits for different BS classes as example:

Table 6.2.1-1: BS type 1-C rated output power limits for BS classes [TS 38.104]

| BS class | Prated,c,AC |
| --- | --- |
| Wide Area BS | (Note) |
| Medium Range BS | ≤ 38 dBm |
| Local Area BS | ≤ 24 dBm |
| NOTE: There is no upper limit for the Prated,c,AC rated output power of the Wide Area Base Station. |  |

For SBFD-capable BS, RAN4 already agreed that “no need to introduce the restriction on PSD scaling for normal and SBFD slots/symbols” and “vendors can declare different TX power values for normal and SBFD slots/symbols”. Another issue is whether or not we expect different BS classes for non-SBFD symbols and SBFD symbols.

Based on our understanding, if two sets of power limits are provided (even with the same limits in these two sets) for non-SBFD symbols and SBFD symbols, it could be possible a contradicting BS classes are given: For example, if the same limits given in Table 6.2.1-1 are used for both non-SBFD symbols and SBFD symbols, there is an SBFD-capable BS with 40dBm output power in non-SBFD symbols and 37dBm output power in SBFD symbols (because of half of the number of antenna elements are used for SBFD symbols). In this example, this SBFD-capable BS shall be categorized as Wide Area BS accordingly to non-SBFD symbols but categorized as Medium Range BS accordingly to SBFD symbols. Obviously, it is not reasonable to categorize one BS implementation into two BS classes based on SBFD and non-SBFD symbols. Therefore, we have the following proposal that only specifying output power limits for different BS classes for non-SBFD symbols for SBFD-capable BS by reusing the existing categorization.

Proposal 2: For SBFD-capable BS, RAN4 will not specify the output power limits for different BS classes for non-SBFD symbols.

Proposal 3: To determine one SBFD-capable BS’s class, the existing requirements for output power limits for BS classes shall be used for non-SBFD symbols, no matter what is the power level declared for SBFD symbols.

### 2.2  Output power dynamics

In the Rel-18 study item, the following conclusion has been reached for output power dynamics:

#### 10.1.2.2 Output power dynamics Regarding the output power dynamic requirement, which mainly consists of RE power control dynamic range requirement and total dynamic range requirement, RAN4 reached the following consensus: - To reuse the existing RE power control dynamic range requirement for SBFD BS; - The total dynamic range requirement is applicable for SBFD-capable BS during normal DL symbols/slots, it is agreed to define the output power dynamic range requirement for SBFD as the ratio of the declared rated output power with all DL RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

Therefore, we propose to setting up the new sub-clauses 6.3.3B (Total power dynamic range for SBFD) and 9.4.3B (OTA total power dynamic range for SBFD) by defining the output power dynamic range requirement for SBFD as the ratio of the declared rated output power with all DL RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

Proposal 4: RAN4 shall specify the new sub-clauses 6.3.3B (Total power dynamic range for SBFD) and 9.4.3B (OTA total power dynamic range for SBFD) by defining the output power dynamic range requirement for SBFD as the ratio of the declared rated output power with all DL RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

### 2.3  Transmit ON/OFF power

For the requirement of transmit ON/OFF power, the following agreement is provided in the study item:

Regarding the transmitter ON/OFF power requirement, RAN4 mainly focus on the ON-OFF time mask and concluded that transmit ON/OFF power requirement is not applicable within SBFD time slot.

For the study item conclusion that “transmit ON/OFF power requirement is not applicable within SBFD time slot”, we expect there are two options for specification implementation: (1) adding Notes in both clause 6.4.1.1 and 9.5.2.1; (2) adding new sub-clauses 6.4.1B and 9.5.2B for SBFD operation. Among these two options, “adding Notes in both clause 6.4.1.1 and 9.5.2.1” is preferred by us for its simplicity.

Proposal 5: To implementation the conclusion of “transmit ON/OFF power requirement is not applicable within SBFD time slot”, RAN4 can add Notes in both clause 6.4.1.1 and 9.5.2.1 to avoid OFF power requirement to be applied for SBFD symbols.

### 2.4  Unwanted emissions

For the ACLR requirement of unwanted emissions, the following agreements is achieved in the study phase [2]:

- For ACLR requirement, it shall be defined outside of the whole carrier instead of sub-band for SBFD DL symbols/slots and ACLR requirement is still defined as the ratio of sum of TX power within the whole carrier to the adjacent carrier.

In the last meeting, some company proposed that the legacy ACLR/ACS requirements are derived based on co-existence study by applying no limitation of grid shift between interfering and victim networks. Based on the co-existence study conducted in study item, the study is performed by focusing on no less than 10% grid shift, and 0% is avoided due to potential RX blocking issue. From that perspective, the proponent proposed that RAN4 shall discuss “whether the co-location ACLR/ACS or equivalent requirement is needed or not”.

In RAN4#111, the following agreement is achieved:

- Agreement: - RAN4 further study the ACLR requirement by applying the existing ACLR requirement for SBFD-capable BS in SBFD symbols/slots - FFS the applicable deployment scenario where different ACLR requirement could be useful, whether it justify the different ACLR requirement.

Firstly, we think we should firstly avoid to use “co-location requirements”, which are requirements which are based on assuming the BS type 1-O is co-located with another BS of the same base station class, they ensure that both co-located systems can operate with minimal degradation to each other. For these co-location requirements, the OTA co-location reference antenna is utilized in the specification. Therefore, the “co-location ACLR/ACS requirement” proposed shall be not relevant to the “co-location requirement” used in TS38.104.

Next, at least for ACLR requirement for BS coexisting with legacy TDD system, we can easily reach the fact that the problem of SBFD TX interfering to other DL reception don’t exist, even 0% grid shift is not evaluated in the study phase, because the SBFD only have to operate in the DL and flexible symbol indicated by TDD-UL-DL-ConfigCommon (Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols) [1].

Proposal 6: If coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the existing ACLR requirement for SBFD-capable BS in SBFD symbols, and confirm this requirement can already guarantee adjacent-channel co-existence for Rel-19 SBFD operation.

For the ACLR requirement for BS coexisting with new SBFD BS in the adjacent channel, RAN4 can further study the existing requirement shall be tightened or not to protect the new SBFD BS’s UL reception.

Proposal 7: If coexisting with new SBFD system in adjacent channel, RAN4 shall further study the ACLR requirement for SBFD-capable BS in SBFD symbols.

### 2.6  Transmitter intermodulation

Based on last meeting’s agreement, there will be no receiver requirement specified for transmitter intermodulation requirement applicable in SBFD symbols, i.e.,

Issue 2-2-1: TX intermodulation requirement - Agreement: - The transmitter intermodulation requirement is applicable in SBFD slots/symbols: - Follow the existing requirement - No receiver requirement is specified.

To implement the above agreement, we have the following simple proposal:

Proposal 8: For transmitter intermodulation requirement, RAN4 can specify a new sub-clause 6.7B (Transmitter intermodulation for SBFD) and a new sub-clause 9.8B (OTA transmitter intermodulation for SBFD), by simply explain that the existing requirement shall be applied while no receiver requirement is specified for SBFD symbols.

# 4. Conclusion

In this contribution, we provided the viewpoints on the modification of existing TX requirements for FR1 and FR2-1 SBFD-capable BS, accordingly with the following observations and proposals obtained:

Proposal 1: For conducted output power and radiated transmit power (radiated transmit power and OTA base station output power), the following new sub-clauses with suffix-B shall be adopted to capture the corresponding declaration for SBFD symbol:

- New clause 6.2B, titled as “Base station output power for SBFD”

- New clause 9.2B, titled as “Radiated transmit power for SBFD”

- New clause 9.3B, titled as “OTA base station output power for SBFD”

Proposal 2: For SBFD-capable BS, RAN4 will not specify the output power limits for different BS classes for non-SBFD symbols.

Proposal 3: To determine one SBFD-capable BS’s class, the existing requirements for output power limits for BS classes shall be used for non-SBFD symbols, no matter what is the power level declared for SBFD symbols.

Proposal 4: RAN4 shall specify the new sub-clauses 6.3.3B (Total power dynamic range for SBFD) and 9.4.3B (OTA total power dynamic range for SBFD) by defining the output power dynamic range requirement for SBFD as the ratio of the declared rated output power with all DL RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

Proposal 5: To implementation the conclusion of “transmit ON/OFF power requirement is not applicable within SBFD time slot”, RAN4 can add Notes in both clause 6.4.1.1 and 9.5.2.1 to avoid OFF power requirement to be applied for SBFD symbols.

Proposal 6: If coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the existing ACLR requirement for SBFD-capable BS in SBFD symbols, and confirm this requirement can already guarantee adjacent-channel co-existence for Rel-19 SBFD operation.

Proposal 7: If coexisting with new SBFD system in adjacent channel, RAN4 shall further study the ACLR requirement for SBFD-capable BS in SBFD symbols.

Proposal 8: For transmitter intermodulation requirement, RAN4 can specify a new sub-clause 6.7B (Transmitter intermodulation for SBFD) and a new sub-clause 9.8B (OTA transmitter intermodulation for SBFD), by simply explain that the existing requirement shall be applied while no receiver requirement is specified for SBFD symbols.

# 5. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2405655, “Discussion on BS RF requirements for SBFD-capable BS”, Samsung, RAN4#110bis.

[4] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[5] R4-2409764, “On the modification of existing requirements for FR1 and FR2-1 SBFD-capable BS”, Samsung, RAN4#111.

[6] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.