3GPP TSG-RAN WG4 Meeting #112                                                                R4-2411641

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:	Discussion on OTA spatial emission requirement

Agenda item:			8.11.2

Document for:	Approval

# 1. Introduction

In ITU-R world radiocommunication conference (WRC) in Nov. 20 - Dec. 15 2023,  the frequency band of 6 425-7 125 MHz was identified in every ITU region as in the RR Footnotes 5.6A12, 5.6B12 and 5.6C12 with associated technical condition of limits on the expected equivalent isotropically radiated power (EIRP) spectral density of IMT base-stations, for the purpose of protecting earth-to-space fixed satellite services (FSS) as in Resolution 220 (WRC-23). Based on the newly released ITU-R regulation, the expected EIRP limits have been established which require the harmonized specification for compliance tests to be developed as the task in 3GPP.

Specifically, the aim of this task is to include the new regulation in BS (and other nodes like IAB, Repeater and NCR) RF core specification (TS 38.104) and a corresponding test requirement in the conformance test specification (TS 38.141-2). Furthermore, having the requirement included in the 3GPP standard would also guarantee a harmonized terminology and conformance test method. Accordingly, the core part objective related to expected EIRP mask for upper 6GHz is provided in WID [1] as:

Expected EIRP mask for upper 6GHz - Introduce the concept of “limits on expected EIRP for angles above horizon” into the RAN4 BS specifications and specify related BS requirement for 6425-7125 MHz based on the WRC-23 outcome. - Note: target completion date of this objective is December 2024.

Based on the approved WF [4] in RAN4#110bis, it is planned to “Target to finalize the core requirement for EIRP mask for U6GHz and endorse the CR if possible” in RAN4#111. And much progress has been achieved in RAN4#111 [6], with the following agreements obtained:

Issue 2-1 The applicability of frequency range within band n104 Agreement: - Option 1: 6425-7075MHz Issue 2-2 The naming for this requirement Agreement: - OTA spatial emission Issue 2-3 Which sub-clause to capture the requirement Agreement: - Use the new clause 9.9 to capture the requirement Issue 2-4 The applicable BS types to fulfill the Expected EIRP requirements Agreement: - BS type 1-H and BS type 1-O Issue 2-5 The coordinate system for Expected EIRP requirement Agreement: Issue 2-6 Mechanical tilts related Agreement: - NOTE 2: The requirement shall apply to all supported mechanical tilts. Agreement: Table 2.2.2-1: [CAR] declarations Parameter; Description. min; Minimum angle along  axis. max; Maximum angle along  axis. min; Minimum angle along  axis. max; Maximum angle along  axis. - NOTE 1: further discuss the definition of min max min max - NOTE 2: reusing the existing declaration is not precluded. Issue 2-7 Expected EIRP calculation in discrete spatial sampling grid Agreement: - PθL , θH=π2MNsinθH-sinθLn=nLnHm=1MP1θn,φm cosθn , where P1 is measured as EIRP - M is the number of sampling points in the azimuth range; - [N is the number of sampling points within the θL , θH bounding range over upper hemisphere in the elevation range ] - nL is the lowest elevation sampling angle within the θL , θH bounding range - nH is the highest elevation sampling angle within the θL , θH bounding range - m=1MP1θn,φm is to produce the global sum over the whole azimuth ranges for EIRP above the horizon in a given bin. - Note: the equation is assuming the angles in radians and the elevation angle is from horizon rather than the coordinate system defined in 3GPP. Note: the above equation should be carefully reviewed by companies.

In this contribution, we would like to further provide our viewpoints on the expected EIRP mask for upper 6GHz, and discuss the related BS requirements (namely, the requirement of OTA spatial emission to be defined in 3GPP) for 6425-7125 MHz based on the WRC-23 outcome.

# 2. Discussion

## 2.1 Definition of Expected EIRP

In the approved WF [6], the draft text proposal is given for the newly introduced clause of OTA spatial emission, in which the concept of EEIRP (expected EIRP) is given ([EEIRP is defined as the average value of the EIRP, with the averaging being performed over azimuth angles from −180° to +180 ° and over the specified elevation angle range [θL ≤ θ < θH] in table 9.9.2-1]). In one company’s draft CR [R4-2409074], a similar definition is provided for the clause 3.1 of TS38.104. However, all the definitions above are not correctly specified because it missed the fact that the averaging shall not only be performed in the azimuth and elevation angle ranges, but also over different beamforming directions within the IMT base station horizontal and vertical steering range.

Observation 1: In the tentatively agreed definition of Expected EIRP, it is not yet mentioned the averaging over different beamforming directions.

Particularly, the expected EIRP spectral density limit is provided in Resolution 220 [2] and extracted as below:

... “resolves 1 that administrations wishing to implement IMT in accordance with Nos. 5.6A12, 5.6B12, and 5.6C12 consider use of the frequency bands referred to in those footnotes, taking into account the most recent versions of the relevant ITU-R Recommendations; 2 that, in order to ensure protection for the FSS (Earth-to-space), and taking into account considering d), the level of expected e.i.r.p. spectral density emitted by an IMT base station as a function of the vertical angle above the horizon shall not exceed the following values (No. 21.5 does not apply): Vertical angle range θL ≤ θ < θH (vertical angle θ above the horizon); Expected e.i.r.p. (dBm/MHz) (See NOTES 1, 2 and 3). 0 ≤ θ < 5; 27. 5 ≤ θ < 10; 23. 10 ≤ θ < 15; 19. 15 ≤ θ < 20; 18. 20 ≤ θ < 30; 16. 30 ≤ θ < 60; 15. 60 ≤ θ ≤ 90; 15. NOTE 1: The expected e.i.r.p. is defined as the average value of the e.i.r.p., with the averaging being performed: ‒ over horizontal angles from −180 to +180, with the IMT base station beamforming in a specific direction within its horizontal and vertical steering range, ‒ over different beamforming directions within the IMT base station horizontal and vertical steering range, and ‒ over the specified vertical angle range θL ≤ θ < θH. NOTE 2: An IMT base station shall comply with the specified limits on expected e.i.r.p. spectral density for all mechanical tilts with which it can be deployed, taking into account considering m). NOTE 3: See the Annex to this Resolution for additional details on how the expected e.i.r.p. can be calculated for this frequency band. …”

Observation 2: Based on Resolution 220 (WRC-23), the definition of expected EIRP is provided as:

- The Expected EIRP is defined as the average value of EIRP (in the specified vertical (elevation) angle range above the horizon, θL ≤ θ < θH) with the averaging being performed:

(a) over horizontal angles from −180 to +180, with the base station beamforming in a specific direction within its horizontal (azimuth) and vertical (elevation) steering range,

(b) over different beamforming directions within the BS horizontal and vertical steering range, and

(c) over the specified vertical angle range θL ≤ θ < θH.

The sequence of the above-mentioned three averaging stages can be further discussed as follows. To represent the above averaging stages (a) and (c), the expected EIRP can be provided in the following equation:

EEIRPθLθH=12πsinθH-sinθLθLθH-ππEIRP1θ,φcosθdφdθ                            (1)

in which EIRP1θ,φ is the averaged value of EIRP with the above averaging stage (b), i.e., weighted averaging over N different beamforming directions within the BS horizontal and vertical steering range,

EIPR1θ,φ=n=1NwnPθ,φ;αn,βn                                                        (2)

where P(θ,φ;αn,βn)  is the EIRP of the base station in the horizontal direction φ and vertical direction θ, with the BS beamforming in the n-th direction of the horizontal direction αn and vertical direction βn.

Based on the following relationship, it has been shown that the averaging sequence of (a) (b) (c) is equivalent to (b) (a) (c)

EEIRPθLθH

=12πsinθH-sinθLθLθH-ππn=1NwnPθ,φ;αn,βncosθdφdθ     (averaging by the seq. of ba(c))

=12πsinθH-sinθLθLθHn=1Nwn-ππPθ,φ;αn,βndφcosθdθ     (averaging by the seq. of ab(c)).

Based on the above relationship, by changing the averaging sequence to (b)(a)(c), the definition of expected EIRP can be given as:

Proposal 1: By rearranging the sequence of three averaging operations, the definition of Expected EIRP is provided as:

- Expected EIRP (EEIRP) is defined as the average value of the EIRP, with the weighted averaging being performed over different beamforming directions within the BS horizontal and vertical steering range and the averaging being performed over horizontal angles from −180° to +180° and the specified elevation angle range θL ≤ θ < θH in Table 9.9.2-1.

Furthermore, the above description of EEIRP definition shall also be provided in the terminology description clause (i.e., Clause 3) of TS 38.104.

Proposal 2: The description of EEIRP definition shall be provided in the terminology description clause (i.e., Clause 3) of TS 38.104.

## 2.2 The necessity of declared CAR (Coverage Angular Range)

In the last RAN4 meeting, the definition of CAR declarations is given in the approved WF:

Agreement: Table 2.2.2-1: [CAR] declarations Parameter; Description. min; Minimum angle along  axis. max; Maximum angle along  axis. min; Minimum angle along  axis. max; Maximum angle along  axis. - NOTE 1: further discuss the definition of min max min max - NOTE 2: reusing the existing declaration is not precluded.

As described in the proponent company’s contribution [R4-2409118], the concept of CAR is defined as “a set of directions describing the intended angular operation range for which the expected EIRP requirement is met”, and “the test beams will be distributed within the declared CAR”. However, by adopting the definition of EEIRP in the Proposal 1, the concept of CAR is not necessary, since it is already emphasized “the weighted averaging being performed over different beamforming directions within the BS horizontal and vertical steering range”.

Since the above agreement is captured in the WF [6] without discriminating that it is for core or performance requirement, we would like to clarify by giving the following proposal:

Proposal 3: The concept of CAR (Coverage Angular Range) could be discussed for the conformance testing, while it is not relevant to the core requirement specified for the expected EIRP.

For the similar reason, the text “[With the BS generating beam peak directions within the OTA peak directions set.]” captured in the WF is also not necessary.

Proposal 4: The text “[With the BS generating beam peak directions within the OTA peak directions set.]” captured in the last meeting’s WF is not necessary in specifying the core requirement of expected EIRP.

## 2.3 Text proposal for OTA spatial emission requirement

For the detailed requirement, we would like to directly give the text proposal below, which provide changes over the agreed text proposal in the WF [6]:

Proposal 5: For OTA spatial emission requirement, the following text proposal is provided for NR base station in TS 38.104:

=================== Start of Text Proposal ==================

  - 9.9	OTA spatial emission

    - 9.9.1	General

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions.

    - 9.9.2	Protection of FSS UL

This requirement shall be applied to BS operating in band n104 to protect FSS (Earth-to-space) satellite receiver.

      - 9.9.2.1	Minimum requirement for BS type 1-H and BS type 1-O

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) for certain vertical angle range in the frequency range 6425 – 7075 MHz, shall not exceed the values specified in Ttable 9.9.2.1-1.

EEIRP is defined as the average value of the EIRP, with the weighted averaging being performed over different beamforming directions within the BS horizontal and vertical steering range and the averaging being performed over horizontal angles from −180° to +180° and the specified elevation angle range θL ≤ θ < θH in Table 9.9.2-1.

The elevation angle range θL ≤ θ < θH is illustrated in Figure 9.9.2.1-1.

[EEIRP is defined as the average value of the EIRP, with the averaging being performed over azimuth angles from −180° to +180 ° and over the specified elevation angle range [θL ≤ θ < θH] in table 9.9.2-1 ]

[With the BS generating beam peak directions within the OTA peak directions set.]

Table 9.9.2.1-1: EEIRP limits for certain vertical angle range as function of elevation above horizon

| Vertical Elevation angular range [θL ≤ θ < θH] (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- |
| 0<<5 | 27 |
| 5<<10 | 23 |
| 10<<15 | 19 |
| 15<<20 | 18 |
| 20<<30 | 16 |
| 30<<60 | 15 |
| 60<<90 | 15 |
| Note 2: The requirement shall apply to all supported mechanical tilts. |  |

Figure 9.9.2.1-1: Illustration of elevation angle range θL ≤ θ < θH Definitions of [θL and θH] angles.

=================== End of Text Proposal ===================

# 3. Conclusion

In this contribution, we provided our analysis on the expected EIRP mask for upper 6GHz, and discuss the related BS requirements for 6425-7075 MHz based on the WRC-23 outcome. Specifically, the following observations and proposals are provided:

Definition of Expected EIRP

Observation 1: In the tentatively agreed definition of Expected EIRP, it is not yet mentioned the averaging over different beamforming directions.

Observation 2: Based on Resolution 220 (WRC-23), the definition of expected EIRP is provided as:

- The Expected EIRP is defined as the average value of EIRP (in the specified vertical (elevation) angle range above the horizon, θL ≤ θ < θH) with the averaging being performed:

(a) over horizontal angles from −180 to +180, with the base station beamforming in a specific direction within its horizontal (azimuth) and vertical (elevation) steering range,

(b) over different beamforming directions within the BS horizontal and vertical steering range, and

(c) over the specified vertical angle range θL ≤ θ < θH.

Proposal 1: By rearranging the sequence of three averaging operations, the definition of Expected EIRP is provided as:

- Expected EIRP (EEIRP) is defined as the average value of the EIRP, with the weighted averaging being performed over different beamforming directions within the BS horizontal and vertical steering range and the averaging being performed over horizontal angles from −180° to +180° and the specified elevation angle range θL ≤ θ < θH in Table 9.9.2-1.

Proposal 2: The description of EEIRP definition shall be provided in the terminology description clause (i.e., Clause 3) of TS 38.104.

The necessity of declared CAR (Coverage Angular Range)

Proposal 3: The concept of CAR (Coverage Angular Range) could be discussed for the conformance testing, while it is not relevant to the core requirement specified for the expected EIRP.

Proposal 4: The text “[With the BS generating beam peak directions within the OTA peak directions set.]” captured in the last meeting’s WF is not necessary in specifying the core requirement of expected EIRP.

Text proposal for OTA spatial emission requirement

Proposal 5: For OTA spatial emission requirement, the following text proposal is provided for NR base station in TS 38.104:

=================== Start of Text Proposal ==================

  - 9.9	OTA spatial emission

    - 9.9.1	General

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions.

    - 9.9.2	Protection of FSS UL

This requirement shall be applied to BS operating in band n104 to protect FSS (Earth-to-space) satellite receiver.

      - 9.9.2.1	Minimum requirement for BS type 1-H and BS type 1-O

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) for certain vertical angle range in the frequency range 6425 – 7075 MHz, shall not exceed the values specified in Ttable 9.9.2.1-1.

EEIRP is defined as the average value of the EIRP, with the weighted averaging being performed over different beamforming directions within the BS horizontal and vertical steering range and the averaging being performed over horizontal angles from −180° to +180° and the specified elevation angle range θL ≤ θ < θH in Table 9.9.2-1.

The elevation angle range θL ≤ θ < θH is illustrated in Figure 9.9.2.1-1.

[EEIRP is defined as the average value of the EIRP, with the averaging being performed over azimuth angles from −180° to +180 ° and over the specified elevation angle range [θL ≤ θ < θH] in table 9.9.2-1 ]

[With the BS generating beam peak directions within the OTA peak directions set.]

Table 9.9.2.1-1: EEIRP limits for certain vertical angle range as function of elevation above horizon

| Vertical Elevation angular range [θL ≤ θ < θH] (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- |
| 0<<5 | 27 |
| 5<<10 | 23 |
| 10<<15 | 19 |
| 15<<20 | 18 |
| 20<<30 | 16 |
| 30<<60 | 15 |
| 60<<90 | 15 |
| Note 2: The requirement shall apply to all supported mechanical tilts. |  |

Figure 9.9.2.1-1: Illustration of elevation angle range θL ≤ θ < θH Definitions of [θL and θH] angles.

=================== End of Text Proposal ===================

# 4. Reference

[1] RP-240829, “NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, Huawei, RAN#103, March 2024.

[2] Final Acts, World Radiocommunication Conference 2023 (WRC-23), 20th Nov.-15th Dec. 2023.

[3] R4-2405656, “Discussion on expected EIRP mask for upper 6GHz”, Samsung, RAN4#110bis, April 2024.

[4] R4-2406088, “Way Forward for [110bis][311] NR_BS_RF”, ZTE, RAN4#110bis, April 2024.

[5] R4-2409766, “Discussion on expected EIRP mask for upper 6GHz”, Samsung, RAN4#111, May 2024.

[6] R4-2409955, “Way Forward for [111][311] NR_BS_RF”, ZTE, RAN4#111, May 2024.