3GPP TSG-RAN WG4 Meeting #112                                                                R4-2411642

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:	Discussion on OTA co-location reference antenna enhancement

Agenda item:			8.11.3

Document for:	Approval

# 1. Introduction

Based on the scope of Rel-19 work item on NR base station (BS) RF requirement evolution for FR1/FR2 and testing, the objective of OTA test enhancement has been identified. Evolving OTA test for BS type 1-O would prepare specifications for facilitating the industry to switch to all OTA conformance testing for AAS and M-MIMO capable BS, especially for new bands defined in the upper region of FR1. Particularly, among objectives of NR BS RF requirement evolution for FR1 and testing, the following objective related to OTA test enhancement is provided in WID [1]:

OTA test enhancement (core) - Investigate whether the BS/IAB OTA co-location reference antenna definition need be improved for FR1, and if feasible, update the definition. OTA test enhancement (performance) - Investigate whether the existing test approach can be improved, and if feasible, improve test methods for BS/IAB OTA co-location requirements and tests for AAS-based test specifications for FR1.

Based on the approved WF [8] in RAN4#110bis, it is planned to “Continue to discuss the applicability of existing co-location reference antenna and identify the problems if any” in RAN4#111, and the following relevant agreements are achieved:

- Issue 3-1: Co-location reference antenna Agreement: - Step 1: further investigate the improvement of the co-location reference antenna if necessary; - Step 2: If this is agreeable, then further discuss e.g. the relevant scenarios, applicable antenna types or define requirements on OTA parameters such as field-strength or power density

Furthermore the approved WF [10] achieved in RAN4#111 also provide the recommendation for further discussion:

- Issue 3-1: Co-location reference antenna - Recommended for further discussion - Further discuss the wideband antennas and check its possibility of translation between reference co-location antenna and wideband antenna; - Further discuss co-location scenario and coupling loss between two AAS BS especially for FR1 high bands; - Further discuss whether the side-by-side deployment is typical scenario for co-location deployment, e.g., in SBFD deployment TX and RX panels could not be in the same height - Other issues are not precluded.

In this contribution, we would like to provide our further analysis and viewpoints on the topic of OTA co-location reference antenna enhancement.

# 2. Discussion

## 2.1 Background of BS/IAB OTA co-location reference antenna

In the current BS RF requirement, co-location requirements are requirements which are based on assuming the BS type 1-O is co-located with another BS of the same base station class, which ensure that both co-located systems can operate with minimal degradation to each other. Co-location requirements are specified at the conducted interface of the co-location reference antenna, and the co-location reference antenna does not form part of the BS under test but is a means to provide OTA power levels (either for measuring emission or for injecting interferer signal) which are representative of a co-located system.

Particularly, the following requirements are regarded as co-location requirement:

- OTA transmit OFF power,

- OTA transmitter spurious emission (optional, depends on declaration)

- OTA transmitter intermodulation

- OTA out-of-band blocking co-location requirement (optional, depends on declaration).

From the definition of co-location reference antenna in the clause 4.9 in TS 38.104 [3], the co-location reference antenna shall satisfy:

- The co-location reference antenna shall be a single column passive antenna which has the same vertical radiating dimension (h), frequency range, polarization, as the composite antenna of the BS type 1-O and nominal 65° horizontal half-power beamwidth (suitable for 3-sector deployment);

- The co-location reference antenna shall be placed at a distance d from the edge of the BS type 1-O, and d shall be set to 0.1m, as shown in Figure 4.9-1 [3];

- The co-location reference antenna shall be aligned with the BS type 1-O composite antenna in a common plane perpendicular to the mechanical bore-sight direction, as shown in Figure 4.9-1 [3];

- The co-location reference antenna shall be aligned with the BS type 1-O composite antenna for the vertical radiating regions;

OTA co-location requirements are based on the power at the conducted interface of a co-location reference antenna, depending on the requirement this interface is either an input or an output. For BS type 1-O with dual polarization the co-location reference antenna has two conducted interfaces each representing one polarization.

Figure 1: Illustration of BS type 1-O enclosure and co-location reference antenna (Figure 4.9-1, [3])

For the above four kinds of co-location requirements, the detailed usage of co-location reference antenna to mimic a base station to a base station co-location scenario is provided in clause 4.12 in TS38.141-2 [4]:

Table 1: Co-location requirements (Table 4.12.1-1, [4])

| Clause number | Requirement | Co-location reference antenna operation | Type |
| --- | --- | --- | --- |
| 6.5 | OTA transmit ON/OFF power for FR1 | Measure emission | Mandatory |
| 6.7.5.3 6.7.5.5 | OTA spurious emission: Protection of the BS receiver of own or different BS | Measure emission | Optional based on declaration |
|  | OTA spurious emission: Co-location with other base stations |  |  |
| 6.8 | OTA transmitter intermodulation | Inject the interferer signal | Mandatory |
| 7.6.3 | OTA out-of-band blocking: Co-location with other base stations | Inject the interferer signal | Optional based on declaration |

## 2.2 Necessity of updating co-location reference antenna definition

Based on the discussion given in previous meetings, especially based on the achieved way forward [10] in RAN4#111, the following factors are proposed as the concerned reasons when RAN4 considers improvement of the BS/IAB OTA co-location reference antenna definition in this work item.

- 30dB MCL assumption does not yet hold between two AAS BS especially for FR1 high bands: As provided in [R4-2405628] and [R4-1706766], antenna port to port isolation would be much increased compared with the MCL 30dB assumption for 2.5-6GHz, where the original 30 dB isolation value was derived from a typical deployment scenario, in which two base stations are assumed to be with passive antennas mounted in two separated masts with main beam pointing down-wards. It should be noted that this isolation calculation can date back to 3G era, when the low range of FR1 and fixed passive base station antennas with equal antenna gain were assumed.

- The side-by-side deployment is not yet the only typical co-location deployment scenario: Whether or not it is a typical deployment scenario to have two base stations from different operator networks being located side-by-side shall be investigated, especially considering the high band BS deployment. In Rel-18 SBFD study item, it is also identified that the collocated BS’s TX panel shall be not side-by-side aligned with BS-Under-Test’s RX panel, with the assumption of TX/RX panel separation. The new product formfactor shall also be considered.

- Logistics and other practical issues when translating Co-Located Reference Antenna to CLTA: During the previous RAN4 discussion, the issue of band-specific CLTA is mentioned, and it is identified as difficult to find commercially signal/multi-column BS antenna with similar characteristics as specified in TS 38.141-2 for FR1 high bands. Furthermore, for NR FR1, the spectrum up to 7125 MHz have been allocated and accordingly AAS antennas with more complex array geometries than that originally assumed are used for base stations in nowadays commercial deployment.

Accordingly, to study whether the OTA co-location reference antenna definition for core requirement needs to be improved, RAN4 shall study the above aspects.

##### 2.2.1 General handling for CLRA definition

Considering that the impact of changing CLRA definition for 5G NR base station is not significant, we suggest the study shall be given by focusing on the AAS-to-AAS coexistence, which could prepare RAN4 for future 6G generation study. Without solid study, RAN4 will have no choice but to reuse what we have in 5G, especially during the busy Rel-20/21 for 6G system specification. Furthermore, we can understand the concern of putting extra effort in CLRA definition, due to the limited TU planned.

Proposal 1: RAN4 shall continue to study/refine CLRA/CLTA definition, by targeting to prepare these definitions for 6G base station specification (i.e., it could be too late to change NR specification).

Proposal 2: RAN4 could consider to study the CLRA definition in a longer timeframe, e.g., to suggest RAN-P to extend this study to next release if more time/study is identified to be needed.

##### 2.2.2 Further study on co-location deployment scenario

In the Rel-15 NR discussion on co-location concept in which the co-location reference antenna was defined, frequencies up to 2.5 GHz were considered in which where the original 30 dB isolation value was derived from a typical deployment scenario, in which two base stations are assumed to be with passive antennas mounted in two separated masts with main beam pointing down-wards. Here we suggest to further study the co-location scenarios between two AAS BS.

Here we suggest to use the three-sector scenario as a baseline, in which the antenna isolation (with the achievable coupling loss) is to be evaluated.

Proposal 3: RAN4 shall set up an evaluation plan to evaluate the antenna isolation in the AAS-to-AAS co-location deployment scenario, with the following parameters considered:

- 3 sector scenario is under consideration:

  - The angle between every two sectors’ boresight directions is 120 degrees;

  - Sector antenna panel’s width is 180mm;

  - Between two sectors’ antenna panel:

    - The center-to-center distance is: 150mm;

    - The nearest distance between edge to edge is: 60mm;

  - For each antenna panel:

    - Non-SBFD case: M column and N rows in each antenna panel;

    - SBFD case: M column and 2*N rows in each antenna panel, with certain structure between TX/RX panels;

  - 3.5GHz/upper 6GHz for operating frequency, with 100MHz bandwidth.

Figure 2: 3-sector scenario for co-channel co-site inter-sector antenna isolation study (Figure 10.2.2.2.1-1 in TR 38.858)

##### 2.2.2 MCL assumption between two AAS BS especially for FR1 high bands

When the co-location isolation between different frequency bands discussed in Rel-15, companies have discussed the antenna port-to-port isolation by investigating the isotropic path-loss at 0.12 m separation and total antenna gain with TX and RX antennas considered. The following table provided in [R4-1706766] is well cited:

Table 2: Antenna port-to-port isolation [R4-1706766]

| Isolation [dB] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Own frequency [GHz] | Co-location frequency [GHz] |  |  |  |  |  |  |
|  | 0.45-2 | 2-3.5 | 3.5-6 | 10 | 20 | 30 | 40 |
| 0.45-2 | 30 | 40 | 50 | 60 | 70 | 80 | 90 |
| 2-3.5 | 40 | 35 | 45 | 54 | 66 | 73 | 83 |
| 3.5-6 | 50 | 45 | 40 | 49 | 60 | 69 | 76 |
| 10 | 60 | 55 | 50 | 44 | 55 | 63 | 72 |
| 20 | 70 | 65 | 60 | 54 | 50 | 58 | 66 |
| 30 | 80 | 75 | 70 | 64 | 60 | 53 | 61 |
| 40 | 90 | 85 | 80 | 74 | 70 | 63 | 56 |

Furthermore, some measurements in the test lab have also been conducted [R4-2409605], in which antennas supporting 1427-2690MHz are used to evaluate the impacts of (1) different antenna separation distances and (2) vertical/horizontal placement. It has been shown that the pair-wise MCL is more than 30dB for 0.1m separation. However, for higher frequency operation point, we have no measurement data to justify or disprove the isolation is achievable with 0.1m separation between BS type 1-O and co-location reference antenna.

Proposal 4: After identifying the required MCL values for the co-location deployment scenario for different frequency bands, RAN4 can use the antenna port-to-port isolation estimation for side-by-side CLRA setup to to identify 0.1m separation is enough or not. (Note: the Table in R4-1706766 could be the exemplary antenna port-to-port isolation estimation).

# 3. Conclusion

In this contribution, we provided our viewpoints and initial analysis on the OTA test enhancement, particularly on the BS/IAB OTA co-location reference antenna definition:

Proposal 1: RAN4 shall continue to study/refine CLRA/CLTA definition, by targeting to prepare these definitions for 6G base station specification (i.e., it could be too late to change NR specification).

Proposal 2: RAN4 could consider to study the CLRA definition in a longer timeframe, e.g., to suggest RAN-P to extend this study to next release if more time/study is identified to be needed.

Proposal 3: RAN4 shall set up an evaluation plan to evaluate the antenna isolation in the AAS-to-AAS co-location deployment scenario, with the following parameters considered:

- 3 sector scenario is under consideration:

  - The angle between every two sectors’ boresight directions is 120 degrees;

  - Sector antenna panel’s width is 180mm;

  - Between two sectors’ antenna panel:

    - The center-to-center distance is: 150mm;

    - The nearest distance between edge to edge is: 60mm;

  - For each antenna panel:

    - Non-SBFD case: M column and N rows in each antenna panel;

    - SBFD case: M column and 2*N rows in each antenna panel, with certain structure between TX/RX panels;

  - 3.5GHz/upper 6GHz for operating frequency, with 100MHz bandwidth.

Proposal 4: After identifying the required MCL values for the co-location deployment scenario for different frequency bands, RAN4 can use the antenna port-to-port isolation estimation for side-by-side CLRA setup to to identify 0.1m separation is enough or not. (Note: the Table in R4-1706766 could be the exemplary antenna port-to-port isolation estimation).

# 4. Reference

[1] RP-240829, “NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, Huawei, RAN#103, March 2023.

[2] 3GPP TR 38.817-02 v15.11.0, “General aspects for Base Station (BS) Radio Frequency (RF) for NR (Release 15)”.

[3] 3GPP TS 38.104 v18.4.0, “Base Station (BS) radio transmission and reception (Release 18)”.

[4] 3GPP TS 38.141-2 v18.4.0, “Base Station (BS) radio transmission and reception (Release 18)”.

[5] 3GPP TR 25.942 v13.0.0, “Radio Frequency (RF) system scenarios (Release 13)”.

[6] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[7] R4-2405657, “Discussion on OTA co-location reference antenna enhancement”, Samsung, RAN4#110bis, April 2024.

[8] R4-2406088, “Way Forward for [110bis][311] NR_BS_RF”, ZTE, RAN4#110bis, April 2024.

[9] R4-2409767, “Discussion on OTA co-location reference antenna enhancement”, Samsung, RAN4#111, May 2024.

[10] R4-2409955, “Way Forward for [111][311] NR_BS_RF”, ZTE, RAN4#111, May 2024.