3GPP TSG-RAN WG4 Meeting #112                                                                R4-2411643

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:	Discussion on OTA spatial emission conformance testing

Agenda item:			8.11.4

Document for:	Approval

# 1. Introduction

In ITU-R world radiocommunication conference (WRC) in Nov. 20 - Dec. 15 2023,  the frequency band of 6 425-7 125 MHz was identified in every ITU region as in the RR Footnotes 5.6A12, 5.6B12 and 5.6C12 with associated technical condition of limits on the expected equivalent isotropically radiated power (EIRP) spectral density of IMT base-stations, for the purpose of protecting earth-to-space fixed satellite services (FSS) as in Resolution 220 (WRC-23). Based on the newly released ITU-R regulation, the expected EIRP limits have been established which require the harmonized specification for compliance tests to be developed as the task in 3GPP.

Specifically, the aim of this task is to include the new regulation in BS (and other nodes like IAB, Repeater and NCR) RF core specification (TS 38.104) and a corresponding test requirement in the conformance test specification (TS 38.141-2). Furthermore, having the requirement included in the 3GPP standard would also guarantee a harmonized terminology and conformance test method. Accordingly, the performance part objective related to expected EIRP mask for upper 6GHz is provided in WID [1] as:

Expected EIRP mask for upper 6GHz - Specify manufacturer declarations, conformance test descriptions, test tolerances and test requirements. - Note: target completion date of this objective is December 2024.

And much progress on performance requirement has been achieved in RAN4#111 [6], with the following relevant agreements obtained:

Agreement: Table 2.2.2-1: [CAR] declarations Parameter; Description. min; Minimum angle along  axis. max; Maximum angle along  axis. min; Minimum angle along  axis. max; Maximum angle along  axis. - NOTE 1: further discuss the definition of min max min max - NOTE 2: reusing the existing declaration is not precluded. Issue 2-7 Expected EIRP calculation in discrete spatial sampling grid Agreement: - PθL , θH=π2MNsinθH-sinθLn=nLnHm=1MP1θn,φm cosθn , where P1 is measured as EIRP - M is the number of sampling points in the azimuth range; - [N is the number of sampling points within the θL , θH bounding range over upper hemisphere in the elevation range ] - nL is the lowest elevation sampling angle within the θL , θH bounding range - nH is the highest elevation sampling angle within the θL , θH bounding range - m=1MP1θn,φm is to produce the global sum over the whole azimuth ranges for EIRP above the horizon in a given bin. - Note: the equation is assuming the angles in radians and the elevation angle is from horizon rather than the coordinate system defined in 3GPP. Note: the above equation should be carefully reviewed by companies.

Furthermore, the following open issues are also summarized and provided [6]:

Issue 2-8 Expected EIRP sampling grid for average EIRP - Recommended for further discussion: - Need further discussions Issue 2-9 Other related with conformance testing declaration and RF channels - Recommended for further discussion: - The following issues to be considered: - the number of mechanical down-tilt and its corresponding angular coverage range; - For each angular coverage range, the number of beams for conformance testing and its corresponding weighting factor; - To balance the number for beams for conformance testing/complexity and test accuracy; - The impacts of potential factors (measurement sampling grid for summation error etc) on EIRP accuracy. - in conformance testing, consideration of converting coordinate system to 3GPP coordinate system, to align with other test cases - Other issues are not precluded. Issue 2-10 Other related with RF channels - Recommended for further discussion: - Need further discussions Issue 2-11 Other related with confidence intervals - Recommended for further discussion: - Need further discussions

In this contribution, we would like to provide our viewpoints on OTA spatial emission conformance testing, based on the WRC-23 outcome.

# 2. Discussion

## 2.1 Expected EIRP calculation

In the Annex to Resolution 220 (WRC-23) [2], details for the calculation of the expected equivalent isotropically radiated power for assessing the compliance of an IMT base station equipment operating within the frequency band 6 425-7 125 MHz with the limit on expected EIRP are provided:

ANNEX TO RESOLUTION 220 (WRC-23) ... The e.i.r.p. of an IMT base station in the horizontal (azimuth) direction −π ≤ φ ≤ π and vertical (elevation) direction 0 ≤ θ ≤ π/2 above the horizon can be written as P(θ, φ; α, β). The parameters α and β are the horizontal and vertical beamforming directions, i.e. the angles towards which the base station electronically steers a beam. These are illustrated in Figure 1 below. The expected e.i.r.p. of an IMT base station within a vertical angle range θL ≤ θ < θH can be calculated by averaging the e.i.r.p. P(θ, φ; α, β) of the base station as follows: 1) Averaging over beamforming directions for a given vertical angle θ0 and horizontal angle φ0: for an AAS base station within a given horizontal and vertical steering range, a sufficient sampling of N beamforming directions (αn, βn) n = 1 ... N is necessary to allow an accurate averaging of the expected e.i.r.p. The beamforming directions (αn, βn) have a uniform statistical angular distribution within the steering range of the IMT base station. In other words: where wn refers to the weight for the nth beamforming direction, i.e. the fraction of the steering range represented by the nth beamforming direction. For example, wn = 1/N in the case that N uniform equispaced beams are assumed in the azimuth and elevation, respectively, and where each beam covers an equal range of angles. The set of base station configurations over which the base station complies with the limits on expected e.i.r.p. (for example, power of steering range as one of the parameters) shall be declared and the BS shall be used within one of these configurations. The set of e.i.r.p. values used to calculate the expected e.i.r.p. for each vertical angle range shall be a mathematical summation of both polarization states of the IMT base station antenna with no polarization discrimination. For a non-AAS base station, P1(θ0, φ0) = P(θ0, φ0; α1, β1) where α1 = 0 and β1 is the electrical tilt. It is noted that the compliance with the limits on expected e.i.r.p. should be limited to a defined range of electrical tilts. 2) Averaging over horizontal and vertical angles: the expected e.i.r.p. is then calculated by averaging the results of step 1 over horizontal angles φ from −π to +π with respect to the base station horizontal boresight, and vertical angles θ within vertical angle measurement window θL ≤ θ < θH with respect to the horizon. In other words: The averaging processes in steps 1 and 2 shall allow for accurate averaging of the expected e.i.r.p (e.g. to the confidence interval of 95%).

Based upon the above detailed description, the expected EIRP over the specified vertical angle range θL ≤ θ < θH is calculated by performing two steps of averaging, i.e., (1) averaging over beamforming directions for a given vertical angle θ and horizontal angle φ, and (2) averaging over horizontal and vertical angles.

To represent the above averaging operation in stage (2), the expected EIRP can be provided as follows:

EEIRPθLθH=12πsinθH-sinθLθLθH-ππEIRP1θ,φcosθdφdθ                                  (1)

in which EIRP1θ,φ is the averaged value of EIRP with the above averaging stage (1), i.e., weighted averaging over NBeam different beamforming directions within the BS horizontal and vertical steering range,

EIPR1θ,φ=n=1NBeamwnPθ,φ;αn,βn                                                               (2)

where P(θ,φ;αn,βn)  is the EIRP of the base station in the horizontal direction φ and vertical direction θ, with the BS beamforming in the n-th direction of the horizontal direction αn and vertical direction βn.

Observation 1: Based on Annex to Resolution 220 (WRC-23), the expected EIRP over the specified vertical angle range θL ≤ θ < θH is calculated by performing two steps of averaging, i.e.,

- Averaging over beamforming directions for a given vertical angle θ and horizontal angle φ,

EIPR1θ,φ=n=1NBeamwnPθ,φ;αn,βn

- Averaging over horizontal and vertical angle ranges:

EEIRPθLθH=12πsinθH-sinθLθLθH-ππEIRP1θ,φcosθdφdθ

### 2.1.1 Averaging over beamforming directions

Particularly in the step of averaging over beamforming directions for a given vertical angle θ and horizontal angle φ, a sufficient sampling of NBeam beamforming directions (αn, βn) n = 1 ... NBeam is necessary to allow an accurate averaging of the expected EIRP and the beamforming directions (αn, βn) have a uniform statistical angular distribution within the steering range of the IMT base station.

Observation 2: Based on Annex to Resolution 220 (WRC-23), when performing the averaging over beamforming directions for a given vertical angle θ and horizontal angle φ, the beamforming directions (αn, βn) n = 1 ... NBeam shall be selected by:

- allowing an accurate averaging of the expected EIRP, and

- having the beamforming directions (αn, βn) with a uniform statistical angular distribution within the steering range of the IMT base station.

Accordingly, the following issue related to conformance testing should be noted: Compared with TRP requirement, which also takes EIRP values performed over the whole sphere, it should be noted that the OTA spatial emission above horizon shall be performed for each beamforming direction, e.g., NBeam = 64 is considered for the potential beamforming direction number, the test effort will be enlarged by 64 times. The much-increased test burden and the according testability issue should be studied by RAN4.

Observation 3: The number of beamforming directions NBeam for a given vertical angle θ and horizontal angle φ could greatly impact the testing effort/burden for EEIRP conformance testing.

Straightforwardly, the following proposal is given for the average value of measured EIRP over NBeam different beamforming directions, which will be used for the expected EIRP calculation in discrete spatial sampling grid.

Proposal 1: For the given direction of the vertical angle θn and the horizontal angle φm, EIRP1θn,φm is the weighted average of measured EIRP values over NBeam different beamforming directions within the BS horizontal and vertical steering range, as follows

EIPR1θn,φm=n=1NBeamwnPθ,φ;αn,βn

### 2.1.2 Averaging over horizontal and vertical angle ranges

For the averaging operation over horizontal and vertical angles, by considering measurement in the discrete spatial sampling grid, we assume the definitions of M and N by generally following the ones given in the approved WF with few wording modifications.

Proposal 2: For the measurement in the discrete spatial sampling grid, the definitions of the numbers of sampling points in the horizontal and vertical range should be revised as:

M is the number of the equally divided intervals over the horizontal (azimuth) range of −180° to +180°;

N is the number of the equally divided intervals over the specified vertical (elevation) range of θL to θH.

By considering the definitions of M and N and assuming the evenly divided intervals over the angle ranges, the measurement grid step sizes are ∆φ=2π/M and ∆θ=(θH-θL)/N, respectively.

Proposal 3: By considering the definitions of M and N and assuming the evenly divided intervals over the angle ranges, the measurement grid step sizes are ∆φ=2π/M and ∆θ=(θH-θL)/N.

As required by the approved WF [6], the equation for the expected EIRP calculation in discrete spatial sampling grid “should be carefully reviewed by companies”. Accordingly, we would like to provide the mathematic deduction here. The expected EIRP in equation (1) can be approximated by considering measurement in the discrete spatial sampling grid, i.e.,

EEIRPθLθH=12πsinθH-sinθLθLθH-ππEIRP1θ,φcosθdφdθ

≈12πsinθH-sinθLθ=θLθHφ=-ππEIRP1θ,φcosθ∆φ∆θ

### 2.1.3 Expected EIRP calculation in discrete spatial sampling grid

Accordingly, we provide the following proposal in which the expected EIRP calculation in discrete spatial sampling grid:

Proposal 4: For measurement in the discrete spatial sampling grid, the expected EIRP over the specified vertical angle range θL ≤ θ < θH  shall be calculated as

EEIRPθLθH=θH-θLMNsinθH-sinθLn=1Nm=1MEIRP1θn,φmcosθn

in which

EIRP1θn,φm is the weighted average of measured EIRP values for the given direction of the vertical angle θn and the horizontal angle φm, with the averaging over NBeam different beamforming directions within the BS horizontal and vertical steering range,

M is the number of the equally divided intervals over the horizontal (azimuth) range of −180° to +180°,

N is the number of the equally divided intervals over the specified vertical (elevation) range of θL to θH,

θn is the vertical angle measured within the n-th interval out of the vertical range of θL to θH, where the interval is from θL+n-1NθH-θL to θL+nNθH-θL,

φm is the horizontal angle measured within the m-th interval out of the horizontal range of −180° to +180°, where the interval is from -π+2π∙m-1M to -π+2π∙mM.

## 2.2 Declared CAR (Coverage Angular Range) in Conformance Test

In the last RAN4 meeting, the definition of CAR declarations is given in the approved WF:

Agreement: Table 2.2.2-1: [CAR] declarations Parameter; Description. min; Minimum angle along  axis. max; Maximum angle along  axis. min; Minimum angle along  axis. max; Maximum angle along  axis. - NOTE 1: further discuss the definition of min max min max - NOTE 2: reusing the existing declaration is not precluded.

As described in the proponent company’s contribution [R4-2409118], the concept of CAR is defined as “a set of directions describing the intended angular operation range for which the expected EIRP requirement is met”, and “the test beams will be distributed within the declared CAR”.

Based on the above-mentioned agreement, the CAR concept could be visualized as a rectangular area in the q/j plane, in which the test beams will be located. Obviously, the number of test beams and the location within the q/j plane will impact the measured EEIRP profile.

Furthermore, as described in Observation 2, based on Annex to Resolution 220 (WRC-23), the beamforming directions (αn, βn) shall have a uniform statistical angular distribution within the steering range of the IMT base station. And for the given beamforming directions, the weight for each beamforming direction, i.e. the fraction of the steering range represented by the nth beamforming direction, is also introduced by WRC-23.

To balance the testing burden and guaranteeing the conformance from practical beam books, we agree that RAN4 could consider to define a concept where the test beam directions are related to the declared Coverage Angular Range (CAR) and the number of test beam directions are specified.

Proposal 5: For conformance testing, RAN4 could consider to define a concept where the test beam directions are related to the declared Coverage Angular Range (CAR), only if:

(1) CAR declaration shall be totally up to BS vendor, i.e., FFS the possibility of not specifying certain CAR in RAN4 specifications;

(2) the number of test beam directions are reasonably small to reduce test burden;

(3) the weight of each test beam shall not be specified as part of CAR.

# 3. Conclusion

In this contribution, we provided our analysis on the expected EIRP mask for upper 6GHz (i.e., the OTA spatial emission requirement), and discuss the related BS conformance testing based on the WRC-23 outcome. Specifically, the following observations and proposals are provided:

Expected EIRP calculation

Observation 1: Based on Annex to Resolution 220 (WRC-23), the expected EIRP over the specified vertical angle range θL ≤ θ < θH is calculated by performing two steps of averaging, i.e.,

- Averaging over beamforming directions for a given vertical angle θ and horizontal angle φ,

EIPR1θ,φ=n=1NBeamwnPθ,φ;αn,βn

- Averaging over horizontal and vertical angle ranges:

EEIRPθLθH=12πsinθH-sinθLθLθH-ππEIRP1θ,φcosθdφdθ

Observation 2: Based on Annex to Resolution 220 (WRC-23), when performing the averaging over beamforming directions for a given vertical angle θ and horizontal angle φ, the beamforming directions (αn, βn) n = 1 ... NBeam shall be selected by:

- allowing an accurate averaging of the expected EIRP, and

- having the beamforming directions (αn, βn) with a uniform statistical angular distribution within the steering range of the IMT base station.

Observation 3: The number of beamforming directions NBeam for a given vertical angle θ and horizontal angle φ could greatly impact the testing effort/burden for EEIRP conformance testing.

Proposal 1: For the given direction of the vertical angle θn and the horizontal angle φm, EIRP1θn,φm is the weighted average of measured EIRP values over NBeam different beamforming directions within the BS horizontal and vertical steering range, as follows

EIPR1θn,φm=n=1NBeamwnPθ,φ;αn,βn

Proposal 2: For the measurement in the discrete spatial sampling grid, the definitions of the numbers of sampling points in the horizontal and vertical range should be revised as:

M is the number of the equally divided intervals over the horizontal (azimuth) range of −180° to +180°;

N is the number of the equally divided intervals over the specified vertical (elevation) range of θL to θH.

Proposal 3: By considering the definitions of M and N and assuming the evenly divided intervals over the angle ranges, the measurement grid step sizes are ∆φ=2π/M and ∆θ=(θH-θL)/N.

Proposal 4: For measurement in the discrete spatial sampling grid, the expected EIRP over the specified vertical angle range θL ≤ θ < θH  shall be calculated as

EEIRPθLθH=θH-θLMNsinθH-sinθLn=1Nm=1MEIRP1θn,φmcosθn

in which

EIRP1θn,φm is the weighted average of measured EIRP values for the given direction of the vertical angle θn and the horizontal angle φm, with the averaging over NBeam different beamforming directions within the BS horizontal and vertical steering range,

M is the number of the equally divided intervals over the horizontal (azimuth) range of −180° to +180°,

N is the number of the equally divided intervals over the specified vertical (elevation) range of θL to θH,

θn is the vertical angle measured within the n-th interval out of the vertical range of θL to θH, where the interval is from θL+n-1NθH-θL to θL+nNθH-θL,

φm is the horizontal angle measured within the m-th interval out of the horizontal range of −180° to +180°, where the interval is from -π+2π∙m-1M to -π+2π∙mM.

Declared CAR (Coverage Angular Range) in Conformance Test

Proposal 5: For conformance testing, RAN4 could consider to define a concept where the test beam directions are related to the declared Coverage Angular Range (CAR), only if:

(1) CAR declaration shall be totally up to BS vendor, i.e., FFS the possibility of not specifying certain CAR in RAN4 specifications;

(2) the number of test beam directions are reasonably small to reduce test burden;

(3) the weight of each test beam shall not be specified as part of CAR.

# 4. Reference

[1] RP-240829, “NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, Huawei, RAN#103, March 2024.

[2] Final Acts, World Radiocommunication Conference 2023 (WRC-23), 20th Nov.-15th Dec. 2023.

[3] R4-2405656, “Discussion on expected EIRP mask for upper 6GHz”, Samsung, RAN4#110bis, April 2024.

[4] R4-2406088, “Way Forward for [110bis][311] NR_BS_RF”, ZTE, RAN4#110bis, April 2024.

[5] R4-2409766, “Discussion on expected EIRP mask for upper 6GHz”, Samsung, RAN4#111, May 2024.

[6] R4-2409955, “Way Forward for [111][311] NR_BS_RF”, ZTE, RAN4#111, May 2024.