3GPP TSG RAN WG4 Meeting #112	R4-2411644

Maastricht, 19 Aug. 2024 – 23 Aug. 2024

Agenda Item:	8.10.4

Source: 	Ericsson

Title:	Discussion on Rel-19 ATG CA RRM requirements

Document for:	Discussion

# Introduction

In Rel-18, RAN4 has finalized the air-to-ground (ATG) network work item (WI), emphasizing the FR1 single carrier scenario. During the RAN plenary meeting #103, a new WI was endorsed to enhance the ATG network for NR by supporting carrier aggregation (CA). The main goals are to enhance throughput, address growing ATG capacity requirements, and optimize the use of operators' spectrum, with the following objectives copied from the WID [1]:

### 4.1 Objective of SI or Core part WI or Testing part WI The core part of the work item includes: - Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]: - FR1 intra-band contiguous CA - Example band combination: n79 - FR1+FR1 inter-band CA - Example band combination: n3+n39 - Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4] - Specify signaling and UE capabilities if necessary [RAN2]. ### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. The performance part of the work item includes: - Specify corresponding RRM performance and demodulation requirements for intra-band co-located and inter-band co-located DL CA. - Specify corresponding demodulation requirements for support of UL MIMO with 2TX for single CC for UE with dual-polarization antenna.

Since this is the first meeting for this WI in RAN4, we look at the RRM requirements that may be impacted by the introduction of ATG DL CA.

# Discussion

## Scenarios

Based on the objectives of Rel-19 WID, the following two scenarios are apparent for both intra-band and inter-band.

- Intra-band co-located contiguous DL CA.

- Inter-band co-located DL CA

However, it is not clearly stated in the text whether the following case should be studied and specified in Rel-19:

- Intra-band co-located non-contiguous DL CA

Therefore, we propose to limit the scope of the ATG-enhanced WI to intra-band co-located contiguous DL CA and inter-band co-located DL CA scenarios.

RAN4 to discuss and agree the scope of the WI is limited to the following two scenarios stated in the WID

  - Intra-band co-located contiguous DL CA.

  - Inter-band co-located DL CA

## Cell Re-selection

The cell re-selection procedure allows the UE to choose a more suitable cell and camp on it. The NR intra-frequency and inter-frequency carriers specification defines the UE's measurement capability for ATG cell re-selection in clause 4.2D of TS 38.133. However, the measurement capability requirements for ATG in IDLE mode do not consider CA. Therefore, we need to study and define clause 4.2D.2 based on the legacy requirements of clause 4.2.2 to support ATG CA.

RAN4 to reuse NR cell re-selection as a baseline for ATG cell re-selection to support CA.

Furthermore, if RAN4 decides to support early measurement reporting (EMR) while the T331 timer is running, the general requirements for ATG DL CA need to be specified. Due to advancements in cell re-selection measurement techniques, leveraging the Rel-18 capabilities would be more appropriate. Also, Rel-18 resolves issues observed in Rel-16. Therefore, it is recommended that RAN4 consider supporting early measurement reporting for ATG by adopting the Rel-18 NR CA EMR as a baseline.

RAN4 to define EMR for ATG DL CA by using the Rel-18 NR CA EMR as a baseline.

## Mobility Requirements for ATG CA

The NR handover for NR ATG involves changing the NR PCell to another NR cell. Hence, the introduction of ATG CA will not impact the handover of ATG in general.

ATG handover requirements are not impacted by the introduction of CA.

## Timing Requirements

## Timing Advanced, Transmit Timing and Timer Accuracy

The timing advance, transmit timing, and timer accuracy are initiated by ATG UE configured with only PCell. Hence, Supporting CA for ATG will not impact UE Timing advanced, transmit timing, and timer accuracy.

The introduction of ATG CA does not impact UE Timing advanced, UE transmit timing, and UE timer accuracy requirements.

## Maximum Receive Time Difference (MRTD)

According to the WID [1], The scope of the WI is co-located cases. In co-located scenarios, the antennas are generally expected to be co-located within a single site. According to [3], co-location means placing antennas at the same physical location or in proximity, such as on different parts of a rooftop where a gNB represents a logical grouping of functionalities typically implemented within a base station at a single site. This definition is valid in the context of 5G networks when using technologies like MIMO, Tx diversity, and intra-band contiguous carrier aggregation. RAN 4 should first discuss the definition of co-location in the context of ATG in Rel-19.

RAN4 should discuss and define co-location in the context of ATG in Rel-19.

Section 7.6.4 of TS 38.133 defines the minimum requirements for NR CA across different configurations, including Intra-band contiguous CA, Intra-band non-contiguous CA, and Inter-band NR CA. In the context of ATG, the specific deployment conditions may have implications for MRTD requirements.

MRTD can be calculated based on the following formula.

MRTD = TAE + ΔRF_prop + Dispersion

Where,

TAE = Time alignment error defined in 38.104 clause 6.5.3.3 for CA.

ΔRF_prop= is the difference in the time taken for signals to travel from the base station to the receiver across different paths or frequencies.

Dispersion = the spread of signal arrival times at the receiver due to multipath propagation.

Now for the inter-band CA in ATG deployments, where base station sites are co-located and the UE is within 100 meters on an aircraft, the MRTD is calculated as case,

TAE = 3 µs.

ΔRF_prop= 300ns, calculated for the ATG cell edge case, as depicted in Fig 1, where the airplane-to-BS distance is 20km, the cell coverage range is 300km, the flight speed is 1200km/hr, and the distance of the collocated antenna is 100m.

Dispersion = 245ns. In the Rel-15 study report [4], section 6.7.2 the Delay Spread (DS) worst case LOS scenario for 20 degrees or higher elevation has a Delay spread (DS) lgDS = log10(DS/1s) with mlgDS = -7.28 and slgDS = 0.67. This means that mlgDS +slgDS = -7.28 + 0.67 = -6.61 and DS < 0.245 µs with 68 % probability, if we assume Gaussian statistics [4, Table 6.7.2-1a].

So, for inter-band co-located CA

MRTD= 3 µs+ .3 µs+.245 µs= 3.545 µs

The MRTD specification of 33 µs for non-colocated deployments is significantly larger than the co-located ATG MTRD requirement. Hence, RAN4 should further study and define MRTD requirements to ensure they are aligned with co-located ATG deployments.

Fig 1: ATG in the cell edge case

For the FR1 intra-band contiguous case we have:

TAE = 0,26 µs

Then the same formula MRTD = TAE + ΔRF_prop + Dispersion, for the same assumptions above gives us:

MRTD > 0.26 µs + 0.3 µs + 0.245 µs = 0.805 µs.

The value of MRTD > 0.805 µs corresponds to 0.805/4.69 ≈ 17 % of CP for SCS = 15 kHz and 35% of CP for SCS = 30 kHz. To have signals within CP is the norm for a working CP-OFDM system.

RAN4 should discuss and define MRTD requirements for co-located ATG CA deployments, using the corresponding NR CA requirements as a baseline.

## SCell Activation/Deactivation

TN Scell activation requirements are defined for standalone NR carrier aggregation in FR1, with known and unknown Scells, intra-band and inter-band serving cells. The procedure and requirements provide a complete framework covering possible ATC DL CA scenarios. Hence, the Scell activation requirements for NR CA can be reused as a baseline for ATG DL CA.

RAN4 to define Scell activation/deactivation requirements for ATG CA by using the corresponding NR CA requirements as a baseline.

## SCell activation delay

From RAN4#111, Fukuoka meeting agreements, we can see that the RAN4 RF agreed there are no limitations on antenna types for ATG CA. The text from the meeting minutes is copied below:

Sub-topic 3-1 antenna type for inter-band CA Issue 3-1: clarify the antenna type for each band for inter-band CA Agreement: - No limitation on antenna types for ATG CA - FFS on whether to assume omni-antenna type can be assumed for both band n3 and n39 in DL CA_n3-n39 - FFS on whether new capability is needed for ATG CA

In accordance with the agreement, we need to explore all potential combinations of omnidirectional and array antennas for the study. Here is a list of the possible antenna combinations:

1. ATG UE with omnidirectional antennas on both PCell and SCell/s.

2. ATG UE with an omnidirectional antenna on PCell and an antenna array on SCell/s.

3. ATG UE with an antenna array on PCC and an omnidirectional antenna on SCell/s.

4. ATG UE with an antenna array on both PCell and SCell/s with only one antenna pattern.

5. ATG UE with an antenna array on both PCell and SCell/s with separate antenna patterns.

To better understand the impact of different antenna types on the SCell activation requirement, let's examine the SCell activation time requirements.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where: THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3] Tactivation_time is the SCell activation delay in millisecond. If the SCell is known and belongs to FR1, Tactivation_time is: - TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms. - TFirstSSB_MAX + Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms. If the SCell is unknown and belongs to FR1, and if one of the following conditions is met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is: - If UE supports shortMeasInterval-r18, then - TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + 5ms, if the following conditions are met, - the SCell is contiguous to an active serving cell in the same band, and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell, and - its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB; - TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + 2*Trs, enhanced + 5ms, otherwise. - Otherwise - TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, if the following conditions are met, - the SCell is contiguous to an active serving cell in the same band, and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell, and - its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB; - TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, otherwise. otherwise, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is: - If UE supports shortMeasInterval-r18, then - 6ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - Otherwise - 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell: - the SCell is contiguous to an active serving cell in the same band, and - A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell - its RTD with contiguous FR1 active serving cell is larger than 260ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB; If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided - The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and - The difference of the reception power with the contiguous active serving cell is <= 6dB, and - The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band. For a UE supporting scellWithoutSSB-InterBandCA-r18if the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell, and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled, - The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and - The [EPRE] difference at the UE is smaller than or equal to [12] dB, where, [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC], and - The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell. where the reference serving cell can be indicated by higherlayer parameter referenceCell-r18. If UE is not indicated with referenceCell-r18, the reference serving cell is assumed to be the QCL-typeC source cell if there is only one active QCL-typeC source cell configured. Editor notes: FFS whether and how to capture the wording “after the compensation for AGC”. Tactivation_time is - Tfirst_TRS + TTRS +5 ms, if aperiodic CSI-RS resources are not configured for SCell activation or UE do not support aperiodicCSI-RS-FastScellActivation-r17 - Tfirst_ATRS + Tgap + TATRS + 5 ms if aperiodic CSI-RS resources are configured for Scell activation for UE supporting aperiodicCSI-RS-FastScellActivation-r17

As marked in the boxed text, the activation delay of SCell may vary depending on the combination of different antenna types in PCell and SCell/s. Hence, SCell activation should be further investigated for ATG CA in Rel-19.

SCell activation requirements shall be further investigated for potential impact, taking into account the band combinations stated in the WID and the antenna types agreed in RF.

## Measurement Procedure

    - Measurement Gap

Measurement gap defined in Rel-18 for NR carrier aggregation should be reused as a baseline for ATG DL CA.

RAN4 to reuse NR carrier aggregation measurement gap as the baseline for ATG DL CA.

## Carrier specific scaling factor

In TS 38.133, carrier carrier-specific scaling factor (CSSF) is specified in clause 9.1D.5. RAN4 needs to study this section and accommodate CSSF for ATG DL CA for FR1.

RAN4 to study and accommodate CSSF requirements for ATG DL CA.

## Scheduling Availability/Restrictions

In Rel-18, different scheduling availabilities are specified for ATG omnidirectional antennas and antenna arrays. However, as no limitations on antenna types for ATG CA were agreed upon in the RAN4#111 meeting, we need to study the possible impact of different antenna combinations on scheduling availability for ATG CA.

RAN4 to further study the impact of antenna pattern on scheduling availability ATG CA.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 aspects for ATG CA and made the following observations.

Observation 1	ATG handover requirements are not impacted by the introduction of CA.

Observation 2	The introduction of ATG CA does not impact UE Timing advanced, UE transmit timing, and UE timer accuracy requirements.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN4 to discuss and agree the scope of the WI is limited to the following two scenarios stated in the WID

	Intra-band co-located contiguous DL CA.

	Inter-band co-located DL CA

Proposal 2	RAN4 to reuse NR cell re-selection as a baseline for ATG cell re-selection to support CA.

Proposal 3	RAN4 to define EMR for ATG DL CA by using the Rel-18 NR CA EMR as a baseline.

Proposal 4	RAN4 should discuss and define co-location in the context of ATG in Rel-19.

Proposal 5	RAN4 should discuss and define MRTD requirements for co-located ATG CA deployments, using the corresponding NR CA requirements as a baseline.

Proposal 6	RAN4 to define Scell activation/deactivation requirements for ATG CA by using the corresponding NR CA requirements as a baseline.

Proposal 7	SCell activation requirements shall be further investigated for potential impact, taking into account the band combinations stated in the WID and the antenna types agreed in RF.

Proposal 8	RAN4 to reuse NR carrier aggregation measurement gap as the baseline for ATG DL CA.

Proposal 9	RAN4 to study and accommodate CSSF requirements for ATG DL CA.

Proposal 10	RAN4 to further study the impact of antenna pattern on scheduling availability ATG CA.

# References

- RP- 240839 “New WID on Enhancements for Air-to-ground network for NR”, Apple

- 3GPP TS 38.133: "NR; Requirements for support of radio resource management"

- RP-180037, LS response on work to support IMT-2020/5G in the Transport Network

- TR 38.811, Study on New Radio (NR) to support non-terrestrial networks (Release 15)