3GPP TSG-RAN WG4 Meeting #112															R4-2411645

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.22.2.2

Source:	Apple

Title:	On Low-power Wake-up Receiver for NR

WI/SI:	NR_LPWUS-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

LP-WUS/WUR shall be introduced to NR in Rel-19. The objectives are covered in the WID [1]. This contribution focuses on the RAN4 core requirements and the way forward [2] from last meeting. The WID lists RAN4 goals as:

# 2  Discussion

##### 2.1 On MDR and FAR metric

The WF [2] captures the state of the discussion as show below:

Issue 1-1-11: Metric for LP_WUR requirements Agreements: - MDR is used as the metric for LP_WUR Rx RF requirements - FFS MDR values. - FAR will be defined as demodulation requirements - FFS FAR values.

During the last meetings it has been discussed that the proposed 1% missed detection rate might lead to demanding test scenarios. It is essential to use 1% value for MDR and FAR in simulations to guarantee optimum performance and conclusions on parameters such as SNR. However, from testing point of view it is challenging to verify a 1% performance metric. To alleviate the testing challenge a value of 10% for X was proposed instead. This number was discussed but appears to be high. As RAN4 couldn’t find an agreement the MDR and FAR values were left FFS. Considering the discussion in RAN4#111 it seems that defining a value of 5% for test purpose could be a compromise achieving balance between actual network performance and test complexity.

Proposal 1: Considering the discussion in RAN4#111 it seems that defining a value of 5% for test purpose could be a compromise to achieve balance between actual network performance and test complexity.

##### 2.1 On noise figure for WUR types

Noise figure and SNR are critical parameters influencing the RF hardware design and network performance. To achieve low noise figure the LNA is typically one of the first components in the receiver chain. Designing a LNA with low power consumption and low noise is immensely challenging. To achieve high linearity and high gain the LNA requires more stages translating into more power consumption. To effectively reduce the power consumption the gain and noise performance needs to be relaxed. RAN4 has held initial discussions on noise figure and captured the following in the WF [2]:

Issue 2-2-4: NF Gap between LR and MR for REFSENS (assume MR as 9dB) Agreement: - RAN4 further discuss potential NF for OOK-based and OFDM-based receiver. - Linked to resolution of Issue 2-2-8 Issue 2-2-6: IM value for REFSENS Agreement: - RAN4 further discuss potential IM value for OOK-based and OFDM-based receiver. Issue 2-2-8: Whether a baseline architecture is needed for LP-WUS receiver Agreement: - RAN4 will further discuss and decide baseline architecture(s) for OOK-based receiver and OFMD-based receiver.

While typical NR FR1 receiver feature a noise figure of approximately 9 dB this value is expected to increase for LP-WUR. An OFDM type receiver is expected to re-use existing RF architecture from the main receiver. The battery lifetime of the device is enhanced by tweaking the LNA setup for reduced current consumption. This decreases the noise figure performance. Therefore, a delta to the typical 9dB of the main receiver is required. We conducted measurements using an OFDM type WUR receiver and found that a delta of 7dB would be required to satisfy the expected current consumption improvements. Regarding implementation margin we propose to set the value no lower than 2.5dB.

Proposal 2: To achieve high linearity and high gain the LNA requires more stages translating in more power consumption. To effectively reduce the power consumption gain and noise performance needs to be reduced. While typical NR FR1 receiver feature a noise figure of approximately 9 dB this value is expected to increase considerably for LP-WUR. Measurements indicate that approximately 7dB delta to main receiver is required to satisfy the expected current consumption improvements. Regarding implementation margin we propose to set the value no lower than 2.5dB.

# 4	Conclusions

This contribution provides a discussion based on last meetings way forward on LP-WUR. The following observations and proposals are made:

Proposal 1: Considering the discussion in RAN4#111 it seems that defining a value of 5% for test purpose could be a compromise to achieve balance between actual network performance and test complexity.

Proposal 2: To achieve high linearity and high gain the LNA requires more stages translating in more power consumption. To effectively reduce the power consumption gain and noise performance needs to be reduced. While typical NR FR1 receiver feature a noise figure of approximately 9 dB this value is expected to increase considerably for LP-WUR. Measurements indicate that approximately 7dB delta to main receiver is required to satisfy the expected current consumption improvements. Regarding implementation margin we propose to set the value no lower than 2.5dB.

# 4	References

- RP-240801, “Low-power Wake-up Signal and Receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony, 3GPP TSG RAN Meeting #103

- R4-2406619, “WF on NR LP-WUS UE requirements”, vivo, 3GPP TSG-RAN WG4 Meeting #110bis