3GPP TSG-RAN WG4 Meeting#112							         			R4-2411646

Maastricht, Netherland, 19 – 23 August 2024

Title: 	On PC1.5 intra-band contiguous ULCA with 2Tx

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.1.1.1.1	Intra-band contiguous and non-contiguous UL CA with PC1.5	[NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN4#111, a way forward [2] captured agreements related to the Release 19 power domain enhancement work item that was further detailed in RAN#104 in [3]. Notably, it covered the PC1.5 intra-band ULCA feature for both contiguous and non-contiguous cases. In this contribution, further developing from our contribution [1] in RAN4#111, we address the MPR requirement for PC1.5 contiguous ULCA based on 2Tx architecture.

# Discussion

## 2.1 Way forward and plenary inputs

In [3], the plenary input sets the objectives that are related to PC1.5 intra-band ULCA which are still aligned with way forward [2]:

- Specify the requirements for intra-band UL contiguous CA with or without UL-MIMO

  - Example band combinations: CA_n41C, CA_n78C, CA_n77C, CA_n79C for intra-band uplink contiguous CA configurations

- Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution.

It should be noted that only bandwidth Class C is requested, however it is unclear if bandwidth Class C can be supported without supporting bandwidth Class B.

Proposal: Bandwidth class B should also be evaluated on top of bandwidth class C to have a complete PC1.5 intra-band contiguous framework.

Below we extracted the agreements and way forwards relative to PC1.5 contiguous intra-band ULCA architecture as captured in [2].

Agreement on Architecture:

- Evaluate MPR/A-MPR numbers for contiguous CA first and then work on non-contiguous CA MPR/A-MPR afterwards.

- The intra-band PC1.5 ULCA R19 work focusses on UE implementing with two 26dBm PAs. Architectures requiring a 29dBm PA are not specified but can be implemented by fulfilling the requirements based on two 26dBm PAs.

- For intra-band contiguous ULCA w/ and w/o UL MIMO, the Architecture#2 is prioritized for MPR/A-MPR evaluation.

  - If time is allowed, architecture #1 can be evaluated.

| Architecture | Description | Indicated capability | Whether can support UL MIMO | Applicable cases |
| --- | --- | --- | --- | --- |
| #1 | 2x26 dBm PA + 2 LO with 100MHz BW | dualPA-Architecture | No | Contiguous CA |
| #2 | 2x26 dBm PA + 1 LO with 200MHz BW | TxD | Yes | Contiguous CA |

Observation: It should be noted that some of the captured architecture attributes are incomplete:

- The power limitation per carrier to 26dBm of the dualPA architecture is not captured, which will impact the SNR at the BS of one of the carriers in case of asymmetric active RB allocation between the CCs.

- The conditions for one CC PC1.5 fallback are not captured.

Proposal on architecture description for PC1.5 contiguous intra-band ULCA: The following table is used, and priority is to develop MPR/A-MPR requirement for architecture #2.

| Architecture | Implementation | Indicated capability | UL MIMO support | Power limitation | Support of 1CC PC1.5 fallback |
| --- | --- | --- | --- | --- | --- |
| #1 | 2x26 dBm PA + 2 LO with 100MHz BW | dualPA-Architecture | No | Each carrier is limited to 26dBm | Requires LO switching and thus interruption |
| #2 | 2x26 dBm PA + 1 LO with 200MHz BW | TxD and/or UL MIMO | Yes | One carrier can reach 29dBm | Can support without any switching |

Some further agreements and way forward were captured in [2]:

Way forward on antenna isolation:  Yes, and assuming 20dB minimum antenna isolation for FWA and 10dB for handheld UE

Way forward on additional power reduction:

- A-MPR requirements are not needed for PC1.5 n77/n78/n79 intra-band ULCA

- Evaluate A-MPR requirements for PC1.5 n41 intra-band UL CA

Way forward on PCmax: FFS in future meetings

Proposals:

  - Proposal 1: Define PCMAX,c limitation for each component carrier considering PC1.5 UE architecture of intra-band UL CA. (LGE)

- For dualPA-architecture

  - PCMAX,C limitation  for each component carrier is 26dBm

- For TxD (dualTx)

  - PCMAX,CC1 = 26+10*log10( LCRB1*SCS1LCRB1*SCS1+LCRB2*SCS2)+3

  - PCMAX,CC2 = 26+10*log10( LCRB2*SCS2LCRB1*SCS1+LCRB2*SCS2)+3

  - Proposal 2: Changes to TS 38.101-1 for dual-PA architecture, inset new equation for PCMAX,c (Ericsson, more details refer to R4-2407721)

In this contribution, we cover initial backoff assessment based on 2Tx architecture and the usual equal PSD assumption. The aspects related to PCmax and equal PWR/PSD assumption are covered in [4].

## 2.2 Existing PC3/PC2 MPR and A-MPR for intra-band contiguous ULCA

The existing MPR for contiguous intra-band ULCA PC3 and PC2 cases are summarized in Table 1 for contiguous inner/outer allocations and in Table 2 for contiguous inner/outer1/outer2 allocations. Here we only cover the bandwidth class C (>100MHz, ≤200MHz) cases.

The underlying assumptions for which those MPR values were derived are:

- PC3 MPR is based on 1Tx/1LO 23dBm PA covering 200MHz BW. It can also be implemented with two 23dBm PAs and two LO, each PA covering 100MHz, in this case DualPA is signalled and LO switching is required to fallback to single CC.

- PC2 (with 1Tx) is based on 1Tx/1LO 26dBm PA covering 200MHz BW. It can also be implemented with two 26dBm (at least one with LO switching) PAs and two LO, each PA covering 100MHz, in this case DualPA is signalled and LO switching is required to fallback to single CC.

- PC2 2Tx is based on 2Tx/1LO with two 23dBm PA covering 200MHz BW. It can support TxD and UL MIMO and can support 2Tx PC2 1CC fall back.

- In all cases equal PSD (power per RB BW) was used for back-off evaluation

- The back-off is applied to the total power, each CC see the same back-off.

Table 1: Contiguous RB allocation for bandwidth class C

| Modulation |  | MPR for PC3 (dB) |  | MPR for PC2 (dB) |  | MPR for PC2 with 2Tx (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 7 | 2.5 | 7 | 3.5 | 8 |
|  | QPSK | 2.5 | 7 | 2.5 | 7 | 3.5 | 8 |
|  | 16QAM | 2.5 | 7 | 2.5 | 7 | 3.5 | 8 |
|  | 64QAM | 5 | 7 | 5 | 7 | 6 | 8 |
|  | 256QAM | 7 | 7.5 | 7 | 7.5 | 8 | 8.5 |
| CP-OFDM | QPSK | 3.5 | 8 | 3.5 | 8 | 4.0 | 8.5 |
|  | 16QAM | 3.5 | 8 | 3.5 | 8 | 4.0 | 8.5 |
|  | 64QAM | 5 | 8 | 5 | 8 | 5.5 | 8.5 |
|  | 256QAM | 7 | 8 | 7 | 8 | 7.5 | 8.5 |

Table 2: non-contiguous RB allocation for bandwidth class C

| Modulation |  |  | MPR for PC3 (dB) |  |  |  |  |  | MPR for PC2 (dB) |  |  |  |  |  | MPR for PC2 with 2Tx (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | inner |  | Outer11 |  | Outer22 |  | inner |  | Outer11 |  | Outer22 |  | inner |  | Outer11 |  | Outer22 |  |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 |  | 6 |  | 13 |  | 33 |  | 7.5 |  | 13.5 |  | 43 |  | 8.5 |  | 14.5 |  |
|  | QPSK | 2.5 |  | 6 |  |  |  | 33 |  | 7.5 |  |  |  | 43 |  | 8.5 |  |  |  |
|  | 16QAM | 3 |  | 6 |  |  |  | 33 |  | 7.5 |  |  |  | 43 |  | 8.5 |  |  |  |
|  | 64QAM | 5 |  | 6 |  |  |  | 5 |  | 7.5 |  |  |  | 6 |  | 8.5 |  |  |  |
|  | 256QAM | 6.5 |  | 6.5 |  |  |  | 6.5 |  | 7.5 |  |  |  | 7.5 |  | 8.5 |  |  |  |
| CP-OFDM | QPSK | 3.5 |  | 7 |  | 14 |  | 3.53 |  | 8 |  | 14.5 |  | 4.53 |  | 9 |  | 15.5 |  |
|  | 16QAM | 3.5 |  | 7 |  |  |  | 3.53 |  | 8 |  |  |  | 4.53 |  | 9 |  |  |  |
|  | 64QAM | 5 |  | 7 |  |  |  | 5 |  | 8 |  |  |  | 6 |  | 9 |  |  |  |
|  | 256QAM | 7.5 |  | 7.5 |  |  |  | 7.5 |  | 8 |  |  |  | 8.5 |  | 9 |  |  |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observations:

- The baseline operation and MPR is related to a single LO with PAs covering both CCs; thus, backoff is applied to both CCs simultaneously and equal PSD is assumed.

- There is no existing case mandating two 26dBm PAs, and the 2Tx case is based on two PC3 PAs.

- For the contiguous RB allocations, the inner/outer allocation limitation is not always dominated by SEM. Thus, PC1.5 contiguous allocation MPR does not necessarily become “3dB more difficult”.

- For the non-contiguous allocations, in most cases, the limitation is based on IMD5 SEM limit at -13dBm/MHz (inner) or -30dBm/MHz (outer1) or IMD3 at -13dBm/MHz (outer2). Thus, PC1.5 non-contiguous allocation MPR becomes “3dB more difficult” in most cases.

## 2.3 Preliminary measurement results

As a first step and only using equal PSD sharing between CCs, we have performed a few 2Tx measurements with two coupled PC2 PAs.

A number of contiguous and non-contiguous allocations were measured, but the set of allocation is too small to derive yet an MPR table. Also, the power sharing shall be extended to encompass at least 6dB PSD imbalance. These first results may at least provide some insight on how they compare to PC2 MPR (both 1Tx and 2Tx). Furthermore, the current measurements do not account for the margin need for ET and APT implementations at significant back-off. Also note that 30kHz SCS is used in the measurement and 15kHz SCS may give more stringent cases. Given these aspects, the only way to derive an MPR table properly will be to measure for both 2Tx PC2 and 2Tx PC1.5 and derive a delta back-off as was done when assessing PC2 MPR in Rel-17 from the PC3 Rel-16 starting point. With these considerations, the numbers reported in table 1 shall be considered optimistic and is only provided to give insights on which limitations are in play for different allocations.

Measured waveforms:

- Based on 160MHz aggregated bandwidth with two 80MHz CCs (similar to n41C for China)

- Only DFT-s-OFDM QPSK waveforms

- Allocation size of 1RB, 15RB, 108RB and 216RB at 30kHz SCS

- Allocation position is varied to create:

  - Contiguous inner and outer allocations

  - Non-contiguous inner, outer1 and outer2 allocations

Table 1: Preliminary backoff evaluation for a limited set of allocations

| Allocation |  |  |  |  |  | Preliminary back-off evaluation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RBL (LCRB) | RBSL (RBstart) | RBH (LCRB) | RBSH (RBstart) | Contiguous/ Non-contiguous | type | ACLR | SEM | Limiting Emission | Source of interference |
| 1 | 216 | 1 | 0 | Contiguous | Inner | 0 | 0 | Na |  |
| 1 | 216 | 15 | 0 | Contiguous | Inner | 0 | 0 | Na |  |
| 1 | 216 | 108 | 0 | Contiguous | Inner | 0 | 0 | Na |  |
| 1 | 216 | 216 | 0 | Contiguous | Outer | 1.5 | 0.8 | ACLR |  |
| 15 | 202 | 15 | 0 | Contiguous | Inner | 0 | 0 | Na |  |
| 15 | 202 | 108 | 0 | Contiguous | Inner | 0 | 0 | Na |  |
| 15 | 202 | 216 | 0 | Contiguous | Outer | 1.8 | 0.7 | ACLR |  |
| 108 | 109 | 108 | 0 | Contiguous | Inner | 0.3 | 0.1 | ACLR |  |
| 108 | 109 | 216 | 0 | Contiguous | Outer | 2.8 | 1 | ACLR |  |
| 216 | 1 | 216 | 0 | Contiguous | Outer | 3.3 | 0.9 | ACLR |  |
| 1 | 0 | 1 | 216 | Non-contiguous | Outer2 | 2.8 | 5.4 | s4 | IMD5 |
| 1 | 140 | 1 | 77 | Non-contiguous | Outer1 | 2.9 | 4.6 | S2 | IMD3 |
| 1 | 147 | 1 | 70 | Non-contiguous | Inner | 0 | 0.2 | S2 | IMD5 |
| 15 | 0 | 1 | 216 | Non-contiguous | Outer2 | 0.8 | 2 | S0 | IMD3 |
| 15 | 0 | 15 | 203 | Non-contiguous | Outer2 | 2.9 | 3.3 | s4 | IMD5 |
| 15 | 140 | 1 | 77 | Non-contiguous | Outer1 | 0 | 0.3 | s1 | IMD3 |
| 15 | 147 | 1 | 70 | Non-contiguous | Inner | 0 | 0 | Na |  |
| 108 | 0 | 1 | 216 | Non-contiguous | Outer2 | 1.2 | 0.9 | s2 |  |
| 108 | 0 | 108 | 109 | Non-contiguous | Outer2 | 3.5 | 1.2 | s2 | IMD3 |
| 108 | 109 | 1 | 108 | Non-contiguous | Outer1 | 0 | 0 | Na |  |
| 216 | 0 | 1 | 216 | Non-contiguous | Outer2 | 1.4 | 0.3 | S2 | IMD3 |
| 216 | 0 | 216 | 1 | Non-contiguous | Outer2 | 3.2 | 0.8 | s2 |  |

## 2.4 Evaluation assumption for PC1.5 intra-band ULCA MPR with 2Tx

To evaluate the PC1.5 intra-band contiguous ULCA MPR for 2Tx, we suggest simulating or measuring the back-off difference versus 2Tx PC1.5 based on two coupled 26dBm PAs and 2Tx PC2 based on two coupled 23dBm PAs.

Proposal on MPR evaluation parameters:

- Post PA loss: 4dB, Antenna isolation: 10dB for smartphone and 20dB for FWA

- PA calibration for DFT-s-OFDM QPSK 20MHz 100RB0 allocation:

  - 30dB ACLR at 26dBm for the “23dBm” PA used in 2Tx PC2

  - 31dB ACLR at 29dBm for the “26dBm” PA used in 2Tx PC1.5

- Measuring back-off needed for SEM and ACLR (for QPSK), EVM for higher order modulations

- Equal PSD power sharing as a starting point but companies are encouraged to also evaluate with 6dB PSD imbalance.

- Both DFT-s-OFDM and CP-OFDM waveforms starting with QPSK

- When the same allocation is used in each CC the waveforms should be uncorrelated (different data)

- Waveforms used in each Tx Path should see TxD processing to have some level of correlation (worst case)

- Emissions are calculated based on the mathematical sum of each path under the assumed coupling.

- Both actual back-off and back-off difference between PC2 and PC1.5 may be reported

# Conclusion

In this contribution, we examined the intra-band contiguous ULCA architectures for PC1.5 and made an initial assessment at back-off needed to meet emissions. This allowed to formulate the following proposals.

Proposal: Bandwidth class B should also be evaluated on top of bandwidth class C to have a complete PC1.5 intra-band contiguous framework.

Proposal on architecture description for PC1.5 contiguous intra-band ULCA: The following table is used, and priority is to develop MPR/A-MPR requirement for architecture #2.

| Architecture | Implementation | Indicated capability | UL MIMO support | Power limitation | Support of 1CC PC1.5 fallback |
| --- | --- | --- | --- | --- | --- |
| #1 | 2x26 dBm PA + 2 LO with 100MHz BW | dualPA-Architecture | No | Each carrier is limited to 26dBm | Requires LO switching and thus interruption |
| #2 | 2x26 dBm PA + 1 LO with 200MHz BW | TxD and/or UL MIMO | Yes | One carrier can reach 29dBm | Can support without any switching |

Proposal on MPR evaluation parameters:

- Post PA loss: 4dB, Antenna isolation: 10dB for smartphone and 20dB for FWA

- PA calibration for DFT-s-OFDM QPSK 20MHz 100RB0 allocation:

  - 30dB ACLR at 26dBm for the “23dBm” PA used in 2Tx PC2

  - 31dB ACLR at 29dBm for the “26dBm” PA used in 2Tx PC1.5

- Measuring back-off needed for SEM and ACLR (for QPSK), EVM for higher order modulations

- Equal PSD power sharing as a starting point but companies are encouraged to also evaluate with 6dB PSD imbalance.

- Both DFT-s-OFDM and CP-OFDM waveforms starting with QPSK

- When the same allocation is used in each CC the waveforms should be uncorrelated (different data)

- Waveforms used in each Tx Path should see TxD processing to have some level of correlation (worst case)

- Emissions are calculated based on the mathematical sum of each path under the assumed coupling.

- Both actual back-off and back-off difference between PC2 and PC1.5 may be reported.

# References

[1] R4-2407371 PC1p5 intra-band ULCA MOP for 2Tx and Dual-PA cases, Skyworks Solutions Inc., RAN4#111

[2] R4-2410750 WF on high power UE, Samsung, RAN4#111

[3] RP‑241656 UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei, HiSilicon, RAN#104

[4] R4‑2411302 On equal PSD vs equal power spectral regrowth, Skyworks Solutions Inc., RAN4#112