3GPP TSG RAN WG4 #112 meeting 	 	R4-2411647

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.1.1.3.1 & 8.1.1.3.2 & 8.1.1.3.3 & 8.1.1.3.4

Source:	Meta Ireland

Title:	Discussion on 6Rx REFSENS and other RF requirements for single carrier in Rel-19

Document for:	Approval

- Introduction

In the last RAN #104 meeting, The RF enhancement at FR1 WI was updated to reflect RAN agreements for the 6Rx related antenna switching issues at RAN1 [1]. In 6Rx WF [3] at RAN4 #111 meeting, RAN4 discussed the REFSENS requirements, SRS antenna switching IL, MIMO layer impact and SRS IL imbalance.

In this contribution, we share our view on the 6Rx core requirements for both HHUE and FWA devices. Based on both RAN and RAN4 agreements, we share our view to specify the detail UE RF requirements to support 6Rx UE at FR1 in Rel-19.

1. RAN agreements for 6Rx for HHUE and FWA [2] - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. Note 1: SRS antenna switching to support 4T6R is precluded. Note 2: RAN1 discussion starts from RAN1#118bis (Oct. 2024) 2. Revised WI Objectives [1] 6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution. 3. WF on RAN4 #111 Meeting [3] Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA Way Forward: Further discuss the following options. - Option 1: RAN4 needs to determine whether to define different ΔRIB,6R value for handheld UE and FWA separately - Option 2: Same value for handheld UE and FWA - Option 3: Different value for handheld UE and FWA Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA Way Forward: Further discuss the following options for the ΔRIB,6R values. Ultimately, the decision to specify separate or same requirements for handheld UE and FWA will be dependent on outcome of Issue 1-1-2. - Option 1: Consider range of ΔRIB,6R in brackets for each case below for further discussion. Operating bands; ΔRIB,6R for FWA; ΔRIB,6R for Handheld. n77, n78, n79, [n104]; [-3.0 ~ -3.3] dB; [-3.0 ~ -3.2] dB. n41; [-3.0 ~ -4.0] dB; [-3.0 ~ -4.0] dB. - Option 2: Consider average value of ΔRIB,6R from company proposals for further discussion. - Option 3: Other options are not precluded. Issue 2-1-1: SRS antenna switching configurations Way forward: RAN4 to focus on discussing ∆TRxSRS for 1tr6 and t2r6 until RAN1 concludes work on 3t6r and 4t6r. Pending RAN1 feedback, RAN4 can consider any necessary updates to the work scope in a Revised WID for the SRS antenna configurations subject to RAN Plenary approval. Issue 2-1-2: Whether to use different ∆TRxSRS based on operating frequency Way forward: RAN4 defines different ∆TRxSRS values based on operating frequency. Further discuss whether to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104. Issue 2-2-1: Proposed ∆TRxSRS values Way forward: Further discuss the following options for the ∆TRxSRS requirements. - Option 1: Consider average value of ∆TRxSRS for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; 4.0 dB; 3.4 dB; 4.4 dB. Band n79, [n104]; 5.3 dB; 4.5 dB; 5.7 dB. - Option 2: Consider range of ∆TRxSRS in brackets for each case below for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; [2.8 ~ 5.0] dB; [2.3 ~ 4.0] dB; [3.3 ~ 5.5] dB. Band n79, [n104]; [3.6 ~ 6.0] dB; [3.0 ~ 5.5] dB; [4.2 ~ 6.5] dB. Note: Maximum level for t1r6 corrected for OPPO proposals per OPPO. - Option 3: Other options are not precluded. Issue 3-1-1: Tightening BS EVM requirement Way Forward: RAN4 to further discuss if the BS EVM requirement needs to be tightened when discussing the feasibility of 6-Layer MIMO for 6Rx UEs. Issue 3-1-2: 6-Layer Performance Evaluation Assumptions Way Forward: RAN4 to further discuss the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios. Issue 3-2-1: 6-layer Support Agreement: - Decision on support of 6 MIMO layers for handheld is deferred until performance level differences in comparison with 4 MIMO layers are known based on evaluations considering realistic antenna correlation assumptions and deployment scenarios Issue 3-2-2: 6-layer Support as optional feature Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature. Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behaviour for SRS transmissions in case of SRS IL. - Companies are encouraged to bring analysis on the existing UE behaviour and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL. - Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation.

- REFSENS for 6Rx in single carrier

  - Reference sensitivity tighten level with ΔRIB for TDD example bands

In this session, we share how to define ΔRIB,6R to allow the tightening REFSENS requirements for 6Rx UE including smart phone type UE and FWA UE from the 2Rx REFSENS requirements in Table 7.3.2-1a and Table 7.3.2-1b in TS38.101-1.

In last RAN4 meeting, RAN4 agreed to collect the proposed ΔRIB,6R as ranges for both FWA and HH UE as follow for 6Rx supporting UEs.

Table 1. Merged ΔRIB,6R for FWA and HHUE according to different operating bands [3]

| Operating bands | ΔRIB,6R for FWA | ΔRIB,6R for Handheld |
| --- | --- | --- |
| n77, n78, n79, [n104] | [-3.0 ~ -3.3] dB | [-3.0 ~ -3.2] dB |
| n41 | [-3.0 ~ -4.0] dB | [-3.0 ~ -4.0] dB |

As formfactor perspective, the ΔRIB,6R might be defined as different levels, but the proposed levels are quite aligned with small differences. Based on the proposed the ΔRIB,6R, RAN4 can specify the same ΔRIB,6R requirements with the average values of HHUE as worst case according to the different operating NR bands.

Proposal 1: Based on the 2Rx REFSENS requirements, the 6Rx REFSENS levels for HHUE and FWA UE would be specified with the same ΔRIB,6R as follow:

| Operating bands | ΔRIB,6R for both FWA and Handheld |
| --- | --- |
| n77, n78, n79 and n104 | -3.1 dB |
| n41 | - 3.5 dB |

Also, RAN4 already specified the detail 2Rx and 4Rx REFSENS requirements according to NR operating bands including n104 as follow:

REFSENS requirements in TS38.101-1

Table 7.3.2-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| n101 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
| --- | --- | --- | --- | --- |
|  | 30 | 10 | -97.1 + 10log10(NRB/24) |  |
| n1041,10 | 15 | 20, 30, 40, 50 | -90.7 + 10log10(NRB/106) | TDD |
|  | 30 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | -90.8 + 10log10(NRB/51) |  |
|  | 60 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | -91.1 + 10log10(NRB/24) |  |
| n10911 | 15 | 5,10,15,20,25,30,40,50 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20,25,30,40,50 | -97.1 + 10log10(NRB/24) |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE and two Rx antenna port XR UEs indicating UE capability [2Rx XR]. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 10: A UE may implement two RX antenna ports for band n104 when conditions are met. The exact conditions are FFS. |  |  |  |  |

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n13, n26, n28, n71, n85, n105 | -2.71 |
| n5, n8, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE. |  |

Hence, RAN4 can define the n104 REFSENS requirements for 6Rx operation based on the example NR bands in WID objectives.

Proposal 2: For n104 of 6Rx supporting, RAN4 can apply the same ΔRIB,6R  as above proposal 1 since RAN4 already defined 2Rx and 4Rx REFSENS requirements in TS38.101-1 even though the Note 10 has been included in REFSENS Table 7.3.2-1b for TDD bands.

  - ΔTRxSRS for TDD example bands

In session 2.2, we are shown the current specification of ΔTRxSRS for 8Rx UE according to the SRS antenna configuration.

Captured in TS38.101-1 [5] The following ∆TRxSRS applies according to the indicated SRS-TxSwitch or srs-AntennaSwitchingBeyond4RX-r17 capabilities: if 't1r8' and 't4r8' are indicated: - The value of ∆TRxSRS is 7.3 dB for bands whose FUL_high is higher than the FUL_low of n79 and 5.8 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16. - The value of ∆TRxSRS is 10.3 dB for bands whose FUL_high is higher than the FUL_low of n79 and 8.8 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is capable of power class 2 in the band and ΔPPowerClass = 0 dB and not indicating txDiversity-r16. else, if 't1r8' and 't2r8' are indicated: - The value of ∆TRxSRS is 6.0 dB for bands whose FUL_high is higher than the FUL_low of n79 and 4.5 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16. - The value of ∆TRxSRS is 9.0 dB for bands whose FUL_high is higher than the FUL_low of n79 and 7.5 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is capable of power class 2 in the band and ΔPPowerClass = 0 dB and not indicating txDiversity-r16. else, if 't1r8' or 't2r8', but not both is indicated: - The value of ∆TRxSRS is 5.5 dB for bands whose FUL_high is higher than the FUL_low of n79 and 4.0 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16. - The value of ∆TRxSRS is 8.5 dB for bands whose FUL_high is higher than the FUL_low of n79 and 7.0 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is capable of power class 2 in the band and ΔPPowerClass = 0 dB and not indicating txDiversity-r16. else, if 't1r2', 't1r1-t1r2', 't1r4', 't1r4-t2r4', 't1r1-t1r2-t1r4', 't2r4', 't1r1-t1r2-t2r2-t2r4', 't1r1-t1r2-t2r2-t1r4-t2r4' or 't4r8' is indicated: - The value of ∆TRxSRS is 4.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16.. - The value of ∆TRxSRS is 7.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 6 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is capable of power class 2 in the band and ΔPPowerClass = 0 dB and not indicating txDiversity-r16. For other SRS transmissions ∆TRxSRS is zero;

In TS38.101-1 specification, RAN4 can define the detail ΔTRxSRS of 6Rx for the candidate SRS antenna configurations based on actual RF architecture to support 6Rx and RAN Plenary already made a consensus to support the antenna switching of t3r6 and removed t4r6 in the revised WID [1] in Rel-19 as follow:

Updated Objectives for 6Rx in revised WID [1]

6Rx for handheld and FWA UE [RAN4, RAN1]

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability

    - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability.

      - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

Hence, RAN4 need to discuss and define SRS antenna switching ∆TRxSRS for 1tr6, t2r6 and t3r6 except t4r6 in Rel-19.

    - ΔTRxSRS antenna switching IL

To derive the TRxSRS IL, we assumed the Switch component parameters in Table 2 as follow

Table 2 Switch component IL for n77/n78 and n79

|  | n41/n77/n78 (<3.5GHz) | n79/n104 (>4.4GHz) |
| --- | --- | --- |
| SP2T IL | 0.7 | 0.9 |
| SP3T IL | 0.8 | 1.0 |
| SP4T IL | 1.0 | 1.2 |
| DPDT IL | 0.8 | 1.1 |
| PCB trace IL (SRS path) | 1.8 | 2.4 |
| Implementation Margin to support multiple operating bands | 0.5 | 0.8 |
| Note1: PCB trace loss used assumes the trace length is 60mm. |  |  |

- t1r6 antenna switching IL

Based on the conventional RF architecture of t14r, 2Rx path can be added as follows, then the expected SRS IL can be analyzed according to supporting NR operating bands

- SRS antenna switching loss for <3.5GHz frequency = 2*SP2T +PCB trace loss + IM = 2*0.7 +1.8 +0.5 = 3.7 dB

- SRS antenna switching loss for > 4.4GHz frequency = 2*SP2T +PCB trace loss + IM = 2*0.9 +2.4 +0.8 =5.0 dB

1-1) t1r6 antenna conf.               1-2) t2r6 antenna conf.               1-3) t1r6-t2r6 antenna conf.

Figure 1. Example RF architecture for t1r, t2r6 and t1r6-t2r6 based on t1r4

- t2r6 antenna switching IL

- SRS antenna switching loss for <3.5GHz frequency = SP2T +PCB trace loss + IM = 0.7 +1.8 +0.5 = 3.0 dB

- SRS antenna switching loss for >4.4GHz frequency = SP2T +PCB trace loss + IM = 0.9 +2.4+0.8 =4.1 dB

- t1r6-t2r6 antenna switching IL

- SRS antenna switching loss for <3.5GHz frequency = 3*SP2T +PCB trace loss + IM = 3*0.7 +1.8+0.5 = 4.4 dB

- SRS antenna switching loss for >4.4GHz frequency = 3*SP2T +PCB trace loss + IM = 3*0.9 +2.4 +0.8 =5.9 dB

2-1) t3r6 antenna conf.                                  2-2) t1r6-t3r6 antenna conf.

Figure 2. Example RF architecture for t3r6 and t1r6-t3r6 based on t1r2

- t3r6 antenna switching IL

- SRS antenna switching loss for <3.5GHz frequency = SP2T +PCB trace loss + IM = 0.7 +1.8 +0.5 = 3.0 dB

- SRS antenna switching loss for >4.4GHz frequency = SP2T +PCB trace loss + IM = 0.9 +2.4+0.8 =4.1 dB

- t1r6-t3r6 antenna switching IL

- SRS antenna switching loss for <3.5GHz frequency = DPDT+2*SP2T+PCB trace loss + IM = 0.8+2*0.7 +1.8+0.5 = 4.5 dB

- SRS antenna switching loss for >4.4GHz frequency = DPDT+2*SP2T +PCB trace loss + IM = 1.1+2*0.9 +2.4 +0.8 =6.1 dB

Table 3 Proposed additional SRS Switch IL for n77/n78 and n79

|  | n77/n78 (< 3.5GHz) | n79 (4.9GHz) |
| --- | --- | --- |
| t1r6 | 3.7 | 5.0 |
| t2r6 | 3.0 | 4.1 |
| t1r6 + t2r6 | 4.4 | 5.9 |
| t3r6 | 3.0 | 4.1 |
| t1r6 + t3r6 | 4.5 | 6.1 |

Proposal 3: RAN4 can define the detailed ΔTRxSRS antenna switching IL values in Table 3. These values will be included in the merged ΔTRxSRS antenna switching IL values in WF [3].

Proposal 4: RAN4 can define the average values of ΔTRxSRS antenna switching IL values from the inputs by interested companies.

  - MIMO layers for 6Rx

In WID scope, RAN4 needs to study the feasibility of 6 MIMO layers and explicit commented to support at least 4 MIMO layers in Rel-19.

The 6 MIMO layer performance is quite dependent on the UE antenna correlation, antenna gain/isolation and UE design implementation perspectives in small form factor. RAN4 need to evaluate the 6Rx benefit compared to the additional RF paths and power consumption.

However, RAN4 did not have any evaluations for the pros & cons the 6Rx MIMO layer supporting as RF and demodulation perspectives.

When 6 MIMO layer have better technical performance gains compared to 4MIMO layers and the impact on the additional RF load and power consumption has been verified, RAN4 can support 6 MIMO layers and 6Rx diversity gain in both Handheld device and FWA UE.

Based on above observation, we propose as follow:

Proposal 5: RAN4 can support 6 MIMO layer in this WID based on the performance evaluation campaign for HHUE and FWA depending on the medium/high correlation of ULA and cross polarized MIMO matrices (i.e., 1x6, 2x6 and 6x6 cases compared to 1x4, 2x4 and 4x4 cases) in Dense urban micro cellular deployment scenarios. It means that RAN4 need to validate the benefits of 6 MIMO layer compared to 4 MIMO layer according to real antenna correlation according to the device type and existing Tx EVM level at gNB.

Furthermore, when the benefits are verified the demodulation performance by simulation based on the real antenna correlation, the existing or expected BS EVM level and deployment scenarios, RAN4 also need to analyse the additional RF complexity and power consumptions perspectives for 6 MIMI layer supporting.

Proposal 6: In RF perspective, RAN4 need to analyse the additional RF complexity and power consumptions to support 6 MIMO layers in both HHUE and FWA device.

  - SRS insertion loss imbalance for 6Rx

Based on the current RAN1 and RAN4 specification and Rel-18 discussion, we do not prefer to define any UE behaviour and capability signalling to compensate SRS power imbalance problem.

Captured in WF R4-2406585 [4] Consider some clarifications according to the RAN1 and RAN4 specs. - RAN1 specification on power control doesn’t require or imply that UE shall apply the same PCMAX for all AS-SRS transmission occasions - Under the minimum requirement ∆TRxSRS defined in RAN4 specification, it is up to UE implementation whether to apply exact value for corresponding AS-SRS transmission occasions.

Based on the above analysis, we propose as follow

Proposal 7: If the system performance is not affected by the SRS IL offset, RAN4 does not need to report SRS IL offset to compensate SRS power imbalance.

- Conclusions

In this contribution, we propose REFSNES requirements for 6Rx, share how to define the SRS IL according to the possible SRS configuration and show our preference for the number of MIMO layer as follows:

Proposal 1: Based on the 2Rx REFSENS requirements, the 6Rx REFSENS levels for HHUE and FWA UE would be specified with the same ΔRIB,6R as follow:

| Operating bands | ΔRIB,6R for both FWA and Handheld |
| --- | --- |
| n77, n78, n79 and n104 | -3.1 dB |
| n41 | - 3.5 dB |

Proposal 2: For n104 of 6Rx supporting, RAN4 can apply the same ΔRIB,6R  as above proposal 1 since RAN4 already defined 2Rx and 4Rx REFSENS requirements in TS38.101-1 even though the Note 10 has been included in REFSENS Table 7.3.2-1b for TDD bands.

Proposal 3: RAN4 can define the detailed ΔTRxSRS antenna switching IL values in Table 3. These values will be included in the merged ΔTRxSRS antenna switching IL values in WF [3].

Proposal 4: RAN4 can define the average values of ΔTRxSRS antenna switching IL values from the inputs by interested companies.

Proposal 5: RAN4 can support 6 MIMO layer in this WID based on the performance evaluation campaign for HHUE and FWA depending on the medium/high correlation of ULA and cross polarized MIMO matrices (i.e., 1x6, 2x6 and 6x6 cases compared to 1x4, 2x4 and 4x4 cases) in Dense urban micro (UMi) cellular deployment scenarios. It means that RAN4 need to validate the benefits of 6 MIMO layer compared to 4 MIMO layer according to real antenna correlation according to the device type and existing Tx EVM level at gNB.

Proposal 6: In RF perspective, RAN4 need to analyse the additional RF complexity and power consumptions to support 6 MIMO layers in both HHUE and FWA device.

Proposal 7: If the system performance is not affected by the SRS IL offset, RAN4 does not need to report SRS IL offset to compensate SRS power imbalance.

Reference

[1] RP-241656, “Revised WID on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4,” Huawei

[2] RP-241621, “Moderator's summary for offline discussion on SRS antenna switching,” RAN1 vice-chair (CMCC)

[3] R4-2410751, “WF on requirements for 6Rx in RAN4 #111 Meeting,” AT&T

[4] R4-2406585, “WF on requirements for 6Rx in RAN4 #110bis Meeting,” AT&T

[5] TS38.101-1