3GPP TSG RAN WG4 #112 meeting 	 	R4-2411649

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.1.1.1.2

Source:	Meta Ireland

Title:	High power inter-band CA/DC UE RF requirements including 2Tx/3Tx within 2 bands

Document for:	Approval

- Introduction

In the last RAN4 #111 meeting, RAN4 agreed the WF [1] on HP inter-band CA/DC including 2Tx/3Tx UE RF enhancements in Rel-19. In the WF, RAN4 has agreed the Rel-18 3Tx related RF requirements will be applied to 3Tx HH UE in Rel-19. Also, RAN4 has decided the baseline of SAR solution for inter-band CA/DC band combinations and the principle of the related RF requirements application between HH UE and FWA at last RAN4 May meeting.

The detail remaining discussion issues are list up based on WF [1] as follow:

3. 2-band Inter-band UL NR-CA/EN-DC with 2Tx and/or 3Tx [1] 3.2 Whether to collect configurations to derive general requirements? Way forward: - General requirements can be specified including considerations of different duplex mode configurations: - For PC2, FDD+FDD, FDD+TDD, TDD+TDD with 2Tx and 3Tx - For PC1.5, [FDD+FDD,] FDD+TDD, TDD+TDD with 2Tx and 3Tx 3.3 3UL CC with 3Tx Way forward: RAN4 to further check whether there is demand for 3 ULCC with 3Tx scenario (such as CA_nXA-nY(2A) and CA_ nXA-nYB), and whether it is desirable to be introduced in Rel-19. 3.4 Release independence Way forward: Discuss the release independence for 3Tx band combination for handheld UE and FWA at later stage of this WI - For FWA, only discuss the configurations that were not introduced in Rel-18

In this paper, we propose how to define the RF requirements for high power CA/DC UE for inter-band including 2Tx/3Tx.

- Consideration for High power inter-band CA/DC UE with 2Tx/3Tx

For PC1.5 inter-band CA/DC with 2Tx/3Tx work scope in the Rel-19 WI, RAN4 will focus on the SAR solution for CA and EN-DC with 2Tx/3Tx and investigate 3UL CC with 3Tx such as CA_nXA-nY(2A) and CA_nXA-nYB.

  - Power combination of PC1.5/PC2 inter-band CA/DC operation with 2Tx and 3Tx

In last RAN plenary, the detail objectives were finalized to focus on SAR solution for 2Tx/3Tx inter-band CA/DC operation.

Also, RAN4 need to investigate whether define some new Power classes to support different PC combination between two inter-band CA/DC UE or not.

In the WI in Rel-19, RAN4 can reuse the existing power class to support new CA/DC band combinations as below in Rel-19.

- Proposed inter-band CA/DC band combinations with 2Tx/3Tx

    - 2Tx CA/DC combinations

      - 1Tx PC2 in Band X (i.e., n77, n78, n79 without UL-MIMO) + 1Tx PC2 in Band Y

        - The power class combination already specified in TS38.101-1 and TS38.101-3 in Rel-18

      - 1Tx PC2 in Band X (i.e., n77, n78, n79 without UL-MIMO) + 1Tx PC3 in Band Y

PC2 CA/DC combinations will be supported in Rel-19

      - PC1.5 FDD+FDD band combination: For 1Tx PC2 FDD Band X + 1Tx PC2 FDD Band Y cannot be considered in Rel-19

PA vendor still investigating and innovating of PC2 Power amplifier Module for FDD bands.

    - 3Tx CA/DC combinations

      - 2Tx PC2 in Band X (i.e., n77, n78, n79 with UL-MIMO) + 1Tx PC2 in Band Y

        - The power class combination already specified in TS38.101-1 and TS38.101-3.

      - 2Tx PC2 in Band X (i.e., n77, n78, n79 with UL-MIMO) + 1Tx PC3 in Band Y

PC2 CA/DC combinations will be supported in Rel-19

      - 2Tx PC1.5 in Band X (i.e., n77, n78, n79 with UL-MIMO) + 1Tx PC2 in Band Y

PC1.5 CA/DC combinations will be supported in Rel-19

      - 2Tx PC1.5 in Band X (i.e., n77, n78, n79 with UL-MIMO) + 1Tx PC3 in Band Y

PC1.5 CA/DC combinations will be supported in Rel-19

      - PC1.5 FDD+FDD band combination: For 2Tx PC2 FDD Band X + 1Tx PC2 FDD Band Y cannot be considered in Rel-19

PA vendor still investigating and innovating of PC2 Power amplifier Module for FDD bands.

Proposal 1: RAN4 can define the above high power inter-band CA/DC band combinations with 2Tx/3Tx in the related Basket WIs in Rel-19. In the Rel-19 UE RF enhancement WI except PC1.5 FDD+FDD band combinations, RAN4 can focus on general RF requirements for high power inter-band CA/DC UE.

For the investigation of whether to define some new Power classes for CA/DC UE to support different PC combination between two inter-band CA/DC UE.  In the WI in Rel-19, RAN4 can reuse the existing power classes as PC1.5/PC2 CA/DC band combinations to support PC1.5 + PC2/PC3 or PC2 + PC3 in Rel-19. However, RAN4 need to open the variable approaches how to support the increase UE transmit power as sum of each power class per band.

Proposal 2: To support the accumulative power increases of inter-band CA/DC band combinations UE, RAN4 can investigate how to apply the actual increased UE transmit power based on the above CA/DC power combinations with the existing power classes in Rel-19.

  - 3 uplink Components carriers with 3Tx

For the high-power inter-band CA with 3UL CC, RAN4 need to follow the updated Rel-19 FR1/FR2 UE RF enhancement phase 4 WID [2]. In the WID, we do not have any specific wording for the number of CCs just mentioned two bands CA/DC combinations with 2Tx/3Tx.

Specially, RAN4 discussed on the following example band combinations at last RAN4 meeting [1].

      - Example Band combinations of CA_nXA-nY(2A) without UL-MIMO in all TDD bands

- PC1.5 CA_nXA-nY(2A)

  - PC2 in Band X + PC2 intra-band NC CA in Band Y  Can be supported with 2Tx/3Tx

  - PC2/PC3 in Band X + PC1.5 intra-band NC CA in Band Y  Can be supported with 3Tx

  - PC1.5 in Band X + PC2/PC3 intra-band NC CA in Band Y  Band X with 2PAs with 1Tx without MIMO + Band Y with 2Tx  Can be supported with 3Tx

- PC2 CA_nXA-nY(2A)

  - PC3 in Band X + PC3 intra-band NC CA in Band Y  Can be supported with 2Tx/3Tx

  - PC3 in Band X + PC2 intra-band NC CA in Band Y  Can be supported with 3Tx

  - PC2 in Band X + PC3 intra-band NC CA in Band Y  Band X with 1Tx + Band Y with 2Tx  Can be supported with 3Tx

      - Example Band combinations of CA_nXA-nYB without UL-MIMO in all TDD bands

- PC1.5 CA_nXA-nYB

  - PC2 in Band X + PC2 intra-band Contiguous CA with class B in Band Y  Can be supported with 2Tx/3Tx

  - PC2/PC3 in Band X + PC1.5 intra-band Contiguous CA with class B in Band Y  Can be supported with 3Tx

  - PC1.5 in Band X + PC2/PC3 intra-band Contiguous CA with class B in Band Y  Band X with 1Tx/2Tx + Band Y with 1Tx  Can be supported with 3Tx

- PC2 CA_nXA-nYB

  - PC2 in Band X + PC2 intra-band Contiguous CA with class B in Band Y  Can be supported with 2Tx/3Tx

  - PC3 in Band X + PC2 intra-band Contiguous CA with class B in Band Y  Can be supported with 2Tx/3Tx

  - PC2 in Band X + PC3 intra-band Contiguous CA with class B in Band Y  Can  be supported with 2Tx/3Tx

Based on the above UE architecture analysis of the example high power CA UE band combinations according to supporting power class, RAN4 can support the two example CA band combinations without uplink MIMO.

Proposal 3: Based on the above UE architecture analysis, RAN4 can support the two example CA band combinations without uplink MIMO in Rel-19.

  - General RF requirements for PC1.5/PC2 inter-band CA/DC UE with 2Tx and 3Tx

For the high-power CA/DC UE with 2Tx/3Tx, RAN4 can reuse the suffix A (2Tx/3Tx CA without MIMO), suffix H (3Tx w/ UL-MIMO) & suffix L (3Tx w/Tx Diversity) in TS38.101-1.

The following General RF requirements will be updated to support the high-power inter-band CA UE in TS38.101-1. The similar specification work for high power inter-band EN-DC band combinations are expected in TS38.101-3.

    - 5.5A.3.1	Configurations for inter-band CA (two bands)

This clause will be updated to support the specific UL/DL configuration for high power inter-band CA band combinations.

    - 6.2A.4.1.3   Configured transmitted power for Inter-band CA

The existing configured Tx power for single band in clause 6.2.4 will be applied per band. For the MPR, A-MPR and P-MPR can be applied per band.

    - 6.2H.3	Transmitter power for inter-band UL CA with UL MIMO

Individual two bands uplink CA band combinations with UL-MIMO in single band will be updated to support the PC1.5/PC2 UE power classes in Table 6.2H.3.1-1. To comply of the SAR regulation, RAN4 will add the SAR solutions with both P-MPR and duty cycle limitation or either way in the clause.

    - 6.2L.3	Transmitter power for inter-band UL CA with Tx Diversity

Individual two bands uplink CA band combinations with Tx diversity in single band will be updated to support the PC1.5/PC2 UE power classes in Table 6.2L.3.1-1. To comply of the SAR regulation, RAN4 will add the SAR solutions with both P-MPR and duty cycle limitation or either way in the clause.

    - 6.3H.3	Output power dynamics for inter-band UL CA with UL MIMO

The same requirements for 3Tx CA UE will be applied for PC1.5/PC2 inter-band CA with UL-MIMO in Rel-19, such as minimum output power, Transmit OFF power, Transmit ON/OFF time mask and power control mechanism per band.

    - 6.3L.3	Output power dynamics for inter-band UL CA with Tx Diversity

The same requirements for 3Tx CA UE will be applied for PC1.5/PC2 inter-band CA with TxD in Rel-19, such as minimum output power, Transmit OFF power, Transmit ON/OFF time mask and power control mechanism per band.

    - 6.4H.3	Transmit signal quality for inter-band UL CA with UL MIMO

The same requirements for 3Tx CA UE will be applied for PC1.5/PC2 inter-band CA with UL-MIMO in Rel-19, such as frequency error, transmit modulation quality (EVM, carrier, leakage, in-band emission, spectrum flatness and phase continuity) per band.

    - 6.4L.3	Transmit signal quality for inter-band UL CA with Tx Diversity

The same requirements for 3Tx CA UE will be applied for PC1.5/PC2 inter-band CA with TxD in Rel-19, such as frequency error, transmit modulation quality (EVM, carrier, leakage, in-band emission, spectrum flatness and phase continuity) per band.

    - 6.5H.3	Output RF spectrum emissions for inter-band UL CA with UL MIMO

The same requirements for 3Tx CA UE will be applied for PC1.5/PC2 inter-band CA with UL-MIMO in Rel-19, such as occupied bandwidth, general SEM, ACLR, general SE and transmit intermodulation per band.

    - 6.5L.3	Output RF spectrum emissions for inter-band UL CA with Tx Diversity

The same requirements for 3Tx CA UE will be applied for PC1.5/PC2 inter-band CA with TxD in Rel-19, such as occupied bandwidth, general SEM, ACLR, general SE and transmit intermodulation per band.

    - 7.3A.2.3	Reference sensitivity power level for Inter-band CA

The band specific MSD study will be treated in future after defining the general RF requirements for PC1.5/PC2 inter-band CA/DC with 2Tx/3Tx.

Proposal 4: RAN4 can consider the above general RF requirements in TS38.101-1 for PC 1.5/PC2 inter-band CA including 2Tx/3Tx UE. Similar approach will be applied to inter-band EN-DC including 2Tx/3Tx UE in TS38.101-3.

      - suffix A (2Tx/3Tx CA without MIMO)

      - suffix H (3Tx w/ UL-MIMO)

      - suffix L (3Tx w/Tx Diversity)

- Conclusions

In this contribution, we propose the power combinations of PC1.5/PC2 inter-band CA/DC with 2Tx/3Tx and propose SAR solution with both P-MPR and duty-cycle limitation approach for inter-band CA/DC with 2Tx/3Tx UE. Also, we share our general RF requirements impact in TS38.101-1 to support the PC1.5/PC2 inter-band CA/DC with 2Tx/3Tx UE. Based on our analysis for the high-power inter-band CA/DC combination with 2Tx/3Tx, we proposed as follow

Proposal 1: RAN4 can define the above high power inter-band CA/DC band combinations with 2Tx/3Tx in the related Basket WIs in Rel-19. In the Rel-19 UE RF enhancement WI except PC1.5 FDD+FDD band combinations, RAN4 can focus on general RF requirements for high power inter-band CA/DC UE.

Proposal 2: To support the accumulative power increases of inter-band CA/DC band combinations UE, RAN4 can investigate how to apply the actual increased UE transmit power based on the above CA/DC power combinations with the existing power classes in Rel-19.

Proposal 3: Based on the above UE architecture analysis, RAN4 can support the two example CA band combinations without uplink MIMO in Rel-19.

Proposal 4: RAN4 can consider the above general RF requirements in TS38.101-1 for PC 1.5/PC2 inter-band CA including 2Tx/3Tx UE. Similar approach will be applied to inter-band EN-DC including 2Tx/3Tx UE in TS38.101-3.

      - suffix A (2Tx/3Tx CA without MIMO)

      - suffix H (3Tx w/ UL-MIMO)

      - suffix L (3Tx w/Tx Diversity)

Reference

[1] R4-2410750, “WF on on HPUE UE,” Samsung

[2] RP-241656, “Revised WID on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4,” Huawei

[3] TS38.101-1

[4] TS38.101-3