3GPP TSG-RAN WG4 Meeting #112	R4-2411653

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	System parameters for LP-WUR

Agenda item:	8.22.2.1

Document for:	Discussion

Introduction

In this paper, we present our views on RF architecture, RF impairments and common simulation parameters. Following agreements were made in RAN4#111 [1].

Issue 1-1-1: Operation bands for LP-WUS feature Agreements: RAN4 confirm LP-WUS is a general feature not limited to specific example band(s) Issue 1-1-2: FR1 example bands for requirements as phase 1 Agreements: No need to list specific example band(s) Issue 1-1-5: Rx diversity gain assumption for LP-WUR Agreements: No diversity gain as baseline for FR1. More Rx could be implementation choice Issue 1-1-7: Channel raster for LP-WUR Agreements: RAN4 further check whether no channel raster is needed for LP-WUR. Issue 1-1-9: Side condition for LP-WUR requirements Agreements: - FFS RAN4 should consider both idle and connected mode conditions in the side conditions for the LPWUR requirements. Issue 1-1-11: Metric for LP_WUR requirements Agreements: - MDR is used as the metric for LP_WUR Rx RF requirements - FFS MDR values. - FAR will be defined as demodulation requirements - FFS FAR values

Discussion

Receiver assumptions

Zero-IF Receiver architecture

The Zero-IF receiver architecture is aimed for multi band usage. This architecture can have a high degree re-use of the NR radio like frontend, antennas, LNA, mixer, ADC (if it is configurable in frequency and sample depth) can be reused.

Figure 1 Basic UE LP-WUR architecture for utilizing a Zero-IF receiver.

As with any direct conversion receiver, DC offset, and self-image are problems here. For the LP-WUS signal, since there can be data at the DC, so any potential DC offset cancellation loop can cause information loss. LO is used for tuning the receiver to different frequencies. Assumed values of the performance parameters of different components used in this architecture are shown in Table 1. These values are taken from published literature ([2]--[8]). As the direct conversion receiver folds the image spectrum, an image reject mixer is used. However, based on the frequency domain content of the signal, an acceptable solution can be designed without an image reject mixer. This however will work only for an envelope detector based receiver. A sequence based detector will need an image reject mixer. Thus, further optimization can be done at the mixer. However, the overall NF level will only increase by around 0.1 dB. The NF of the whole receiver is being decided by the front-end filters and the LNA, which will be common for both sequence based and envelope based receivers.

Table 1 Gain, and NF values for a reference Zero-IF based receiver for a sequence detector LP_WUR.

| Component | Gain (dB) | NF (dB) | Remarks |
| --- | --- | --- | --- |
| RF BPF/Duplex filter | -1.5 | 1.5 |  |
| LNA | 15 | 7 | Low power LNA with large NF. |
| Mixer | 3 | 10 |  |
| BB amplifier | 40 | 25 | BB amplifier providing the majority of required gain. |
| Total | 56.5 | 11.6 |  |

Zero-IF architecture supports a high degree of reuse of the NR main radio components.

To support more than one band, the receiver could use a wideband LNA or multiple LNAs supporting smaller frequency area.

As the bandwidth of the WUS signal is expected to be scaled according to the sub carrier spacing the LP filter will most likely be required to have different cut off frequencies, e.g., one configuration for each sub-band spacing configuration.

The NF of the receive chain is being governed by front-end components and LNA, which are common for both sequence based and envelope based receivers.

Agree to use zero-IF receiver as a baseline architecture for LP_WUR.

Agree to use the estimated NF of 12dB as a baseline for both envelope and sequence based LP_WUR.

Metric for LP_WUR requirements

It was agreed to use both FAR and MDR as a metric. However, the values have not been decided yet. During the study phase, companies had studied the performance of LP_WUS using link-level simulations with a metric of 1% MDR with FAR ranging from 0% to 1%. With these metrics, it was shown that the LP_WUS has power saving gain as high as 98%, subjected to effective UE paging rate [9]. As this feature is meant for saving UE power, it makes sense to set the MDR and FAR metric for requirement purposes as close to what values were used in study phase.

Use 1% MDR and 1% FAR values for defining the requirements.

Coverage assumptions

According to the WID [10], target coverage of LP-WUS and LP-SS shall be the coverage of the PUSCH message 3. To evaluate that, RAN1 had done link evaluations based on MIL calculations and following agreements were made in RAN1#117

Agreement​ For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: ​ - SNR=-3dB for NF of LR = NF of MR+ 8dB​ - SNR= -0.5dB for NF of LR = NF of MR+ 5dB​ - SNR=2dB for NF of LR = NF of MR+ 2dB​ - Note 1: The NF of MR is assumed as 7dB

This argument implies that RAN1 had reached on consensus regarding the evaluation for uplink coverage methodology and came with these three SNR and NF tuples. RAN4 is going to do its own evaluation to figure out SNR values based on link-level simulations and NF values based on system architecture. These numbers will then be used for REFSENS calculations. However, RAN4 should not repeat RAN1 work for evaluating the coverage of LP-WUS. RAN4 can do a sanity check on the RAN4 derived SNR and NF values with the above mentioned RAN1 equations. If that fails, then RAN4 can inform RAN1 that the coverage of LP-WUS might not be sufficient keeping in mind the hardware limitation and RAN1 should think about coverage enhancements techniques for LP-WUS.

RAN4 will evaluate SNR values based on link-level simulations and NF values based on system architecture.

Do not repeat the coverage evaluation work done by RAN1 in RAN4.

Inform RAN1 at a later date if RAN4 find out that coverage of LP-WUS is not sufficient.

Conclusion

The following Observations and Proposals were made:

Observation 1: Zero-IF architecture supports a high degree of reuse of the NR main radio components.

Observation 2: To support more than one band, the receiver could use a wideband LNA or multiple LNAs supporting smaller frequency area.

Observation 3: As the bandwidth of the WUS signal is expected to be scaled according to the sub carrier spacing the LP filter will most likely be required to have different cut off frequencies, e.g., one configuration for each sub-band spacing configuration.

Observation 4: The NF of the receive chain is being governed by front-end components and LNA, which are common for both sequence based and envelope based receivers.

Proposal 1: Agree to use zero-IF receiver as a baseline architecture for LP_WUR.

Proposal 2: Agree to use the estimated NF of 12dB as a baseline for both envelope and sequence based LP_WUR.

Proposal 3: Use 1% MDR and 1% FAR values for defining the requirements.

Observation 5: RAN4 will evaluate SNR values based on link-level simulations and NF values based on system architecture.

Proposal 4: Do not repeat the coverage evaluation work done by RAN1 in RAN4.

Proposal 5: Inform RAN1 at a later date if RAN4 find out that coverage of LP-WUS is not sufficient.

- References

- R4-2410569, WF for [111][136] NR_LPWUS_UERF, Vivo, RAN4#111, Fukuoka, Japan, 20 – 24 May 2024.

- Pletcher, N.M., Gambini, S. and Rabaey, J., 2008. A 52uW Wake-Up Receiver with 72 dBm Sensitivity Using an Uncertain-IF Architecture. IEEE Journal of solid-state circuits, 44(1), pp.269-280

- Cook, B.W., Berny, A.D., Molnar, A., Lanzisera, S. and Pister, K.S., 2006, February. An ultra-low power 2.4 GHz RF transceiver for wireless sensor networks in 0.13um CMOS with 400mV supply and an integrated passive Rx front-end. In 2006 IEEE International Solid State Circuits Conference-Digest of Technical Papers (pp. 1460-1469).

- E. -H. V. Yeh, A. -H. Lo, W. -S. Chen, T. -J. Yeh and M. Chen, "A 16 nm FinFET 0.4 V inductor-less cellular receiver front-end with 10 mW ultra-low power and 0.31 mm2 ultra-small area for 5G system in sub-6 GHz band," 2017 Symposium on VLSI Circuits, Kyoto, Japan, 2017, pp. C72-C73.

- Du, Y., Hu, W., Li, G.F. and Liu, S., 2018. Low power hybrid PG_Filter-AGC analog baseband for wireless receivers. IEICE Electronics Express, 15(3), pp.20171197-20171197.

- P. -H. P. Wang et al., "A 400 MHz 4.5 nW −63.8 dBm sensitivity wake-up receiver employing an active pseudo-balun envelope detector," ESSCIRC 2017 - 43rd IEEE European Solid State Circuits Conference, Leuven, Belgium, 2017, pp. 35-38..

- N. M. Pletcher, S. Gambini and J. Rabaey, "A 52 μ W Wake-Up Receiver With − 72 dBm Sensitivity Using an Uncertain-IF Architecture," in IEEE Journal of Solid-State Circuits, vol. 44, no. 1, pp. 269-280, Jan. 2009.

- H. Bialek et al., "A Passive Wideband Noise-Canceling Mixer-First Architecture With Shared Antenna Interface for Interferer-Tolerant Wake-Up Receivers and Low-Noise Primary Receivers," in IEEE Journal of Solid-State Circuits, vol. 57, no. 9, pp. 2611-2625, Sept. 2022.

- 3GPP TR 38.869: “Study on low-power wake-up signal and receiver for NR”.

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN#103, Maastricht, Netherlands, 18 – 21 March 2024.