3GPP TSG-RAN WG4 Meeting #112	R4-2411654

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	Fundamental RX requirements for LP-WUR

Agenda item:	8.22.2.2

Document for:	Discussion

Introduction

In this paper, we present our views on LLS simulation parameters, REFSENS, ASCS, and ACS requirements. Following agreements were made in RAN4#111 [1].

### Sub-topic 2-1 Alignment of LLS parameters to specify ACS/ASCS requirements Issue 2-1-1: Center frequencies for LLS simulation Agreement: - For ACS and ASCS simulation, select 900MHz, 2.6GHz and 3.5GHz as example frequencies for FR1. - FFS on FR2 example band(s) Issue 2-1-2: Performance metric (MDR/BLER value) for LLS simulation (apply to ACS/ASCS and REFSENS) Agreement: - For LLS simulation (apply to ACS/ASCS and REFSENS), the metric includes - 1% MDR/BLER as baseline and 5% MDR/BLER as optional - The following false alarm rate can be considered - 1% - 5% - Providing the information whether the false alarm rate is considered or not - Further down-select the performance metric for the requirements and testing Issue 2-1-3: Waveform for LLS simulation Agreement: - Use both OOK1 and OOK4 based on RAN1 agreements for link level simulations Issue 2-1-4: number of LP-WUS RBs for LLS simulation Agreement: - Assume 11PRB for LP-WUS signal with 30kHz SCS for simulations - FFS on RB number(s) for 15KHz SCS depending on RAN1 decision Issue 2-1-5: number of ADC assumption for LLS simulation Agreement: - Number of ADC assumption for LLS simulation - Option 1: 8 bit - Option 2: 4 bit - Encourage companies to provide simulation results with both options for comparison Issue 2-1-6: Frequency error assumption for LLS simulation Agreement - Frequency error assumption for LLS simulation - Up to 20ppm Issue 2-1-7: Phase noise model for LLS simulation Agreement: - Not needed in LLS. Consider as RF impairment of implementation. Issue 2-1-8: For ASCS, the BWinterferer for ASCS evaluation Agreement: - For ASCS evaluation, only 10MHz and 20MHz NR CBWs are assumed and the BWinterferer is - All RBs between WUS edge to channel edge Issue 2-1-10: The overall simulation parameters for LLS simulation Agreement: - Wait for RAN1 conclusion on FR2 to decide the parameters for evaluations - Adopt the following parameters in table below for FR1 simulation evaluations Attributes; Assumptions. Case name (waveform); OOK-1 waveform; OOK-4 waveform. Center frequency; 900MHz, 2.6GHz and 3.5GHz. Channel structure; Total 8/16 bits. Chip rate; M=1; M=1/2/4. WUS duration; FFS. Coding; 1/2 rate Manchester coding. Time error; 0. Residual Frequency error; 0/10/20 ppm. SCS; 30kHz. UE Channel BW; 20MHz (51 RB)-case 1 10MHz (24 RB)-case 2 5MHz (11 RB)-case 3. WUS RB; - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases. Position within channel; - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel. Guardband of NR channel, both wanted cell and interfer cell (ACS); - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz. Guard RB; - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB. Filter; - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case. ASCS; PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal. ACS; PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [20~33] dB NOTE: decide the interference level depending on SNR. Wanted signal level; For ACS, REFSENS + 14 dB for LP-WUS. Sampling rate; 7.68MHz. ADC bit width; 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison. Phase noise; Not modelled. Non-linearities; Not modelled. Power boosting; EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation. Channel Model; Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options. Performance metric; - 1% MDR/BLER as baseline and 5% MDR/BLER as optional - The following false alarm rate can be considered - 1% - 5% Providing the information whether the false alarm rate is considered or not. ### Sub-topic 2-2 REFSENS requirements Issue 2-2-3: SNR value (not requirement) for REFSENS Agreement: - Stick to last meeting agreements: Wait for RAN1 SNR progress Issue 2-2-4: NF Gap between LR and MR for REFSENS (assume MR as 9dB) Agreement: - RAN4 further discuss potential NF for OOK-based and OFDM-based receiver. - Linked to resolution of Issue 2-2-8 Issue 2-2-6: IM value for REFSENS Agreement: - RAN4 further discuss potential IM value for OOK-based and OFDM-based receiver. Issue 2-2-8: Whether a baseline architecture is needed for LP-WUS receiver Agreement: - RAN4 will further discuss and decide baseline architecture(s) for OOK-based receiver and OFMD-based receiver. ### Sub-topic 2-3 ASCS requirements Issue 2-3-1: Align the definition of ASCS requirements Agreement: - RAN4 can use the following ASCS definition as starting point. - Adjacent SubCarrier Selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its assigned channel frequency in the presence of adjacent subcarrier NR signal(s) at a given frequency offset (guard RB) between LP-WUS and NR. ASCS is the ratio of the receive filter attenuation on the assigned LP-WUR channel frequency to the receive filter attenuation on the adjacent NR subcarrier Issue 2-3-2: ASCS requirements value Agreement: - RAN4 further discuss whether ASCS requirements value should be specified, or just the test parameter are specified, i.e., LP_WUS and NR signal bandwidths and power levels, and guard RBs. Issue 2-3-3: Required number of guard RB for ASCS Agreement: - RAN4 further discuss required number of guard RB for ASCS based on LLS simulation outcome next meeting. Issue 2-3-4: Test case for ASCS Agreement: - RAN4 further discuss detailed test parameters for ASCS. ### Sub-topic 2-4 ACS requirements Issue 2-4-1: Whether to update the ACS definition for LP-WUS Agreement: - RAN4 consider the following clarified ACS requirements definition for LP-WUS as starting point. - Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR LP-WUS signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the center frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s). Issue 2-4-2: ACS requirements value Agreement: - RAN4 consider the how to specify ACS requirements. FFS same as MR. - Decision will be made based on LLS outcome Issue 2-4-3: Required guard RB for ACS case Agreement: - RAN4 further discuss required number of guard RB for ACS based on LLS simulation outcome next meeting.

Discussion

LLS parameters

The miss detection ratio (MDR) and WUS duration are two parameters that were not agreed in the last meeting. MDR value of 1% had been used for LLS during the study phase to evaluate the performance by both RAN1 and RAN4.

Agree to use 1% MDR value for the LLS evaluation.

REFSENS

For E-UTRA and NR in FR1, 2.5dB of IM had been used for the main receiver in case of 5 MHz channel bandwidth. However, for a LP-WUR there is no interference from its own transmitter as the transmitter should not be working because UE is in deep or ultra-deep sleep. Further, there is only a single receiver. Thus, IM can be significantly tightened to a value less than 2.5dB.

Agree to have implementation margin to be 1dB.

NF value depends on the receiver architecture assumptions. Further, this has consequences for the coverage and power consumption of the LR. In our companion document under agenda 8.22.2.1 we have provided reference receiver architecture and NF values for sequence based and envelope detector based LP_WUR [2]. The proposed value is 12dB for both sequence based and envelope detector based LP-WUR.

NF has an impact on the coverage and power consumption of the LR.

Agree to use the estimated NF of 12dB as a baseline for LP-WUR.

ASCS

Link level simulations were performed with both AWGN and TDL-C 300 channel. Simulated performance for OOK4 M=4 modulation in AWGN channel is shown below. A flat PSD is used i.e., LP-WUS and NR signals have the same PSD. Rest of the results for the AWGN channel can be seen from the Annex A of this document. Simulations were performed with zero guard RB and no ADC quantization. Different curves in a single figure corresponds to different band filter order (bfo). Major observations from the results are following:

Figure 1 ASCS performance with envelope detector with zero guard RB for OOK4 M=4.Different curves correspond to different band filter order (bfo).

The SNR required for achieving a MDR of 1% does not improve much after third order filter.

Frequency offset has no significant impact on the envelope detector performance.

A SNR value of -2.5dB is sufficient to achieve a MDR of 1% in case of an envelope detector based LP-WUR.

Agree to have zero guard RB for the ASCS case.

To understand the impact of NR signal on LP-WUR performance, LLS were performed with the REFSENS test setup i.e., with no additional NR interference signal. The results from the LLS are shown below in Figure 2. Comparing results from Figure 1 and Figure 2, there is barely any performance degradation because of the NR signal provided the PSD of NR signal and LP-WUS is same.

Agree to have no separate ASCS requirement.

Figure 2 LLS performance in REFSENS test environment i.e., no NR interferer.

ACS

Preliminary simulation results for ACS are shown in Annex B. The impact of limited number of ADC bits hasn’t been considered so these results cannot be used to conclude a guard RB number. However, they do show that between three to four guard RBs might be sufficient to meet 1% MDR.

As the LP-WUR is going to face the similar interferer as the main receiver, the side conditions for the ACS test case can be the same as the side conditions used for ACS tests for the normal UE. Further, as Pinterferer depends on the REFSENS, and given the limited dynamic range of the LP-WUR, for LP-WUR it should depend on the REFSENS of the LP-WUR.

Test parameters defined in Table 7.5-3, 7.5-4, 7.5-5, and 7.5-6 of TS 38.101-1 apply for LP-WUR ACS test case.

In test case where Pinterferer depends on REFSENS, LP-WUR REFSENS should be used.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Agree to use 1% MDR value for the LLS evaluation.

Proposal 2: Agree to have implementation margin to be 1dB.

Observation 1: NF has an impact on the coverage and power consumption of the LR.

Proposal 3: Agree to use the estimated NF of 12dB as a baseline for LP-WUR.

Observation 2: The SNR required for achieving a MDR of 1% does not improve much after third order filter.

Observation 3: Frequency offset has no significant impact on the envelope detector performance.

Observation 4: A SNR value of -2.5dB is sufficient to achieve a MDR of 1% in case of an envelope detector based LP-WUR.

Proposal 4: Agree to have zero guard RB for the ASCS case.

Proposal 5: Agree to have no separate ASCS requirement.

Proposal 6: Test parameters defined in Table 7.5-3, 7.5-4, 7.5-5, and 7.5-6 of TS 38.101-1 apply for LP-WUR ACS test case.

Proposal 7: In test case where Pinterferer depends on REFSENS, LP-WUR REFSENS should be used.

- References

- R4-2410569, WF for [111][136] NR_LPWUS_UERF, Vivo, RAN4#110bis, Changsha, China, 15 – 19 April 2024.

- R4-2411653, System parameters for LP-WUR, Nokia, RAN4#112, Maastricht, Netherlands, Aug 19 – Aug 23, 2024.

- Annex A: ASCS Results

Simulated results with zero guard RB for selected few cases are collated below. The detector used is envelope based. TDL-C results followed the same trend as for AWGN channel and are shown after AWGN channel results.

- Annex B: ACS Results

Simulated results with different guard RBs for selected few cases are collated below. The detector used is envelope based. The are the results with no quantization i.e., impact of limited number of ADC bits hasn’t been included here.